3GPP TSG-RAN WG1 Meeting #95	R1- 1812926

# Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.9.2.2

Source:	Apple Inc.

Title:	Power Saving Techniques based on Explicit Indication

Document for:	Discussion, Decision

# Introduction

In NR, we have new features to support higher data rates applications (eMBB) entailing higher power consumption. To overcome increased power consumption, techniques such as BWP (in frequency domain) and periodic PDCCH monitoring (in time domain) were introduced in R15. In this paper, we propose enhancements based on explicit indication from network to UE. Our companion paper [5] includes techniques for UE-assisted techniques for UE power saving.

# Potential Enhancements for UE Power Saving

In this section, we discuss explicit signal based power saving scheme.

## Explicit Signal based Sleep Indication

### Motivation

In R15 MTC and NB-IoT, it was agreed to introduce power efficient wake up type signal for UE power saving. The focus was on IDLE mode power saving enhancement since IoT devices stay mostly in IDLE state for extending battery life. In this sense, the context is different from NR UE power saving, where we think eMBB is most popular use case. Accordingly, the approach for power saving signal in NR could be very different from that in MTC/NB-IoT.

To achieve meaningful power saving gain, NR needs power saving techniques for CONNECTED mode. Considering that total power contribution from Idle mode is relatively small compared to that in CONNECTED mode [1][2][3], it becomes clear that power saving techniques for CONNECTED mode could be much more effective than that for IDLE mode. So, it should be the focus of NR UE power saving study.

When a UE in RRC CONNECTED mode is scheduled by gNB to receive DL/UL data, the grant transmission time to the UE of interest could be random. This is because that traffic arrival for the UE from application server is random and the gNB should serve multiple UEs in the cell. Figure 1 shows an exemplary CDF of grant inter arrival time of video and web-browsing in LTE. It is observed the mean grant-inter arrival time is 5ms for video traffic and 6.2ms for web-browsing. Note that the tail of distribution reaches to ~ 20ms for video and ~40ms for web-browsing. This indicates that there could be time gap of order of ms to tens of ms during which UE can enter sleep state for power saving. The further analysis could found in [4].

Figure 1 CDF of grant inter-arrival time for video (left) and web-browsing (right)

### Proposals

To take advantage of such time gap between grants, we could think of explicit sleep indication based UE power saving mechanism as shown in Figure 2. Due to the dynamic change of traffic arrivals, configuring static PDCCH monitoring period or CDRX parameters may not be enough to effectively handle dynamic data transmission / reception in power efficient manner. With explicit signaling approach, network can dynamically handle such situation while maximizing power saving gain.

Figure 2 Expected UE power consumption with explicit signal based sleep indication

Observation 1

- The explicit signal-based sleep mechanism is effective in adapting to dynamic traffic arrivals for UE power saving.

Proposal 1

- NR supports explicit-signal based sleep indication for UE power saving.

### Performance Evaluation

For evaluation, we consider a statistical model with random grant arrivals. The model is simple yet good enough to capture the interplay between sleep and traffic load. For simple analysis, we make following assumptions.

#### Assumptions

- Traffic pattern: We model a grant arrival event with a Bernoulli random variable with parameter p, i.e., the grant arrival for any given slot occurs with probability p in i.i.d fashion. Accordingly, inter-grant arrival time can be modeled as a random variable D with geometric distribution as shown in Figure 3.

Figure 3 Illustration of grant arrivals and inter-grant arrival time

- Sleep indication : We assume that gNB can send Genie sleep indication to UE to indicate the for sleep duration D. This indicates that grant arrives with probability 1 after sleep period ends. Due to this assumption, the results we generate below should be understood as an upper bound of performance gain.

- BLER and ACK/NACK : For simplicity, we assume BLER=0 and ACK/NACK can be sent in the same slot as grant received (i.e., self-contained slot) without an error.

#### Analysis

Average Power Consumption with Sleep indication

Figure 4 shows an example of power state transitions in a single time duration of length 1+D including one grant slot and one following sleep duration D. Note that the inter-grant time D could be equal to or larger than 0. P0 denotes power consumption for normal PDCCH monitoring and PDSCH decoding (P0=300). PDi is sleep power for i-th duration Di.  Note that PDi is a function of Di, i.e., different length of sleep duration has different sleep power. The random variable ΔDi accounts for the energy cost for state transition.

Figure 4 Illustration of power consumption with sleep between two consecutive grants

Under this mode, one can compute the average power consumption Pavg over N such periods. The numerator is the total energy consumed for processing N grants and N sleep oepartion. The denominator corresponds to the total time for N periods. Note that each period includes one slot for a grant and one sleep duration of length D.

If we send N infinity, then by law of large numbers Pavg converges to the ratio of mean energy consumed and mean duration of D+1.

Now we calculate Pavg with followings. D is a geometric random variable with support {0,1,2,…} and pmf

.

Then, the mean of D is given by

.

The expected value of the product PDD is given as

,

where Pd the sleep power is given as

.

The expected value of transitional power cost is given by

,

where

.

Average Power with No Sleep

In case there is no sleep operation, the average power is determined based on the portion of PDCCH-only states and that of PDSCH+PDCCH decoding states. Since a grant arrives with probability p, the portion of slots with PDSCH is given as p. Accordingly, PDCCH-only monitoring slots take the portion of 1-p. Then, the average power is given as

Pavg = (1-p)×PPDCCH-only + p×PPDCCH-PDSCH = (1-p) ×100+p×300 = 100+200p.

#### Results

Figure 5 and Figure 6 show the power saving performance of sleep indication based on the statistical model described in the previous section. From the results, one can see that by dynamically indicating the sleep duration, significant gain can be attained especially when packet arrival rate is low (e.g., ~55% at p=0.1). Power saving gain depends on the grant arrival probability p. When p is close 0, a UE with no-sleep keep monitors PDCCHs which consumes on average power of 100. Whereas, a UE with sleep indication mechanism can sleep for a long time so the average power consumption approaches to 0. When p is close 1, both UEs receives grants almost every slot and there is no time to enter sleep. So, the power consumption converges to 300.

Figure 5 Average power consumption vs. grant arrival rate (p)

Figure 6 Power saving percentage of sleep indication vs. grant arrival rate (p)

Observation 2

- When the grant arrival probability is 0.05, the power saving gain of the explicit signal based sleep indication mechanism is 71%.

- When the grant arrival probability is 0.1, the power saving gain of the explicit signal based sleep indication mechanism is 55%.

- When the grant arrival probability is 0.2, the power saving gain of the explicit signal based sleep indication mechanism is 37%.

# Conclusions

In this contribution, we have analyzed the performance of the explicit signal based sleep indication mechanism. We made following observations and proposals.

Observation 1

- The explicit signal-based sleep mechanism is effective in adapting to dynamic traffic arrivals for UE power saving.

Proposal 1

- NR supports explicit-signal based sleep indication for UE power saving.

Observation 2

- When the grant arrival probability is 0.05, the power saving gain of the explicit signal based sleep indication mechanism is 71%.

- When the grant arrival probability is 0.1, the power saving gain of the explicit signal based sleep indication mechanism is 55%.

- When the grant arrival probability is 0.2, the power saving gain of the explicit signal based sleep indication mechanism is 37%.

# References

[1] R1-166368 UE Power Consideration based on Days-of-Use, Qualcomm, Aug, 2016

[2] R1-1609557 On Techniques for Enhanced UE Power Efficiency, MediaTek, Oct, 2016

[3] RP-172318 Motivation for Study on UE Power Saving in NR, VIVO, Dec, 2017

[4] R1-1714092 Control Information for UE Power Saving, Apple, Aug, 2017

[5] R1-1812925 Power Saving Techniques based on UE Adaptation, Apple, Nov, 2018