3GPP TSG RAN WG1 Meeting #95	R1-1812955

# Spokane, USA, November 12th – 16th, 2018

Agenda item:	7.1.1.3

Source:	Samsung

Title:	Remaining Issues on Measurements for Mobility Management

Document for:	Discussion and decision

# Introduction

This contribution discusses the remaining issues on measurements for mobility management including;

- RLM RS when Lmax = 4 and RadioLinkMonitoringRS is not provided

# RLM RS when Lmax = 4 and RadioLinkMonitoringRS is not provided

In RAN1#94bis, UE behaviour to select RLM RS was agreed for the case when Lmax=4 and RadioLinkMonitoringRS is not provided:

Agreements: - Adopt the following proposal to Section 5 of 38.213 - the UE is not required to use for radio link monitoring an aperiodic or semi-persistent RS - For L = 4, the UE selects the NRLM RS provided for active TCI states for PDCCH reception in the CORESETs associated with the search spaces with shortest monitoring periodicity. If more than one CORESETs are associated with search spaces with same periodicity, the UE selects the CORESETs with the highest CORESET ID.

However, during post-RAN1#94bis email discussion, an ambiguity issue was raised and noted in the latest endorsed CR [1]. For example, {SS set 0, SS set 1, SS set 2} map to {CORESET 0, CORESET 1, CORESET 2} and have monitoring periodicities {1, 1, 2} slots. Then, according to the wording of the agreement, CORESETs 1 and 2 are selected which is not the original intention.

In order to address the raised ambiguity, two points need to be clarified, i.e., CORESET ordering and UE selection of RLM RS from the ordered CORESET:

- CORESET ordering:

  - the CORESETs are sorted in ascending order from the shortest monitoring periodicity for corresponding search space sets.

  - Then, if more than one CORESETs are associated with search space sets having same monitoring periodicity, the more than one CORESETs are ordered in descending order of CORESET index

- UE selection of RLM RS:

  - UE selects NRLM RS from the NRLM first ordered CORSETs

A draft CR is provided in a separate document based on the latest endorsed CR.

# Reference

[1] R1-1812092, CR to 38.213