3GPP TSG RAN WG1 Meeting #95	R1-1812964

# Spokane, USA, November 12th – 16th, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on PHR for PUSCH mapping Type-B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-2 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In case of UL CA with different SCSs, it is possible that CC(s) for PHR calculation use Type-B scheduling and a CC carrying PHR uses Type-A scheduling. In such a case, how to calculate PHR is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If a PUSCH on the CC for PHR calculation uses Type-B sceduling, the PUSCH including the first OFDM symbol of the first slot on the CC for PHR calculation which is overlapped with the slot on the PHR-carrying CC is used to calculate PH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior on PHR is incomplete in case of UL CA with different SCSs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

7.7.1			Type 1 PH report

<*** Unchanged parts omitted ***>

------------------------------------------------------ Start of Text Proposal --------------------------------------------------------

If a UE is configured with multiple cells for PUSCH transmissions, where a SCS configuration  on active UL BWP  of carrier  of serving cell  is smaller than a SCS configuration  on active UL BWP  of carrier  of serving cell with PUSCH mapping Type A, and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP  that overlaps with multiple slots on active UL BWP , the UE provides a Type 1 power headroom report for the first slot of the multiple slots on active UL BWP  that fully overlaps with the slot on active UL BWP .

If a UE is configured with multiple cells for PUSCH transmissions, where a subcarrier spacing configurationµ1  on active UL BWP b1  of carrier f1  of serving cell c1 is equal or smaller than a subcarrier spacing configuration µ2 on active UL BWP b2 of carrier f2 of serving cell c2 with PUSCH mapping Type B, and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP b1 that overlaps with one or multiple slots on active UL BWP b2, the UE provides a Type 1 power headroom report for the first OFDM symbol of first slot of the multiple slots on active UL BWP b2 that fully overlaps with the slot on active UL BWP b1.

If a UE is configured with multiple cells for PUSCH transmissions, the UE does not consider for computation of a Type 1 power headroom report in a first PUSCH transmission that includes an initial transmission of transport block on active UL BWP  of carrier  of serving cell , a second PUSCH transmission on active UL BWP  of carrier  of serving cell  that overlaps with the first PUSCH transmission if

-	the second PUSCH transmission is scheduled by a DCI format 0_0 or a DCI format 0_1 in a PDCCH received in a second PDCCH monitoring occasion, and

-	the second PDCCH monitoring occasion is after a first PDCCH monitoring occasion where the UE detects the earliest DCI format 0_0 or DCI format 0_1 scheduling an initial transmission of a transport block after a power headroom report was triggered

------------------------------------------------------- End of Text Proposal ---------------------------------------------------------

<*** Unchanged parts omitted ***>