3GPP TSG RAN WG1 Meeting #95				R1-1812965

Spokane, USA, November 12th – 16th, 2018

Agenda item:	7.1.7

Source: 	Samsung

Title: 				    UE Features for Rel-15

Document for:	Discussion and Decision

Introduction

Up to RAN1#94bis, the UE feature issues have been discussed and concluded [1], [2]. There are still remaining issues not decided yet. This contribution discusses the remaining issues on UE features.

Discussion on existing features

In the following, the features highlighted as yellow in [2] and not-decided features are discussed.

Feature 2-4

For FR2, dynamic beam change of 64 beams is a crucial feature to prevent frequent RRC reconfiguration for beam change when the UE moves within the cell. Since 64 TCI states should be divided into two groups for beam indication of TRS and PDCCH/PDSCH/CSI-RS, only up to 32 beams can be utilized via L1/L2 signalling. To cover entire cell area without RRC reconfiguration, 128 TCI states shall be made mandatory.

| 2-4 | TCI states for PDSCH | 1. Support number of active TCI states per BWP per CC , including control and data 2. maximum number of configured TCI states per CC for PDSCH | 2-1 | Yes | Only one TCI state can be supported | Type 1 | N.A. | N.A. |  | Note: UE is required to track only the active TCI states |  | Mandatory with capability Component-1: Candidate value set: {1, 2, 4, 8 } Component-2: candidate value set: {4, 8, 16, 32, 64, [128]} UE is mandated to signal [64/128] for FR2. For FR1, UE is mandated to report at least the max number of allowed SSB in the band | Component-1: Candidate value set: {1, 2, 4, 8 } Component-2: candidate value set: {4, 8, 16, 32, 64, 128} UE is mandated to signal [64, 128] for FR2 and FFS the values for FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 1: UE is mandated to signal 128 for FR1 and FR2.

Feature 2-20

| 2-20 | Beam correspondence | 1. Support Beam correspondence 2. When CA is configured, whether the same beam correspondence relationship for beam management is supported across CCs. Note: RAN4 to check the feasibility for component-2, e.g. intra-band or inter-band |  | Yes | Beam correspondence is not supported | Type 1 | No need | N.A. |  | Note: Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports |  | [Mandatory/optional] with capability signaling Component-2, candidate value: {Yes, No} | [Mandatory/optional] with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: For FR2 in Feature 2-20, mandatory with capability signaling which shall be set to ‘1’

Proposal 3: For UEs which reported component 2 in Feature 2-20, prioritization of beam indication signaling shall be supported [3]

Feature 2-24

| 2-24 | SSB/CSI-RS for beam measurement | 1. The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot shall not exceed MB_1 1a. The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP shall not exceed MC_1 2. The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 3. Supported density of CSI-RS 4. The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-RSRP shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a | Yes | RSRP measurement is not supported | Type 1 | No need | Yes |  |  |  | Mandatory with capability signaling Component-1, candidate value set for MB_1 is {0, 8, 16, 32, 64} On FR2, UE is mandated to signal MB_1 >=8 On FR1, MB_1 >=8 is supported mandatory with capability signaling. Component-1a, candidate value set for MC_1 is {0, 4, 8, 16, 32, 64} Component-2, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-3: candidate value set: {“not supported”, “1 only”, “3 only”, “both 1 and 3”} On FR2, UE is mandated to signal either “3 only” or “both 1 and 3” On FR1, either “3 only” or “both 1 and 3” is mandatory with UE capability signaling. Component-4, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} For FR2, UE is mandated to report at least 4 FFS the mandated value for FR1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: For component 1 and 1-a in Feature 2-24, UE is mandated to support at least 64 SSB/CSI-RS resources.

Feature 2-34

Proposal 5: For Feature 2-34, optional with capability signaling

Feature 2-38

| 2-38 | CSI report without PMI | Support CSI report without PMI | 2-35 | Yes | CSI report without PMI is not supported | Type 4 | No need | Yes |  | RAN1 to clarify whether it depends on SRS Tx switch |  |  | [Mandatory/Optional] with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6: For 2-38, optional with capability signaling

Feature 2-51

In addition to configuration of 128 TCI states, 64 TRS resource sets per CC should be made mandatory to support 64 beams. In order to provide time/frequency tracking, each beam should be associated to a TRS resource set. Note that 64 TRS resource set should be supported with 128 TCI states, otherwise beam information cannot be configured due to lack of available TCI state configuration.

| 2-51 | TRS (CSI-RS for tracking) | 1. Support TRS BW 2. TRS burst length (X), 3. Max # of TRS resource sets (per CC) UE is able to track simultaneously 4. Max # of TRS resource sets configured to UE per CC 5. Max # of TRS resource sets configured to UE across CCs | 2-50 | Yes | TRS is not supported | Type 1 | N.A. | N.A. |  |  |  | Component-1: candidate values set: {BWP, min(52,BWP), both} Component-2: candidate values {1,2} Component-3: Candidate value set: {1 to 8} Component-4: Candidate value set: {1 to 64} Component-5: Candidate value set: {1 to 128} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 7: A UE is mandated to signal 64 and 64*#CC TRS resource sets for Component-4 and 5, respectively.

Feature 2-56

| 2-56 | SRS carrier switch | 1. Report inter-cell switching time capability | 2-53 | Yes | SRS carrier switch is not supported | Type 1 | No need | N.A. |  | RAN4 reply LS, R1-1805817, includes candidate value sets | RAN1/4 | candidate values set is up to RAN4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 8: For 2-56, optional with capability signaling

Feature 3-1

Regarding the number of search space in a PCell or PSCell per BWP, we prefer to have 8 considering possibility of scheduling with many DCI formats in the same slot.

| 3-1 | Basic DL control channel | 1) One configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - TCI state(s) for a CORESET configuration 2) CSS and USS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - Up to [7], [8] search spaces in a slot in a PCell or PSCell per BWP This search space limits are before applying dropping rules. - UP to 3 search spaces in a slot for a scheduled SCell per BWP This search space limit is before applying all dropping rules. - For type 1 with dedicated RRC configuration, type 3, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 PDCCH CSS without dedicated RRC configuration and for type 0, 0A, and 2 PDCCH CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1-PDCCH CSS without dedicated RRC configuration, or Types 0, 0A, or 2 PDCCH-CSS configurations within a single span of three consecutive OFDM symbols within a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD 6) Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot per scheduled CC for TDD |
| --- | --- | --- |

Proposal 9: For 3-1, choose 8 in “Up to [7], [8] search spaces in a slot in a PCell or PSCell per BWP.”

Discussion on features for Rel-15 late drop

In the following, the features highlighted as yellow in [2] and not-decided features are discussed. It is seen that the following features are supported in Rel-15.

- Architecture Opt 4: NR-LTE DC connected to 5GC with NR anchor

- Architecture Opt 7: LTE-NR DC connected to 5GC with LTE anchor

- NR-NR DC (FR1+FR2 and synchronous)

Therefore, we need to reflect the above things to the UE feature discussion.

For Features 6-7, 6-8, 6-9, 6-13, and 6-23, mentioning NE-DC is also added to the titles of feature group and components. Also, the following features are needed to be newly defined.

New features

| 6-24 | Basic operations for FR1-FR2 synchronous NR-NR DC |  |  | Yes |  | Type 3 (per band combination) | N.A. | N.A. |  |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 8-1b | Dynamic power sharing for NR-LTE DC | When total transmission power exceeds Pcmax, UE scales NR transmission power. | NE-DC | Yes |  |  | Type 3 |  | N.A. |  | N.A. |  |  |  |  |  |  |  | Mandatory with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8-1c | Operation A with single UL Tx case 1 for NR-LTE DC |  | NE-DC | Yes |  | Type 2 |  | Yes |  | Yes |  |  |  |  |  |  |  | Mandatory with capability signaling conditioned that UE does not support dynamic power sharing, i.e., UE indicate “0” as non-support for 8-1b, optional for UEs supporting dynamic power sharing |  |

Proposal 10. Reflect NE-DC and NR-NR DC related issues to the features.

Conclusions

This contribution considered aspects for UE features, and the followings are proposed.

Proposal 1: UE is mandated to signal 128 for FR1 and FR2.

Proposal 2: For FR2 in Feature 2-20, mandatory with capability signaling which shall be set to ‘1’

Proposal 3: For UEs which reported component 2 in Feature 2-20, prioritization of beam indication signaling shall be supported [3]

Proposal 4: For component 1 and 1-a in Feature 2-24, UE is mandated to support at least 64 SSB/CSI-RS resources.

Proposal 5: For Feature 2-34, optional with capability signaling

Proposal 6: For 2-38, optional with capability signaling

Proposal 7: A UE is mandated to signal 64 and 64*#CC TRS resource sets for Component-4 and 5, respectively.

Proposal 8: For 2-56, optional with capability signaling

Proposal 9:  For 3-1, choose 8 in “Up to [7], [8] search spaces in a slot in a PCell or PSCell per BWP.”

Proposal 10. Reflect NE-DC and NR-NR DC related issues to the features.

- References:

- R1-1812037, “LS on RAN1 NR UE features update,” RAN1#94bis, Oct. 2018.

- R1-1812064, “NR UE feature list,” RAN1#94bis, Oct. 2018.

- R1-1812959, “Simultaneous reception of physical channels and reference signals,” RAN1#95, Nov. 2018.