3GPP TSG RAN WG1 Meeting #95	R1-1813007

Spokane, USA, 12th – 16th November 2018

Agenda item:	7.2.8.6

Source: 	Samsung

Title: 	SLS evaluation on Type II overhead reduction

Document for:	Discussion and Decision

# Introduction

The following agreement was made in RAN1#94bis [1].

Agreement On the issue of Type II overhead reduction (rank 1, 2), to further progress, interested companies are to submit evaluation results (especially performance-overhead tradeoff) in RAN1#95 once the evaluation methodology is finalized in RAN1#94B. - Focus on proposals based on linear combination codebook as in Rel-15 - Also investigate potential common ground between frequency domain and time domain approaches, e.g. merging these two into one category

This contribution provides simulation results for the Type II CSI overhead reduction scheme proposed in the companion contribution [2].

# Simulation results for Type II CSI overhead reduction

For performance evaluation of the proposed Type II overhead reduction scheme in [2], the non-full-buffer system-level evaluation is carried out for Dense Urban (Macro only) channel model in medium (50% target RU) traffic loading scenario, and dynamic switching between SU-MIMO and MU-MIMO is considered in the simulation. The results are provided for 16 antenna ports at the gNB. The relevant simulation assumptions and parameters are according to the agreement made in RAN1#94bis [1], and are enlisted in Table 1 in Appendix.

Evaluation 1: L vs. M

We first provide simulation results for different L (number of beams for spatial domain compression) and M values (number of beams for frequency domain compression) in order to determine a set of L and M values which achieves a reasonable performance-overhead tradeoff. The results are provided in Figure 1 for the following parameters.

- Spatial compression: L = {2, 3, 4, 6}

- Frequency compression: M = {2, 4, 6, 8}

- Coefficient quantization: (Amp, phase) = (3,3) bits

As reference, Rel. 15 Type II with L = 2, WB+SB amplitude, and 8-PSK phase is considered. We can observe the following.

Observation 1:

- Large overhead reduction is achieved with the proposed frequency domain compression scheme – 38% overhead reduction with L=M=4 when compared with Rel. 15 Type II with L=4.

- M=4-6 achieves good performance-overhead tradeoff.

- L=6 does not show any significant gain at small M values.

.

Figure 1: Performance-overhead trade-off for different L and M values

Evaluation 2: Common vs independent basis and per SB vs per RB precoder

We next provide evaluation results for the following variations of the proposed scheme.

- M frequency domain (FD) basis vectors for each spatial domain beams i={0,1,..,2L-1}:

  - Common: M basis vectors are selected common for all spatial domain beams.

  - Independent: M basis vectors are selected independently for each spatial domain beam

- Frequency granularity of precoder:

  - Per SB: similar to Rel. 15 Type II, the precoder is reported in a SB manner, i.e., a precoder is reported common for all RBs in a SB.

  - Per RB: as proposed in [3], precoder is reported for each RB, i.e., if SB size is four, then a precoder is reported for each of the four RBs in a SB. Note that the CQI reporting is assumed to be per SB.

- Spatial compression: L = 4

- Frequency compression: M = {2, 4, 6, 8}

- Coefficient quantization: (Amp, phase) = (3,3) bits

The results are provided in Figure 2. We can observe the following.

Observation 2:

- For a given (L, M) value, the independent basis shows performance gain over common basis but the overhead is also higher, which implies that independent basis has no noticeable benefit over common basis in terms of performance-overhead tradeoff.

- For rank 1, per RB precoder does not show any gain over per SB precoder. For dynamic rank 1-2, per RB precoder shows small gain (~2%) over per RB precoder in high overhead regime.

Figure 2: Performance-overhead tradeoff for common vs independent basis, and per SB vs. per RB precoder

Evaluation 3: Subset of coefficient reporting

We next provide evaluation results to study whether the UE needs to report all of 2LM coefficients or a subset is sufficient. The main motivation behind reporting a subset of coefficients is the fact that the number of strong spatial domain beams is likely to vary across SBs or vice versa, i.e., the number of strong frequency domain beams is likely to vary across ports. This implies that a subset of 2LM coefficients can be set to zero without any significant loss in performance. To verify this, the following evaluation is conducted.

- L = 4, M = 8

- A subset comprising K (out of 2LM = 64) coefficients is reported. The remaining 2LM – K coefficients are set to zero value.

- Frequency granularity of precoder: per SB, per RB

- Common frequency domain basis

- Coefficient quantization: (Amp, phase) = (3,3), (3,4) bits

The results are provided in Figure 3. For comparison, the results for independent FD basis vectors and per RB precoder (from Figure 2) is also shown. We can observe the following.

Observation 3:

- It is sufficient to report a subset of K (out of 2LM) coefficients without losing performance.

- A subset of K=28 (out of 2LM = 64) coefficient reporting achieves 10% more compression when compared with independent FD basis, and achieves similar performance.

- 4 bit phase achieves ~2% additional gain over 3 bit phase.

Figure 3: Performance-overhead tradeoff for a subset of K < 2LM coefficients reporting

# Conclusions

In this contribution, simulation results are provided for the Type II overhead reduction scheme proposed in [2]. The observations made are summarized as follows.

Observation 1:

- Large overhead reduction is achieved with the proposed frequency domain compression scheme – 38% overhead reduction with L=M=4 when compared with Rel. 15 Type II with L=4.

- M=4-6 achieves good performance-overhead tradeoff.

- L=6 does not show any significant gain at small M values.

Observation 2:

- For a given (L, M) value, the independent basis shows performance gain over common basis but the overhead is also higher, which implies that independent basis has no noticeable benefit over common basis in terms of performance-overhead tradeoff.

- For rank 1, per RB precoder does not show any gain over per SB precoder. For dynamic rank 1-2, per RB precoder shows small gain (~2%) over per RB precoder in high overhead regime.

Observation 3:

- It is sufficient to report a subset of K (out of 2LM) coefficients without losing performance.

- A subset of K=28 (out of 2LM = 64) coefficient reporting achieves 10% more compression when compared with independent FD basis, and achieves similar performance.

- 4 bit phase achieves ~2% additional gain over 3 bit phase.

# References

- Chairman’s notes, RAN1#94bis

- R1-1813001, “CSI enhancement for MU-MIMO,” Samsung

- R1-1811193, “On CSI Enhancements for MU-MIMO support,” Ericsson

# Appendix

Table 1: Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1, 4GHz. |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS SB size = 4 and #SBs = 13 |
| Simulation bandwidth |  | 10 MHz,15kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation |
| MIMO layers |  | Up to 4 MU layers |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | DMRS, CSI-RS, PDCCH |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 50% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput vs CSI feedback overhead (bits) |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook |