3GPP TSG RAN WG1 Meeting #95	                R1-1813014

Spokane, USA, November 12th – 16th, 2018

Source:	Panasonic

Title: 	Discussion on physical layer structures of NR sidelink

Agenda Item:		7.2.4.1.1

Document for:	Discussion, Decision

Introduction

This contribution is update of R1-1810951 and shows our views on physical layer structures in NR sidelink accordingly, especially on PSSCH and PSCCH multiplexing, necessity of BWP, Shortened TTI and beam sweeping, based on the agreements in last meeting.

Discussion

PSSCH and PSCCH multiplexing

In our view option 1A where PSSCH and PSCCH have aligned frequency size but is TDMed should be precluded due to restriction on PSCCH size. Then the question is how to down select from Option 1B and Option 3. The detailed analysis on these two options is shown in following table.

Table 1 Comparison between option 1B and option 3

|  | Option 1B | Option 3 |
| --- | --- | --- |
| Requirement on AGC adjustment time for the receiver and transient time for the transmitter | Some period may be necessary | Some period may be necessary |
| Latency on receiving PSSCH | Depending on assumption on gap between PSCCH and PSSCH. If this gap is small, latency is similar as option 3 | If the gap between PSCCH and PSSCH is small, similar latency as Option 1B |
| Power boosting of PSCCH | More flexible | More difficult if same PSD assumption between PSCCH and PSSCH |
| Buffering size | Smaller as no need to buffer PSSCH in advance | Larger due to the need to buffer both PSSCH and PSCCH |
| MIMO operation differentiation between PSCCH and PSSCH | Easy | Possible depending on DMRS design |
| DMRS sharing | Difficult | Possible depending on DMRS design |
| Front-loaded DMRS for PSSCH | Easier to use | Possible depending on DMRS design |
| Half duplex | More issue due to longer duration of control channel + data channel | Less issue due to shorter duration of control channel + data channel |
| Cross-slot scheduling i.e. different slot between PSCCH and PSSCH | Easy to support such functionality. The same design between cross-slot scheduling and same-slot scheduling is supported | No support on cross-slot scheduling. Option 1B like design is required in addition in case cross-slot scheduling is required. |

Based on above table, Option 1B seems to have more merits but we are fine to further discuss these two options. Therefore, we propose to establish working assumption of option 1B. To support both option 1B and 3 are not precluded. We have following proposal,

Proposal 1: Option 1A should be precluded due to restriction on PSCCH size. Working assumption is option 1B.

Regarding whether to support Option 2 in addition, our view is to support it. In LTE V2X, option 2 like operation has been proved to work well for broadcast traffic so it is straightforward to reuse it in NR V2X. Different option can be used to support different operations depending on factors like latency and MIMO schemes. It would be just a (pre)configuration issue.

Proposal 2: Option 2 is supported in addition to Option 1B/Option 3. Which option is selected can be (pre)configured.

Necessity of BWP

Resource pool was agreed in last meeting and the remaining issue is whether to define BWP for NR V2X sidelink. In our view the discussion should focus on functionality first and then discuss such necessity. In the end, BWP may or may not be necessary. The standardization situation in different layer (e.g., RAN1 and RAN2) as well as between sidelink only carrier and sidelink/Uu co-existence carrier may be different. Following Table 2 and Table 3 are some detailed analysis on functionality supported BWP in case of sidelink only carrier and sidelink/Uu co-existence carrier, respectively.

Table 2 Analysis on functionality supported by BWP in sidelink only carrier

| Functionality supported by BWP | Comments |
| --- | --- |
| Linkage to numerology | Resource pool can be configured by numerology |
| Resource allocation | Some solutions are necessary to solve the ambiguity between Tx UE and Rx UE |
| Assumption of UE RF/BB bandwidth | It can be UE implementation issue or up to RAN4. Another simpler way is just to use whole carrier as UE RF/BB bandwidth. Power consumption is not prioritized target for V2X. |
| RRC signalling structure related with BWP | It is up to RAN2 how to modify the spec. To define some relation like “Resource pool = BWP” would be possible. |
| BWP switching | Resource pool switching can be equal function with numerology change and frequency bandwidth change. |

Table 3 Analysis on functionality supported by BWP in the carrier to support both Uu and sidelink

| Functionality supported by BWP | Comments |
| --- | --- |
| Linkage to numerology | Sidelink and Uu should be allowed to be independent in a carrier. To constraint them in the same BWP can simplify the implementation and interference handling but Rx BWP should be wide enough. |
| Resource allocation | The discussion should be separate for mode 1 and mode 2 |
| Assumption of UE RF/BB bandwidth | Up to UE implementation. UE could declare DC carrier position to network which is aligned with sidelink operation. |
| RRC signalling structure related with BWP | Independent between Uu and sidelink and up to RAN2 discussion |
| BWP switching | To be applied only for Uu. For sidelink, no BWP switching. |

Proposal 3:  Regarding necessity of BWP, individual functionality supported by BWP should be discussed first respectively for sidelink only carrier and sidelink/Uu co-existence carrier.

Shortened TTI

In LTE V2X, only subframe level resource allocation is supported although there were some discussions on introducing shortened TTI. On the other hand, existing NR framework is very flexible and any duration/starting positions are supported for PUSCH mapping type B as shown in Table 4. DMRS pattern is also flexible in NR. It can be configured with single/two symbols or front loaded/additional DMRSs.

Table 4 Flexible resource allocation for NR PUSCH

For NR sidelink, we see the need to support shortened TTI due to latency requirement. It is also required to have more flexible operation on TTI transmission compared with LTE. But on the other hand, certain limitation on the transmission flexibility is needed considering receiver UE’s implementation complexity. Taking duration as an example, only the durations targeting for certain use case (e.g., coverage or latency) can be supported. For each use case, only fixed or very limited durations (including starting positions) of TTIs are supported. It can reduce both transmitter or receiver side’s complexity. In addition, we don’t see the need to support overlapped TTI transmissions.

Following figure shows one example on limited durations for NR sidelink: one is mode A which has 13-symbol TTI and targets for large coverage and another one is mode B which has 4 symbol TTI and targets for short latency. DMRS pattern is fixed according to different mode to further reduce the complexity. Other possibilities on limiting duration/starting positions of TTI are not precluded. The amount of limitation on duration/starting positions depends on how duration/starting positions are signalled to the receiver UE and what kind of resource selection procedure are assumed. Above discussions rather assume aligned or quasi-aligned duration of PSCCH and PSSCH (FDMed). In case PSCCH and PSSCH are TDMed, it is possible that PSCCH’s duration/starting position is limited but PSSCH’s duration/position is more flexible.

Figure 1 Limited durations for NR sidelink

In addition, for sidelink physical channel design, it is desirable to reuse existing NR framework as much as possible to save the standardization efforts. On the other hand, necessary modification should be carried out.

Based on above analysis, we propose

Proposal 4: Shortened TTI is supported for NR sidelink but its starting position and/or duration are restricted within a slot.

Beam sweeping

Beam sweeping is supported for mmWave in existing NR in case analogue/ hybrid beamforming is applied. Figure 2 shows one example of beam sweeping for SSB in NR. For NR sidelink, analogue/ hybrid beamforming may be used as well in case the operated frequency is high. In this case, it would have certain impact to sensing or resource selection procedure. In Figure 3, it shows one example on two-beam sweeping where initial transmissions or retransmissions of two beams are consecutive in time domain and have same frequency position. On the other hand, the number of antennae supported in vehicles should be taken into account [3]. More details should be discussed further.

Proposal 5: Discuss whether and how to support beam sweeping for NR sidelink

Figure 2 Beam sweeping of SSB in NR [2]

Figure 3 Beam sweeping of PSSCH/PSCCH

Other aspects

Regarding waveform, our view is both CP-OFDM and DFT-S-OFDM can be considered. The merit and demerit of these two waveforms were largely discussed before and well known. DFT-S-OFDM is beneficial for coverage case while CP-OFDM is beneficial for Rx complexity and MIMO operation for example.

Regarding CP length, extended CP would be needed to handle propagation delay especially for larger SCS case. FFS on exact CP length.

Regarding RS design, we think PT-RS is needed especially for higher modulation case and/or higher frequency. Current design in NR could be the baseline.

Conclusion

In this contribution we discussed physical layer structures in NR sidelink, especially on PSSCH and PSCCH multiplexing, necessity of BWP, Shortened TTI and Beam sweeping. Based on these discussions, we propose followings,

Proposal 1: Option 1A should be precluded due to restriction on PSCCH size. Working assumption is option 1B.

Proposal 2: Option 2 is supported in addition to Option 1B/Option 3. Which option is selected can be (pre)configured.

Proposal 3:  Regarding necessity of BWP, individual functionality supported by BWP should be discussed first respectively for sidelink only carrier and sidelink/Uu co-existence carrier.
Proposal 4: Shortened TTI is supported for NR sidelink but its starting position and/or duration are restricted within a slot.

Proposal 5: Discuss whether and how to support beam sweeping for NR sidelink

Reference

[1] RP-181480, “New SID: Study on NR V2X”, Vodafone

[2] 5G New Radio: Unveiling the Essentials of the Next Generation Wireless Access Technology, Xingqin Lin etc.

[3] RP-181467, " Evaluation for 2 RX exception in Rel-15 vehicle mounted UE", SAMSUNG, Volkswagen AG, LG Electronics

- Appendix

Figure 4 Four options on PSSCH and PSCCH multiplexing

Agreements in RAN1#94bis meeting,

Agreements:

- NR sidelink supports the SCSs supported by Uu in a given frequency range, i.e., {15, 30, 60 kHz} in FR1 and {60, 120 kHz} in FR2.

  - FFS the supported CP length

  - Baseline is that a UE is not required to receive sidelink transmissions using different SCSs simultaneously in a given carrier.

    - FFS if this applies to sidelink synchronization signals/channels

  - Baseline is that a UE is not required to transmit sidelink transmissions using different SCSs simultaneously in a given carrier.

    - FFS if this applies to sidelink synchronization signals/channels

Continue discussion on the wavefom till next meeting – companies are encouraged to perform more analysis/evaluations.

Agreements:

For PSCCH and associated PSSCH multiplexing

- At least one of Option 1A, 1B, and 3 is supported.

  - FFS whether some options require transient period between PSCCH and PSSCH.

- FFS whether to support Option 2

Agreements:

Sidelink control information (SCI) is defined.

SCI is transmitted in PSCCH.

SCI includes at least one SCI format which includes the information necessary to decode the corresponding PSSCH.

NDI, if defined, is a part of SCI.

Sidelink feedback control information (SFCI) is defined.

SFCI includes at least one SFCI format which includes HARQ-ACK for the corresponding PSSCH.

FFS whether a solution will use only one of “ACK,” “NACK,” “DTX,” or use a combination of them.

FFS how to include other feedback information (if supported) in SFCI.

FFS how to convey SFCI on sidelink in PSCCH, and/or PSSCH, and/or a new physical sidelink channel

FFS in the context of Mode 1:

whether/how to convey information for SCI on downlink

whether/how to convey information of SFCI on uplink

Agreements:

At least resource pool is supported for NR sidelink

Resource pool is a set of time and frequency resources that can be used for sidelink transmission and/or reception.

FFS whether a resource pool consists of contiguous resources in time and/or frequency.

A resource pool is inside the RF bandwidth of the UE.

FFS how gNB and other UEs know the RF bandwidth of the UE

FFS if BWP (if defined) can be used to in defining at least part of resource pool

FFS if the numerology of a resource pool is indicated as a part of (pre-)configuration for resource pool, carrier, band, or BWP (if defined)

UE assumes a single numerology in using a resource pool.

Multiple resource pools can be configured to a single UE in a given carrier.

FFS how to use multiple resource pools when (pre-)configured.

FFS BWP is supported for NR sidelink

FFS whether RAN1 can assume that at most one BWP is configured in a carrier from the system perspective.

It is RAN1 understanding that, in some cases, the entire system bandwidth is covered by a single BWP.

FFS the details of BWP configurations, including the possibility of restricting the number of BWPs

FFS whether BWP for TX and RX is separated or a common BWP applied to both TX and RX

There is at most one activated sidelink BWP for a UE in a given carrier as in the Uu case

Further study the feasibility, benefit, and impact of sidelink BWP switching

Aim to conclude in RAN1#95

Companies are encouraged to provide more analysis, including checking current Rel-15 specification regarding BWP related text