3GPP TSG RAN WG1 meeting #95	R1-1813019

Spokane, WA, USA, November 12th – 16th, 2018

Agenda item:	7.1.3.1

Source:	NEC

Title:	Maintenance for physical downlink control channel

Document for:	Discussion and decision

Introduction

In RAN1#94-bis, following agreements were made:

Agreements:

- To adopt the following TP to Section 10 of TS 38.213:

-	a Type0-PDCCH common search space set configured by pdcch-ConfigSIB1 in MasterInformationBlock or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG;

-	a Type0A-PDCCH common search space set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG;

[…]

-	a Type2-PDCCH common search space set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG;

In this contribution, we discuss the necessity of the agreed change.

Discussion

In our view, it is already clear that the procedure described in section 10 and its sub-clauses for Type0-PDCCH CSS, Type0A-PDCCH CSS and Type2-PDCCH is not applicable to SCG according to the following description at the beginning of section 10 of TS38.213:

“If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG except for PDCCH monitoring in Type0/0A/2-PDCCH CSS sets where the UE is not required to apply the procedures in this clause for the SCG”

Therefore, adding “of the MCG” is not needed.

Conclusion

We propose not to adopt adding “of the MCG”.

Reference

[1] RAN1#94bis Chairman’s notes