3GPP TSG-RAN WG1 Meeting #95	R1-1813149

# Spokane, WA, USA, November 12 - 16, 2018

| CR-Form-v11.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | V15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BD/CEE limit for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Three issues discussed in Section 2.1 of attached summary |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes required to adress the three issues |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE does not know what are the BD/CCE limits for the scheduled cell if configured with multiple cells in CA, if UE is capable of dynamic BWP switching or one of the configured serving cells is deactivated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 10.1	UE procedure for determining physical downlink control channel assignment

<unchanged parts omitted >

If a UE

-	is capable for operation with carrier aggregation with a maximum of 4 downlink cells or indicates through pdcch-BlindDetectionCA a capability to monitor PDCCH candidates for  downlink cells, and

-	is configured with  downlink cells with DL BWPs having SCS configuration  where  or , respectively, and

-	for cross-carrier scheduling over  downlink cells, including a scheduling cell, from the  downlink cells where the scheduling cell and scheduled cell(s) have DL BWPs with same SCS configuration

the UE is expected to be able to monitor  PDCCH candidates and  non-overlapped CCEs per slot on the active DL BWP of the scheduling for each scheduled cell. The case of  corresponds only to self-scheduling for the scheduling cell.

If a UE

-	indicates through pdcch-BlindDetectionCA a capability to monitor PDCCH candidates for  downlink cells, and

-	is configured with  downlink cells with DL BWPs having SCS configuration  where , and

the UE is expected to be able to monitor a total of  PDCCH candidates and a total of  non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) for the scheduled cell(s) from the  downlink cells.

UE determines the numerology of a configured serving cell as the numerology of active BWP in the serving cell. For deactivated configured cell, UE determines the numerology as numerology of firstActiveDownlinkBWP-Id BWP. The number of PDCCH candidates and non-overlapped CCEs, the UE is expected to monitor before an active DL BWP switching is triggered, are valid until the last slot of the active DL BWP switching.

<unchanged parts omitted >