3GPP TSG-RAN WG1 Meeting #95	R1-1813149

# Spokane, WA, USA, November 12 - 16, 2018

| CR-Form-v11.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | V15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On half-duplex operation in CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Issue discussed in Section 4.1 of attached summary |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes required to adress the issues |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If no UE behavior is specified, the UE could see the conflict between PDCCH monitoring on one cell and UL transmission in another cell, and the UE behavior is undefined in this case. This causes uncertainty for the gNB and creates issues for gNB scheduling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1 and 11.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

11.1	Slot configuration

<unchanged parts omitted >

If a UE is configured with multiple serving cells in a band combination, the UE is not capable of simultaneous reception and transmission in the serving cells in the band combination, and the UE is not configured to monitor PDCCH for DCI format 2_0 for the reference cell,

- if a symbol in the reference cell is indicated as downlink by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated, or if the UE receives or monitors any DL transmissions in the symbol, the UE shall not transmit any UL transmissions on another cell in the band combination in the symbol;

- else if a symbol in the reference cell is indicated as uplink by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated, or if the UE transmits any UL transmissions in the symbol, the UE shall not receive or monitor any downlink transmissions on another cell in the band combination in the symbol

- otherwise, the UE shall not transmit any UL transmissions on another cell in the band combination in the symbol.

The reference cell is the PCell if the PCell belongs to the band combination, or the serving cell with the smallest cell index across the serving cells in the band combination otherwise.

<unchanged parts omitted >

### 11.1.1	UE procedure for determining slot format

<unchanged parts omitted >

If a UE is configured with multiple serving cells in a band combination, the UE is not capable of simultaneous reception and transmission in the serving cells in the band combination, and the UE is configured to monitor PDCCH for DCI format 2_0 for the reference cell,

- if a symbol in the reference cell is indicated as downlink by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated, or DCI format 2_0, or if the UE receives or monitors any DL transmissions in the symbol, the UE shall not transmit any UL transmissions on another cell in the band combination in the symbol;

- else if a symbol in the reference cell is indicated as uplink by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated, or DCI format 2_0, or if the UE transmits any UL transmissions in the symbol, the UE shall not receive or monitor any downlink transmissions on another cell in the band combination in the symbol

- otherwise, the UE shall not transmit any UL transmissions on another cell in the band combination in the symbol.

The reference cell is the PCell if the PCell belongs to the band combination, or the serving cell with the smallest cell index across the serving cells in the band combination otherwise.

<unchanged parts omitted >