3GPP TSG-RAN WG1 Meeting #95	R1-1813155

# Spokane, WA, USA, November 12 - 16, 2018

| CR-Form-v11.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | V15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Error Cases in PUCCH/PUSCH multiplexing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 02.11.2018 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-14 (Release 15) Rel-14 (Release 16) |  |
| Reason for change: |  | The HARQ-ACK information is multiplexed on PUSCH after the first DMRS symbol of each frequency hop. If there are no non-DMRS symbols after the first DMRS symbol of each frequency hop it would not be possible to multiplex HARQ-ACK on PUSCH UCI information is multiplexed on PUSCH in a non-DMRS symbol. If there are no non-DMRS symbols in a each frequency hop (or slot), it would not be possible to multiplex UCI on PUSCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Calarify that if there are no non-DMRS symbols after the first symbol of each frequency hope, the UE is not expected to multiplex HARQ-ACK information on PUSCH. Calarify that if there are no non-DMRS symbols in each frequency hop (or slot), the UE is not expected to multiplex UCI information on PUSCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec will be ambiguous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 9.3	UCI reporting in physical uplink shared channel

If frequency hopping is configured for a PUSCH, and if there is no symbol in each frequency hop that doesn’t carry DMRS, the UE is not expected to multiplex UCI in the PUSCH. If there is no symbol in each frequency hop after the first set of consecutive OFDM symbol(s) carrying DMRS, the UE is not expected to multiplex HARQ-ACK information in the PUSCH.

If frequency hopping is not configured for a PUSCH, and if there is no symbol in the PUSCH that doesn’t carry DMRS, the UE is not expected to multiplex UCI in the PUSCH. If there is no symbol in the PUSCH after the first set of consecutive OFDM symbol(s) carrying DMRS, the UE is not expected to multiplex HARQ-ACK information in the PUSCH.

<omitted parts>