3GPP TSG RAN WG1 Meeting #95		R1-1813233

Spokane, USA, November 12th – 16th, 2018

Source:		SoftBank

Title:	Necessity of PUCCH/PUSCH enhancements for additional SRS symbols

Agenda Item:	6.2.3.1.1

Document for:	Discussion

Introduction

In RAN#81, the modification of WID (highlighted part with yellow shadow) was briefly discussed because the restriction is too strong. However, no decision was made due to lack of time.

- Enhance SRS capacity and coverage [RAN1]

  - Introduce more than one symbol for SRS for one UE or for multiple UEs on a UL normal subframe

      - Baseline: the minimum SRS resource allocation granularity for a cell is one slot, when more than one symbol in a normal subframe is allocated for SRS for the cell

      - Enhancements on PUCCH and PUSCH are not in scope

This contribution discusses the necessity of PUCCH and PUSCH enhancements. Note this contribution is just for information to RAN1, and the final decision shall be made in RAN Plenary.

This is a revised contribution of R1-1811134, and the new/modified sentences are highlighted by the track changes.

Discussion

Non-CA case

1-A. Collision between additional SRS and PUSCH on the same carrier

Additional SRS resources obviously prevent UEs from transmitting PUSCH, and hence DL performance gain can be obtained at the expense of UL throughput. The degradation highly depends on the amount of additional SRS resources. It is anticipated that the degradation may reach (roughly) 50% when no PUSCH enhancements are allowed, as show in the top figure of Figure 1 (i.e. TDD UL/DL Config.2, 5ms periodicity of 2-consecutive-slot/half-system-bandwidth additional SRS region). If 1-slot PUSCH is available, we can get an option to optimize the occupied additional SRS resources depending on the traffic situation. More concretely, the degradation of PUSCH resources will be relaxed to (roughly) 25% if 1-slot PUSCH for Rel-16 UEs is available as shown in the bottom figure of Figure 1.

Figure 1. Collision between PUSCH and additional SRS on the same carrier

1-B. Collision between additional SRS and PUCCH on the same carrier

An SRS transmission on an additional SRS region may block a HARQ-ACK transmission and the corresponding PDSCH transmissions are consequently interrupted, resulting in DL performance degradation. The degradation will reach 50% degradation when TDD UL/DL Config.2 and 10ms transmission interval of additional SRS are configured for the UE. This is very critical because the intention of additional SRS is enhancement of DL throughput while the PUCCH blocking will degrade the DL throughput conversely. To overcome this problem, a simple existing solution such as HARQ-reference-config may be useful.

Given the discussions above, enhancements on PUCCH and PUSCH are considered as an essential part of this WI for non-CA case. If RAN (or RAN1) decides not to allow PUCCH/PUSCH enhancements, it would be good to deprioritize non-CA use case itself to reduce our standardization efforts. This can be discussed in the next RAN1 meeting taking into account the decision in RAN#82.

Observation 1

For non-CA case,

  - Allow necessary PUSCH/PUCCH enhancements, for example

    - Rate matching for PUSCH (i.e. 1-slot PUSCH)

    - Introduction of HARQ reference config

CA case

In this section, we focus on FDD-TDD CA with FDD PCell and single uplink SRS carrier switching because this is one of the most important scenarios (see R1-1809215 for details). In this scenario, the following two cases (2-A and 2-B) need be considered.

2-A. Collision between additional SRS on SCell and PUSCH on PCell

As shown in Figure 2, PUSCH transmission on PCell will be blocked due to additional SRS transmission on SCell (including switching time). The Rel-14 specifications for SRS carrier switching allows not transmitting a part of PUSCH symbols to maximize the SRS transmission opportunity, but this is limited to non-DMRS/non-UCI symbols to minimize the specification impact. However for additional SRS symbols, it is difficult to apply the same PUSCH rate matching rule as Rel-14 because the SRS position is now more flexible. Hence, an enhancement for PUSCH (e.g. 1-slot PUSCH) might be helpful to solve this issue.

Figure 2. Collision between PUSCH on PCell and additional SRS on SCell with SRS carrier switching

2-B. Collision between additional SRS on SCell and PUCCH on PCell

As shown in Figure 3, Additional SRS (including switching time) on SCell may collide with PUCCH on PCell when SRS carrier switching is used. Note the same problem has identified in Rel-14 SRS carrier switching WI, and RAN1 has already introduced harq-ReferenceConfig. We can reuse this functionality in principle, but we need more study whether or not any optimization is necessary. For example, the Rel-14 mechanism is optimized for TDD-TDD CA because the same mechanism as eIMTA is reused. The modified HARQ transmission is always performed in TDD UL subframe. On the other hand, it would be beneficial to utilize the TDD DL time for HARQ transmission in the case of FDD-TDD CA because the simultaneous transmission of SRS on an SCell and HARQ-ACK can be completely avoided. It is noted that the same feature has already been introduced in Rel-15 for EN-DC, and we can minimize the standardization effort by reusing it.

Figure 3. Collision between PUCCH on PCell and additional SRS on SCell with SRS carrier switching

Observation 2

For CA case 	(especially FDD-TDD CA with SRS carrier switching) ,

  - Rate matched PUSCH transmission on PCell will be useful to make full use of UL resources on PCell.

  - HARQ-ReferenceConfiig for SRS carrier switching is useful to avoid the collision between PUCCH and additional SRS, and RAN1 needs further study on the necessity of optimization.

Conclusion

Our observation in this contribution is summarized as follows:

  - For non-CA case,

  - Allow necessary PUSCH/PUCCH enhancements, for example

    - Rate matching for PUSCH (i.e. 1-slot PUSCH)

    - Introduction of HARQ reference config

For CA case 	(especially FDD-TDD CA with SRS carrier switching) ,

  - Rate matched PUSCH transmission on PCell will be useful to make full use of UL resources on PCell.

  - HARQ-ReferenceConfiig for SRS carrier switching is useful to avoid the collision between PUCCH and additional SRS, and RAN1 needs further study on the necessity of optimization.

Therefore, it would be appropriate to consider the WID modification to address this issue. The example changes are given below, and companies are encouraged to consider it more carefully until RAN#82.

- Enhance SRS capacity and coverage [RAN1]

  - Introduce more than one symbol for SRS for one UE or for multiple UEs on a UL normal subframe

      - Baseline: the minimum SRS resource allocation granularity for a cell is one slot, when more than one symbol in a normal subframe is allocated for SRS for the cell

      - Enhancements on PUCCH are not in scope except the introduction of HARQ reference timing to avoid the collision between SRS and UCI, which are transmitted on the same or different cell

      - Enhancements on PUSCH are not in scope except the PUSCH rate matching to avoid the collision between SRS and PUSCH, which are transmitted on the same or different cell

  - Introduce virtual cell ID for SRS

- Specify higher layer support of enhancements listed above [RAN2]

- Specify RF requirement of UE for the enhancements listed above [RAN4]