3GPP TSG RAN WG1 Meeting #95 R1-1813303 Spokane, USA, November 12th – 16th, 2018 CR-Form-v11.2. CHANGE REQUEST. 38.212; CR; DRAFT; rev; -; Current version:; 15.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on 38.212 for BWP switching with cross-carrier scheduling. Source to WG:; NTT DOCOMO, INC.. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2018-11-02. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; For carrier aggregation with cross-carrier scheduling, UE is configured to monitor DCI format 0_1 or 1_1 with carrier indication field. The carrier indication field indicates for which carrier the DCI format schedules a PDSCH or a PUSCH. If the UE is further configured with dynamic BWP switching, the UE is further configured to monitor DCI format 0_1 or 1_1 with bandwidth part indication field. It is then not clear yet how the UE in this case assumes the bitwidth of the bandwith part indication field. For example, if the scheduling cell has 2 BWP configurations and the possible scheduled cell has 4 BWP configurations, whether this field has 1 bit or 2 bits.. Summary of change:; It is clarified that the bitwidth of the bandwidth part indication field is according to the BWP configuration of the scheduled cell which is indicated by the carrier indication field.. Consequences if not approved:; UE may switch BWP in a wrong cell based on the bandwidth part indication field.. Clauses affected:; 7.3.1.1.2 & 7.3.1.2.2. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

7.3.1.1.2	Format 0_1

<<Unchanged part omitted>>

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL BWPs  configured by higher layers, excluding the initial UL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

-	If the carrier indicator is present and set to the value other than 0, the bandwidth part indicator designates the UL BWP of the scheduled cell given by the carrier indicator, where PUSCH is scheduled. Otherwise (i.e. the carrier indicator is set to 0 or absent), the bandwidth part indicator designates the UL BWP of the scheduling cell where this DCI format is received.

-	If the carrier indicator is present and set to the value other than 0, the bitwidth of the bandwidth part indicator is determined by the number of UL BWPs  configured for the scheduled cell given by the carrier indicator. Otherwise (i.e. the carrier indicator is set to 0 or absent), the bitwidth of the bandwidth part indicator is determined by the number of UL BWPs  configured for the scheduling cell where this DCI format is received.

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

<<Unchanged part omitted>>

7.3.1.2.2	Format 1_1

<<Unchanged part omitted>>

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of DL BWPs  configured by higher layers, excluding the initial DL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

-	If the carrier indicator is present and set to the value other than 0, the bandwidth part indicator designates the DL BWP of the scheduled cell given by the carrier indicator, where PDSCH is scheduled. Otherwise (i.e. the carrier indicator is set to 0 or absent), the bandwidth part indicator designates the DL BWP of the scheduling cell where this DCI format is received.

-	If the carrier indicator is present and set to the value other than 0, the bitwidth of the bandwidth part indicator is determined by the number of DL BWPs  configured for the scheduled cell given by the carrier indicator. Otherwise (i.e. the carrier indicator is set to 0 or absent), the bitwidth of the bandwidth part indicator is determined by the number of DL BWPs  configured for the scheduling cell where this DCI format is received.

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

<<Unchanged part omitted>>