3GPP TSG RAN WG1 Meeting #95 R1-18xxxxx Spokane, USA, November 12th – 16th, 2018 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; DRAFT; rev; -; Current version:; 15.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on 38.213 for BWP switching with cross-carrier scheduling. Source to WG:; NTT DOCOMO, INC.. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2018-11-02. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; 1) It is not clear for which serving cell the bandwidth part indicator field indicates a DL BWP or a UL BWP when the cross-carrier scheduling is configured. 2) there is a description “on the PCell, or on the PSCell, of the MCG”. Obviously PSCell is not in the MCG. This should be typo.. Summary of change:; 1) Clarify that the bandwidth part indicater field indicates a DL BWP or a UL BWP of the scheduled cell. 2) Delete “or on the PSCell, of the MCG” from the sentence.. Consequences if not approved:; 1) gNB and UE cannot have common understanding for which serving cell the bandwidth part indicater field indicates a DL BWP or a UL BWP. 2) new cell, “PSCell of the MCG” is created.. Clauses affected:; 12. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

12	Bandwidth part operation

<<Unchanged part omitted>>

For each DL BWP in a set of DL BWPs on the primary cell, a UE can be configured CORESETs for every type of CSS sets and for USS as described in Subclause 10.1. The UE does not expect to be configured without a CSS set on the PCell in the active DL BWP.

<<Unchanged part omitted>>

If a bandwidth part indicator field is configured in DCI format 1_1, the bandwidth part indicator field value indicates the active DL BWP, from the configured DL BWP set, of the scheduled cell, for DL receptions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in DCI format 0_1, the bandwidth part indicator field value indicates the active UL BWP, from the configured UL BWP set, of the scheduled cell, for UL transmissions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in DCI format 0_1 or DCI format 1_1 and indicates an UL BWP or a DL BWP different from the active UL BWP or DL BWP of the scheduled cell, respectively, the UE shall

-	for each information field in the received DCI format 0_1 or DCI format 1_1

-	if the size of the information field is smaller than the one required for the DCI format 0_1 or DCI format 1_1 interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, respectively, the UE prepends zeros to the information field until its size is the one required for the interpretation of the information field for the UL BWP or DL BWP prior to interpreting the DCI format 0_1 or DCI format 1_1 information fields, respectively

-	if the size of the information field is larger than the one required for the DCI format 0_1 or DCI format 1_1 interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, respectively, the UE uses a number of least significant bits of DCI format 0_1 or DCI format 1_1 equal to the one required for the UL BWP or DL BWP indicated by bandwidth part indicator prior to interpreting the DCI format 0_1 or DCI format 1_1 information fields, respectively

-	set the active UL BWP or DL BWP to the UL BWP or DL BWP indicated by the bandwidth part indicator in the DCI format 0_1 or DCI format 1_1, respectively

<<Unchanged part omitted>>