3GPP TSG RAN WG1 Meeting #95	R1-18nnnn

# Spokane, WA, USA, November 12-16, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to 38.213 capturing the number of closed-loop power control processes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction for the number of PUSCH/PUCCH/SRS closed-loop power control (CL-PC) processes simultaneously maintained at the UE. The current specificaution implies up to 8 (or 16, for the case of supplemnatry uplink -- SUL) CL-PC processes for PUSCH, up to 8 (or 16) CL-PC processes for PUCCH, and up to 12 (or 24) CL-PC processes for SRS, per serving cell. This does not capture the agreements in RAN1#90-Bis and RAN1#91 of up to 2 CL-PC processes for PUSCH, up to 2 for PUCCH, and up to 3 for SRS, per serving cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capturing RAN1#90bis and RAN1#91 agreements regarding the number of PUSCH/PUCCH/SRS closed-loop power control processes per serving cell in endorsed editors’ CR in R1-1812092. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will be in conflict with RAN1 agreements. Increased UE complexity, contrary to RAN1 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1, 7.2.1, 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR … |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR … CR … |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR … CR … |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 7.1.1	UE behaviour

<Unchanged parts are omitted>

For the PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  in PUSCH transmission occasion

-	 is a TPC command value included in a DCI format 0_0 or DCI format 0_1 that schedules the PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell  or jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, as described in Subclause 11.3

-	 if the UE is configured with twoPUSCH-PC-AdjustmentStates and  if the UE is not configured with twoPUSCH-PC-AdjustmentStates or if the PUSCH transmission is scheduled by a RAR UL grant as described in Subclause 8.3. A UE is not expected to maintain more than two PUSCH power control adjustment states per serving cell.

<Unchanged parts are omitted>

### 7.2.1	UE behaviour

<Unchanged parts are omitted>

For the PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  and PUCCH transmission occasion

-	 is a TPC command value and is included in a DCI format 1_0 or DCI format 1_1 for active UL BWP  of carrier  of the primary cell  that the UE detects for PUCCH transmission occasion  or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 36.212], as described in Subclause 11.3

-     if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo and  if the UE is not provided twoPUCCH-PC-AdjustmentStates or PUCCH-SpatialRelationInfo. A UE is not expected to maintain more than two PUCCH power control adjustment states per serving cell.

<Unchanged parts are omitted>

### 7.3.1	UE behaviour

<Unchanged parts are omitted>

For the SRS power control adjustment state for active UL BWP  of carrier  of serving cell  and SRS transmission occasion

- 	A UE is not expected to maintain more than three SRS power control adjustment states per serving cell.

-	, where  is the current PUSCH power control adjustment state as described in Subclause 7.1.1, if srs-PowerControlAdjustmentStates indicates a same power control adjustment state for SRS transmissions and PUSCH transmissions; or

-	 if the UE is not configured for PUSCH transmissions on active UL BWP  of carrier  of serving cell , or if srs-PowerControlAdjustmentStates indicates separate power control adjustment states between SRS transmissions and PUSCH transmissions, and if tpc-Accumulation is not provided, where

<Unchanged parts are omitted>