3GPP TSG RAN WG1 Meeting #95	R1-181nnnn

# Spokane, WA, USA, November 12-16, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to 38.213 capturing PUCCH power control parameters after successful link recovery (a.k.a., beam failure recovery -- BFR) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction for PUCCH power control parameters after successful BFR to ensure beam-specific power control. - If the spatial filter for PRACH transmission for BFR is a configured spatial filter for PUCCH, then the corresponding open-loop and closed-loop PUCCH parameters should be used. - If the spatial filter for PRACH transmission for BFR is not a configured spatial filter for PUCCH, then P0=0dBm should be used, as in the case of PUCCH before configuration. - Corrected typo in RRC parameter PUCCH-SpatialRelationInfo |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capturing beam-specific power control parameters for PUCCH after successful BFR in endorsed editors’ CR in R1-1812092. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior will not be in compliance with the agreed principle of beam-specific power control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6, 7.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR … |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR … CR … |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR … CR … |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 6	Link recovery procedures

<Unchanged parts are omitted>

After K symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId for which the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI and until the UE receives an activation command for PUCCH-SpatialRelationInfo [11, TS 38.321] or is provided PUCCH-SpatialRelationInfo for PUCCH resource(s), the UE transmits a PUCCH using

-	a same spatial filter as for the last PRACH transmission

-	a power determined as described in Subclause 7.2.1 with

-	, and

- the UE determines the value of  and  respectively from the value of p0-PUCCH-Id and closedLoopIndex provided in a configured PUCCH-SpatialRelationInfo if a referenceSignal in the PUCCH-SpatialRelationInfo is same as the reference signal associated with index , = 0 dBm , = 0 dBm and l = 0 otherwise

<Unchanged parts are omitted>