3GPP TSG-RAN WG1 Meeting #94bis 	R1-1813449

Spokane, USA, November 12th – 16th, 2018

Agenda item:	7.2.9.3

Source: 	Qualcomm Incorporated

Title: 	UE Power Consumption Reduction in RRM Measurements

Document for:	Discussion/Decision

# Introduction

This contribution presents our discussion on UE power reduction in RRM measurements for Idle states in Section 2 and Connected state in Section 3.

# RRM Measurement Enhancement in Idle State

When UE operates in Idle states (RRC-IDLE and RRC-INACTIVE), UE might need to perform RRM measurement, cell re-selection if triggered and paging montoring every I-DRX. In particular, NR UE is expected to perform the following key functions:

- Loop convergence (e.g., AGC/TTL/FTL and etc.)

- RRM monitoring/measurement for cell reselection.

- Multi-beam monitoring for beam selection

- Paging monitoring during paging occasion

In LTE, all above functions could be performed using CRS in each configured paging monitoring occasion (I-DRX). For NR, the functions are based on SSBs in the idle state, which may not be present during ON duration of I-DRX. Furthermore, multiple SSBs may be necessary for loop convergence and RRM monitoring/beam selection. The main power contributions in idle state are from loop convergence, RRM measurement, paging monitoring and cell re-selection if triggered. Here, we first point out some key disadvantages of Rel-15 idle mode operation compared to LTE counterparts in idle state before going more details into possible enhancements. More specifically, for Rel-15:

  - UE might need longer RF ON time and additional processing for RRM measurement since UE might need to perform measurement over multiple beams.

  - For LTE, CRS is always present during ON duration of DRX. However, for NR, SSB might be present outside ON duration of DRX. As the result, NR UE might need to wake-up twice for RRM measurement and paging monitoring.

  - Paging inefficiency at the network due to multi-beam paging beam-sweeping, and inefficient paging monitoring/acquisition at the UE since UE needs to determine a suitable beam e.g., the strongest beam to receive paging

Furthermore, NR has been designed for both FR1 and FR2. Due to lower carrier frequency, FR1 has wider coverage than FR2. Furthermore, it could be shown that UE power consumption for FR1 operation is much lower than that for FR2 operation. As a result, it is likely that typical idle UEs will camp on FR1 for coverage and power savings.

Observation 1: It is likely that typical idle UEs will camp on FR1 for coverage and power savings.

NR has 3 RRC states whose state transition is specified in [1]. Compared to LTE, the RRC INACTIVE is a new RRC state for idle UEs to camp. It is different from RRC IDLE in the sense that UE operating in RRC INACTIVE has RAN context; hence the RRC message setup/release latency could be significantly shortened. From UE power perspectives, such shortened latency could make a good contribution to UE power savings especially when UE needs to wake up for setting up RRC connection for background application sync. Further analysis shows in [2] that typically, for the power associated with intermittent RRC connections, roughly a quarter of the time is spent in establishing RRC connections. This signaling overhead can be reduced if UE camps on RRC INACTIVE mode instead of RRC IDLE mode. As a result, it would be beneficial for idle UEs to camp in RRC INACTIVE.

Observation 2: An idle UE could have lower RRC connection setup latency and lower power consumption by camping in RRC-INACTIVE.

RAN1#94b made the following agreements to address the limitation of Rel-15 mobility procedure and to further study on additional resource for power savings from mobility perspectives:

Agreements: - Focus on stationary (e.g., 0km/h), pedestrian (e.g., 3km/h) and vehicular (e.g., 30km/h) scenarios for further studies when considering UE power saving techniques for RRM measurements, - Further studies UE power saving for RRM measurement by considering the following issues - Multi-beam/multi-cell related RRM measurement issues. - Possible misalignment between SSB(s)/SMTC and DRX ON duration Agreements: - Further study on the benefit/impact for additional resource for RRM measurements for power saving for mobility performance not inferior to that of Rel-15 - The schemes proposed should at least include the following aspects: - The signals/channels used as additional resource - Procedure on how to use additional resources. - Overhead from network perspective should be considered.

Figure 1: SFN synchronization

Here, we are discuss possible enhancements related to mobility and paging that could address the limitation of Rel-15 on aspects of multi-beam/multi-cell related RRM measurement and possible misalignment between SSB(s)/SMTC and DRX ON duration (e.g., for paging acquisition and RRM). The enhancements (illustrated in Figure 1), which enables a single beam operation at UE for mobility measurement and paging for FR1, help the idle UEs in RRC INACTIVE further extend its standby time. In particular,

  - Network side: cells within an area (e.g., RNA or RAN area) send SFN (single frequency network) synchronization signals and SFN paging over a single beam

    - We show in Section 5 that the coverage of SFN transmission is comparable to the coverage of multi-beam transmission even in very coverage-limited scenerios.

  - UE side: UE performs measurement of SFN synchronizations signal for RRM and SFN paging reception via a single beam only

The benefits of such enhancement:

  - UE performs single-beam mobility measurement (shorter RF ON time and lower processing overhead). It is worth noting that UE is expected to perform RRM measurement every I-DRX.

  - UE is not required to perform cell reselection when moving between cells within an area (saving UE power in measuring/searching neighbor cells as well as acquiring SI from candidate cells).

  - NW could transmit paging over a single-beam (saving NW power in paging transmission and UE power in beam selection in paging reception).

Figure 2: 30kHz SSB composition

To further understand the UE power savings of our enhancements, we use the agreed RRM power model for FR1 (see Section 6) to analyse the power that UE consumes when performing SSB measurement over a single SSB and over multiple SSBs of 30kHz subcarrier spacing illustrated in Figure 2. More specifically, the measurement power saving gain is computed as

G=1-Em,1Em,L*100

Here, Em,L is the UE energy consumed for measuring L SSBs:

Em,L= Pfr1*NL*Ts

Pfr1 is the agreed slot-averaged power for SSB measurement (e.g., 150 units for measuring 8 cells in synchronous scenarios), Ns is the number of slots that UE needs to perform measurement of L SSBs, and Ts is the slot duration (e.g., 0.5msec for 30kHz SCS).

Em,1 is the UE energy consumed for measuring only one SSB:

Em,1= P1*α*Ts

Here, α is the scalling factor to be applied on the serving cell SSB measurement power P1 agreed in RAN1 #94b i.e., P1=100. We assume α=0.75 (please see [2]). Table 1 summarizes the UE power savings gain of the RRM measurement based on SFN synchronization signals. This power savings gain is from the RRM measurement perspective. From the stand-by use case, other operation/processing aspects (e.g., AGC, time/frequency tracking, paging monitoring etc.) need to be accounted for. RRM measurement might contribute some fraction of the use case power. We might scale the above power saving gains accordingly from stand-by use case perspectives if needed.

Table 1: The power savings gain of RRM measurement based on SFN synchronization signals

| L | NL | Power saving gain G (%) |
| --- | --- | --- |
| 4 | 2 | 75 |
| 8 | 4 | 87.5 |

Our analysis in Section 5 shows that, with the enhancement, 100% UEs are not required to perform cell reselection when moving between cells within an area but not on the SFN boundary. In addition, only around 3% UEs moving on SFN boundary are required to perform an area re-selection. This avoidance of cell re-selections is expected to provide significant additional UE power savings on top of RRM measurement power savings.

Observation 3: RRM enhancement based on SFN synchronization signals could provide a substantial UE power savings gain for RRM measurement. Furthermore, additional UE power savings could come from no cell reselection when UE moves across cells within an area.

Based on above observations, we make the following proposals:

Proposal 1: RAN1 considers RRM measurement based on SFN synchronization signals in RRC-INACTIVE.

Proposal 2: RAN1 considers SFN paging for enabling power savings at both NW and UE in RRC-INACTIVE.

# RRM Measurement Enhancement in Connected State

## Adaptive RRM Measurement Configuration

For Rel-15 RRM, some scheduling restriction is specified in [3]. This restriction is due to different Rx beamforming and/or different sub-carrier spacings of reference signals (SS/PBCH block or CSI-RS) and/or measurement gap for intra/inter frequency measurements.

Observation 4: Rel-15 RRM might require some scheduling restrictions due to the use of different Rx beamforming and/or different sub-carrier spacings of RRM reference signals and/or measurement gap for intra/inter frequency measurements.

Furthermore, RRM measurement configuration is RRC-configured. It is unlikely that the measurement configuration is reconfigured often via RRC. The UE might experience different conditions (e.g., stationary vs. mobile, cell center vs. cell edge, etc.). If a fixed configuration is configured to UE regardless of UE conditions, network may need to configure meausrement configuration based on the worst case which may waste scheduling opportunities due to the scheduling restriction in NR measurements. Depending on minimum requirements, it may also require UE to perform unnecessary measurements. Also note that RAN1#94b made the following agreements:

Agreements: - Further study on relaxing/adapting RRM measurement for UE power saving, - Relaxing/adapting the number of RRM measurements in time domain for serving cell and neighbor cells based on certain conditions, e.g., number of SSB/CSI-RS/measurement occasions. - Relaxing/adapting the number of measured cells/frequencies for neighbor cells based on different conditions - Schemes considered should disclose the mechanism used to determine the mobility/speed assumed - FFS conditions, e.g., channel condition, UE location, UE speed, measurement configuration, measurement results, cell type, etc. - FFS impact on measurement/mobility performance - Relaxed/adapting measurement report (if any) - Other solutions are not precluded.

As a result, we make the following proposal.

Proposal 3: RAN1 considers adaptive RRM measurement configuration in RRC-CONNECTED to minimize scheduling restriction and unnecessary UE power consumption.

- Multiple RRM measurement configurations (e.g., measurement duration, periodicity and measurement gap) can be configured

- A suitable measurement configuration can be dynamically indicated to UE.

## CSI-RS for RRM Measurement

A UE may be configured with the higher layer parameter CSI-RS-Resource-Mobility to use CSI-RS for neighboring cell measurement. Although the UE may predominantly use SS/PBCH blocks for RRM, the CSI-RS for mobility provides the UE with additional opportunities to reinforce the measurement. Especially, in FR2, the CSI-RS for mobility can be transmitted with beam sweeping, which is important both for cell range expansion and UE power saving.

For example, the neighboring cell can use refined (narrow) beams with higher beamforming gain for the CSI-RS for mobility, compared to the (wide) beams used for the associated SSBs. Over multiple CSI-RS-based RRM opportunities, the UE may also refine its Rx beam to further enhance the measurement. The benefit of using refined beam is supported by the fact that the gNB may use narrow beams for data and dedicated control channels, while using wide beams for SSB and common control channels. Hence, a UE being outside the coverage of SSB does not always imply being outside the coverage of data channels.

Now, let us think of the power saving aspect of the CSI-RS for mobility. A UE can measure the CSI-RS for mobility during configured measurement gaps, as shown in Figure 3. The measurement gap is configured with a certain periodicity. Thus, the UE can refine its Rx beam across multiple measurement occasions, as long as the channel variation across occasions is not significant.

Figure 3: RRM measurement with CSI-RS for mobility (different color for different beams)

Note that each measurement includes any RF re-tuning overhead of the UE, which may incur a substantial power penalty, especially when the UE is in C-DRX mode or when the periodicity of the measurement gap is short. The basic principle of UE power saving is to compact the active timeline. In this respect, repetition of beams within a CSI-RS burst can be helpful. When the repetition is enabled, the UE can assume the same spatial domain transmission filter is used for multiple CSI-RS resources. Therefore the UE may sweep its Rx beam within a measurement occasion and the periodicity of measurement can be increased for power saving.

Proposal 4: RAN1 considers repetition of CSI-RS resources for mobility within a resource set.

A conceptual diagram of the proposal is shown in Figure 4.

Figure 4: Repetition of CSI-RS for mobility (different color for different beams)

## Multi-panel RRM Measurement in FR2

A UE with hybrid-beamforming capability, e.g., in FR2, may be equipped with multiple antenna subarrays or panels to support a spherical coverage regardless of the UE orientation relative to the gNB. In some cases, at least for a certain class of UEs, multiple antenna panels are also required to support multi-link or multi-TRP communication. An example of a two-panel UE supporting different multi-panel modes is illustrated in Figure 5.

Figure 5: Multi-panel UE modes

As already stated in the previous section, the principle of RRM power saving is reducing the overall time consumed for measurement. In Rel-15, a measurement gap for a UE is TDMed with the serving cell time and interrupts the communication with the serving cells. This may extend the overall UE timeline and will result in increased power consumption. One enhancement related to the multi-panel mode in Figure 5 (c) is spatially multiplexing the RRM measurement.

For example, in fig (c), TRP1 and TRP2 are two non-collocated neighboring cells and they can be measured simultaneously, cutting down the measurement time in half. In another example, TRP1 is the serving cell and TRP2 is a neighboring cell to measure. In this case, the link to the serving cell may not be interrupted by the measurement gap, although the link quality may be affected. Therefore, during this “soft” gap, the network can still schedule the UE, possibly with restrict max rank, MCS, etc.

Proposal 5: At least for FR2, RAN1 studies multi-panel RRM measurement options for UE power savings:

- The link to the serving cell may not be interrupted during the RRM measurement.

# Conclusion

This contribution provides our discussion on UE power consumption reduction in RRM measurement. For RRC-IDLE and RRC-INACTIVE, we have made the following observations and proposals:

Observation 1: It is likely that typical idle UEs will camp on FR1 for coverage and power savings.

Observation 2: An idle UE could have lower RRC connection setup latency and lower power consumption by camping in RRC-INACTIVE.

Observation 3: RRM enhancement based on SFN synchronization signals could provide a substantial UE power savings gain for RRM measurement. Furthermore, additional UE power savings could come from no cell reselection when UE moves across cells within an area.

Proposal 1: RAN1 considers RRM measurement based on SFN synchronization signals in RRC-INACTIVE.

Proposal 2: RAN1 considers SFN paging for enabling power savings at both NW and UE in RRC-INACTIVE.

For RRC-CONNECTED, we have made the following observations and proposals:

Observation 4: Rel-15 RRM might require some scheduling restrictions due to the use of different Rx beamforming and/or different sub-carrier spacings of RRM reference signals and/or measurement gap for intra/inter frequency measurements.

Proposal 3: RAN1 considers adaptive RRM measurement configuration in RRC-CONNECTED to minimize scheduling restriction and unnecessary UE power consumption.

- Multiple RRM measurement configurations (e.g., measurement duration, periodicity and measurement gap) can be configured

- A suitable measurement configuration can be dynamically indicated to UE.

Proposal 4: RAN1 considers repetition of CSI-RS resources for mobility within a resource set.

Proposal 5: At least for FR2, RAN1 studies multi-panel RRM measurement options for UE power savings:

- The link to the serving cell may not be interrupted during the RRM measurement.

# Appendix 1: SFN vs. Multi-beam Transmission Analysis

One possible RRM enhancement as being discussed in Section 2 is to enable SFN transmission of the synchronization signals for cells within an area. In this section, we provide an analysis on coverage of SFN transmission and multiple-beam transmission schemes (single beam and 8 beams). For SFN modelling, we model a network consisting of multiple areas and each area corresponds to one SFN. With this SFN modelling methodology, we could properly capture the interference impact of neighbour SFNs on the serving SFN.

The analysis is performed for IMT-2020 Dense Urban and 3GPP Urban Macro (UMA). Table 2 provides system-level simulation parameters. In addition, it is worth noting that in our analysis we modelled scenarios with 100% high loss building type which should be the worst cases for coverage analysis. The IMT 2020 Dense Urban has 80% indoor UEs and 20% outdoor UEs while UMA has 100% indoor UEs. Together with longer ISD, UMA is the worst coverage-limited scenarios which could properly represent the scenarios like basement or deep coverage holes.

The performance metric we used to evaluate coverage of different transmission schemes is PDCCH BLER which should work well as a proxy for other channels (e.g., SFN synchronization signals). Figure 6 provides the coverage performance for IMT-2020 Dense Urban and Urban Macro (UMA) for single beam, SFN and 8 beam transmission. It shows that the coverage of SFN transmsison is comparable to 8-beam transmission even in scenario with very limited coverage. In fact, SFN transmission is a bit better since most UEs does not experience the interference from cells within an area.

Figure 7 provides the CDF of SINR for UEs in a dense urban deployment (3GPP UMI). Assuming we have RSRQ level 5 which is a typical setting for RSRQ in practice. RSRQ level 5 corresponds to RSRQ around -17.5dB, which maps to SINR = -6dB when there is 100% loading, and SINR = -3dB when there is 50% loading. It is shown that for SFN-based transmission, ~97% of UEs has SINR higher than -3dB when UE is moving in cells in the SFN boundary while 100% of UEs has SINR higher than -3dB when UE is moving in cells that are not in the SFN boundary (i.e., SFN interior).

- IMT-2020 Dense Urban

- Urban Macro (UMA)

Figure 6: PDCCH coverage performance

Figure 7: CDF of Geometry in 3GPP Urban Micro (UMI) scenario

Table 2: System-level simulation parameters

| Parameter | Value |
| --- | --- |
| Carrier frequency (GHz) | 3.5 |
| Subcarrier spacing (kHz) | 30 |
| System bandwidth (MHz) | 100 |
| UE speed (km/h) | 3 |
| UE deployment | 80% indoor and 20% outdoor (IMT 2020 Dense Urban) 100% indoor (UMA) |
| Percentage of high loss and low loss building type | 100% high loss |
| Total transmit power per TRP (dBm) | 51 |
| Inter-site distance | 200m (IMT-2020 Dense Urban), 500m (UMA) |
| Number of antenna elements per TRP | 128 |
| Number of UE antenna elements | 4 |

# Appendix 2: RAN1#94b Aggreements on RRM power model

Reference configuration and assumptions for RRM measurements:

- SSB periodicity is 20msec

  - 2 SSBs per slot are measured

- SMTC periodicity is 20msec

  - SMTC duration is TBD.

- For inter-band

  - Up to [2] frequency layers and

  - Gap pattern ID = 0 defined in TS 38.133 (i.e., measurement gap length of 6msec and measurement gap periodicity of 40msec)

For intra-frequency measurements:

| N: Number of cells for intra-frequency measurement | Synchronous case |  | Asynchronous case |  |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2 | FR1 | FR2 |
| N=8 | 150 | FFS | 170 | FFS |
| N=4 | 120 | FFS | 140 | FFS |

Table 3: UE power consumption for the RRM measurements

- All above values are slot-averaged power (Pfr1 or Pfr2 for FR1 or FR2 respectively).

- Synchronous case means actual SSB tranmissions from cells are time-aligned e.g., timing of SSBi from cell i is aligned with timing of SSBi from cell j.

- Asynchronous case means actual SSB transmissions from cells are not time-aligned e.g., timing of SSBi from cell i is not aligned with timing of SSBi from cell j

For inter-frequency measurement as function of the number of frequency layers:

UE needs to monitor Nf frequency layers within a measurement gap.

E3=i=0Nf-1Ei+Et∙Nf+1

Where

- Ei is either Pfr1*Ns (for FR1) or Pfr2*Ns (for FR2) for each frequency layer i

  - Ns is the number of slot over which measurements (for each frequency layer i) are carried out

- Nf is the number of frequency layers measured

- Et = Pt * Tt, where

  - Pt is the switching power consumption

    - [Assume microsleep power for Pt]

  - Tt is 0.5ms for FR1 and 0.25ms for FR2 (from RAN4)

It can be simplified to the following if Ei is same across frequency layers (i.e. Ei = E for all i).

E3 = E*Nf + Et*(Nf+1)

Note: RAN4 requirement assumes only one frequency layer per measurement gap.

For full neighboring cell search:

The baseline neighbor cell search power is independent of the number of cells on the first order and can be approximated as:

| FR1 | FR2 |
| --- | --- |
| [180] | FFS |

Table 4: UE power consumption of neighboring cell search

- It is expressed as the power averaged over a slot during which search is performed.

- For combined measurement and search: FFS

# Reference

- TS 38.331 v15.3.0

- R1-1813446, UE Power Evaluation Methodology, Qualcomm Incorporated, RAN1#95

- TS 38.133 v15.3.0