3GPP TSG-RAN WG1 Meeting #95	R1-1813451

# Spokane, USA, 12 – 16 November 2018

Agenda item:	7.2.10.2

Source: 	Qualcomm Incorporated

Title: 	Positioning evaluation methodology and results

Document for:		Discussion and Decision

# 1. 	Introduction

At RAN#80 the study item on NR positioning – "Study on NR Positioning Support" – was approved [1], and revised at RAN#81 [2]. The objectives of this SI include (among others) the following:

- Select the requirements and study corresponding evaluation scenarios/methodologies to enable positioning in regulatory and commercial use cases [RAN1]

  - Identify requirements such as accuracy, latency, capacity, coverage, and etc. (in RAN1 #94bis)

    - For evaluation purpose, radio layer level latency is considered rather than end-to-end latency.

  - Define a representative number of evaluation scenarios for indoor and outdoor

    - One use case representing indoor (e.g. Indoor Office as a baseline)

    - One use case representing outdoor (Umi-street canyon and Uma scenario as baseline)

    - One macro deployment from TR37.857 for FR1

    - Note: Any specific deployment scenarios are also studied including evaluation scenarios for FR2.

  - Define evaluation methodologies considering the above evaluation scenarios including:

    - System parameters including operating bands for both FR1 and FR2 at least for RAT-dependent (NR-based) positioning and for hybrid of RAT-dependent and RAT-independent positioning

    - User dropping procedures

    - Performance metrics to evaluate vertical/horizontal positioning and the above identified requirements

    - The evaluation scenarios/methodologies developed for above regulatory aspects can be a baseline for other positioning evaluations at least by taking TR 37.857 into account.

At RAN1#94bis, some simulation assumptions were agreed on for InH, UMi (both FR1 & FR2) and UMa FR1 [3]. In this contribution, we address some remaining aspects that were not yet addressed previously. We also provide simulation results for the horizontal positioning error for the UMI and Indoor Hotspot scenario for the following three positioning procedures:

- OTDOA

- UTDOA

- Multi-RTT

# 2. Scenarios, System Models and Assumptions

## 2.1  TxRU virtualization, beamforming and antenna configuration

UE Antenna Configuration for FR2 scenarios was agreed in [3] to be virtualized as follows: “The antenna elements of the same polarization of the same panel is virtualized into one TXRU.”  For gNB Antenna Configuration, such explicit agreement was not arrived at, except in the case of FR2 for Indoor office scenario, wherein the following was agreed: “One TXRU per polarization per panel is assumed”. This configuration could be extended to other cases as well.

Proposal 1: Assume 1 TXRU per polarization per panel for all scenarios agreed in RAN1#94bis (UMi and Indoor for both FR1 and FR2, and UMa FR1).

Behavior with multiple panels is also not explicitly defined. We propose the following:

Proposal 2: When UE has 2 panels, one of the panels is selected for each UE to gNB link. The selection is based on maximizing the average channel gain based on the long-term statistics of the channel. Multiple panels on a gNB can be treated as a single large panel with Tx power equal to the sum of the per panel Tx powers.

Note that if PRS from multiple gNBs are FDMed, the above panel selection rule at UE may select different panels for different gNBs. Since virtualization is per panel per polarization, this is feasible. However, gNBs to be received using the same TXRU at the same time must then share the same receive analog beam. TDMing the PRS from different gNBs avoids this constraint. Also note that for the agreed multipanel gNB cases, the agreed parameters obey dgH=N*dH if Ng>1, and dgV=M*dV if Mg>1. This implies that the single large panel also has uniform antenna-element spacing given by dH and dV.

For the indoor office scenario, gNB Antenna Radiation pattern for FR2 was agreed as “3-sector antenna configuration [TR 38.802]”. Table A.2.1-7 of TR38.802 has three antenna configurations for indoor BS antenna besides ‘Single sector’, namely ‘3-sector’, ‘Wall-mount’, and ‘Ceiling-mount’, and the previous agreement leaves it unclear as to which of these is to be selected. We propose the following:

Proposal 3: Adopt ‘Ceiling-mount’ configuration of Table A.2.1-7 of TR38.802 for FR2 indoor office gNB antenna radiation pattern, with antenna panel facing downwards from the ceiling (i.e., orientation angle beta of TR38.901 is 90 degrees).

## 2.1  Other assumptions

For our evaluation results, we also assume the following:

- Analog beamforming weights for both gNB and UE were chosen to create a co-phased beam in the direction of the earliest cluster for each gNB-UE link.

- PRS is a QPSK PN sequence occupying all subcarriers and is TDMed across different gNBs, i.e., perfect PRS muting is assumed.

- Position is estimated based on a single observation of PRS from all gNBs in the layout.

- Network synchronization error is applied by shifting all estimated TOAs from all gNBs relative to the OTDOA reference gNB prior to using the TOAs to estimate position.

- The removal of weak clusters (25dB weaker than the strongest) as per the final procedure in Step 6 in channel generation of TR38.901 was not performed. It is unclear whether this should be done before or after including the LOS power in the first cluster. Also, separate subcluster delays for the two strongest clusters was not modelled, following the recommendations for link-level evaluations in TR38.901. However, the cluster powers and delays were created following the system simulation framework in TR38.901 (as opposed to using fixed profiles based on CDL channels).

### 2.1.1 Network Synchronization

In the figure shown below we provide the probability density function used as a network synchronization error in the estimate TOA for T1=50 nsec. Network synchronization is related to TDOA-based procedures, either OTDOA or UTDOA.

### 2.1.3 Description of applied algorithms

The blocks shown in the below diagram correspond to the main aspects of the applied algorithm with regards to TOA-based positioning procedures: OTDOA, UTDOA, multi-RTT.

RS Transmission from each gNB to a specific UE and/or from a UE to the gNB

CER reception, TOA estimation/pruning

Perform position estimation

Fix “good” Tx/Rx Beam pair for each gNB/UE link

### 2.1.3.1 TOA Estimation & Pruning

Identify the reference TOA as the TOA with the highest quality metric, and derive the TDOA vector

Prune the TOAs with a quality metric smaller than a threshold, ensuring that

- At least X different cell-sites are used (e.g., 3)

Sort the TOAs based on a Quality metric, e.g.,:

- the estimated SNR

- median/TOA-peak ratio

- median/main peak ratio

After all TOAs are estimated (e.g., from 57 cells for UMI/UMA), the TOAs are sorted according to a quality metric, and all the TOAs with a quality metric smaller than a specific threshold are removed while ensuring that at least 3 different cell-sites are used. Then, in the OTDOA and UTDOA procedures, the reference TOA is identified as the TOA with the highest quality metric.

### 2.1.3.2 TDOA positioning algorithm

The TDOA-based estimation requires an algorithm to solve the multilateration (MLAT) system. This is a widely studied problem in the literature [7]. In this paper, the following two algorithms have been implemented, and the best one is chosen to present the following results:

- Chan's Algorithm [5]: A non-iterative solution to the TDOA position estimation problem.

- Taylor Series Algorithm [4]: Linearizes the set of equations by Taylor-series expansion, and then uses an iterative method to solve them. The iterative method begins with an initial guess and improves the estimate at each iteration by determining the local linear least-square (LS) solution.

In both algorithms, tha covariance matrix of the TOA measurement could be used to improve further the performance, but in these results we only assume that equal weight is used to all the chosen TOA measurements that make it to the TDOA vector.

### 2.1.3.3 multi-RTT positioning algorithm

For multi-RTT positioning, the positioning engine should be one solving the trilateration problem. Several procedures can be found in the literature, some statistical, numerical, or algebraic. For its simplicity and its low computational complexity, we chose an algebraic solution proposed in [6]. Similarly, to the TDOA-based approaches, the covariance matrix of the TOA measurements could be used to improve the results, but for simplicity, we assume that equal weight is applied to all the TOA measurements which have not been removed ny the TOA pruning procedure.

Proposal 4: Discuss further whether a common baseline algorithm across different procedures is needed to compare different positioning schemes and calibrate the results across companies.

# 3. 	Simulation results – Horizontal Positioning Error (CDF)

All results are for 100 MHz in FR1 and 400 MHz in FR2.

## 3.1 UMI

## 3.1.1 FR1

##### 3.1.1.1 OTDOA

##### 3.1.1.2 UTDOA

##### 3.1.1.3 multi-RTT

## 3.1.1 FR2

##### 3.1.1.1 OTDOA

##### 3.1.1.2 UTDOA

##### 3.1.1.3 multi-RTT

## 3.2 	Indoor hotspot

## 3.1.1 	FR1

##### 3.1.1.1 OTDOA

##### 3.1.1.2 UTDOA

##### 3.1.1.3 multi-RTT

## 3.1.1 	FR2

##### 3.1.1.1 OTDOA

##### 3.1.1.2 UTDOA

##### 3.1.1.3 multi-RTT

## 3.2 Summary of the Simulation Results

| 90%-ile [m] | UMi 4GHz Perf sync ¦ T1=50ns |  | InO 4GHz Perf sync ¦ T1=50ns |  | UMi 30GHz Perf sync ¦ T1=50ns |  | InO 30GHz Perf sync ¦ T1=50ns |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OTDOA | 9.5m | 13.5m | 3.5m | 44m | 3.5m | 13m | 0.8m | 37m |
| UTDOA | 12m | 18m | 4m | 44m | 11m | 21m | 0.8m | 37m |
| Multi-RTT | 11.5m | 11.5m | 4.5m | 4.5m | 4.5m | 4.5m | 0.8m | 0.8m |

| 80%-ile [m] | UMi 4GHz Perf sync ¦ T1=50ns |  | InO 4GHz Perf sync ¦ T1=50ns |  | UMi 30GHz Perf sync ¦ T1=50ns |  | InO 30GHz Perf sync ¦ T1=50ns |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OTDOA | 6.5m | 11m | 2m | 25m | 2m | 11m | 0.5m | 23m |
| UTDOA | 8.5m | 14.5m | 2m | 25m | 3.5m | 15m | 0.5m | 23m |
| Multi-RTT | 6.5m | 6.5m | 3m | 3m | 2.2m | 2.2m | 0.5m | 0.5m |

| 67%-ile [m] | UMi 4GHz Perf sync ¦ T1=50ns |  | InO 4GHz Perf sync ¦ T1=50ns |  | UMi 30GHz Perf sync ¦ T1=50ns |  | InO 30GHz Perf sync ¦ T1=50ns |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OTDOA | 4.5m | 9m | 1.5m | 17m | 1m | 9m | 0.4m | 17m |
| UTDOA | 5.5m | 11.5m | 1.5m | 17m | 1.5m | 11.5m | 0.4m | 17m |
| Multi-RTT | 4.5m | 4.5m | 2m | 2m | 1m | 1m | 0.4m | 0.4m |

| 50%-ile [m] | UMi 4GHz Perf sync ¦ T1=50ns |  | InO 4GHz Perf sync ¦ T1=50ns |  | UMi 30GHz Perf sync ¦ T1=50ns |  | InO 30GHz Perf sync ¦ T1=50ns |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OTDOA | 3m | 7m | 1m | 12m | 0.6m | 7m | 0.3m | 12m |
| UTDOA | 3.5m | 9m | 1m | 12m | 0.7m | 8.5m | 0.3m | 12m |
| Multi-RTT | 3m | 3m | 1.3m | 1.3m | 0.6m | 0.6m | 0.3m | 0.3m |

## 3.3 Observations based on simulation results

Based on the above results, we make the following qualititavely observations:

Observation 1: For multi-RTT approach, the positioning error is not affected by the network synchronization.

Observation 2: In the perfect synchronizaiton scenario, both OTDOA and multi-RTT demonstrate similar performance.

Observation 3: In the realistic synchronization scenario, multi-RTT demonstrates better performance compared to OTDOA and UTDOA.

# 4. 	Summary

In this contribution, we proposed:

Proposal 1: Assume 1 TXRU per polarization per panel for all scenarios agreed in RAN1#94bis (UMi and Indoor for both FR1 and FR2, and UMa FR1).

Proposal 2: When UE has 2 panels, one of the panels is selected for each UE to gNB link. The selection is based on maximizing the average channel gain based on the long-term statistics of the channel. Multiple panels on a gNB can be treated as a single large panel with Tx power equal to the sum of the per panel Tx powers.

Proposal 3: Adopt ‘Ceiling-mount’ configuration of Table A.2.1-7 of TR38.802 for FR2 indoor office gNB antenna radiation pattern, with antenna panel facing downwards from the ceiling (i.e., orientation angle beta of TR38.901 is 90 degrees).

Proposal 4: Discuss further whether a common baseline algorithm across different procedures is needed to compare different positioning schemes and calibrate the results across companies.

Observation 1: For multi-RTT approach, the positioning error is not affected by the network synchronization.

Observation 2: In the perfect synchronizaiton scenario, both OTDOA and multi-RTT demonstrate similar performance.

Observation 3: In the realistic synchronization scenario, multi-RTT demonstrates better performance compared to OTDOA and UTDOA.

# References

[1]	RP-181399, "New SID: Study on NR positioning support", Intel Corporation, Ericsson.

[2]	RP-182155, "Revised SID: Study on NR positioning support", Intel Corporation, Ericsson.

[3]	3GPP RAN1#94bis Chairman’s Notes

[4] 	Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994.

[5]	W. H. Foy, " Position-Location Solutions by Taylor-Series Estimation", IEEE Transactions on Aerospace and Electronic Systems, vol. AES-12, pp. 187-194, March 1976.

[6]	Norrdine, Abdelmoumen. (2015). An Algebraic Solution to the Multilateration Problem. 10.13140/RG.2.1.1681.3602.

[7] 	Mantilla-Gaviria, I.A., Leonardi, M., Galati, G. et al. SIViP (2015) 9: 1549. https://doi.org/10.1007/s11760-013-0608-1

[8]	3GPP TR 38.802: "Study on New Radio Access Technology; Physical Layer Aspects".

[9]	3GPP TR 37.857: "Study on indoor positioning enhancements for UTRA and LTE".

[10]	3GPP TR 36.873: "Study on 3D channel model for LTE".

[11]	R1-1811285: "Requirements and Performance Metrics for Positioning Evaluations", Qualcomm Incorporated.