3GPP TSG-RAN WG1 Meeting #95	<TDoc#>

# Spokane, WA, USA, November 12 - 16, 2018

| CR-Form-v11.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | V15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Failure Detection Resources for BFR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 02.11.2018 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-14 (Release 15) Rel-14 (Release 16) |  |
| Reason for change: |  | Define UE actions in implicit BFD-RS configuration when number of active TCI States for PDCCH exceeds the maximum number of beam failure detection resources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following logic is proposed for selecting subset of BFD-RS signals when number of active TCI states for PDCCH exceeds the maximum number - Selection of BFD-RS to be included in the set of q0 is determined based on shortest search space periodicity associated with CORESET with active TCI state for PDCCH. - SS periodicity of a CORESET is used first as criteria for selecting the BFD-RS, and CORESETs are selected in order according to shortest SS periodicity - if more than one selected CORESETs are associated with same SS periodicity, the respective CORESETs are selected in order according to the highest CORESET ID |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not accepted, there will be ambiguity at UE and network side which signals are used for beam failure detection when BFD-RS signals are implicitly configured |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 38.213, 6 Link Recovery Procedures |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 6	Link Radio link monitoring

A UE can be provided, for a serving cell, a set  of periodic CSI-RS resource configuration indexes by failureDetectionResources and a set  of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList for radio link quality measurements on the serving cell. If the UE is not provided failureDetectionResources, the UE determines the set  to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-states for respective CORESETs that the UE uses for monitoring PDCCH. The UE expects the set  to include up to two RS indexes and, if there are two RS indexes in a TCI state, the set  includes RS indexes with QCL-TypeD configuration for the corresponding TCI states. The UE expects single port RS in the set .

=== Start of change request  ===

If the UE is not provided failureDetectionResources, and the UE configured with more than two activated TCI States for PDCCH, the UE selects the two RS to be included in the set of q0, provided for active TCI states for PDCCH reception in CORESETs associated with the search space sets with the lowest monitoring periodicities. If more than one selected CORESETs are associated with search spaces sets having same monitoring periodicity, the UE selects the CORESETs in order according to CORESET ID starting from highest CORESET ID.

=== End of change request ===