3GPP TSG-RAN WG1 Meeting #95	<TDoc#>

# Spokane, WA, USA, November 12 - 16, 2018

| CR-Form-v11.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | V15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Selection of RS to be include in set fo q0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 02.11.2018 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-14 (Release 15) Rel-14 (Release 16) |  |
| Reason for change: |  | Define UE actions in implicit BFD-RS configuration when number of active TCI States for PDCCH exceeds the maximum number of beam failure detection resources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following logic is proposed for selecting subset of BFD-RS signals when number of active TCI states for PDCCH exceeds the maximum number - Selection of BFD-RS to be included in the set of q0 is determined based on shortest search space periodicity associated with CORESET with active TCI state for PDCCH. - SS periodicity of a CORESET is used first as criteria for selecting the BFD-RS, and CORESETs are selected in order according to shortest SS periodicity - if more than one selected CORESETs are associated with same SS periodicity, the respective CORESETs are selected in order according to the lowest CORESET ID |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not accepted, there will be ambiguity at UE and network side which signals are used for beam failure detection when BFD-RS signals are implicitly configured and number of activated TCI States for PDCCH is 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 38.213, 6 Link Recovery Procedures |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 6	Link Recovery Procedures

TCI state or any of the parameters TCI-StatesPDCCH-ToAddlist and/or TCI-StatesPDCCH-ToReleaseList. After the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI in the search space set provided by recoverySearchSpaceId, the UE continues to monitor PDCCH candidates in the search space set provided by recoverySearchSpaceId until the UE receives a MAC CE activation command for a TCI state or TCI-StatesPDCCH-ToAddlist and/or TCI-StatesPDCCH-ToReleaseList.

=== Start of change request ===

After K symbols from a last symbol of a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI that a UE detects in a search space set provided by recoverySearchSpaceId or after successful completion of contention based random access procedure for beam failure recovery and until the UE receives an activation or reconfiguration of PUCCH-Spatialrelationinfo for PUCCH resource(s), the UE transmits PUCCH using

-	a same spatial filter as for the PRACH transmission

-	a power determined as described in Subclause 7.2.1 with , , and

=== End of change request ===