3GPP TSG RAN WG1 Meeting #95	           	R1-1813488

Spokane, USA, November 12th – 16th, 2018

Agenda item:       7.2.8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	CSI feedback overhead reduction for MU-MIMO enhancements

Document for:     Discussion and Decision

# 1	Introduction

Enhancements on MU-MIMO support were approved to be studied and specified as part of the MIMO Enhancements WID in RAN#80 [1] and revised slightly in RAN#81 [2]. The objectives for enhancing MU-MIMO support are as follows [1] [2]:

The work item aims to specify the enhancements identified for NR MIMO. The detailed objectives are as follows. - Extend specification support in the following areas [RAN1] - Enhancements on MU-MIMO support - Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead - Perform study and, if needed, specify extension of Type II CSI feedback to rank >2 … …

In this contribution we specifically target the first goal of the MU-MIMO enhancement WI. In particular, we propose an overhead reduction framework which could be adopted to meet the performance/overhead target and serve as basis for future studies and developments in view of the second goal of the WI.

# 2	Overhead reduction by Fourier-based compression

Fourier methods provide a convenient way to compress the information carried by the N3 matrices  W2n, with N3 defined as number of frequency resources over which CSI is available, e.g., subbands in the context of Rel. 15 Type II CSI reporting. A two-fold advantage characterizes these methods:

- They allow the capture and exploitation of correlation present between different matrices  W2n across the N3 frequency resources to reduce the overall number of coefficients to be reported.

- Information about the adopted compression approach can be signaled to the gNB using a rather low number of bits. This allows the design of efficient reporting mechanisms for the PMI component of CSI reporting.

In general, W2n can be represented with or without layers compression. In this regard, three major possibilities exist:

- If layers compression is not performed, then W2n can be represented as:

  - A redundant set of complex numbers;

  - A redundant set of real-valued coefficients, i.e., amplitude and phase of each element.

- If layers compression is performed, then W2n can be represented with the minimum number of real-valued coefficients, i.e., two sets of angular values in the form of Givens and phase rotations, respectively. This has the advantage of eliminating any redundancy in the layer representation and provides increasing compression gain as the number of layers grows [3].

Suitable Fourier methods can be applied at this stage to compress the information present in the N3 matrices W2n, depending on the adopted representation of the latter. In particular, when each W2n is composed of complex numbers then DFT-based methods can be applied. Conversely, when each W2n is represented with two sets of real numbers then either DFT-based or DCT-based methods can be applied.

Before proceeding with the detailed description of DFT-based and DCT-based method, a brief introduction to layers compression is given for the sake of clarity.

## 2.1	Layers compression

Before applying frequency compression, the number of linear combination coefficients resulting from the spatial compression can be reduced by exploiting the unit-norm and orthogonality properties of layers. This so-called layers compression simply aims at removing the redundancy in the layers representation before applying compression in frequency. By doing so we can save 2 real coefficients in the representation of layer 1, 4 real coefficients in the representation of layer 2, etc. Note that the saving achieved for layer 1 is equivalent to the amplitude and phase normalization performed by Type II CSI.

This layers compression can be achieved in either of two ways:

- By normalising the complex-valued W2n for each n=0,…,N3-1, in amplitude and phase, thereby saving 1 complex coefficient per layer, which yields a complex matrix W2,l of reduced size (2L-1)×N3 for layer l=1,2

- By compressing W2n for each n=0,…,N3-1, in the layer domain, i.e., across its Nri columns (with  Nri=2 in this document). This yields two real-valued matrices W2,lA,W2,lP of reduced size (2L-l)×N3 for layer l=1,2.

Table 1 shows an example of the additional overhead reduction achieved by layers compression combined with frequency compression in the case L=4, where M is the number of selected/compressed frequency-domain components.

Table 1. Overhead reduction for freq. compression w/ and w/o layer compression, with L=4

|  | No freq. compression (M=13) |  | M=8 |  | M=4 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Layer 1 | Layer 2 | Layer 1 | Layer 2 | Layer 1 | Layer 2 |
| No freq. compression | 100% | 100% | -- | -- | -- | -- |
| DCT/DFT freq. compression | 100% | 100% | 61.5% | 61.5% | 30.8% | 30.8% |
| layers compression + DCT/DFT freq. compression | 87.5% | 75% | 53.8% | 46.1% | 26.9% | 23.1% |

Layers compression can be achieved efficiently for any number of layers by representing W2n by means of Givens parameters before applying frequency compression. This operation does not require any additional signalling.

Figure 1 illustrates the sequence of spatial-layers-frequency compression operations applied to reduce overhead of CSI reports. In the following two sections we will describe in more details how the frequency compression can be achieved by means of Fourier-based transformations (DFT/DCT).

Figure 1. Diagram showing the sequence of compression operations to reduce overhead in CSI feedback

Observation 1: Prior to frequency compression, the number of linear combination coefficients for each frequency resource can be reduced by exploiting the unit-norm and orthogonality of layers. These coefficients are then rearranged by layers ready for applying frequency compression

Proposal 1: Aspects of the mapping of coefficients between spatial compression and frequency compression are FFS. A layers compression stage should be considered between spatial and frequency compression to further reduce overhead and included in the evaluation results

## 2.2 DFT-based frequency compression

We now switch our focus to the first proposed compression approach, which is DFT-based. A suitable method to exploit the correlation across different matrices W2n, when the latter are composed by complex numbers, is to resort to an oversampled DFT codebook F to realize compression across the frequency resources. If we let O3 be the oversampling factor, then such oversampled codebook would be defined as:

Fnm=1N3e-j2πnmO3N3n∈[0,1,…, N3-1]m∈[0,1,…,O3N3-1],

where a regular DFT codebook would be obtained if O3=1. It is worth observing that the construction of F is performed following the same logic followed to construct the codebook which the spatial compression matrix W1 is extracted from. This codebook can be used to compress the matrices W2n, in several ways. In the general case, the compression starts by constructing the Nri aggregated matrices W2,l of size 2L×N3 for each layer l, by horizontally concatenating the l-th column of each W2n. Then a set of M<N3 column vectors is selected from the codebook F to form a frequency compression matrix Wf of size N3×M, which is common to different layers. We note that an equivalent way of realizing such construction is to build Wf directly, without performing a prior construction of F, as

Wfnm=1N3e-j2πnkmO3N3n∈[0,1,…, N3-1]m∈[1, M]km∈0,1,…,O3N3-1 km≠km', if m≠m' .

Regardless of the approach adopted for its construction, such Wf is applied to each W2,l to yield a compressed matrix W2,l=W2,lWf, where W2,l has size 2L×M.

In this case, the UE would need to signal to the gNB a set of Nri W2,l matrices and the information on the vectors used to construct Wf, e.g., in the form of their indices inside the oversampled codebook. Several options can be adopted for the quantization of the matrices W2,l, ∀l∈0,Nri-1.  In particular, one possibility could be the quantization of the 2L×M coefficients of the l-th layer, i.e., present in the matrix W2,l, in terms of amplitude and phase. In this context, a “wideband/subband” approach could be followed, as for Rel. 15 Type II CSI. Accordingly, for each layer, amplitude quantization could employ a differential subband feedback structure, and the index of the strongest wideband amplitude for each layer would be used to indicate the strongest one in each column of the matrix W2,l. In this case, and since each column of the matrix W2,l would be no longer normalized after frequency compression, all the strongest coefficients in each column of matrix W2,l, totally Nri×M, should be quantized separately. All the coefficients other than the strongest ones in matrix W2,l, totally up to 2L-1×Nri×M, would then be signaled separately in terms of differential subband amplitude and subband phase. Information on the M vectors used to construct Wf may then be fed back following the same or different periodicity as W2,l. This approach could provide a quantization framework similar for Rel. 15 Type II CSI, however other quantization approaches could be adopted (possibly more advantageous in terms of performance/overhead tradeoff). This aspect should be subject to future study for optimization.

Finally, the gNB can reconstruct an estimation of each matrix W2n. More precisely, this last operation would be performed as follows. First, the Nri matrices W2,l=W2,lWfH, i.e., the estimates of the original Nri  W2,l matrices, are computed. Then, the estimates of the N3 matrices W2n, i.e., W2n, are obtained by horizontally concatenating the n-th column of each W2,l matrix.

This approach can be applied in case of both complex and real representation of the elements in W2n, with or without layers compression, and allows the reduction of the number of real coefficients necessary to represent the Type II PMI.

Observation 2: Fourier transformation by means of DFT is a well-established technique to describe a correlated signal by means of a few dominant components in the transformed domain. The selection of columns in Wf allows the location of the M dominant components, which are reported as linear combination coefficients. It is the same technique used for spatial compression through the selection of W1.

Proposal 2: Achieve frequency compression using a Fourier-based transformation and reporting of the dominant transformed components.

Observation 3: The quantization of the linear combination coefficients after frequency compression needs to be optimized for best trade-off performance/overhead.

Proposal 3: Aspects of quantization of the linear combination coefficients are to be optimized.

## 2.3	DCT-based frequency compression

The purpose of compressing the W2n matrices across the frequency resources is to exploit the correlation of their coefficients in frequency. The discrete cosine transform (DCT) is a well-known Fourier-based transformation for real-valued signals with very strong “energy” concentration properties, i.e., the signal information tends to be concentrated in a few “low-frequency” components of the DCT. For strongly correlated signals DCT can approach the compression efficiency of the optimal Karhunen-Loève transform (a.k.a., principal component analysis), without requiring knowledge of the correlation function. Moreover, the W2n complex coefficients are naturally represented by two sets of real-valued coefficients prior to scalar quantization, hence it makes sense to consider compression directly in the real domain.

There are some advantages in applying the DCT as frequency compression transformation over DFT, notably:

- The compression matrix Wf is fixed and does not need to be signaled, which further reduces the feedback overhead. In other words, a DCT-based solution is codebook-free.

- The absence of signaling related to Wf simplifies the signaling structure, i.e., feedback periodicity is not affected by its presence and is fully compliant with Rel-15.

- Because Wf is fixed, there is no need for the UE to build this matrix before frequency compression, for example by codebook selection, which reduces the computational burden of the UE

It is worth observing that the reason why the first advantage is possible lies in the strong decorrelation properties of the DCT: the signal information is typically concentrated in the first M samples, so there is no need to locate and report the indices of the transformed coefficients that give the most accurate representation. This is illustrated in Figure 2 where the PDF is plotted for the first 6 coefficients of the DCT. The example is with L=4, Nri=4. The figure shows that the signal energy is consistently concentrated in the first few samples, i.e., the standard deviation decreases sharply with the DCT coefficient index, particularly for the first 2 layers. Typically, the first M=4 DCT samples carry most information and the rest can be discarded.

Figure 2. Distribution of the first 6 DCT coefficients after frequency compression for layers 1 to 4.

The compression matrix is simply given by

Wfnm=1K,  n=02Kcosπ2m+1n2K,  n=1,…K-1,  m=0,…,M-1

and the compressed linear combination coefficients for layer l are given by W2,l=W2,lWf, where W2,l contains the parametric representation of layer l of W2n for all frequency resources (one column vector per frequency resource). Because the DCT is applied to real valued coefficients, the parametrization of W2n ensures that W2n is exactly represented by 2 real-valued matrices W2,l and the DCT is applied to each of them separately.

The frequency compressed feedback that the UE needs reporting for each layer l is given by a quantized version of the linear combination coefficients W2,l. As for the DFT-based case, several options can be adopted for the quantization, with different impact on the total number of bits forming the CSI feedback message for each UE.

Switching the focus to the operations performed by the gNB, we note that the parameters to reconstruct the set of N3 complex-valued W2n from W2,l are obtained from the inverse DCT operation, i.e., W2,l=W2,lWfT for each real dimension. The rest of the reconstruction is performed as described in Sec. 2.

Observation 4: DCT is a well-established Fourier-based transformation with strong “energy” concentration properties for correlated real-valued signals. It has the advantage of having a fixed Wf, which does not require signaling and avoids the complexity of codebook selection and reporting at the UE.

Proposal 4: The columns of the compression matrix Wf are DFT or DCT vectors.

## 2.4	Performance

The simulation settings used to obtain all the results in the section are as per EVM configuration agreed in RAN1 #94b [4], summarized in Table 2 in the appendix for convenience. The following labels are used to refer to the proposed solutions:

- FD-DFT: denoting DFT-based frequency domain compression

- LD: denoting layers compression (based on Givens parameters)

- LD-DFT: denoting layers compression followed by DFT-based frequency domain compression

- LD-DCT: denoting layers compression followed by DCT-based frequency domain compression

In Figures 3 and 4, the performance of the proposed CSI feedback schemes is depicted, for the cases of 16 and 32 antenna ports, respectively. The performance of Type I and Type II CSI feedback, the latter configured with L=4 and 8-PSK phase, WB+SB amp, is plotted as a benchmark.

Normalized Mean User Spectral Efficiency

Normalized 5th percentile user SE

Figure 3 – Performance with 16 Aps

As shown in Figures 3 and 4, LD compression shows almost no loss compared to Type II CSI, noting that the overhead of LD compression can be further reduced by exploiting the correlation of the compressed Givens parameters.

The frequency compression variants without LD compression, show 10%-20% performance loss for M=8 and M=4, respectively. We note, however, that a simple scalar quantization scheme was used to quantize each compressed matrix W2,l, hence these results should not be considered as the outcome of an optimized solution. Further strategies for designing an optimized quantization scheme are FFS and may include:

- Higher bit resolution for the Kf strongest frequency beams;

- Higher bit resolution for the Ks strongest spatial beams (as in Type II CSI);

- Different number of frequency beams, i.e., columns of the matrix Wf per layer.

In Figures 5 and 6, we assess the SINR performance, without quantization, of the different variants of LD compression, compared to the baseline case with perfect knowledge of the W2 matrix. As shown therein, baseline LD without quantization shows almost no loss compared to the baseline case. Moreover, the following observations can be made:

- Combining LD compression with DFT or DCT based FD compression with M=4, results in less than 1dB SINR loss, with a significant overhead reduction (cfr. Table 1).

- Performing a frequency-domain compression with M=8, results in less than 0.5dB SINR loss.

- DCT-based frequency compression consistently outperforms DFT-based compression, with larger performance gap for M=4.

Normalized 5th percentile user SE

Normalized Mean User Spectral Efficiency

Figure 4 – Performance with 32 APs

Figure 5- SINR CDF for the unquantized case with 16APs

Figure 6- SINR CDF for the unquantized case with 32APs

# 3	Conclusions

In this contribution, we have presented our proposal for overhead reduction of the NR Type II codebook.  Our observations and proposals are:

Observation 1: Prior to frequency compression, the number of linear combination coefficients can be reduced by exploiting the orthogonality of layers. These coefficients are then separated by layers into real or complex-valued matrices ready for applying frequency compression.

Proposal 1: Aspects of the mapping of coefficients between spatial compression and frequency compression are FFS. A layers compression stage should be considered between spatial and frequency compression to further reduce overhead and included in the evaluation results.

Observation 2: Fourier transformation by means of DFT is a well-established technique to describe a correlated signal by means of a few dominant components in the transformed domain. The selection of columns in Wf allows the location of the M dominant components, which are reported as linear combination coefficients. It is the same technique used for spatial compression through the selection of W1.

Proposal 2: Achieve frequency compression using a Fourier-based transformation and reporting of the dominant transformed components.

Observation 3: The quantization of the linear combination coefficients after frequency compression needs to be optimized for best trade-off performance/overhead.

Proposal 3: Aspects of quantization of the linear combination coefficients are to be optimized.

Observation 4: DCT is a well-established Fourier-based transformation with strong “energy” concentration properties for correlated real-valued signals. It has the advantage of having a fixed Wf, which does not require signaling and avoids the complexity of codebook selection and reporting at the UE.

Proposal 4: The columns of the compression matrix Wf are DFT or DCT vectors.

# References

| [1] | Samsung, RP-181453, “New Draft WI proposal on NR MIMO enhancements”, 2018. |
| --- | --- |
| [2] | Samsung, RP-182067, “Revised WID: Enhancements on MIMO for NR”, 2018. |
| [3] | Nokia/NSB, R1-1811406, "CSI Enhancements for MU-MIMO Support", 2018. |
| [4] | 3GPP, Chairman's notes RAN1 94b final, 2018. |

# Appendix

Table 2.  System Simulation Parameters

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban |
| Carrier frequency |  | 4GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | UMa, according to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | As in TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 |
| CSI Feedback bit allocation | LD | Nϕ=3 bits and Nψ=2 bits |
|  | FD-DFT | Namp=3 bits and Nphase=3 bits |
| Simulation bandwidth |  | 10 MHz for 15kHz |
| Frame structure |  | Slot Format 0 |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation. |
| MIMO layers |  | Maximum MU layers = 12 |
| CSI feedback |  | CSI feedback periodicity: 5 ms Scheduling delay: 4 ms |
| Overhead |  | 2 symbols/slot |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% target utilization. Arrival rates are: - 4 users/sec for 16 antenna ports results in 62% RU - 5 users/sec for 32 antenna ports results in 60% RU |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Non-ideal |