3GPP TSG RAN WG1 Meeting #95	R1-1813850

Spokane, USA, November 12th – 16th, 2018

Agenda item:	7.2.6.4

Source: 	ZTE Corporation

Title: 	Use Cases and Simulation Assumptions on 700MHz

Document for:	Discussion/Decision

Motivations to Support for Scenarios with 700MHz

The Rel-16 URLLC requires enhancement for both Duplex Modes. However, the frequency band of 4 GHz is basically for TDD as commonly observed in deployment. For simplicity of evaluation, RAN1 has concluded to simulate UL and DL independently. However, for TDD mode this would be too much simplification. The UL/DL URLLC performance in TDD mode can impact to each other due to the limit that only one transmission direction is allow in one time. In that sense, FDD would be more suitable for those independent evaluations. Also for 700MHz, it is also motivated by operator for deploying even since Rel-15 NR. Another motivation to consider 700 MHz as one of the deployment bands for URLLC is due to its lower coupling loss and good coverage.

After previous meeting discussion for simulation assumption, 4 GHz is explicitly listed as the carrier frequency for all use cases (i.e., transportation industry, power distribution, factory automation and Rel-15 enabled use cases) and 30 GHz can also be applied for factory automation use case [1]. Due to time limit of URLLC topic, the 700MHz frequency band and FDD duplex mode are concluded to be allowed but companies need to report the exact setup of the scenarios.

Still, we should take FDD @ 700MHz for some URLLC evaluation scenarios to achieve comprehensive system level performance evaluations.

In case that some companies are evaluating 700MHz frequency band, one common simulation assumption would be helpful for comparing different enhancement schemes.

Basically, we can refer the simulation assumptions of ITU self-evaluation [2], in which both 4 GHz and 700 MHz are selected for URLLC Urban-Macro scenario.  It will reduce the workload by reusing most of the parameters. The detailed simulation assumptions of ITU self-evaluation are also present in Appendix 6.1.

700 MHz should be included in evaluating Rel-16 URLLC enhancement with the better supporting of real FDD system.

Simulation Assumptions for 700 MHz

Among the four use cases in Rel-16 URLLC evaluation, i.e., (1) power distribution, (2) factory automation, (3) transportation industry and (4) Rel-15 enabled use cases, power distribution, transportation industry and Rel-15 enabled use cases can support outdoor scenarios. The factory automation only supports indoor scenarios.

To reduce the simulation cases, we can consider 700 MHz mainly for outdoor. Urban Macro for 700 MHz is firstly defined for power distribution as we listed in Table 1. The parameters are mostly from the agreed 4GHz power distribution scenario. Antenna configurations of 700 MHz in ITU self-evaluation replace those for 4GHz.

Take the simulation assumptions in Table 1 for power distribution use case in 700 MHz for Rel-16 URLLC simulation.

- Table 1. Simulation assumptions for power distribution in 700 MHz

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m Note: Other value (e.g. 150 m) is not precluded |
| Carrier frequency | 700 MHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23 dBm |
| BS antenna configurations | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5 dB |
| UE antenna configuration | 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) 0°, 90° polarization 2TXRU (1-to-1 mapping) Omni-directional |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 0dBi as starting point |
| UE receiver noise figure | 7 dB |
| Total transmit power per TRxP | 49 dBm |
| BS receiver | MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded. |
| Parameters with the value not defined in 38.802 |  |
| Number of UEs per cell | Up to 10 Note: Example of the number of users can be 5, 10 |
| Simulation bandwidth | 10 MHz [Note1] |
| SCS | 30 KHz [Note2] Note: Other values for evaluation are not precluded. |
| UE distribution | 100% of users are outdoors Use 3km/h for modeling fading channel |
| UE power control | Companies report the PC mechanisms used for URLLC. |
| HARQ/repetition | Companies report (including HARQ mechanisms). |
| Channel estimation | Companies report (including HARQ mechanisms). |

Note1: The original simulation bandwidth for 4 GHz is 40 MHz. 40 MHz bandwidth is relatively large in 700 MHz. It seems more realistic to set the simulation bandwidth to 10 MHz from the operator perspective.

Note2: 15 KHz SCS may not be sufficient to support the strict latency requirements of URLLC applications. Thus, we align the SCS of 700 MHz with the SCS of 4 GHz, i.e., SCS = 30 KHz.

The Urban Macro scenarios for transportation industry and Rel-15 enabled use cases can be with the same modifications on antenna configurations, noise figures and bandwidth when 700 MHz applied.

Regarding to the UE distribution, UE number and UE antenna height, the Urban Macro scenarios for transportation industry and Rel-15 enabled use cases with 700 MHz can reuse the corresponding configuration wtih 4 GHz concluded in last meeting.

Geometry CDF

Overall, all the geometry CDFs in this contribution are based on the simulation assumptions summarized in Section 2 with the following clarification.

Minimum distance of TRxP and UE

In the URLLC simulation assumptions finalized in last meeting, the minimum distance of TRxP and UE has not been explicitly provided in UMa scenarios. To better simulate the real deployment, we configure the minimum distance of TRxP and UE in UMa scenarios as 10m according to [2], i.e., d2D_min=10m.

UE Re-dropping principle

In last discussion of last meeting, companies agreed that UEs may be re-dropped or discarded if they do not satisfy certain channel quality. In our simulation assumptions, the UEs not satisfying certain channel quality are re-dropped. If all the frequency-time domain resource within the target latency requirement are allocated for this UE and it still can’t finish one packet transmission based on the code rate estimated from Geometry SINR.

Take for example: the packet size and air interface latency for power distribution grid fault are 100 bytes and 2 ms, respectively. 2 ms air interface latency corresponds to 4 slots with 30 KHz SCS. If network allocates all the frequency-time resource within 4 slots to this UE, and if this UE still can’t reach its target reliability requirement, then it means this UE needs to be re-dropped.

For better calibration with companies, both the geometry CDF with UE re-dropping and without re-dropping are provided in this contribution.

Downlink

The downlink geometry CDF figures of UMa scenarios in 700 MHz are elaborated in Appendix 6.2 Table 3.

Most of the UEs dropped in AR/VR (UMa) scenario can satisfy above mentioned UE re-dropping principle in downlink, thus the geometry CDF of AR/VR (UMa) scenario without re-dropping is almost the same as that with re-dropping. Similarly, the geometry CDF of power distribution scenario without re-dropping is almost the same as that with re-dropping. While for transportation industry scenario, the latency and reliability requirement are too stringent to satisfy with 10 MHz bandwidth, thus the Q value with re-dropping of transportation industry is much higher than that without re-dropping.

| Scenarios | Q value without re-dropping (dB) | Q value with re-dropping (dB) |
| --- | --- | --- |
| AR/VR (UMa) | -0.71 | -0.51 |
| Power Distribution | -0.58 | -0.04 |
| Transportation Industry | -0.73 | 4.93 |

Uplink

The uplink geometry CDF figures of UMa scenarios in 700 MHz are elaborated in Appendix 6.3 Table 4.

Most of the UEs dropped in AR/VR (UMa) scenario can satisfy above mentioned UE re-dropping principle, thus the geometry CDF of AR/VR (UMa) scenario without re-dropping is almost the same as that with re-dropping. Similarly, the geometry CDF of power distribution scenario without re-dropping is almost the same as that with re-dropping.

| Scenarios | Q value without re-dropping (dB) | Q value with re-dropping (dB) |
| --- | --- | --- |
| AR/VR (UMa) | -1.43 | -1.40 |
| Power Distribution | -0.79 | -0.79 |

Conclusion

To sum up, the following proposals are put forward in this contribution, i.e.,

- 700 MHz should be included in evaluating Rel-16 URLLC enhancement with the better supporting of real FDD system.

- Take the simulation assumptions in Table 1 for power distribution use case in 700 MHz for Rel-16 URLLC simulation.

- In addition, the downlink and uplink geometry CDFs of UMa scenarios in 700 MHz are present in Section 3 for calibration.

References

- “Draft Report of 3GPP TSG RAN WG1 #94bis v0.1.0”, October 2018.

- RP-180524, “Summary of calibration results for IMT-2020 self-evaluation”, Huawei, March 2018.

Appendix

ITU Self-evaluation Simulation Assumptions for URLLC

- Table 2. Baseline parameter for Urban Macro – URLLC [2]

| Urban Macro - URLLC | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 4 GHz | 700 MHz |
| BS antenna height | 25 m | 25 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) |
| Inter-site distance | 500 m | 500 m |
| Number of antenna elements per TRxP | 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) |
| Number of UE antenna elements | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°, 90° polarization | 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) 0°, 90° polarization |
| Number of TXRU per UE | 4TXRU (1-to-1 mapping) | 2TXRU (1-to-1 mapping) |
| Device deployment | 80% outdoor, 20% indoor Randomly and uniformly distributed over the area | 80% outdoor, 20% indoor Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction |
| UE speeds of interest | 3 km/h for indoor and 30 km/h for outdoor | 3 km/h for indoor and 30 km/h for outdoor |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| BS antenna element pattern | See Table 6 in Section 4.6 | See Table 6 in Section 4.6 |
| UE antenna element gain | 0 dBi | 0 dBi |
| UE antenna element pattern | Omni-directional | Omni-directional |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer |
| Simulation bandwidth | 10 MHz | 10 MHz |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | 1.5 m | 1.5 m |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B |
| TRxP number per site | 3 | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | [99°] in LCS | [99°] in LCS |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) | 0 (i.e., the strongest cell is selected) |
| TRxP boresight | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 |

Downlink geometry CDF

- Table 3 Downlink geometry CDF of UMa scenarios in 700 MHz

| Without re-dropping |  |
| --- | --- |
| With re-dropping |  |

Uplink geometry CDF

- Table 4 Uplink geometry CDF of UMa scenarios in 700 MHz

| Without re-dropping |  |
| --- | --- |
| With re-dropping |  |