3GPP TSG RAN WG1 Meeting #95		R1-1813852
Spokane, USA, November 12th – 16th, 2018

Source:	CATT

Title:	Summary of Remaining NR RMSI Issues

Agenda Item:	7.1.1.1

Document for:	Discussion and Decision

# Introduction

This document provides a summary of the remaining RMSI issues and the proposed solutions, which are discussed the contributions submitted to this meeting [1-?].

# Frequency offset between SSB and CORESET0 in multiplexing pattern 1

Background

In R1-1812178, it points out that “For CORESET#0 configuration for SSB/CORESET multiplexing pattern 1 (TDM) in TS38.213, there are certain configurations in which the initial DL BWP (CORESET#0) does not fully contain the SSB in frequency domain. They are identified for some RB offset values when k_SSB > 0, shown in Tables 13-1 through 13-10. For example, as illustrated in Figure 1, for CORESET size of 24 RBs in Table 13-1”.

Figure 1. An example in Table 13-1 where CORESET#0 does not fully contain SSB for multiplexing pattern 1.

Submitted Proposals

It was proposed in the draft CR in R1-1812178 that “In TS38.213 section 13, the value of 4 in Table 13-1 should be changed to “4 if  k_SSB=0, 3 if k_SSB > 0”. In addition, the problematic value of x in any of the tables 13-4, 13-5, 13-6, 13-7, 13-8, 13-9, and 13-10 should be changed to “x if  k_SSB=0, x-1 if k_SSB>0”.

Comments

| Company Name |  | Comments |
| --- | --- | --- |
| CATT | - The frequency offset between SSB and CORESET0 is used for the UE to locate the CORESET0 after the detection of the SSB. Having a fraction of an RB of the SSB not covered by the RMSI has no impact this function. - The configuration with a fraction of an RB of the SSB is not covered by the RMSI CORESET seems not causing any other problem for NR initial access. Thus, for RMSI CORESET multiplexing pattern 1, the last RB of the SSB for some RMSI CORESET configurations may be partially contained within the bandwidth of the RMSI CORESET. These are valid configurations. |
| Samsung | The design principle is to have at least one configuration such that there exists one SSB and the associated CORESET confined in the carrier bandwidth. There is no intention to make every configuration compatible with the floating sync actually. I the example above, there is no issue with the configuration of setting the offset = 4 RB itself, and the issue comes out because the network didn’t choose the configuration correctly (or say choose the SS raster correctly). To be more precise, the network should choose offset = 0 RB to be compatible with k_SSB>0 in this example, such that both SSB and CORESET are confined within the carrier bandwidth, and this configuration is totally feasible since the SS rater is 4 RB. Hence, there is no need to make any change. |

Offline Conclusion

No change is needed for the RMSI requirement.

References

- R1-1812178, 	Remaining issues on downlink signals and channels,  Huawei, HiSilicon