3GPP TSG RAN WG1 Meeting #95	 R1-1813893

Spokane, USA, 12th – 16th November, 2018

Agenda Item:	7.2.2.3.2

Source:	Ericsson

Title:	Feature lead summary for UL Signals and Channels

Document for:	Discussion and decision

# Introduction

This document summarizes the contributions made under the “UL Signals and Channels” agenda item of the Rel-16 study item on NR-based access to unlicensed spectrum. Generally speaking, most of the submitted Tdocs contained similar (if not the same) content as RAN1#94b, since many of the topics under this agenda item were not discussed in the previous two meetings, e.g. PUCCH/PRACH/SRS design.

Please note:

On all of the open issues, I have updated individual company views if the corresponding Tdoc had a proposal. If there was no proposal, I left the company view as it was in the feature lead summary from last meeting. As such, please carefully check that your company view is captured accurately.

Recommendation:

Depending on the time available for offline discussion on this agenda item in Spokane, any of the below issues is open for further discussion to try to narrow down alternatives. However, if consensus cannot be reached, then it will be necessary to simply list the alternatives that have been identified in the SI and capture those in a text proposal for the TR. Please see a starting point for such a text proposal in Section 7. This can be adapted based on any progress that is made in offline discussions during the meeting.

| Area | Issue | Status |
| --- | --- | --- |
| General UL Design Aspects | 2.1 Number interlaces; PRBs/interlace | Proposal |
|  | 2.2 Sub-PRB interlacing | Further offline discussion required - Potential for agreement on prioritizing PRB-based design? |
|  | 2.3 DFT-s-OFDM waveform | Proposal |
|  | 2.4 Interlace design for wideband | Recommend no further discussion until progress is made in frame structure agenda item on single wideband carrier operation |
|  | 2.5 Partial interlace allocation | Offline discussion required (new issue) |
| PUCCH Design | 3.1 Legacy PUCCH formats | Proposal |
|  | 3.2 Enhanced PUCCH formats | Text Proposal for TR |
| PRACH Design | 3.4 Frequency mapping of PRACH sequences | Text Proposal for TR |
|  | 3.5 PRACH numerology | Proposal |
| SRS Design | 3.6 Time domain configuration | Proposal |
|  | 3.7 Frequency Mapping | Further offline discussion required |

# General UL Design Aspects

## Number of interlaces and PRBs per interlace

Description:

The following text appears in the TR [23], based on an agreement made in RAN1#94b:

Within a 20 MHz bandwidth, the following candidate PRB-based interlace designs have been identified where M is the number of interlaces and N is the number of PRBs per interlace in a 20 MHz bandwidth. Where two values are listed for N, it means that some interlaces have one more PRB than others (non-uniform interlace design)

| SCS | M | N |
| --- | --- | --- |
| 15 kHz | 12 | 8 or 9 |
|  | 10 | 10 or 11 |
|  | 8 | 13 or 14 |
| 30 kHz | 6 | 8 or 9 |
|  | 5 | 10 or 11 |
|  | 4 | 12 or 13 |
| 60 kHz | 4 | 6 |
|  | 3 | 8 |
|  | 2 | 12 |
| 60 kHz (assuming 26 PRBs is agreed by RAN4 in a 20 MHz bandwidth) | 4 | 6 or 7 |
|  | 2 | 13 |
|  | 3 | 8 or 9 |

Several sources have expressed their preference for which option to support, and this is captured in the table below. One source proposes removing the options that assume 26 PRBs in a 20 MHz bandwidth due to that RAN4 has not made a decision on this. Some sources have pointed out that N should be chosen to be “friendly” to DFT-s-OFDM implementation. A majority of companies support M = 10/5 and N = {10,11} for 15/30 kHz SCS.

| Company(s) | View/position |
| --- | --- |
| Huawei | For 15 kHz: M = 10, N = {10,11}; for 30 kHz: M = 5, N = {10,11}; and for 60 kHz: M = 2, N = 12 or M = 3, N = {8,9} |
| Intel | N = 12 for 15/30/60 kHz with e.g., M = 8/4/2. Additional REs can be assigned to subset of interlaces. |
| LG | M = 10/5 for 15/30 kHz |
| Nokia | M = 10/5, N = 10 for 15/30 kHz; M = 5, N =10 ½ PRBs for 60 kHz |
| Samsung | For 15 kHz: M = 12, N = {8,9}; for 30 kHz: M = 6, N = {8,9}; for 60 kHz: M = 3, N = 8. |
| Spreadtrum | For 15 kHz: M = 10, N = {10,11}; for 30 kHz: M = 5, N = {10,11}; |
| Ericsson | For 15 kHz: M = 10, N = {10,11}; for 30 kHz: M = 5, N = {10,11} |
| Mediatek | For 15 kHz: M = 10, N = {10,11}; for 30 kHz: M = 5, N = {10,11}; for 60kHz: M = 2, N = 12 |
| OPPO | For 15 kHz: M = 8, N = {13,14}; for 30 kHz: M = 4, N = {12,13}; for 60 kHz: M = 2, N = 12. |
| Lenovo, Motorola | For 15 kHz: M = 10, N = {10,11}; for 30 kHz: M = 5, N = {10,11}; |
| Interdigital | For 15/30/60 kHz: fixed number of PRBs per interlace, e.g., M = 12/6/3, min(N) = 8 |
| vivo | Includes guard band PRB in interlace design and partial interlace is supported. Remove the following text from the TR since RAN4 has not agreed on supporting 26 PRBs in a 20 MHz bandwidth for 60 kHZ SCS - 60 kHz (assuming 26 PRBs is agreed by RAN4 in a 20 MHz bandwidth): - M = 4, N = 6 or 7 - M = 2, N = 13 - M = 3, N = 8 or 9 |

Proposal:

- It has been identified that at least the following PRB-based interlace design is beneficial for NR-U:

  - For 15 kHz: M = 10, N = {10,11}

  - For 30 kHz: M = 5, N = {10,11}

## Sub-PRB Interlacing for 60 kHz SCS

Description:

The following text appears in the TR [23], based on agreements made in RAN1#94 and RAN1#94b:

For sub-PRB block interlace designs, in some scenarios, sub-PRB block interlacing can be beneficial in terms of power boosting. However, the sub-PRB block interlace design has at least the following specification impacts: Reference signal design (e.g., DMRS); Channel estimation aspects; Resource allocation.

Both PRB and sub-PRB interlacing for 60 kHz have been studied. For sub-PRB interlacing the following aspects have been considered:

-	Power boosting potential depending on resource allocation size

-	PUSCH DMRS configuration aspects

-	Channel estimation performance

-	Number of REs per interlace unit

Several sources have provided views on whether or not sub-PRB interlacing should be supported. Some companies argue that sub-PRB interlacing should not be supported due to concerns on specification impact. Some companies argue that it is not needed, since the power boosting potential is available only for small resource allocations. Other companies argue that the specification impact is not a concern. There is no consensus yet: 7 sources are not supportive of sub-PRB interlacing; 4 sources are supportive; and two sources suggest further study is needed.

Alternatives:

- Alt-1: Support sub-PRB interlacing for 60 kHz SCS

- Alt-2: Do no support sub-PRB interlacing

| Company(s) | View/position |
| --- | --- |
| OPPO | Alt-1 |
| Huawei | Alt-2 |
| LG | FFS on Alt-1 vs. Alt-2 |
| Lenovo, Motorola | Alt-2. FFS on Alt-1 |
| Mediatek | Alt-2. Use CDM or TDM instead to achieve power boosting |
| Nokia | Alt-1. # of sub-PRBs per interlace should have multiple of 12 REs |
| Panasonic | Alt-1. Restrictions on what DMRS type depending on sub-PRB size |
| Qualcomm | Alt-1 |
| Samsung | Alt-2 |
| ZTE | Alt-2 |
| Vivo | Alt-2 |
| Ericsson | Alt-2 |
| InterDigital | Alt-2 |
| Intel | Alt-2 |

Proposal:

- Further offline discussion

- Can there be consensus that the TR recommends prioritizing PRB-based interlace design in the WI?

## DFT-s-OFDM waveform

Description:

Several contributions have discussed whether or not DFT-s-OFDM is beneficial for NR-U. Some companies argue that it is needed for coverage and maintenance of lower PAPR/CM. However, other companies point out that the use of an interlace design degrades the PAPR/CM, so the benefits of DFT-s-OFDM are reduced. Some companies have pointed out that the number of PRBs per interlace N needs to be specified to be “friendly” to DFT-s-OFDM, i.e., N should be able to be factored as 2^n1*3^n2*5^n3 where n1, n2, n3 are integers >= 0. While there is no strict consensus, a majority of companies prefer to support DFT-s-OFDM for NR-U.

Alternatives:

- Alt-1: DFT-s-OFDM supported for PUSCH/PUCCH

- Alt-2: Usage of DFT-s-OFDM should be restricted

- Alt-3: CP-OFDM is primary waveform; DFT-s-OFDM is secondary WF option

| Company(s) | View/position |
| --- | --- |
| OPPO | Alt-1 |
| Huawei | Alt-2 |
| LG | Alt-1 |
| NEC | Alt-1 |
| Nokia | Alt-3 |
| Qualcomm | Alt-1 |
| Samsung | Alt-1 |
| Ericsson | Alt-2 considering that PUCCH performance may be negatively impacted due to use of DFT-s-OFDM |
| Mediatek | Alt-2 |
| Panasonic | Alt-1 |
| Lenovo, Motorola | Alt 1 |
| vivo | Alt-2 |
| InterDigital | Alt-1 |

Proposal:

- DFT-s-OFDM is supported for NR-U, but any potential restrictions on its use are to be identified in the WI.

## Interlace design for wideband (> 20 MHz) carriers

Description:

The following text appears in the TR [23], based on an agreement made in RAN1#94b:

For carriers with bandwidth larger than 20 MHz, two candidate interlace designs have been identified:

-	Alt-1: Same interlace spacing for all interlaces regardless of carrier BW. This alternative uses Point A as a reference for the interlace definition

-	Alt-2: Interlacing defined on a sub-band (20 MHz) basis. (Note: Possible interlace spacing discontinuity at edges of sub-band).

Several sources have provided views on which alternative should be supported. For Alt-1, the interlace spacing is the same across the whole carrier. For Alt-2, the interlaces are defined on a per 20 MHz basis with possible discontinuities in the interlace spacing across 20 MHz boundaries. Some sources argue that Alt-1 is beneficial considering users configured with overlapping bandwidth parts. Other sources argue that Alt-2 is beneficial due to the use of sub-band (20 MHz) LBT. This issue has a dependency on the open issue being discussed in the Frame Structure agenda item where 4 options for single carrier wideband operation have been identified for the DL, with further down-selection in RAN1#95. It is FFS on whether some or all of the same options apply to the UL.

Alt-1

Alt-2

Example of 30kHz SCS, 40MHz bandwidth, number of interlaces M=10 (source: R1-1810587)

Alternatives:

- Alt-1: Same interlace spacing for all interlaces regardless of carrier BW

  - Alt-1b: Same number of PRBs per interlace (N) for all interlaces regardless of carrier BW

- Alt-2: Interlacing defined on a sub-band (20 MHz) basis. (Note: Possible interlace spacing discontinuity at edges of sub-band).

| Company(s) | View/position |
| --- | --- |
| Fujitsu | Alt-2 |
| Oppo | Alt-2 |
| Huawei | Alt-1 |
| Interdigital | Alt-2 |
| Nokia | Alt-2 |
| Qualcomm | Alt-1 |
| WILUS | Alt-2 |
| Ericsson | Alt-1 |
| Mediatek | Alt-1 |
| Intel | Alt-1b (For carriers with bandwidth larger than 20 MHz, retain the same number of PRBs per interlace (N) for all interlaces regardless of carrier BW) Reference: R1-1813863 [revised version of R1-1812481] |
| Panasonic | Alt-2 for CA of multiple 20 MHz CCs. FFS for single CC wider than 20 MHz (wait for frame structure agenda). |
| Lenovo/Motorola | Alt-2 |
| LG | Alt-1 |

Proposal:

- Recommend no further discussion until progress has been made on narrowing down options on single carrier wideband operation in the Frame Structure agenda item.

## Partial interlace allocation

Description:

Several sources have highlighted that a user can be allocated a partial interlace. This has not been discussed previously, but is a valid issue in the case where there is a large number of PRBs in an interlace, e.g., within a wideband carrier.

| Company(s) | View/position |
| --- | --- |
| Nokia | - Support almost contiguous resource allocation based on UL Type 0/1. - Interlaces designed for 20 MHz sub-band - Support user multiplexing of wideband transmission based on almost contiguous allocation + 20 MHz interlaced allocation - Support partial interlace allocation |
| LG | Support partial interlace allocation |
| MediaTek | Support partial interlace allocation |
| Panasonic | Support partial interlace allocation based on VRB-based mapping |
| WILUS | Support multiplexing of users onto same interlace for PUCCH (partial interlace allocation) |
| vivo | Inclusion of guard bands in interlaced structure design is needed if RAN4 identifies the need for guardbands (as discussed in LS to RAN4) |

Proposal:

- Offline discussion on partial interlace allocation considering at least the following aspects:

  - Which channels (PUSCH/PUCCH/PRACH/SRS)?

  - Minimum bandwidth allocation?

  - Dependency of CA vs. single wideband carrier operation?

# PUCCH Design

## Supported Legacy PUCCH formats

Description:

The following agreement was made in RAN1#93:

Agreement:

- An interlaced waveform can have benefits in some scenarios including

  - Link budget limited cases with given PSD constraint

  - As one option to efficiently meet the occupied channel bandwidth requirement.

- A waveform contiguous in frequency may be adequate in some scenarios

  - To inherit legacy contiguous allocation designs.

Note: It is RAN1’s understanding that the temporal allowance of not meeting occupied channel bandwidth by regulation can be exploited if the minimum bandwidth requirement, e.g., 2 MHz, is satisfied.

Agreement:

- Support for Rel-15 NR PUCCH formats can be considered. Exclusion of the support of certain formats is to be identified.

  - Note: It is RAN1’s understanding that certain formats do not meet the minimum bandwidth requirement by regulation.

- It is identified that block-interlaced based PUSCH can be beneficial.

- It is beneficial to use the same interlace structure for PUCCH and PUSCH.

- The following aspects can be considered for interlace waveform based PUCCH design:

  - Flexible number of OFDM symbols

  - Flexible payload size

  - User multiplexing

  - Number of formats

As highlighted above, the open issue is that support of certain format to be excluded for unlicensed operation is to be identified. Multiple companies have pointed out that certain existing formats do not satisfy the minimum 2 MHz bandwidth for any numerology since only a single PRB may be configured for these formats (PF0,1, and 4). In this section, the focus is on what legacy formats are to be excluded assuming no modifications/enhancements are made. This is in-line with the above agreement where it is stated that “A waveform contiguous in frequency may be adequate in some scenarios in which case legacy contiguous allocation designs are inherited”. In the next section the focus is on modified/enhanced PUCCH formats considering an interlaced waveform.

Alternatives:

- Alt-1: Support only PF2,3 for unlicensed operation. Restrict the configuration to a sufficient number of PRBs to satisfy the minimum temporal allowance of 2 MHz.

- Alt-2: Support all PUCCH formats

| Company(s) | View/position |
| --- | --- |
| Huawei | Alt-2 |
| Intel | Alt-1 |
| Lenovo, Motorola | Alt-1 |
| Nokia | Alt-1 |
| Qualcomm | Alt-2 |
| Sharp | Alt-1 |
| Spreadtrum | Alt-1 |
| ZTE | Alt-1 |
| Vivo | Alt-1 |
| Ericsson | Alt-1 |
| CATT | Alt-1 |
| MediaTek | Alt-1 |
| Samsung | Alt-1 |
| InterDigital | Alt-1 |

Proposal:

- It has been identified that legacy PUCCH formats PF2 and PF3 are beneficial for NR-U for the scenario of contiguous allocations due to the fact that they may be configured with bandwidth that meets the minimum temporal allowance of 2 MHz (12/6/3 PRBs for 15/30/60 kHz SCS).

- It has been identified that legacy PUCCH formats PF0/1/4 are not well-suited for NR-U for the scenario of contiguous allocations since they support only single PRB.

## Enhanced PUCCH formats

Description:

Multiple companies have provided views on enhancing PUCCH to support a block-interlaced structure. There appears to be reasonable consensus that both short and long PUCCH formats are needed, but there is less consensus about which PUCCH formats should be the starting point for enhancement. Some companies propose that all formats are enhanced, an others propose that 2 or fewer are enhanced.

Several companies have discussed alternatives for the encoding of the UCI payload to be carried by an enhanced PUCCH format. Some companies that propose enhancing legacy PUCCH formats 0/1/4 (single PRB) propose that those formats can be extended to multiple PRBs by repetition of the coded bit. One company suggests applying different cyclic shifts for the different repetitions to control the PAPR. Other companies suggest that a legacy PUCCH format intended for payloads for 3 or more bits (PF2/3/4) can be modified to carry 1 or 2 bit payloads either by zero padding the 1 or 2 bit payloads or by extension of the encoder to small payloads.

For a block-interlaced PUCCH design, there is a loss in multiplexing capacity due to spreading the PUCCH transmission across the band. To regain the lost multiplexing capacity, some form of code division multiplexing is needed in either the frequency domain, time domain, or both. Legacy NR PUCCH formats 1 and 4 support a limited degree of multiplexing. Several companies point out the need to extend this functionality for an enhanced PUCCH design based on block interlacing.

Alternatives:

- Alt-1: Design one enhanced short format and one enhanced long format, e.g., based on PF2/3

- Alt-2: Design single enhanced format with flexible duration

- Alt-3: Enhance three or more legacy NR PUCCH formats (PF0,1,2,3,4)

| Company(s) | View/position |
| --- | --- |
| CATT | Alt-1. Short format based on PF2; long format based on PF3 |
| Huawei | Enhance PF2,3,4. For PF4, repeat single PRB. Apply PRB-specific processing to reduce PAPR. |
| Interdigital | Alt-3. Design of new sequences for PF0/1. PAPR/CM control for the extensions of PF2/3/4; include OCC; ; define a power-efficient format for 2-11 bits and define another format for large payloads |
| LG | Alt-3. Enhance PF 0/1/2/3 |
| Lenovo, Motorola | Alt-1. Enhance PF2/3 |
| MediaTek | Alt-1. Enhance PF2/3 |
| Nokia | Alt-1. Define separate formats for short and long. Short format uses FDM between data and RS. Long format uses TDM between data and RS |
| Panasonic | Alt-3 (PF2, PF3) + PF0 |
| Intel | Alt-1. Extend PF2 and 3 Extend encoder to handle small payloads (1-2 bits) |
| Qualcomm | Alt-3 Enhance PF0/1/2/3/4 For non-sequence-based formats: coding across all RBs For sequence based formats: longer sequence across all RBs |
| Samsung | Alt-1: Short format based on PF2; long format based on PF3 or 4. CS/OCC for user-multiplexing |
| Sharp | Alt-1. Starting point is PF2/3 |
| Spreatrum | Alt-1 |
| WILUS | Alt-3. Enhance PF 0/1/4. |
| ZTE | Alt-3 |
| vivo | Alt-1: Short format based on PF2; long format based on PF3 Extend encoder to handle small payloads (1-2 bits) |
| Ericsson | Alt-2 Extend encoder to handle small payloads (1-2 bits) To support user multiplexing, OCCs for control data, cyclic shifts for RS |
| Interdigital | For PF4, apply OCC’s in both time and frequency across PRBs of an interlace |
| NEC | If enhanced PF0/1 adopted, use bit level or frequency sample level spreading for user multiplexing and PAPR control. Bit level rate matching with larger SF for enhanced PF4. |
| NTT DOCOMO | Alt-3 |

Proposal:

- See Text Proposal in Section 7

# PRACH Design

## Frequency mapping of PRACH sequences

Description:

Four fundamental approaches have been identified for the frequency mapping of PRACH sequences:

- Alt-0: Uniform PRB-level interlace mapping

  - In this approach a PRACH sequence for a particular PRACH occasion is mapped to all of  the PRBs of one or more of the interlaces in the same PRB-based block interlace structure used for PUSCH/PUCCH. Within a PRB, either all or a subset of REs are used. Different PRACH occasions are defined using an orthogonal set of PRBs, or an orthogonal set of REs within the PRBs, from one or more same/different interlaces.

  - It has been identified that a uniform mapping (equal spacing of PRBs) in the frequency domain produces a zero-autocorrelation zone, of which the duration is inversely proportional to the frequency spacing between the PRBs. If the receiver detection window is longer than the zero-autocorrelation zone, accurate time synchronization is not guaranteed.

- Alt-1: Non-uniform PRB-level interlace mapping

  - In this approach a PRACH sequence for a particular PRACH occasion is mapped to some or all of  the PRBs of one or more of the interlaces in the same PRB-based block interlace structure used for PUSCH/PUCCH. Within a PRB, either all or a subset of REs are used. Different PRACH occasions are defined using an orthogonal set of PRBs, or an orthogonal set of REs within the PRBs, from one or more same/different interlaces.

  - It has been identified that an irregular mapping (non-equal spacing of PRBs/REs) in the frequency domain is beneficial to reduce the false peaks in the PRACH preamble auto-correlation function.

- Alt-2: Uniform RE-level interlace mapping

  - In this approach, a PRACH sequence for a particular PRACH occasion consists of a “comb-like” mapping in the frequency domain with equal spacing between all used REs. Different PRACH occasions are defined by way of different comb offsets.

  - Since this approach does not fit with the common PUSCH/PUCCH interlace structure, one source suggests that only TDM multiplexing of PUSCH/PUCCH and PRACH should be supported. Another source suggests that puncturing/rate matching PUSCH/PUCCH around the used PRACH REs may be used.

- Alt-3: Contiguous mapping

  - In this approach, a PRACH sequence for a particular PRACH occasion is mapped to a number of contiguous PRBs as in NR Rel-15.

  - Some sources propose that to fulfill the minimum OCB requirement, that the PRACH sequence is repeated across the frequency domain.

Most companies have identified that the long PRACH sequence length defined in NR-Rel-15 (L = 839) is not useful for NR-U, since it is tailored more toward very large cells. However, when it comes to shorter sequence lengths, some companies wish to reuse the short sequence length (L = 139) defined in NR-Rel-15, whereas other companies suggest defining new sequence lengths depending on which of the 3 alternatives above is supported.

Deciding on which approach to specify should be left to the WI phase; however, the above alternatives can be listed in the TR. Additionally, some companies have studied a number of important aspects that need to be considered in the WI phase including PRACH capacity, max Tx power, PAPR/CM, multiplexing of PUSCH/PUCCH and potential puncturing/rate matching, etc.

Alternatives:

- Alt-0: Uniform PRB-level interlace mapping

- Alt-1: Non-uniform PRB-level interlace mapping

  - Alt-1a: Same as Alt-1, but with non-uniform mapping to a subset of REs within a PRB

- Alt-2: Uniform RE-level interlace mapping

- Alt-3: Contiguous mapping

  - Alt-3a: Same as Alt-3, but with repetition in the frequency domain

| Company(s) | View/position |
| --- | --- |
| Apple | Alt-2 TDM mux of PRACH with interlaced PUSCH/PUCCH |
| CATT | Support legacy PRACH sequence length L = 139 |
| OPPO | Alt-3/3a |
| Huawei | Alt-0/1/1a Allow new PRACH sequence lengths |
| Intel | Alt-3a. Repetitions are cyclically shifted and/or phase rotated. PUSCH/PUCCH rate matched/punctured on PRBs occupied by PRACH Support only legacy PRACH sequence length L = 139 Alt-3 when temporal 2 MHz BW is allowed by regulation. |
| Interdigital | Alt-2/Alt-3 |
| LG | Alt-3a / Alt-0 |
| DOCOMO | Alt-3 / Alt-2 Support new PRACH sequence lengths |
| Nokia | Alt-3/Alt-3a |
| Panasonic | Alt-1 Support legacy PRACH sequence length L = 139 |
| Samsung | Alt-3 TDM mux of PRACH with interlaced PUSCH/PUCCH Support legacy PRACH sequence length L = 139 |
| ZTE | Alt-3a |
| vivo | Alt-3 / Alt-3a |
| Ericsson | Alt-1 FDM mux of interlaced PRACH with interlaced PUSCH/PUCCH (common interlace structure) Support legacy PRACH sequence length L = 139 |
| Qualcomm | Not clear which option |
| WILUS | FDM mux of interlaced PRACH with interlaced PUSCH/PUCCH (common interlace structure) |
| MediaTek | Alt-3 |
| Fujitsu | Support only legacy PRACH sequence length L = 139 |

Proposal:

- See Text Proposal in Section 7

## PRACH Numerology

Description:

In legacy NR for FR1, 15 and 30 kHz SCS are supported for PRACH, whereas 60 and 120 kHz are supported for FR2. Several companies have provided views on what PRACH numerologies should be supported for NR-U in their contributions.

Alternatives:

- Alt-1: 15 kHz

- Alt-2: 30 kHz

- Alt-3: 60 kHz

| Company(s) | View/position |
| --- | --- |
| Nokia | Alt-1, 2, 3 |
| Panasonic | Alt-1, 2, 3 |
| Samsung | Alt- 2 |
| Ericsson | Alt-1, 2 |
| MediaTek | Alt- 2, 3 |
| NTT DOCOMO | Alt-1, 2, 3 |
| Huawei | Alt-1, 2, 3 |
| CATT | Alt-1, 2 |
| InterDigital | Alt-1, 2, 3 |
| Intel | Alt-1, 2 |
| LG | Alt-1, 2 |
| vivo | Alt-1, 2 |

Proposal:

- For NR-U operation in FR1, it has been identified that use of 30 kHz for PRACH is beneficial. Some sources also support 15 kHz and others support 60 kHz.

# SRS Design

Several companies provided views on SRS design, but compared to PRACH and PUCCH there was much less discussion. The main issues that have been discussed are the configuration of the SRS bandwidth of SRS, the time domain properties (periodic/aperiodic), and the location of an SRS resource within a slot. Very few companies have commented on a very basic element of SRS design for NR-U: interlacing vs. contiguous. Some companies have proposed that only wideband SRS (full CC/BWP) is supported; however, it is unclear if this is in the context of an interlaced design or a contiguous design. Further discussion is needed on these points.

## Time domain configuration

Description:

Several companies have pointed out that aperiodic SRS is suitable for NR-U operation due to uncertainties on when the channel is available thus making periodic and semi-persistent SRS less useful. However, since all three types (aperiodic, periodic, and semi-persistent) are supported in NR Rel-15, it does not seem necessary to capture an agreement in the TR. If ap-SRS is beneficial, an implementation can ensure that only this type is configured.

It has also been identified that for operation in unlicensed spectrum, it is beneficial to have more flexibility in which OFDM symbols the SRS may be configure, i.e., not constrained to only the last 6 symbols of the slot. This can help reduce/eliminate potential gaps between LBT and UL transmissions (PUSCH/PUCCH/SRS).

Alternatives:

- Alt-1: Support additional starting symbols in slot for SRS resource

| Company(s) | View/position |
| --- | --- |
| CATT | Alt-1 |
| Intel | Alt-1 ap-SRS only |
| Nokia | Alt-1 |
| Qualcomm | Alt-1 No restrictions on p/sp/ap-SRS |
| Samsung | Alt-1 TDM multiplexing between SRS and other channels Further study of p/sp/ap-SRS. |
| ZTE | Alt-1 ap-SRS suitable for NR-U |
| Ericsson | Alt-1 SRS ap-SRS beneficial for NR-U SRS on same common interlace structure used for PUSCH/PUCCH |
| Spreadtrum | ap-SRS beneficial |
| vivo | ap-SRS beneficial |

Proposal:

- It has been identified as beneficial for NR-U to introduce flexibility in configuring/triggering SRS to be transmitted in OFDM symbols other than the last 6 symbols of a slot.

## SRS frequency mapping

Description:

A few sources have provided views on frequency mapping of SRS. Three companies have proposed that an interlaced waveform is introduced for SRS  to allow multiplexing with other channels. One company suggests that a minimum SRS bandwidth is introduced to satisfy temporal minimum occupied channel bandwidth regulations. Two companies propose support for legacy NR Rel-15 SRS design. Insufficient discussion has occurred so far in order to make a concrete proposal.

| Company(s) | View/position |
| --- | --- |
| Nokia | Allow FDM multiplexing of SRS with PUSCH/PUCCH – interlaced waveform for SRS? |
| Intel | For 15/30/60 kHz SCS, minimum sounding BW is 12/8/4 PRBs Support Rel-15 legacy NR SRS as the baseline For wideband SRS, support multiple SRS resource configurations in the frequency domain. UE performs LBT on each unit 20 MHz bandwidth within the sounding band and dynamically adapts SRS transmission bandwidth depending on LBT outcome |
| Samsung | Support legacy Rel-15 SRS mapping |
| Spreadtrum | Wideband and 20 MHz sub-band supported |
| vivo | Support interlaced waveform |
| Ericsson | Support interlaced waveform |

Proposal:

- Further offline discussion required on frequency mapping of SRS

# References

[1]	R1-1812193	UL PHY channels for NR unlicensed	Huawei, HiSilicon

[2]	R1-1812299	Discussion on physical UL channel design in unlicensed spectrum	vivo

[3]	R1-1812355	UL Signals and Channels for NR-U operation	MediaTek Inc.

[4]	R1-1812404	On UL signals and channels design for NR-U	Fujitsu

[5]	R1-1812434	Considerations on UL Reference Signals and Channels Design for NR-U	ZTE

[6]	R1-1812481	Enhancements to NR UL signals and channels for unlicensed operation	Intel Corporation

[7]	R1-1812522	NR-U UL channels	Panasonic Corporation

[8]	R1-1812559	Physical layer design of UL signals and channels for NR unlicensed operation	LG Electronics

[9]	R1-1812653	Discussion on UL Signals and Channels in NR-U	NEC

[10]	R1-1812660	On uplink signal and channel structures for NR-U	Nokia, Nokia Shanghai Bell

[11]	R1-1812780	UL signals and channels for NR-U	Lenovo, Motorola Mobility

[12]	R1-1812800	Considerations on UL signals and channels for NR-U	OPPO

[13]	R1-1812915	On UL signals and channels for NR-U	Apple Inc.

[14]	R1-1812976	Uplink signal and channel design for NR-U	Samsung

[15]	R1-1813061	Discussion on UL signals and channels in NR-U	Spreadtrum Communications

[16]	R1-1813205	UL channel and signal design for NR unlicensed operation	Sharp

[17]	R1-1813219	Design aspects of NR-U uplink signals and channels 	InterDigital, Inc.

[18]	R1-1813280	UL Physical Channel and Signal Design for NR Unlicensed Operations	CATT

[19]	R1-1813312	UL Signals and Channels for NR-U operation	NTT DOCOMO, INC.

[20]	R1-1813412	UL signals and channels for NR-U	Qualcomm Incorporated

[21]	R1-1813457	On UL signals and channels	Ericsson

[22]	R1-1813533	UL signals and channels for NR-U operation	WILUS Inc.

[23]	3GPP TR 38.889, “Study on NR-based Access to Unlicensed Spectrum,” V0.2.0, October 2018.

# Text Proposal for TR

>>> Begin Text Proposal for Section 7.2.1.2 of 38.889 >>>

<<< Text omitted >>>

#### 7.2.1.2	Physical layer channel designs

For physical layer channel design, NR design will be used as baseline, and the following potential design changes are to be studied to support the following channels/signals in NR-U.

-	PDCCH/PDSCH

-	PUCCH/PUSCH

-	PSS/SSS/PBCH

-	PRACH

-	DL and UL reference signals applicable to the operational frequency range

For SS/PBCH block transmission, extended CP is not supported for NR-U operation. [Editor notes: This is a working assumption and will be confirmed if there is no issue identified in terms of coverage and delay spread]

For PSS/SSS/PBCH transmission, NR-U should have a signal that contains at least SS/PBCH block burst set transmission. The design of this signal should consider the following characteristics specific to unlicensed band operation:

-	There are no gaps within the time span the signal is transmitted at least within a beam

-	The occupied channel bandwidth is satisfied (although this may not be a requirement)

-	Strive to minimize the channel occupancy time of the signal

-	Characteristics that may facilitate fast channel access

Inclusion of the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) associated with SS/PBCH block(s) in addition to the SS/PBCH burst set in one contiguous burst (tentatively referred to as the NR-U DRS) can be beneficial for

-	Meeting OCB requirement

-	Compacting signals in time domain to limit the required number of channel access and for short channel occupancy

-	Support of stand-alone NR-U deployments

-	Support of automatic neighbour relations (ANR) functionality in an NR-U deployment

-	Resolution of PCI confusion in an NR-U deployment

The transmission of additional signals such as OSI and paging within the NR-U DRS is allowed and can be beneficial.

For UL waveform for PUSCH, PUCCH, and PRACH, it has been identified that an interlaced waveform can have benefits in some scenarios including link budget limited cases with given PSD constraint, and as one option to efficiently meet the occupied channel bandwidth requirement.

On the other hand, it is RAN1's understanding that the temporal allowance of not meeting occupied channel bandwidth by regulation can be exploited if the minimum bandwidth requirement, e.g., 2 MHz, is satisfied. Therefore, a waveform contiguous in frequency may be adequate in some scenarios, which implies that Release 15 NR contiguous allocation designs can be used for NR-U as well.

[Placeholder for text proposal on DFT-s-OFDM depending on agreement(s) in RAN1#95]

Support for Rel-15 NR PUSCH can be considered. However, it has been identified that block-interlaced based PUSCH can be beneficial.

Support for Rel-15 NR PUCCH formats can be considered, however, not necessarily all Release 15 NR PUCCH formats are applicable to NR-U. It is RAN1's understanding that certain formats do not meet the minimum bandwidth requirement by regulation. Exclusion of the support of certain formats is to be identified.

[Placeholder for text proposal on legacy PUCCH formats depending on agreement(s) in RAN1#95]

When new block interlace waveform for PUCCH is to be defined, it is beneficial to use the same block interlace structure for PUCCH and PUSCH. For the block interlace waveform design for PUCCH, the following aspects can be considered:

-	Flexible number of OFDM symbols

-	Flexible payload size

-	User multiplexing

-	Number of formats

It has been identified that enhancement of one or more legacy PUCCH formats is feasible to support block interlaced PUCCH transmission. There is consensus that enhanced PUCCH with both short and long duration is beneficial for NR-U; however, no consensus has been achieved about which legacy PUCCH format(s) should be the starting point for an enhanced PUCCH design. Some sources suggest introducing just one or two new enhanced PUCCH formats, while other sources suggest enhancing all or almost all legacy PUCCH formats (PF0,1,2,3,4). Regardless of which format(s) is(are) chosen as a starting point for enhancement, the following common aspects have been identified as important to consider in the detailed design of the enhanced PUCCH format(s):

  - Flexible number of OFDM symbols

    - Short duration, e.g., 1 or 2 OFDM symbols

    - Long duration, e.g., 4 – 14 OFDM symbols

  - Flexible UCI payload

    - Small payload, e.g., 1 or 2 bit

    - Larger payloads, e.g., > 2 bits

  - Coding of UCI payload, e.g.,

    - Extend legacy (NR Rel-15) PUCCH encoder to handle small payloads

    - Repetition of coded UCI bits across PRBs of an interlace

    - UCI Codebits over all PRBs, i.e. no repetition coding.

  - Number of supported PUCCH formats

  - Support for user multiplexing of both UCI payload and DMRS on an interlace, e.g.,

    - OCCs

    - Cyclic shifts

    - FDM within an interlace

  - Multiplexing method of UCI payload and DMRS, e.g,

    - TDM

    - FDM

  - Mechanism to control PAPR, e.g.,

    - OCC cycling

    - Bit level processing

    - PRB level processing

    - Sequence hopping

  - PUCCH waveform

  - Performance, e.g.,

    - Required SNR to achieve a target BLER

    - Required SNR to achieve target ACK to NACK rate, NACK to ACK rate and DTX to ACK rate.

Several sources provide simulation results on various aspects of enhanced PUCCH design.

Support for Rel-15 NR PRACH formats can be considered, however, not necessarily all Release 15 NR PRACH formats are applicable to NR-U. It is RAN1's understanding that certain formats do not meet the minimum bandwidth requirement by regulation. Exclusion of the support of certain formats is to be identified.

It is identified that interlaced based PRACH can be beneficial. The following aspects can be considered for Interlace waveform based PRACH design for 4-step random access:

-	Interlacing based on PRB or REs

-	Targeted cell sizes

-	Targeted PRACH capacity

-	Targeted false alarm and detection rates

-	Targeted timing estimation accuracy

-	Number of formats

-	Multiplexing with other channels such as block interlaced PUCCH and PUSCH

It has been identified that enhancement of one or more legacy PRACH formats is feasible to support interlaced PRACH transmission. Four potential design alternatives, including no interlacing, have been identified for the frequency mapping of PRACH sequences for NR-U, where consensus on which one(s) to support for NR-U has not yet been achieved:

- Alt-1: Uniform PRB-level interlace mapping

  - In this approach a PRACH sequence for a particular PRACH occasion is mapped to all of  the PRBs of one or more of the interlaces in the same PRB-based block interlace structure used for PUSCH/PUCCH. Within a PRB, either all or a subset of REs are used. Different PRACH occasions are defined using an orthogonal set of PRBs, or an orthogonal set of REs within the PRBs, from one or more same/different interlaces.

  - It has been identified that a uniform mapping (equal spacing of PRBs) in the frequency domain produces a zero-autocorrelation zone, of which the duration is inversely proportional to the frequency spacing between the PRBs. If the receiver detection window is longer than the zero-autocorrelation zone, accurate time synchronization is not guaranteed.

- Alt-2: Non-uniform PRB-level interlace mapping

  - In this approach a PRACH sequence for a particular PRACH occasion is mapped to some or all of  the PRBs of one or more of the interlaces in the same PRB-based block interlace structure used for PUSCH/PUCCH. Within a PRB, either all or a subset of REs are used. Different PRACH occasions are defined using an orthogonal set of PRBs, or an orthogonal set of REs within the PRBs, from one or more same/different interlaces.

  - It has been identified that an irregular mapping (non-equal spacing of PRBs/REs) in the frequency domain is beneficial to reduce the false peaks in the PRACH preamble auto-correlation function.

- Alt-3: Uniform RE-level interlace mapping

  - In this approach, a PRACH sequence for a particular PRACH occasion consists of a “comb-like” mapping in the frequency domain with equal spacing between all used REs. Different PRACH occasions are defined by way of different comb offsets.

  - Since this approach does not fit with the common PUSCH/PUCCH interlace structure, one source suggests that only TDM multiplexing of PUSCH/PUCCH and PRACH should be supported. Another source suggests that puncturing/rate matching PUSCH/PUCCH around the used PRACH REs may be used.

- Alt-4: Contiguous mapping

  - In this approach, a PRACH sequence for a particular PRACH occasion is mapped to a number of contiguous PRBs as in NR Rel-15.

  - Some sources propose that to fulfill the minimum OCB requirement, that the PRACH sequence is repeated across the frequency domain.

Several sources provide simulation results on various aspects of one or more of the above alternatives.

It has been identified that the long PRACH sequence length defined in NR Rel-15 (L = 839) is not beneficial for NR-U, since PRACH formats based on this length are tailored toward large cells not expected in an NR-U deployment. However, when it comes to shorter sequence lengths, some sources propose reusing the short sequence length (L = 139) defined in NR-Rel-15, whereas other sources propose defining new sequence lengths depending on which of the 3 alternatives above is supported.

It has been identified that the following common design attributes need to be considered in the detailed design of an interlaced PRACH waveform for 4-step random access for NR-U:

  - Multiplexing of PRACH and PUSCH/PUCCH, considering block interlaced structure used for PUSCH/PUCCH, e.g.,

    - FDM

    - TDM

  - Supported PRACH sequence and PRACH sequence length(s)

  - PRACH capacity

    - Number of PRACH preambles per cell

    - Number of root sequences

    - Number of cyclic shifts

    - Number of PRACH occasions

  - Maximum supported Tx power

  - PAPR/CM

  - Number of PRACH formats

  - Performance

    - Timing estimation error

    - Miss-detection probability

    - False-detection probability

    - Sensitivity to interference

[Placeholder for text proposal on RACH numerology depending on agreement in RAN1#95]

For scenarios in which a block-interlaced waveform is used for PUCCH/PUSCH, it has been identified that from FDM-based user-multiplexing standpoint it can be beneficial to have UL channels on a common interlace structure, at least for PUSCH, PUCCH, associated DMRS, and potentially PRACH

[Placeholder for text proposal on SRS depending on agreement(s) in RAN1#95]

On the other hand, for scenarios in which a contiguous allocation for PUSCH and PUCCH is used, it is beneficial to use contiguous resource allocation for PRACH

For scenarios in which a block-interlaced waveform is used for UL transmission, a PRB-based block-interlace design has been identified as beneficial at least for 15 and 30 kHz SCS, and potentially for 60 kHz SCS. One identified benefit is better link budget with given PSD constraint. However, it has been observed that power boosting gains decrease with increasing SCS. Another identified benefit is as one option to efficiently meet the occupied channel bandwidth requirement. Compared with sub-PRB interlace design, the PRB-based block-interlace design has comparatively less specification impact.

For sub-PRB block interlace designs, in some scenarios, sub-PRB block interlacing can be beneficial in terms of power boosting. However, the sub-PRB block interlace design has at least the following specification impacts: Reference signal design (e.g., DMRS); Channel estimation aspects; Resource allocation.

Both PRB and sub-PRB interlacing for 60 kHz have been studied. For sub-PRB interlacing the following aspects have been considered:

-	Power boosting potential depending on resource allocation size

-	PUSCH DMRS configuration aspects

-	Channel estimation performance

-	Number of REs per interlace unit

It has been identified as beneficial to support a block-interlaced structure in which the number of interlaces (M) decreases with increasing SCS, and the nominal number of PRBs per interlace (N) is similar for each SCS (in a given bandwidth) at least for 15 and 30 kHz SCS, and potentially 60 kHz depending on supported interlace design.

From a RAN1 perspective it has been identified that supporting a non-uniform interlace structure in which the number of PRBs per interlace is allowed to be different for different interlaces is beneficial from a spectrum utilization point of view. It is up to RAN4 to investigate whether or not the non-uniform interlace structure has an impact on MPR/A-MPR requirements for PUSCH.

Within a 20 MHz bandwidth, the following candidate PRB-based interlace designs have been identified where M is the number of interlaces and N is the number of PRBs per interlace in a 20 MHz bandwidth. Where two values are listed for N, it means that some interlaces have one more PRB than others (non-uniform interlace design)

| SCS | M | N |
| --- | --- | --- |
| 15 kHz | 12 | 8 or 9 |
|  | 10 | 10 or 11 |
|  | 8 | 13 or 14 |
| 30 kHz | 6 | 8 or 9 |
|  | 5 | 10 or 11 |
|  | 4 | 12 or 13 |
| 60 kHz | 4 | 6 |
|  | 3 | 8 |
|  | 2 | 12 |
| 60 kHz (assuming 26 PRBs is agreed by RAN4 in a 20 MHz bandwidth) | 4 | 6 or 7 |
|  | 2 | 13 |
|  | 3 | 8 or 9 |

For carriers with bandwidth larger than 20 MHz, two candidate interlace designs have been identified:

-	Alt-1: Same interlace spacing for all interlaces regardless of carrier BW. This alternative uses Point A as a reference for the interlace definition

-	Alt-2: Interlacing defined on a sub-band (20 MHz) basis. (Note: Possible interlace spacing discontinuity at edges of sub-band).

It has been identified that support of different numerology candidates at least has the following specification impacts:

-	For PRB-based block-interlace design for 15, 30, and 60 kHz SCS, the following spec impacts have been identified: Number of interlaces and number of PRBs per interlace need to be defined; the resource allocation mechanism needs to be defined; channel estimation aspects need to be considered, such as impact on PRG. In addition to the above impact, for sub-PRB-based block-interlace design for 60 kHz SCS, reference signal design (such as DMRS) needs to be revisited and alternative resource allocation mechanism is needed.

-	For NR-U DRS design for 15 and 30 kHz SCS, the SS/PBCH block time domain pattern is already supported in Rel-15. For 60 kHz SCS, there is no SS/PBCH block time domain pattern defined in Rel-15. SS/PBCH block to CORESET configuration tables (38.213 Section 13) need to be defined as well.

#### 7.2.1.3	Physical layer procedure designs

<<< Text omitted >>>

>>> End Text Proposal >>>