# 3GPP TSG RAN WG1 Meeting #95	      			                           R1-1814067
Spokane, US, 12th - 16th Nov. 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE procedure for transmitting the physical uplink shared channel in 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UE behaviour is ambiguous when a UE receives a DCI format 0_1 with “UL-SCH indicator” set to “0”, and with a non-zero “CSI request” where associated “reportQuantity” in ReportConfig set to “none”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that a UE does not transmit the PUSCH scheduled by a DCI format 0_1 with “UL-SCH indicator” set to “0”, and with a non-zero “CSI request” where associated “reportQuantity” in ReportConfig set to “none”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behaviour is not clearly defined when a UE receives a DCI format 0_1 with “UL-SCH indicator” set to “0”, and with a non-zero “CSI request” where associated “reportQuantity” in ReportConfig set to “none”. With this ambiguity, the feature of eNB issue an UL grant to trigger UE to only monitor A-TRS for tracking loops or monitor A-CSI-RS for P3 beam refinement is disallowed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated impact analysis This is a backward compatible CR. The behavior of UEs compliant with the 15.3.0 version of the specification in this case is not clearly defined, although a reasonable UE is like to adopt the behavior compliant with this CR. The CR clarifies/specified the UE behavior for this case. |  |  |  |  |  |  |  |  |

## 6.1	UE procedure for transmitting the physical uplink shared channel

PUSCH transmission(s) can be dynamically scheduled by an UL grant in a DCI, or semi-statically configured to operate according to Subclause 6.1.2.3 and according to Subclause 5.8.2 of [10, TS 38.321] upon the reception of higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant without the detection of an UL grant in a DCI, or configurdGrantConfig not including rrc-ConfiguredUplinkGrant semi-persistently scheduled by an UL grant in a DCI after the reception of higher layer parameter configurdGrantConfig not including rrc-ConfiguredUplinkGrant.

A UE shall upon detection of a PDCCH with a configured DCI format 0_0 or 0_1 transmit the corresponding PUSCH as indicated by that DCI. Upon detection of a DCI format 0_1 with “UL-SCH indicator” set to “0” and with a non-zero “CSI request” where the associated “reportQuantity” in CSI-ReportConfig set to “none” for all CSI report(s) triggered by “CSI request” in this DCI format 0_1, the UE ignores all fields in this DCI except the “CSI request” and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1. For any two HARQ process IDs in a given cell, if the UE is scheduled to start a PUSCH transmission in symbol j by a PDCCH in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than symbol j by a PDCCH starting later than symbol i.

For PUSCH scheduled by DCI format 0_0 on a cell, the UE shall transmit PUSCH according to the spatial relation, if applicable, corresponding to the PUCCH resource with the lowest ID within the active UL BWP of the cell, as described in sub-clause 9.2.1 of [6, TS 38.213].

For uplink, 16 HARQ processes per cell is supported by the UE.