3GPP TSG RAN WG1 Meeting #95				                          R1-1814136

Spokane, USA, Nov. 12th - 16th, 2018

Source:                ZTE

Title:                    Summary of performance evaluation for NOMA

Agenda item:      7.2.1.4

Document for:    Discussion and Decision

# Background

In RAN1#94bis meeting and the post-meeting email discussion, the following observations on LLS were agreed [1].

The following observations have be drawn from the case-by-case evaluation results under “ideal conditions” which means equal SNR, zero TO/FO and fixed MA signature allocation:

- with ideal channel estimation, the LLS results for Case 1 with 12 or 24 UEs show a similar performance for most of curves provided, at target BLER = 0.1, with appropriate configurations.

- with ideal channel estimation, the LLS results for Case 2 with 6 or 12 UEs show a similar performance for most of curves provided for coding rates no more than 0.2, at target BLER = 0.1.

- with ideal channel estimation, the LLS results for Case 2 with 6 or 12 UEs show a similar performance for most of curves provided for coding rate ~0.4, at target BLER = 0.1.

- for Case 3 with 10 UEs and ideal channel estimation,

  - the LLS results for simulated schemes with the Chip EPA hybrid PIC or MMSE-hybrid IC receiver or ESE-SISO receiver show a similar performance for most of curves provided with code rate up to 0.4, at target BLER = 0.1.

  - the LLS results for simulated schemes with the MMSE-hard IC receiver show a similar performance for most of curves provided with code rate up to 0.4, at target BLER = 0.1.

  - the LLS results with the Chip EPA hybrid PIC or MMSE-hybrid IC receiver or ESE-SISO receiver show better performance than the results with the MMSE-hard IC receiver.

- for Case 4 with 6 UEs and ideal channel estimation,

  - the LLS results for simulated schemes with the Chip EPA hybrid PIC or MMSE-hybrid IC receiver show a similar performance for most of curves provided with code rate up to 0.6, at target BLER = 0.1.

  - the LLS results for simulated schemes with the MMSE-hard IC receiver or ESE-SISO receiver show a similar performance for most of curves provided with code rate up to 0.6, at target BLER = 0.1.

  - the LLS results with the Chip EPA hybrid PIC or MMSE-hybrid IC receiver show better performance than the results with the MMSE-hard IC receiver or ESE-SISO receiver.

- for Case 5 with 4 UEs and ideal channel estimation,

  - when the code rate is similar, the LLS results for simulated schemes with the Chip EPA hybrid PIC or MMSE-hybrid IC receiver show a similar performance for most of curves provided, at target BLER = 0.1.

  - the LLS results for simulated schemes with the MMSE-hard IC receiver and ESE-SISO receiver show a similar performance for most of curves provided, at target BLER = 0.1.

  - When the code rate is round 0.36, the LLS results with the Chip EPA hybrid PIC or MMSE-hybrid IC receiver show better performance than the results with the MMSE-hard IC receiver or ESE-SISO receiver.

  - When the code rate is round 0.71, the LLS results with the Chip EPA hybrid PIC or MMSE-hybrid IC receiver show similar performance to the results with the MMSE-hard IC receiver or ESE-SISO receiver

- for Case 5 with 6 UEs and ideal channel estimation,

  - the LLS results for linear-spreading based schemes (SF>1) with the MMSE-hard IC receiver show a similar performance, at target BLER = 0.1.

- Under ideal conditions for Case 18 with 12 or 24 UEs, the LLS results for simulated schemes show a similar performance for most of curves provided, at target BLER = 0.1, with appropriate configurations.

- Under ideal conditions for Case 19 with 8 UEs, the LLS results for simulated schemes show a similar performance for most of curves provided, at target BLER = 0.1, with appropriate configurations.

- Under ideal conditions for Case 19 with 16 UEs, the LLS results for simulated schemes show a similar performance for most of curves provided for coding rates no more than 0.4, at target BLER = 0.1.

- Under ideal conditions for Case 20 with 4 or 8 UEs,

  - the LLS results for simulated schemes with the code rate round 0.35 show a similar performance for most of curves, at target BLER = 0.1.

  - the LLS results for simulated schemes with the code rate round 0.7 show a similar performance for most of curves, at target BLER = 0.1.

  - the LLS results with the code rate round 0.35 show better performance than the results with the code rate round 0.7.

- Under ideal conditions for Case 14 with 6 or 12 UEs, the LLS results for simulated schemes show a similar performance for most of curves provided, at target BLER = 0.001, with appropriate configurations.

- Under ideal conditions for Case 15 with 4 or 6 UEs,

  - the LLS results for simulated schemes with the code rate round 0.3 show a similar performance for most of curves, at target BLER = 0.001.

  - the LLS results for simulated schemes with the code rate round 0.6 show a similar performance for most of curves, at target BLER = 0.001.

  - the LLS results with the code rate round 0.3 show better performance than the results with the code rate round 0.6.

- Under ideal conditions for Case 16 with 6 or 12 UEs, the LLS results for simulated schemes show a similar performance for most of curves provided, at target BLER = 0.001, with appropriate configurations.

- Under ideal conditions for Case 17 with 4 or 6 UEs,

  - the LLS results for simulated schemes with the code rate round 0.3 show a similar performance for most of curves, at target BLER = 0.001.

  - the LLS results for simulated schemes with the code rate round 0.6 show a similar performance for most of curves, at target BLER = 0.001.

  - the LLS results with the code rate round 0.3 show better performance than the results with the code rate round 0.6.

- Under ideal conditions for Case 26 with 6 or 8 UEs,

  - the LLS results for simulated schemes with the code rate round 0.3 show a similar performance for most of curves, at target BLER = 0.1.

  - the LLS results for simulated schemes with the code rate round 0.6 show a similar performance for most of curves, at target BLER = 0.1.

  - the LLS results with the code rate round 0.3 show better performance than the results with the code rate round 0.6.

- Under ideal conditions for Case 27 with 4 UEs,

  - the LLS results for simulated schemes with the code rate round 0.35 show a similar performance for most of curves, at target BLER = 0.1.

  - the LLS results for simulated schemes with the code rate round 0.7 show a similar performance for most of curves, at target BLER = 0.1.

  - the LLS results with the code rate round 0.35 show better performance than the results with the code rate round 0.7.

- Under ideal conditions for Case 27 with 6 UEs,

  - the LLS results for simulated schemes with the code rate round 0.35 show a similar performance for most of curves, at target BLER = 0.1.

  - the LLS results for simulated schemes with the code rate round 0.7 show a similar performance for most of curves, at target BLER = 0.1.

  - the LLS results with the code rate round 0.35 show better performance than the results with the code rate round 0.7.

The following observations can be drawn from the case-by-case evaluation results under unequal SNR distributions:

- For Case 6 with 6 or 12 UEs, ideal channel estimation, unequal SNR (SNR offset is within +/-3dB), TO is within [0, 0.5*NCP], and non-zero FO, the LLS results for simulated schemes show a similar performance for most of curves provided, at target BLER = 0.1, with appropriate configurations.

- For Case 8 with 12 or 24 UEs, ideal channel estimation, unequal SNR (SNR offset is within +/-3dB), TO is within [0, 0.5*NCP], and non-zero FO, the LLS results for simulated schemes show a similar performance for most of curves provided, at target BLER = 0.1, with appropriate configurations.

- For Case 9 with 6 or 12 UEs, ideal channel estimation, unequal SNR (SNR offset is within +/-3dB), TO is within [0, 0.5*NCP], and non-zero FO, the LLS results for simulated schemes show a similar performance for most of curves provided, at target BLER = 0.1, with appropriate configurations.

- For Case 21 with 12 or 24 UEs, ideal channel estimation, unequal SNR (SNR offset is within +/-3dB), TO is within [0, 0.5*NCP], and non-zero FO, the LLS results for simulated schemes show a similar performance for most of curves provided, at target BLER = 0.1, with appropriate configurations.

Some general observations can be drawn from the simulated cases:

- For LLS in mMTC/eMBB/URLLC scenarios with ideal channel estimation, equal SNR, zero TO/FO. and fixed MA signature allocation

  - For low TBS (per UE SE is less than 0.15 bps/Hz and total SE is less than 1.8 bps/Hz), as long as the simulation configuration, e.g., reasonable code rate, is appropriate, the performance difference between NOMA schemes/MA signatures is small, even when different receiver types are used.

- For LLS in the simulated case 15/17/19/20/26/27 with ideal channel estimation, equal SNR, zero TO/FO and fixed MA signature allocation

  - For medium to high TBS (per UE SE is within [0.3, 0.55] bps/Hz, and total SE is less than 3.6 bps/Hz), as long as the simulation configuration, e.g., reasonable code rate, is appropriate, the performance difference between NOMA schemes/MA signatures is small.

    - Results with lower code rate (e.g. LDPC coding rate < 0.5) show better performance than the results with higher code rate (e.g. LDPC coding rate > 0.5).

By comparing the evaluation results between realistic channel estimation and ideal channel estimation based on cases 1~5 and 14~20, the following observations are made.

- For LLS with realistic channel estimation, equal SNR distribution, zero TO/FO and fixed MA signature allocation

  - Up to [2-4]dB performance degradation is observed compared to ideal channel estimation for mMTC/eMBB scenario.

  - Up to [5]dB performance degradation is observed compared to ideal channel estimation for URLLC scenario.

  - Different performance degradation levels may be due to different channel estimation algorithm and DMRS extension methods.

  - The lower the SNR operation point is, the larger the performance degradation due to realistic CE can be observed

  - Higher number of UEs have larger performance degradation than lower number of UEs under the same channel condition and the same TBS for each individual case.

There are some remaining issues for both the link-level and system-level simulation results, which are summarized in this contribution and to be discussed in RAN1#95 meeting.

# Remaining issues for the Link level simulation results

## For unequal SNR transmission

### Observations for the newly added cases 32-35

- Case 32, 12 UEs

| Company | ZTE |  |  |  | CATT |  | DOCOMO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scheme | MUSA-SIC-5dB | MUSA-EPA-5dB | MUSA-SIC-9dB | MUSA-EPA-9dB | PDMA | PDMA | UGMA |
| Code rate | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| Receiver | MMSE- hard IC | EPA | MMSE- hard IC | EPA | EPA | MMSE- hard SIC | MMSE- hard SIC |
| Unequal distribution | Option 1 | Option 1 | Option 2 | Option 2 | Option 1 | Option 1 | Option 4 |

- Case 33, 8 UEs

| Company | ZTE |  |  |  | CATT |  | DOCOMO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scheme | MUSA-SIC-4dB | MUSA-EPA-4dB | MUSA-SIC-9dB | MUSA-EPA-9dB | PDMA | PDMA | UGMA |
| Code rate | 0.43 | 0.43 | 0.57 | 0.57 | 0.57 | 0.57 | 0.55 |
| Receiver | MMSE- hard IC | EPA | MMSE- hard IC | EPA | EPA | MMSE- hard SIC | MMSE- hard SIC |
| Unequal distribution | Option 1 | Option 1 | Option 2 | Option 2 | Option 1 | Option 1 | Option 4 |

- Case 34, 8 UEs

| Company | ZTE |  |  |  | CATT |  |
| --- | --- | --- | --- | --- | --- | --- |
| Scheme | MUSA-SIC-4dB | MUSA-EPA-4dB | MUSA-SIC-9dB | MUSA-EPA-9dB | PDMA | PDMA |
| Code rate | 0.29 | 0.29 | 0.29 | 0.29 | 0.57 | 0.57 |
| Receiver | MMSE- hard IC | EPA | MMSE- hard IC | EPA | EPA | MMSE- hard SIC |
| Unequal distribution | Option 1 | Option 1 | Option 2 | Option 2 | Option 1 | Option 1 |

- Case 35, 12 UEs

| Company | ZTE |  |  |  | CATT |  | DOCOMO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scheme | MUSA-SIC-5dB | MUSA-EPA-5dB | MUSA-SIC-9dB | MUSA-EPA-9dB | PDMA | PDMA | UGMA |
| Code rate | 0.1 | 0.2 | 0.1 | 0.2 | 0.2 | 0.2 | 0.19 |
| Receiver | MMSE- hard IC | EPA | MMSE- hard IC | EPA | EPA | MMSE- hard SIC | MMSE- hard SIC |
| Unequal distribution | Option 1 | Option 1 | Option 2 | Option 2 | Option 1 | Option 1 | Option 4 |

For the curves with the same SNR distribution, the performance of different schemes are quite similar.

Significant performance loss can be observed when the SNR deviation is enlarged, e.g. from 5dB to 9dB (or using a mixed Gaussian and deterministic distribution). This is reasonable because there are more UEs suffering very low SNR and thus limit the overall performance. It is not necessary to say the performance loss is due to NOMA operation, since even for single UE randomly selected SNR from ±9dB in each simulation drop, the performance loss can be observed.

### General observations for the unequal SNR distribution

During the email discussion, some individual observations have been made for the typical cases with unequal SNR distribution. There are still some parts such as case 22 needs further checks. And we were waiting for the results for the newly added cases 32-35. So we leave the general observation to be further discussed in RAN1#95 meeting.

Actually if we look at the results submitted for Case 22, most of the curves are quite aligned with less than 1 dB deviation.

- Case 22, 8UEs

- Case 22, 16UEs

If still there are different views for some of the unequal cases, an alternative solution is to explicitly list the cases corresponding to the observations, similar as what we have done for the general observation for equal SNR distribution.

  - For LLS in the simulated cases 32/33/34/35 with ideal channel estimation, under unequal SNR, and fixed MA signature allocation, as long as the simulation configuration is appropriate, the performance difference between NOMA schemes/MA signatures is small, even when different receiver types are used

  - Performance loss of 1.1-3.2 dB can be observed with real channel estimation  in multipath, where the losses are greater for the larger  number of UEs or with greater SNR variation with for link level simulations in the simulated cases of 32 and 33.

To compare BLER results with realistic channel estimation vs. ideal channel estimation for Case 32 and 33.

| Case No. & # of UEs | SNR range for ICE [dB] | SNR range for RCE [dB] | SNR gap @ target BLER |
| --- | --- | --- | --- |
| Case 32, N=6 (mMTC, TDL-A) | [-1.5, 3] | [-0.5, 4] | ~1 dB |
| Case 32, N=12 (mMTC, TDL-A) | [-1, 3] | [0.5, 5] | 1.5~3dB |
| Case 33, N=6 (mMTC, TDL-C) | [2.5, 8] | [3.5, 11] | 1~3dB |
| Case 33, N=8 (mMTC, TDL-C) | [3.5, 9.5] | [5, 12.5] | 1.5~3dB |

## For asynchronous transmissions

No conclusion for the simulated cases with asynchronous transmission was made during the email discussion after RAN1#94bis, since there was not enough results collected for the asynchronous cases.

There are more results submitted to RAN1#95, including the following:

- Case 9, 12 UEs, fixed, ICE

ICE

RCE

| Company | ZTE | HW |  |  | Qualcomm |  | Intel |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scheme | MUSA-SIC-5dB | SCMA | MUSA | LCRS | RSMA | RSMA | LCRS |
| Code rate | 0.2 | 0.2 | 0.2 | 0.1 | 0.2 | 0.2 | 0.1 |
| Receiver | MMSE- hard IC | EPA | EPA | EPA | MMSE- hard SIC | MMSE- hard SIC | MMSE- hard SIC |
| Max TO | 0.5 CP | 0.5 CP | 0.5 CP | 0.5 CP | 1.5 CP | 1.5 CP | 0.5 CP |

- Case 11, 6UEs, random, ICE

| Company | HW |  |  |  | CATT |  | DOCOMO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scheme | SCMA | LCRS | MUSA | SCMA | PDMA | PDMA | UGMA | MUSA | SCMA |
| Code rate | 0.2 | 0.1 | 0.2 | 0.2 | 0.41 | 0.41 | 0.19 | 0.19 | 0.25 |
| Receiver | EPA | EPA | EPA | MMSE-hard IC | EPA | MMSE-hard IC | MMSE-hard IC | MMSE-hard IC | MMSE-hard IC |
| Max TO | 0.5 CP | 0.5 CP | 0.5 CP | 0.5 CP | 0.5 CP | 0.5 CP | 0.5 CP | 0.5 CP | 0.5 CP |
| MA signature allocation / collision | Random activation / No | Random activation / No | Random activation / No | Random activation / No | Random activation / No | Random activation / No | Random selection / Yes | Random selection / Yes | Random selection / Yes |

- Case 11, 6UEs, random, RCE

| Company | HW |  |  |  |  | Samsung | Samsung | ZTE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scheme | SCMA | LCRS | MUSA | SCMA | MUSA | IGMA | MUSA | MUSA |
| Code rate | 0.2 | 0.1 | 0.2 | 0.2 | 0.2 | 0.18 | 0.37 | 0.17 |
| Receiver | EPA | EPA | EPA | MMSE-hard IC | MMSE-hard IC | MMSE-hard IC | MMSE-hard IC | MMSE-hard IC |
| Max TO | 0.5 CP | 0.5 CP | 0.5 CP | 0.5 CP | 0.5 CP | 1.5 CP | 1.5 CP | 1.5 CP |
| MA signature allocation / collision | Random activation / No | Random activation / No | Random activation / No | Random activation / No | Random activation / No | Random activation / No | Random activation / No | Random selection / Yes / (50% RS overhead) |

Proposed observation 2:

- For case 9 with ideal or realistic channel estimation and timing offset is within [0, 1.5*NCP] (fixed allocation, no MA signature collision, non-zero FO, and SNR offset is within +/-3dB)

  - When the TBS is small (i.e. 20bytes), 12 simultaneous activated UEs can be supported with no significant performance loss at the target BLER, compared to timing offset is within [0, 0.5*NCP].

Note: this observation is drawn based on a single company’s result

- For case 11 with ideal channel estimation and random selection with potential MA signature collision (timing offset is within [0, 0.5*NCP], non-zero FO, and SNR offset is within +/-3dB)

  - When the TBS is small (i.e. 20bytes), 12 simultaneous activated UEs can be supported with no significant performance loss at the target BLER, compared to random activation with no MA signature collision.

- For case 11 with realistic channel estimation and random selection with potential MA signature collision (timing offset is within [0, 1.5*NCP], non-zero FO, and SNR offset is within +/-3dB)

  - When the TBS is small (i.e. 20bytes), with 6 simultaneous activated UEs, based on the realistic UE detection by using 50% RS overhead, with less than 3dB performance loss at the target BLER, compared to random activation with no MA signature collision and timing offset is within [0, 0.5*NCP].

## PAPR

Some contributions provide the PAPR comparison between spreading and non-spreading based schemes. The values for PAPR @ CCDF=1% and 0.1% are summarized in the following figures for CP-OFDM and DFT-s-OFDM respectively.

PAPR for CP-OFDM

| Schemes | Frequency domain spreading, SF=4 (R1-1810206) | Time domain spreading, SF=4 (R1-1810206) | Frequency domain spreading, SF=4 + scrambling (R1-1810206) | Non-spreading (R1-1810206) | Spreading (R1-1811243) | Spreading + scrambling (R1-1811243) | Non-spreading (R1-1811243) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PAPR [dB] @ CCDF=1% | 9.9 | 6.6 | 6.6 | 6.6 | 9.5 | 6 | 6.2 |
| PAPR [dB] @ CCDF=0.1% | 12.2 | 8.4 | 8.5 | 8.4 | 12 | 7.5 | 8 |
| Schemes | Spreading, SF=4 (R1-1810762) | Spreading, SF=4, K=8 (R1-1810762) | Non-spreading, (R1-1810762) | Spreading (R1-1810849) | Spreading + interleaving/ scrambling (R1-1810849) | Non-Spreading (R1-1810849) | Interleaving with zero-padding (R1-1810849) |
| PAPR @ CCDF=1% | 9.8 | 9.5 | 8.1 | 13.5 | 9.8 | 9.8 | 9.5 |
| PAPR [dB] @ CCDF=0.1% | 10.5 | 10.2 | 8.6 |  | 10.8 | 10.8 | 10.4 |
| Schemes | Spreading + scrambling (R1-1811360) | Spreading (R1-1811360) | Spreading, SF=6 (R1-1811101) | Spreading + hopping, SF=6 (R1-1811101) | Non-spreading (R1-1811101) |  |  |
| PAPR @ CCDF=1% | 9.6 | 12 | 12.1 | 9.6 | 9.9 |  |  |
| PAPR [dB] @ CCDF=0.1% |  |  | 13.1 | 10.4 | 10.8 |  |  |

PAPR for DFT-s-OFDM

| Schemes | Non-spreading (R1-1810206) | Time domain spreading, SF=4 (R1-1810206) | Spreading (R1-1811243) | Spreading + scrambling (R1-1811243) | non-spreading (R1-1811243) | Spreading, SF=2 (R1-1810261) | Non-spreading (R1-1810261) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PAPR [dB] @ CCDF=1% | 4.5 | 4.5 | 4.4 | 3.8 | 4.4 | [3.1~5.1] | 4.5 |
| PAPR [dB] @ CCDF=0.1% | 5.8 | 5.8 | 6.7 | 4.8 | 6 | [3.5~6.2] | 5.7 |
| Schemes | Spreading (R1-1810849) | Spreading + interleaving/ scrambling (R1-1810849) | Non-Spreading (R1-1810849) | Interleaving with zero-padding (R1-1810849) |  |  |  |
| PAPR @ CCDF=1% | 8.1 | 7 | 7 | 6 |  |  |  |
| PAPR [dB] @ CCDF=0.1% | 9.2 | 7.5 | 7.5 | 6.4 |  |  |  |
| Schemes | Spreading, SF=2, QPSK (R1-1810120) | Spreading, SF=4, 16QAM (R1-1810120) | Spreading, SF=2, QPSK, 2branches (R1-1810120) | Spreading, SF=2, QPSK, 4branches (R1-1810120) | 16point modulation, SF=4 (R1-1810120) | 16point modulation, SF=4, 2branches (R1-1810120) | Non-spreading, QPSK |
| PAPR @ CCDF=1% | 4.5 | 5.1 | 5.4 | 6 | 4.6 | 4.6 | 4.5 |
| PAPR [dB] @ CCDF=0.1% | 5.7 | 6.4 | 6.7 | 7.5 | 5.9 | 5.9 | 5.7 |

The following figures are based on the collection of companies’ results submitted in the form of ‘template – 3’.

CP-OFDM, QPSK

DFT-s-OFDM, QPSK

In addition, in [R1-1813254] Ericsson pointed out that PAPR/CM should not be considered in isolation, since the PA backoff does not depend only on PAPR/CM, but also on the number of allocated PRBs and on the required transmit power for the transmission scheme.

Proposed observation 3:

- In general, frequency-domain spreading increases the PAPR for CP-OFDM

  - OFDM-symbol level (time-domain) spreading has the same PAPR as non-spreading scheme

  - Sequence hopping can be applied to reduce the PAPR

- Symbol-level scrambling/interleaving can be applied to reduce the PAPR for both CP-OFDM and DFT-s-OFDM

- Sparsity can be beneficial for PAPR reduction for DFT-s-OFDM

- FFS the impact of PAPR/CM on PA backoff, taking into account the required transmit power and the occupied bandwidth for the transmission.

Any comment?

| Company | View |
| --- | --- |

# Remaining issues for the L2S mapping methods

Three different PHY abstraction methods have been agreed during the email discussion and were captured in [2]. One remaining issue is whether or not to capture the modelling for DMRS collision (agreed in principle), based on the further refinement on the wording (especially considering further analysis/evaluations, random activation vs. selection, optionality, etc.).

## Modeling of channel estimation error under DMRS collision

========================= Start of Text Proposal =========================

<Unchanged part omitted>

2.1.3 (Optional) Modeling for DMRS collision

When one DMRS is transmitted by multiple UEs, for example, there are N users, if User 1 and User 2 transmit with the same DMRS (e.g., same DMRS sequence initialization, same DMRS antenna port), , and OFDM symbols, only N-1 users can be identified using the DMRS correlation, and a combined channel  will be obtained from the DMRS in the receiver, where H1 and H2  are the ideal channel estimates of User 1 and User 2, respectively. Further, the channel estimation error  can be modelled based on Eq. (2.1-8) described above, then we can derive the realistic channel estimationand . H12,R and are used to calculate the weight of the linear MMSE receiver, respectively. Finally, the post-processing SINR for PUSCH will be calculated using Eq (2.1-6), where ideal channel is still used for the target signal and the interfering signal.

The rest of the procedure of PHY abstraction is the same as Section 2.1.1.

Note: the above model considers only one-shot transmission (e.g., without HARQ retransmissions) and does not address issues such as UE identification, specific DMRS design, contention resolution details, etc.

<Unchanged part omitted>

========================= End of Text Proposal =========================

## More validation results

It is proposed in R1-1813545 to add more validation results for the L2S mapping method described in section A.2.3.

========================= Start of Text Proposal =========================

<Unchanged part omitted>

A.2.3 Link-to-system mapping for EPA-hybrid IC and MMSE-Hard IC receiver

…

Unequal SNR

With unequal SNR of different SNR variations, the BLER performance comparisons between real UL LLS evaluation with realistic channel estimation and the BLER obtained from the described PHY abstraction method for different TBSs are shown in Figure A.2.3-10, Figure A.2.3-11 and Figure A.2.3-12 respectively. The results with Gaussian power distribution with 9dB variation are presented in Figure A.2.3-13 and Figure A.2.3-14.

…

| 2UE | 6UE | 14UE |
| --- | --- | --- |

Figure A.2.3-13 TB size=20 Bytes, SCMA, Gaussian, 9dB

| 2UE | 4UE | 6UE |
| --- | --- | --- |

Figure A.2.3-14 TB size=60 Bytes, SCMA, Gaussian, 9dB

<Unchanged part omitted>

========================= End of Text Proposal =========================

Possible Proposal 4:

- Agree on the text proposals in section 3, to be captured in the Annex of TR38.812.

Any views or comments?

| Company | View |
| --- | --- |

# Performance evaluation at system level

The following tables summarize the system-level evaluation results as well as the additional assumptions submitted to RAN1#95 meeting from multiple companies.

Table 1 SLS evaluation results for NOMA in mMTC scenario

| Source | ZTE - Case 1 | ZTE - Case 2 | ZTE - Case 3 | ZTE - Case 4 | ZTE - Case 5 | HW - Case 1 | HW - Case 2 | HW - Case 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | 700MHz, 1732m | 700MHz, 1732m | 700MHz, 1732m | 700MHz, 1732m | 700MHz, 1732m | 700MHz, 1732m | 700MHz, 1732m | 700MHz, 1732m |
| Simulation bandwidth | 6 PRBs | 6 PRBs | 6 PRBs | 6 PRBs | 6 PRBs | 6 PRBs | 6 PRBs | 6 PRBs |
| BS antenna number | 2Rx | 2Rx | 2Rx | 2Rx | 2Rx | 2Rx | 2Rx | 2Rx |
| BS downtilt | 92 | 92 | 92 | 92 | 92 | 92 | 92 | 92 |
| Number of UEs per cell | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| UE power control | P0 = -100 dBm, alpha = 1 | P0 = -100 dBm, alpha = 1 | P0 = -95 dBm, alpha = 1 | P0 = -100 dBm, alpha = 1 | P0 = -95 dBm, alpha = 1 | P0=-110 dBm, alpha = 1 | P0=-110 dBm, alpha = 1 | P0=-110 dBm, alpha = 1 |
| HARQ/repetition | Maximum number of HARQ transmissions is 8; non-adaptive re-transmissions | Maximum number of HARQ transmissions is 8; non-adaptive re-transmissions | UEs are classified to 3 groups, different repetitions are assumed respectively | Maximum number of HARQ transmissions is 8 non-adaptive re-transmissions are assumed | 4 repetitions | non-adaptive re-transmissions, maximum number of HARQ transmission=8, repetition dependents on coupling loss, can be 1,2 | non-adaptive re-transmissions, maximum number of HARQ transmission=8, repetition dependents on coupling loss, can be 1,2 | non-adaptive re-transmissions, maximum number of HARQ transmission=8, repetition dependents on coupling loss, can be 1,2 |
| Channel estimation | Realistic | Realistic | Realistic | Realistic | Realistic | Realistic | Realistic | Realistic |
| BS advanced receiver | Baseline: MMSE-IRC or MMSE-PIC MUSA: MMSE-PIC | Baseline: MMSE-IRC or MMSE-PIC MUSA: MMSE-PIC | MMSE-IC for baseline and MUSA | MMSE-IC for baseline and MUSA | MMSE-IC for baseline and MUSA | EPA receiver for all schemes | EPA receiver for all schemes | EPA receiver for all schemes |
| TB size | 25 bytes | 25 bytes | 25 bytes | 25 bytes | 20 bytes | 20 bytes | 60 bytes | 40 bytes |
| Packet dropping criterion | If one TB is not received correctly after HARQ/repetition, the packet is dropped | If one TB is not received correctly after HARQ/repetition, the packet is dropped | If one TB is not received correctly after HARQ/repetition, the packet is dropped | If one TB is not received correctly after HARQ/ repetition, the packet is dropped; | If one TB is not received correctly after HARQ/ repetition, the packet is dropped; | 8 HARQ transmissions or 1s latency | 8 HARQ transmissions or 1s latency | 8 HARQ transmissions or 1s latency |
| DMRS setting and allocation | 24 DMRS each RB assumed, preconfigured, no collision | 24 DMRS each RB assumed, preconfigured, no collision | Baseline: 24 DMRS, random selection MUSA: 64 DMRS, random selection | 24 DMRS each RB assumed, preconfigured, no collision | No DM-RS; Baseline and MUSA: Preamble 64 Random selection | the DMRS pool size is 24 for each resource unit; no DMRS collision | the DMRS pool size is 24 for each resource unit; no DMRS collision | the DMRS pool size is 24 for each resource unit; no DMRS collision |
| MA signature and allocation | 24 spreading codes of length 4 are used for MUSA, preconfigured | 24 spreading codes of length 4 are used for MUSA, preconfigured | 64 spreading codes of length 4 are used for MUSA, random selection | 24 spreading codes of length 2, 4 and 6 are used for MUSA, preconfigured. | 64 spreading codes of length 4 are used, random selection | spreading codes of length 2 are used for MUSA | spreading codes of length 2 are used for MUSA | spreading codes of length 2 are used for MUSA |
| Details on configured grant, e.g. periodicity, offset, and #of UEs assigned on the same resource | periodicity = 4 ms for baseline and MUSA | periodicity = 5 ms for baseline and MUSA | - | periodicity = 6 ms for baseline and MUSA | - | periodicity = 6 ms | periodicity = 6 ms | periodicity = 6 ms |
| Other assumptions for baseline | 1 PRB per Tx for a UE | 6 PRBs per Tx for a UE | 1 PRB per Tx for a UE; 1, 4 and 16 repetitions for the 3 groups respectively | 1 PRB + 6 ms per Tx for a UE | Basic channel structure: 6 PRBs + 1ms for preamble, 6 PRBs + 1ms for data; TO belong to [0, 1.5*NCP] | a resource unit of 1 PRB+6 ms for each TB | a resource unit of 1 PRB+6 ms for each TB | a resource unit of 1 PRB+6 ms for each TB |
| Other assumptions for NOMA | 1 PRB + 4 ms per Tx for a UE; the energy of spreading codes for MUSA is normalized to 1 | 6 PRBs per Tx for a UE | 1 PRB + 4 ms per Tx for a UE; 1, 1 and 4 repetitions for the 3 groups respectively, the energy of spreading codes used by UEs in the first group is normalized to 1 | 1 PRB + 6 ms per Tx for a UE | Basic channel structure: 6 PRBs + 1ms for preamble, 6 PRBs + 1ms for data; TO belong to [0, 1.5*NCP] | a resource unit of 1 PRB+6 ms for each TB; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA | a resource unit of 1 PRB+6 ms for each TB; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA | a resource unit of 1 PRB+6 ms for each TB; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA |
| Supported PAR for baseline at PDR=1% (packet/s/cell) | 300 for baseline with MMSE-IRC; 350 for baseline with MMSE-PIC | <100 for baseline with MMSE-IRC; <100 for baseline with MMSE-PIC | 25 | 400 | <50 for baseline with MMSE-SIC | 590 | 740 | 860 |
| Supported PAR for NOMA at PDR=1% (packet/s/cell) | 600 | 200 | 50 | 700 for SF = 2 900 for SF = 4 1000 for SF = 6 | 100 | 590 for SCMA 580 for MUSA; 580 for SL-RSMA | 740 for SCMA 620 for MUSA; 620 for SL-RSMA 720 for ML-RSMA | 790 for MUSA, 780 for SL-RSMA, 860 for ML-RSMA,860 for SCMA |
| Gain (relative to baseline) | 100% (baseline with MMSE-IRC) 71% (baseline with MMSE-PIC) | 100% | 100% | 75% for SF = 2 125% for SF = 4 150% for SF = 6 | 100% | No gain | No gain | No gain |
| Source | HW - Case 4 | HW - Case 5 | HW - Case 6 | Samsung | Nokia | InterDigital |  |  |
| Carrier frequency | 700MHz, 1732m | 700MHz, 1732m | 700MHz, 1732m | 700MHz, 1732m | 700MHz, 1732m | 700MHz, 1732m |  |  |
| Simulation bandwidth | 6 PRBs | 6 PRBs | 6 PRBs | 6 PRBs | 6 PRBs | 6 PRBs |  |  |
| BS antenna number | 2Rx | 2Rx | 2Rx | 2Rx | 2Rx | 2Rx |  |  |
| BS downtilt | 92 | 92 | 92 | - | 92 | 92 |  |  |
| Number of UEs per cell | 100 | 100 | 100 | 20 | 20 | 20 |  |  |
| UE power control | P0=-110 dBm, alpha = 1 | P0=-110 dBm, alpha = 1 | P0=-110 dBm, alpha = 1 | P0=-90 dBm | P0=-110 dBm, alpha = 1 | P0 = -100 dBm, alpha = 1 |  |  |
| HARQ/repetition | non-adaptive re-transmissions, maximum number of HARQ transmission=8, repetition dependents on coupling loss, can be 1,2 | non-adaptive re-transmissions, maximum number of HARQ transmission=8, repetition dependents on coupling loss, can be 1,2 | non-adaptive re-transmissions, maximum number of HARQ transmission=8, repetition dependents on coupling loss, can be 1,2 | HARQ combining with random back-off. The maximum number for transmissions is 8 | Back off, max 8, no repetition | 8 Chase combining |  |  |
| Channel estimation | Realistic | Realistic | Realistic | Ideal & Realistic | Realistic | Ideal & Realistic |  |  |
| BS advanced receiver | SCMA/ LCRS: chip-wise MMSE hard IC receiver; MUSA and RSMA: block MMSE hard IC receiver | SCMA/ LCRS: chip-wise MMSE hard IC receiver; MUSA and RSMA: block MMSE hard IC receiver | SCMA/ LCRS: chip-wise MMSE hard IC receiver; MUSA and RSMA: block MMSE hard IC receiver | Baseline: MMSE-IRC IGMA: ESE receiver | Baseline: MMSE-IRC or MMSE-SIC NOCA: MMSE-SIC | EPA receiver for IDMA |  |  |
| TB size | 20 bytes | 60 bytes | 40 bytes | 40 bytes | - | 45 bytes (40 + 5 bytes) |  |  |
| Packet dropping criterion | 8 HARQ transmissions or 1s latency | 8 HARQ transmissions or 1s latency | 8 HARQ transmissions or 1s latency | 8 HARQ transmissions | Max 8 transmissions | maximum number of HARQ transmissions |  |  |
| DMRS setting and allocation | the DMRS pool size is 24 for each resource unit; no DMRS collision | the DMRS pool size is 24 for each resource unit; no DMRS collision | the DMRS pool size is 24 for each resource unit; no DMRS collision | DMRS are semi-static configured | - | - |  |  |
| MA signature and allocation | spreading codes of length 2 are used for MUSA | spreading codes of length 2 are used for MUSA | spreading codes of length 2 are used for MUSA | MA signatures are semi-static configured | spreading codes of length 6 are used for NOCA | - |  |  |
| Details on configured grant, e.g. periodicity, offset, and #of UEs assigned on the same resource | periodicity = 6 ms | periodicity = 6 ms | periodicity = 6 ms | - | - | - |  |  |
| Other assumptions for baseline | a resource unit of 1 PRB+6 ms for each TB | a resource unit of 1 PRB+6 ms for each TB | a resource unit of 1 PRB+6 ms for each TB | one RB is allocated for each UE | each UE selects one PRB out of the allocated PRBs; QPSK 1/2 is used for all users | - |  |  |
| Other assumptions for NOMA | a resource unit of 1 PRB+6 ms for each TB; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA | a resource unit of 1 PRB+6 ms for each TB; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA | a resource unit of 1 PRB+6 ms for each TB; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA | all 6 RBs are occupied by each UE | each UE always occupies the whole allocated resources for transmission; QPSK 1/2 is used for all users | - |  |  |
| Supported PAR for baseline at PDR=1% (packet/s/cell) | 580 | 700 | 840 | 400 (ICE) | 100 for baseline with MMSE-IRC or MMSE-SIC (5 packet/s/UE * 20 UE/cell) | - |  |  |
| Supported PAR for NOMA at PDR=1% (packet/s/cell) | 580 for SCMA/MUSA/ML-RSMA/SL-RSMA | 610 for MUSA, 610 for SL-RSMA, 700 for ML-RSMA,700 for SCMA | 780 for MUSA, 770 for SL-RSMA, 840 for ML-RSMA,840 for SCMA | 800 (ICE) 750 (RCE) | 200 (10 packet/s/UE * 20 UE/cell) | 540 (ICE) 270 (RCE) |  |  |
| Gain (relative to baseline) | No gain | No gain | No gain | 100% (ICE) 87.5% (IGMA with RCE relative to baseline with ICE) | 100% | - |  |  |

Table 2 SLS evaluation results for NOMA in URLLC scenario

| Source | ZTE - Case 1 | ZTE - Case 2 | ZTE - Case 3 | ZTE - Case 4 | ZTE - Case 5 | Huawei - Case 1 | Huawei - Case 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | 4GHz, 200m | 700MHz, 500m | 4GHz, 200m | 700MHz, 500m | 4GHz, 200m | 4GHz, 200m | 4GHz, 200m |
| Simulation bandwidth | 12 PRBs, SCS = 60kHz | 12 PRBs, SCS = 60kHz | 12 PRBs, SCS = 60kHz | 12 PRBs, SCS = 60kHz | 12 PRBs, SCS = 60kHz | 12 PRBs, SCS = 60kHz, 7OS | 12 PRBs, SCS = 60kHz, 7OS |
| BS antenna number | 4 Rx | 4 Rx | 4 Rx | 4 Rx | 4 Rx | 4 Rx | 4 Rx |
| BS downtilt | 102 | 98 | 102 | 98 | 102 | 102 | 102 |
| Number of UEs per cell | 20 | 20 | 20 | 20 | 20 | 10 | 10 |
| UE power control | P0 = -90 dBm, alpha = 1 | P0 = -90 dBm, alpha = 1 | P0 = -90 dBm, alpha = 1 | P0 = -90 dBm, alpha = 1 | P0 = -90 dBm, alpha = 1 | P0 = -85 dBm, alpha = 0.93 | P0 = -85 dBm, alpha = 0.93 |
| HARQ/repetition | Number of transmission(s) = 1, no HARQ/ repetition | Number of transmission(s) = 1, no HARQ/ repetition | Number of transmission(s) = 1, no HARQ/ repetition | Number of transmission(s) = 1, no HARQ/ repetition | Number of transmission(s) = 1, no HARQ/repetition | 1 repetition, no retransmission | 2 repetition, no retransmission |
| Channel estimation | Realistic | Realistic | Realistic | Realistic | Realistic | Realistic | Realistic |
| BS advanced receiver | Baseline: MMSE-IRC or MMSE-PIC (2 iterations) MUSA: same as baseline | Baseline: MMSE-IRC or MMSE-PIC (2 iterations) MUSA: same as baseline | Baseline: MMSE-IRC or MMSE-PIC (2 iterations) MUSA: same as baseline | Baseline: MMSE-IRC or MMSE-PIC (2 iterations) MUSA: same as baseline | Baseline: MMSE-IRC or MMSE-PIC (2 iterations) MUSA: same as baseline | EPA receiver for all schemes | EPA receiver for all schemes |
| TB size | 60 bytes | 60 bytes | 200 bytes | 200 bytes | 60 bytes | 60 bytes | 60 bytes |
| Packet dropping criterion | - | - | - | - | - | - | - |
| DMRS setting and allocation | 24 DMRS each RB assumed, preconfigured, no collision | 24 DMRS each RB assumed, preconfigured, no collision | 24 DMRS each RB assumed, preconfigured, no collision | 24 DMRS each RB assumed, preconfigured, no collision | 24 DMRS each RB assumed, preconfigured, no collision | the DMRS pool size is 12, no DMRS collision | the DMRS pool size is 12, no DMRS collision |
| MA signature and allocation | 24 spreading codes of length 4 are used for MUSA, preconfigured | 24 spreading codes of length 4 are used for MUSA, preconfigured | 24 spreading codes of length 4 are used for MUSA, preconfigured | 24 spreading codes of length 4 are used for MUSA, preconfigured | 24 spreading codes of length 4 are used for MUSA, preconfigured | spreading codes of length 2 are used for MUSA | spreading codes of length 2 are used for MUSA |
| Details on configured grant, e.g. periodicity, offset, and #of UEs assigned on the same resource | periodicity = 0.5 ms for baseline and MUSA | periodicity = 0.5 ms for baseline and MUSA | periodicity = 2 ms for baseline and MUSA | periodicity = 2 ms for baseline and MUSA | periodicity = 0.5 ms for baseline and MUSA | 7 OFDM symbols | 7 OFDM symbols |
| Other assumptions for baseline | 3 PRBs per Tx for a UE | 3 PRBs per Tx for a UE | 3 PRBs per Tx for a UE | 3 PRBs per Tx for a UE | 12 PRBs + 0.25ms per Tx for a UE | 12PRBs per UE | 12PRBs per UE |
| Other assumptions for NOMA | 12 PRBs per Tx for a UE; Same power consumption as baseline | 12 PRBs per Tx for a UE; Same power consumption as baseline | 12 PRBs per Tx for a UE; Same power consumption as baseline | 12 PRBs per Tx for a UE; Same power consumption as baseline | 12 PRBs + 0.25ms per Tx for a UE | 12PRBs per UE; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA | 12PRBs per UE; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA |
| Supported PAR for baseline at 95% of UEs satisfying requirement (packet/s/cell) | 750 | 1000 | 400 | 500 | <500 | 2320 | 2810 |
| Supported PAR for NOMA at 95% of UEs satisfying requirement (packet/s/cell) | 2500 | 3500 | 1200 | 1200 | 2000 | 2350 for SCMA 1690 for MUSA 1690 for SL-RSMA 2290 for ML-RSMA | 2930 for SCMA 2170 for MUSA 2170 for SL-RSMA 2810 for ML-RSMA |
| Gain (relative to baseline) | 233% | 250% | 200% | 140% | 300% | 1.3% for SCMA, no gain for other schemes | 4.3% for SCMA, no gain for other schemes |
| Source | Huawei - Case 3 | Huawei - Case 4 | CATT |  |  |  |  |
| Carrier frequency | 4GHz, 200m | 4GHz, 200m | 4GHz, 200m |  |  |  |  |
| Simulation bandwidth | 12 PRBs, SCS = 60kHz, 7OS | 12 PRBs, SCS = 60kHz, 7OS | 12 PRBs |  |  |  |  |
| BS antenna number | 4 Rx | 4 Rx | 4 Rx |  |  |  |  |
| BS downtilt | 102 | 102 | 102 |  |  |  |  |
| Number of UEs per cell | 10 | 10 | 20 |  |  |  |  |
| UE power control | P0 = -85 dBm, alpha = 0.93 | P0 = -85 dBm, alpha = 0.93 | P0 = -90 dBm, alpha = 1 |  |  |  |  |
| HARQ/repetition | 1 repetition, no retransmission | 2 repetition, no retransmission | No HARQ/repetition |  |  |  |  |
| Channel estimation | Realistic | Realistic | Realistic |  |  |  |  |
| BS advanced receiver | SCMA/LCRS: chip-MMSE hard IC MUSA/RSMA: block-MMSE hard IC | SCMA/LCRS: chip-MMSE hard IC MUSA/RSMA: block-MMSE hard IC | MMSE-SIC |  |  |  |  |
| TB size | 60 bytes | 60 bytes | 60 bytes |  |  |  |  |
| Packet dropping criterion | - | - | - |  |  |  |  |
| DMRS setting and allocation | the DMRS pool size is 12, no DMRS collision | the DMRS pool size is 12, no DMRS collision | no DMRS collision |  |  |  |  |
| MA signature and allocation | spreading codes of length 2 are used for MUSA | spreading codes of length 2 are used for MUSA | - |  |  |  |  |
| Details on configured grant, e.g. periodicity, offset, and #of UEs assigned on the same resource | 7 OFDM symbols | 7 OFDM symbols | - |  |  |  |  |
| Other assumptions for baseline | 12PRBs per UE | 12PRBs per UE | - |  |  |  |  |
| Other assumptions for NOMA | 12PRBs per UE; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA | 12PRBs per UE; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA | - |  |  |  |  |
| Supported PAR for baseline at 95% of UEs satisfying requirement (packet/s/cell) | 2290 | 2810 | 1900 |  |  |  |  |
| Supported PAR for NOMA at 95% of UEs satisfying requirement (packet/s/cell) | 2290 for SCMA 1690 for MUSA 1690 for SL-RSMA 2290 for ML-RSMA | 2810 for SCMA 2150 for MUSA 2150 for SL-RSMA 2810 for ML-RSMA | 2700 for MUSA 2950 for PDMA |  |  |  |  |
| Gain (relative to baseline) | No Gain | No Gain | 42.1% for MUSA 55.3% for PDMA |  |  |  |  |

Table 3 SLS evaluation results for NOMA in eMBB scenario

| Source | ZTE - Case 1 | ZTE - Case 2 | Huawei - Case 1 | Huawei - Case 2 | Huawei - Case 3 | Huawei - Case 4 | CATT | Samsung |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency | 4GHz, 200m | 4GHz, 200m | 4GHz, 200m | 4GHz, 200m | 4GHz, 200m | 4GHz, 200m | 4GHz, 200m | 4GHz, 200m |
| Simulation bandwidth | 12 PRBs | 12 PRBs | 12 PRBs | 12 PRBs | 12 PRBs | 12 PRBs | 12 PRBs | 12 PRBs |
| BS antenna number | 4 Rx | 4 Rx | 4 Rx | 4 Rx | 4 Rx | 4 Rx | 4 Rx | 4 Rx |
| BS downtilt | 102 | 102 | 102 | 102 | 102 | 102 | 102 | - |
| Number of UEs per cell | 100 | 100 | 20 | 20 | 20 | 20 | 40 | 20 |
| UE power control | P0 = -95 dBm, alpha = 1 | P0 = -95 dBm, alpha = 1 | P0 = -90 dBm, alpha = 0.9 | P0 = -90 dBm, alpha = 0.9 | P0 = -90 dBm, alpha = 0.9 | P0 = -90 dBm, alpha = 0.9 | P0 = -90 dBm, alpha = 1 | - |
| HARQ/repetition | Number of transmission(s) = 1, no HARQ/repetition | Number of transmission(s) = 1, no HARQ/repetition | Maximum number of retransmission=8, no repetition | Maximum number of retransmission=8, no repetition | Maximum number of retransmission=8, no repetition | Maximum number of retransmission=8, no repetition | Random back-off, max 4 HARQ, no repetition | HARQ combining with random back-off. The maximum number for transmissions is 8 |
| Channel estimation | Realistic | Realistic | Realistic | Realistic | Realistic | Realistic | Realistic | Ideal & Realistic |
| BS advanced receiver | Baseline: MMSE-IRC or MMSE-PIC MUSA: MMSE-PIC | Baseline: MMSE-IRC or MMSE-PIC MUSA: MMSE-PIC | EPA receiver for all schemes | EPA receiver for all schemes | SCMA/LCRS: chip-MMSE hard IC MUSA/RSMA: block-MMSE hard IC | SCMA/LCRS: chip-MMSE hard IC MUSA/RSMA: block-MMSE hard IC | MMSE-SIC | Baseline: MMSE-IRC IGMA: ESE receiver |
| TB size | 70 bytes | 70 bytes | 60 bytes | 80 bytes | 60 bytes | 80 bytes | 60 bytes | - |
| Packet dropping criterion | If one TB is not received correctly after HARQ/ repetition, the packet is dropped | If one TB is not received correctly after HARQ/ repetition, the packet is dropped | 8 HARQ transmissions or 1s latency | 8 HARQ transmissions or 1s latency | 8 HARQ transmissions or 1s latency | 8 HARQ transmissions or 1s latency | - | 8 HARQ transmissions |
| DMRS setting and allocation | 24 DMRS each RB assumed, preconfigured, no collision | 24 DMRS each RB assumed, preconfigured, no collision | the DMRS pool size is 24, no DMRS collision | the DMRS pool size is 24, no DMRS collision | the DMRS pool size is 24, no DMRS collision | the DMRS pool size is 24, no DMRS collision | no DMRS collision | DMRS are semi-static configured |
| MA signature and allocation | 24 spreading codes of length 4 are used for MUSA, preconfigured | 24 spreading codes of length 4 are used for MUSA, preconfigured | spreading codes of length 2 are used for MUSA | spreading codes of length 2 are used for MUSA | spreading codes of length 2 are used for MUSA | spreading codes of length 2 are used for MUSA | - | MA signatures are semi-static configured |
| Details on configured grant, e.g. periodicity, offset, and #of UEs assigned on the same resource | periodicity = 5 ms for baseline and MUSA | periodicity = 5 ms for baseline and MUSA | periodicity = 1 ms | periodicity = 1 ms | periodicity = 1 ms | periodicity = 1 ms | - | - |
| Other assumptions for baseline | 3 PRB per Tx for a UE | 12 PRBs per Tx for a UE | 12PRBs per UE | 12PRBs per UE | 12PRBs per UE | 12PRBs per UE | - | 2 RBs is allocated for each UE |
| Other assumptions for NOMA | 12 PRBs per Tx for a UE; Same power consumption as baseline | 12 PRBs per Tx for a UE | 12PRBs per UE; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA | 12PRBs per UE; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA | 12PRBs per UE; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA | 12PRBs per UE; Multi-layer transmission for SCMA; Single-layer or multi-layer transmission for RSMA | - | all 12 RBs are occupied by each UE |
| Supported PAR for baseline at PDR=1% (packet/s/cell) | 800 for baseline with MMSE-IRC; 1200 for baseline with MMSE-PIC | 400 for baseline with MMSE-IRC; 600 for baseline with MMSE-PIC | 5050 | - | 4800 | - | 2100 | 450 (ICE) |
| Supported PAR for NOMA at PDR=1% (packet/s/cell) | 2000 | 1500 | 5100 for SCMA 4950 for MUSA; 4900 for SL-RSMA 5050 for ML-RSMA | 4850 for SCMA 4800 for MUSA; 4650 for SL-RSMA 4600 for ML-RSMA | 4800 for SCMA/MUSA/RSMA | 4500 for SCMA 4500 for MUSA; 4500 for SL-RSMA 4300 for ML-RSMA | 2500 for MUSA 3000 for PDMA | 900 (ICE) 860 (RCE) |
| Gain (relative to baseline) | 150% (baseline with MMSE-IRC) 66.7% (baseline with MMSE-PIC) | 275% (baseline with MMSE-IRC) 150% (baseline with MMSE-PIC) | 0.99% for SCMA, no gain for other schemes | - | no gain | - | 19.1% for MUSA 42.9% for PDMA | 100% (ICE) 91.1% (IGMA with RCE relative to baseline with ICE) |
| Source | Qualcomm-Case 1 (configured grant) | Qualcomm-Case 2 (grant-based) | DCM |  |  |  |  |  |
| Carrier frequency | 4GHz, 200m | 4GHz, 200m | 4GHz, 200m |  |  |  |  |  |
| Simulation bandwidth | 12 PRBs | 20/80 MHz (25% assumed for PDCCH, over 2 OFDM symbols) | 12 PRBs |  |  |  |  |  |
| BS antenna number | 4 Rx | 4 Rx | 4Rx |  |  |  |  |  |
| BS downtilt | 102 | 102 | 102 |  |  |  |  |  |
| Number of UEs per cell | 32 | 32 | 40 |  |  |  |  |  |
| UE power control | P0 = -99.4 dBm, alpha = 0.9 | P0 = -99.4 dBm, alpha = 0.9 | P0=-90 dBm, alpha = 1 |  |  |  |  |  |
| HARQ/repetition | Max 8 HARQ attempts | Max 8 HARQ attempts | maximum number of HARQ transmission=8, no repetition |  |  |  |  |  |
| Channel estimation | Realistic | Realistic | Realistic |  |  |  |  |  |
| BS advanced receiver | MMSE with hard parallel IC | MMSE with hard parallel IC | Baseline: MMSE-PIC UGMA: MMSE-SIC |  |  |  |  |  |
| TB size | 89 bytes | - | 85 bytes |  |  |  |  |  |
| Packet dropping criterion | Packet is dropped after 8 HARQ attempts | Packet is dropped after 8 HARQ attempts | 8 HARQ transmissions or 1s latency |  |  |  |  |  |
| DMRS setting and allocation | 16 DMRS assumed, preconfigured, no collision | 16 DMRS assumed, preconfigured, no collision | No collision |  |  |  |  |  |
| MA signature and allocation | 16 spreading codes of spreading factor 4, preconfigured | 4 spreading codes of spreading factor 2 | spreading codes of length 2 are used for UGMA |  |  |  |  |  |
| Details on configured grant, e.g. periodicity, offset, and #of UEs assigned on the same resource | TDD Config: 3 DL, 1UL Periodicity: Every 2 UL slots (i.e. every 8 slots = 4 ms) Overloading (# UEs per resource): Up to 4 for baseline; Up to 16 for NOMA | TDD Config: 3 DL, 1UL 2 PDCCH symbols used to convey UL grants for each UL slot |  |  |  |  |  |  |
| Other assumptions for baseline | 3 PRBs assigned to a UE for each arrival rate, the best MCS is selected ensuring that the packet drop rate is within the 1% limit | Frequency-selective (Granularity: 4 sub-bands of 3 RBs each) PDCCH constrained to 18 CCEs for UL Grants | Inter-cell interference is treated as white Gaussian noise |  |  |  |  |  |
| Other assumptions for NOMA | 12 PRBs assigned to a UE, SF = 4 for each arrival rate, the best MCS is selected ensuring that the packet drop rate is within the 1% limit | Frequency-selective (Granularity: 4 sub-bands of 3 RBs each) PDCCH constrained to 18 CCEs for UL Grants | Inter-cell interference is treated as white Gaussian noise |  |  |  |  |  |
| Supported PAR for baseline at PDR=1% (packet/s/cell) | supported PAR = 640 UPT (Mbps) at PAR = 480: 5%: 0.0256 50%: 0.0882 95%: 0.158 | supported PAR = 1120 UPT (Mbps) at PAR = 1120: 5%: 0.0378 50%: 0.1115 95%: 0.3173 | 1000 |  |  |  |  |  |
| Supported PAR for NOMA at PDR=1% (packet/s/cell) | supported PAR = 800 UPT (Mbps) at PAR = 480: 5%: 0.0442 50%: 0.1045 95%: 0.1645 | supported PAR = 1150 UPT (Mbps) at PAR = 1120: 5%: 0.0142 50%: 0.0982 95%: 0.2619 | 1200 |  |  |  |  |  |
| Gain (relative to baseline) | Gain on PAR: 25% Gain on UPT at PAR = 480: 5%: 72.3% 50%: 18.4% 95%: 4% | Gain on PAR:2% No Gain on UPT at PAR = 1120 | 20% |  |  |  |  |  |

Some possible observations can be made based on the provided results.

Possible Propose 5: Capture the table 1-3 and the following observations for SLS in TR38.812.

Observation 1: for mMTC scenario, under the system-level evaluation assumptions as detailed in Table 1 in R1-1814075, relative to the evaluated OFDM waveform (using configured grant with multiple users in the same time and frequency resources) with MMSE-IRC or advanced receiver, the evaluated NOMA schemes with configured grant (without DMRS collision) can provide the results in Table 1 as in R1-1814075.

  - In some simulated cases in Table 1 as in R1-1814075,

    - time and frequency resource configuration per UE for the baseline is different from that per UE for evaluated NOMA schemes;

    - Receivers used for the baseline and for the evaluated NOMA schemes are in some cases different and in other cases the same.

    - Resource utilization of simulated NOMA schemes is 3 to 5 times than baseline.

  - In some other simulated cases,

    - the time and frequency resource configuration per UE for the baseline is the same as that per UE for the evaluated NOMA scheme

    - the same type of receiver is assumed for the baseline and the evaluated NOMA schemes

    - Resource utilization of simulated NOMA schemes is comparable to baseline.

  - Different L2S mappings are used.

  - In R1-1813900, it assumes ideal assumptions of inter-cell interference covariance matrix (non-block diagonal and genie-known to the receiver).

  - Different baselines, different amount of optimization, and different choice of receiver types are used by companies

  - Note: Table 1 in R1-1814075 may need to be further updated, e.g., by incorporating any missing evaluation results, etc.

- Observation 3: for eMBB scenario, under the system-level evaluation assumptions as detailed in Table 3 in R1-1814075, relative to the evaluated OFDM waveform (using configured grant with multiple users in the same time and frequency resources) with MMSE-IRC or advanced receiver, the evaluated NOMA schemes with configured grant (without DMRS collision) can provide the results in Table 3 as in [R1-1814075].

  - In some simulated cases in Table 3 as in R1-1814075,

    - time and frequency resource configuration per UE for the baseline is different from that per UE for evaluated NOMA schemes;

    - Receivers used for the baseline and for the evaluated NOMA schemes are in some cases different and in other cases the same.

    - [Resource utilization of simulated NOMA schemes is 3 to 5 times than baseline].

  - In some other simulated cases,

    - the time and frequency resource configuration per UE for the baseline is the same as that per UE for the evaluated NOMA scheme

    - the same type of receiver is assumed for the baseline and the evaluated NOMA schemes

    - [Resource utilization of simulated NOMA schemes is comparable to baseline].

  - Different L2S mappings are used.

  - In R1-1813900, it assumes ideal assumptions of inter-cell interference covariance matrix (non-block diagonal and genie-known to the receiver).

  - Different baselines, different amount of optimization, and different choice of receiver types are used by companies

  - Note: Table 3 in R1-1814075 may need to be further updated, e.g., by incorporating any missing evaluation results, etc.

Whether PDR vs. PAR metric or/and throughput metric is used for calculate the gain should be reported by companies when updating Table 3.

Tdoc numbers as well as the sources will replace the companies names in columns.

- Observation 2: for URLLC scenario, under the system-level evaluation assumptions as detailed in Table 2 in R1-1814075, relative to the evaluated OFDM waveform (using configured grant with multiple users in the same time and frequency resources) with MMSE-IRC or advanced receiver, the evaluated NOMA schemes with configured grant (without DMRS collision) can provide the results in Table 2 as in R1-1814075.

  - In some simulated cases in Table 2 as in R1-1814075,

    - time and frequency resource configuration per UE for the baseline is different from that per UE for evaluated NOMA schemes;

    - Receivers used for the baseline and for the evaluated NOMA schemes are in some cases different and in other cases the same.

    - [Resource utilization of simulated NOMA schemes is 3 to 5 times than baseline].

  - In some other simulated cases,

    - the time and frequency resource configuration per UE for the baseline is the same as that per UE for the evaluated NOMA scheme

    - the same type of receiver is assumed for the baseline and the evaluated NOMA schemes

    - [Resource utilization of simulated NOMA schemes is comparable to baseline].

  - Different L2S mappings are used.

  - In R1-1813900, it assumes ideal assumptions of inter-cell interference covariance matrix (non-block diagonal and genie-known to the receiver).

  - Different baselines, different amount of optimization, and different choice of receiver types are used by companies

  - Note: Table 2 in R1-1814075 may need to be further updated, e.g., by incorporating any missing evaluation results, etc.

SLS results for mMTC

| Source | R1-1813900 | R1-1812969 | R1-1813159 |
| --- | --- | --- | --- |
| NOMA Tx Scheme | MUSA | IGMA | NOCA |
| Resource Configuration | Configured without collision | Configured without collision | Configured without collision |
| NOMA Receiver | MMSE Hard-SIC | ESE | MMSE Hard-SIC |
| Baseline Receiver | MMSE-IRC or MMSE-hard-SIC | MMSE-IRC | MMSE-IRC or MMSE-hard-SIC |
| L2S Mapping | Method-1 | Method-2 | Method-1 |
| Gain | For MMSE-IRC: 100% For MMSE-hard SIC: 71%~150% | ~87.5% | ~100% |

SLS results for eMBB

| Source | R1-1813900 | R1-1812612 | R1-1812969 | R1-1813894 | R1-1814077 |
| --- | --- | --- | --- | --- | --- |
| NOMA Tx Scheme | MUSA | PDMA/ MUSA | IGMA | RSMA | UGMA |
| Resource Configuration | Configured without collision | Configured without collision | Configured without collision | Configured without collision | Configured without collision |
| NOMA Receiver | MMSE Hard-SIC | MMSE Hard-SIC | ESE | MMSE Hard-SIC | MMSE Hard-SIC |
| Baseline Receiver | MMSE-IRC or MMSE-hard-SIC | MMSE-hard-SIC | MMSE-IRC | MMSE Hard-SIC | MMSE Hard-SIC |
| L2S Mapping | Method-1 | Method-1 | Method-2 | Method-1 | Method-1* |
| Gain | For MMSE-IRC: 150%~275% For MMSE-hard SIC: 66.7%~150% | 19.1% for MUSA 42.9% for PDMA | ~91.1% | 25% for PAR 4%~72.3% for UPT | 20% |

SLS results for uRLLC

| Source | R1-1813900 | R1-1812612 |
| --- | --- | --- |
| NOMA Tx Scheme | MUSA | PDMA/ MUSA |
| Resource Configuration | Configured without collision | Configured without collision |
| NOMA Receiver | MMSE Hard-SIC | MMSE Hard-SIC |
| Baseline Receiver | MMSE-IRC or MMSE-hard-SIC | MMSE-hard-SIC |
| L2S Mapping | Method-1 | Method-1 |
| Gain | 140%~300% | 42.1% for MUSA 55.3% for PDMA |

# Conclusions

Proposed observation 1:

- For LLS in the simulated case 6/8/9/21/32/33/34/35 with ideal channel estimation, unequal SNR distribution, and fixed MA signature allocation

As long as the simulation configuration, e.g., reasonable code rate, is appropriate, the performance difference between NOMA schemes/MA signatures is small, even when different receiver types are used

Proposed observation 2:

- For case 9 with ideal channel estimation and timing offset is within [0, 1.5*NCP] (fixed allocation, no MA signature collision, non-zero FO, and SNR offset is within +/-3dB)

  - When the TBS is small (i.e. 20bytes), 12 simultaneous activated UEs can be supported with no significant performance loss at the target BLER, compared to timing offset is within [0, 0.5*NCP].

- For case 11 with ideal channel estimation and random selection with potential MA signature collision (timing offset is within [0, 0.5*NCP], non-zero FO, and SNR offset is within +/-3dB)

  - When the TBS is small (i.e. 20bytes), 12 simultaneous activated UEs can be supported with no significant performance loss at the target BLER, compared to random activation with no MA signature collision.

- For case 11 with realistic channel estimation and random selection with potential MA signature collision (timing offset is within [0, 1.5*NCP], non-zero FO, and SNR offset is within +/-3dB)

  - When the TBS is small (i.e. 20bytes), 6 simultaneous activated UEs can be supported based on the realistic UE detection by using 50% RS overhead, with less than 3dB performance loss at the target BLER, compared to random activation with no MA signature collision and timing offset is within [0, 0.5*NCP].

Proposed observation 3:

- In general, frequency-domain spreading increases the PAPR for CP-OFDM

  - OFDM-symbol level (time-domain) spreading has the same PAPR as non-spreading scheme

  - Sequence hopping can be applied to reduce the PAPR

- Symbol-level scrambling/interleaving can be applied to reduce the PAPR for both CP-OFDM and DFT-s-OFDM

- Sparsity can be beneficial for PAPR reduction for DFT-s-OFDM

- FFS the impact of PAPR/CM on PA backoff, taking into account the required transmit power and the occupied bandwidth for the transmission.

Possible Proposal 4:

- Agree on the text proposals in section 3, to be captured in the Annex of TR38.812.

Possible Proposal 5: Capture the table 1-3 and the following observations for SLS in TR38.812.

Observation 1: Relative to OFDMA based baseline with MMSE-IRC or advanced receiver, the evaluated NOMA schemes with configured grant can provide significant gain at target performance metrics in mMTC, URLLC, and eMBB scenarios.

Observation 2: Relative to low code rate based baseline with advanced receiver, significant gain can be observed from one source for NOMA with configured grant in at least mMTC and eMBB scenarios, while insignificant gain is observed from another source for NOMA with configured grant.

Observation 3: At least in mMTC grant free with random selection scenario, NOMA can also provide significant gain at target PDR = 1%.

# Appendix A1

SLS results for mMTC

| Source | R1-1813900 | [R1-] | [R1-] | [R1-] |
| --- | --- | --- | --- | --- |
| NOMA Tx Scheme |  |  |  |  |
| Resource Configuration |  |  |  |  |
| NOMA Receiver |  |  |  |  |
| Baseline Receiver |  |  |  |  |
| L2S Mapping |  |  |  |  |
| Gain |  |  |  |  |

SLS results for eMBB

| Source | R1-1813900 | [R1-] | [R1-] | [R1-] | [R1-] |
| --- | --- | --- | --- | --- | --- |
| NOMA Tx Scheme |  |  |  |  |  |
| Resource Configuration |  |  |  |  |  |
| NOMA Receiver |  |  |  |  |  |
| Baseline Receiver |  |  |  |  |  |
| L2S Mapping |  |  |  |  |  |
| Gain |  |  |  |  |  |

SLS results for uRLLC

| Source | R1-1813900 | [R1-] | [R1-] |
| --- | --- | --- | --- |
| NOMA Tx Scheme |  |  |  |
| Resource Configuration |  |  |  |
| NOMA Receiver |  |  |  |
| Baseline Receiver |  |  |  |
| L2S Mapping |  |  |  |
| Gain |  |  |  |