3GPP TSG RAN WG1 #95 			R1-1814218

Spokane, USA, November 12th – 16th, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for d1,2 in cancellation timeline in slot configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, VIVO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | From specification, the value of d2,1 is not explicitly stated to be 0 or 1 in some cases involving cancellation of RRC configured transmissions for PUCCH, SRS, and PRACH. This is because the value of d2,1 is dependent upon the PUSCH DMRS location, but in these latter cases there is no PUSCH to determine this value. A value definition is needed to clarify operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define d2,1=1 in cancellation timeline where dynamic downlink scheduling cancels RRC configured uplink transmissions. This would disambiguate operation for both the UE and network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification on which value to assume between d2,1 = 0 or 1. gNB and UE may be out of sync regarding minimum action time required for cancellation of RRC configured signals, whenever network dynamically scheduled downlink data to the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclause 11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Impact analysis: This clarification is not expected to impact UE behavior according to current specification since the value of d2,1=1 sets the largest possible value which was previously ambiguous. Since the action time for PUSCH preparation is large relative to d2,1=1, there is no expectation of performance or latency degradation. Moreover, specification would indicate operation clearly for both UE and gNB and allow them to use the minimal possible action time. |  |  |  |  |  |  |  |  |

-----------Text Proposal for 38.213 Subclause 11.1--------

If a UE is configured by higher layers to transmit periodic SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	the UE does not expect to cancel the transmission in symbols from the subset of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 1_0 or the DCI format 1_1 or the DCI format 0_1, after a number of symbols that is smaller than the PUSCH preparation time  for the corresponding UE processing capability [6, TS 38.214] assuming d2,1 = 1;

-	the UE cancels the PUCCH, or PUSCH, or PRACH transmission in remaining symbols from the set of symbols and cancels the periodic SRS transmission in remaining symbols from the subset of symbols

-----------End of Text Proposal--------