3GPP TSG RAN WG1 #95																					   R1-1814341

Spokane, USA November 12th - 16th, 2018

Agenda item:	7.2.1.5

Source: 	InterDigital Inc.

Title:	On NOMA mMTC SLS Evaluation

Document for:	Discussion and Decision

# Introduction

In WG1 Meeting #94b [1], RAN1 continued the discussion about evaluation and operation of NOMA. A bulk part of the discussion was focused on compiling observations based on link level simulations, and also further details related to system level aspects.  Based on the discussion the following agreements were reached:

- Channel structure consisting of preamble and data can be considered for supporting the asynchronous transmission: - Preamble in Rel-15 can be considered as the starting point. - Additional components can be included if necessary, e.g., the UL channel for assisting the UE detection or GP. - Study further the case when a UE is configured with one or more set(s) of MA signature/resource - FFS principle for MA signature/resource configuration/selection among MA signature/resource belonging to same/different set(s). - e.g. different MA signatures/resources may be considered for different TBSs/MCSs/retransmissions/UE grouping/measurements, etc. - FFS signaling - FFS how to handle the collision of MA signature/resource - FFS the mapping between RS and other MA signatures - For NOMA SLS, companies to report how HARQ re-transmissions are performed (e.g., non-adaptive, adaptive, etc.)

In RAN1 #92b, #93 and #94 meetings [1, 2, 3], system level simulation assumptions for NOMA were agreed. In RAN1 #94 [3], simulation assumptions were provided through an e-mail discussion to calibrate the system level simulator. In this contribution, we provide our calibration and preliminary results from our system level simulator with IDMA. In an accompanying contribution [5], we provided our SLS results with assumption of P0=-100dbm. In this contribution, we provide our evaluation results for P0=-110dBm along with our baseline evaluation.

# Preliminary PDR Results

In this section, we provide simulation results for IDMA with CP-OFDM in the mMTC use case. In this evaluation, we assume that there is no contention in DMRS and MA signature sequences. Therefore, it is assumed that the system operates with a configured grant where DMRS and NOMA signatures are preconfigured to avoid collisions.

The deployment consists of 20 single antenna mMTC UEs per sector. For NOMA, the transmission is based on IDMA and gNB relies on an EPA-based receiver  with 2 receive antennas. For the baseline OMA, the scheduling is done using random resource selection. When a UE has a packet to transmit, it randomly chooses one of the 6 RBs from the resource pool and transmits. If more than two UEs are scheduled to transmit on the same resource, a collision occurs and UEs retransmit on a new randomly selected RB. A packet is dropped if it reaches the maximum number of retransmissions.

Packets are randomly generated with Poisson arrival rate and Pareto distributed size with an additional higher layer overhead of 29 bytes. They are then segmented into 40 bytes chunks and an additional segmentation overhead of 5 bytes is added per segment [4]. The segmentation plays an important role in the performance of the system. For a given size of the transmission resources, i.e., REs, a smaller size of packets leads to a lower code rate, and thus a higher multiplexing capability. However, such a selection also extends the duration of a transmission to several TTIs that may lead to additional contentions in the system. Therefore, the choice of a packet size should be optimized based on the overall system performance. In our evaluation, we found for NOMA that a segmentation size of 40 bytes provides a satisfactory performance for the given size of the resources and the assumed traffic pattern. For the baseline, we assume packets are segmented into 20B.

We are providing our evaluation results with both real and ideal channel estimation assumptions. The impact of channel estimation is considered through appropriate calibration of beta values for the link-to-system mapping. The dropping criterion is based on decoding failure after 8 transmissions, i.e., the maximum number of HARQ retransmissions.

Detailed simulation assumptions are provided in the appendix as well as the curves used in the link-to-system mapping to show the accuracy of the link prediction model

Figure 1 provides the packet dropping rate (PDR) plotted as a function of the packet arrival rate (PAR) as arrivals per second per sector. For NOMA with ideal CHEST, the target PDR of 1% is achieved around 900 PAR while for real CHEST it occurs around850. It is shown that NOMA can handle significantly high PAR and maintain the PDR below 1%. At even higher PAR such as 1000 the PDR remains below 5%. Due to the significant overloading capability of NOMA, a high number of concurrent transmissions on the same resources with random arrival can be handled in the system with low dropping rate.

For the baseline OMA case, the target PDR of 1% is achieved around 580 PAR.

The degradation due to realistic channel estimation is more significant at higher PAR. At high PAR, more users are simultaneously multiplexed which increases the interference in the system and reduces the accuracy of the channel estimates. This affects the performance of iterative receivers used by NOMA as they rely on channel estimates to remove interference from co-scheduled users. Further optimization is possible considering channel estimation enhancements and the tradeoff between segmentation overhead and overloading capability of NOMA. For the baseline, at high PAR, the number of collisions increases and the gNB cannot decode the users anymore.

Figure 1 PDR vs PAR for IDMA-EPA in mMTC

Observation 1: IDMA-EPA with ideal (realistic) channel estimation can handle a PAR of ~900 (850)packets/s/sector with ~1% PDR.

To enhance the performance of a NOMA system, it is important to incorporate mechanism to reduce collision of resources, and improve the robustness of the system. Without loss of generality, we could consider some characteristics of a NOMA transmission to determine the resource to be used for NOMA transmission. For example, a UE could select a set of NOMA resources based on its SNR condition to enable a grouping mechanism to reduce the impact of near/far effect on channel estimation and the overall system performance.

Proposal 1: RAN1 considers SNR-based grouping mechanism for system level analysis of NOMA systems.

.

# Conclusions

In this contribution, we presented IDMA-EPA performance results with real and ideal channel estimation in mMTC use case. We make the following observation and proposal:

Observation 1: IDMA-EPA with ideal (realistic) channel estimation can handle a PAR of ~900(850)packets/s/sector with ~1% PDR.

Proposal 1: RAN1 considers SNR-based grouping mechanism for system level analysis of NOMA systems.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #92b, Sanya, China, April 2018

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #93, Busan, South Korea, May 2018

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #94, Gothenburg, Sweden, August 2018

- R1-1810048, “Email discussion on the SLS calibration and template for collecting SLS evaluation results”, ZTE

- R1-1813214, NOMA System Level Evaluation, RAN1 #95, Spokane, US, November 2018

# Appendix

Table 1 System level simulation assumptions

| Parameters | mMTC |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid 19 sites |
| Inter-BS distance | 1732 m |
| Carrier frequency | 700 MHz |
| Simulation bandwidth | 6 PRBs |
| Number of UEs per cell | 20 |
| Channel model | UMa in TR 38.901 |
| UE Tx power | Max 23 dBm |
| BS antenna configurations | 2 Rx for 700 MHz; 2 ports: (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), 2 TXRU; dH = dV = 0.5λ; BS antenna downtilt: 92 degrees |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | 8 dBi, including 3 dB cable loss |
| BS receiver noise figure | 5 dB |
| UE antenna configuration | 1Tx |
| UE antenna height | Follow the modelling of TR 38.901 |
| UE antenna gain | 0 dBi as starting point |
| UE distribution | For mMTC: 20% of users are outdoors (3 km/h), 80% of users are indoor (3 km/h) Users dropped uniformly in entire cell |
| UE power control | Open loop PC, P0 = -110 dBm, alpha = 1. |
| HARQ/repetition | 8 Chase combining |
| Channel estimation | Ideal and Real 1/7 OFDM symbol DMRS overhead |
| BS receiver | NOMA: EPA OMA: MMSE-IRC |
| Packet dropping criterion | maximum number of HARQ transmissions |

# L2S Mapping

Figure 2 Link-to-system mapping curves