3GPP TSG RAN WG1 Meeting #95                                                             	                  R1-1814345

Spokane, USA, November 12th – 16th, 2018

Source:	NTT DOCOMO, INC.

Title:	DCI size alignment

Agenda Item:	7.1.3.3

Document for: 	Discussion and Decision

- Discussions

Regarding the DCI size alignment between the DCI format 0_1 with CRC scrambled by CS-RNTI and DCI format 0_1 with CRC scrambled by C-RNTI. Based on the views, three options are proposed in general:

  - Option 1: the bit widths of configurable fields in DCI format 0_1 are determined by pusch-Config, and UE is not expected that the bit width of each configurable field in DCI format 0_1 with CRC scrambled by CS-RNTI is larger than that of DCI format 0_1 with CRC scrambled by C-RNTI.

    - Supported by DCM, CATT, Sharp, QC, Intel, LG, HW, Samsung,

  - Option 2': A UE shall assume that the number of information bits in the DCI format 0_1 with CRC scrambled by CS-RNTI prior to padding is equal to or less than the payload size of the DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI or MCS-C-RNTI. If the number of information bits in the DCI format 0_1 with CRC scrambled by CS-RNTI prior to padding is less than the payload size of the DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI or MCS-C-RNTI, zeros shall be appended to the DCI format 0_1 with CRC scrambled by CS-RNTI until the payload size equals that of DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI or MCS-C-RNTI.

    - Supported by LG, DCM, Nokia, Intel, Ericsson

  - Option 3: Network ensures that the bit-width of each field in DCI format 0_1 are aligned between the DCI with CRC scrambled by C-RNTI, and the DCI with CRC scrambled by CS-RNTI

    - Supported by QC, Ericsson, MTK