3GPP TSG RAN WG1 Meeting #95			R1-1814367

Spokane, United States, November 12th – 16th, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On maximum payload size for transmission on PUCCH formats 4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UCI bits including CRC bits are used for encoding using the maxCodeRate in Subclause 9.2.5.2 of 38.213. The procedure in this subclause determines when dropping of a CSI report is needed to carry UCI on a PUCCH format 2 or 3 or 4 based on e.g. the number of REs with DMRS, modulation, configured coderate and number of PRBs. For PUCCH format 4, the number of PRBs is equal to one and spreading factor is additionally consdiered. Therefore, it is clear how many UCI bits including CRC bits a PUCCH format, e.g.PUCCH format 4, can carry. The maximum number depends on the largest code rate, etc. Therefore, the description in this subclause related to capacity of PUCCH fromat 4, for the purpose of this subclause is redundant. Similar description is inlcuded in Subclause 5.2.4 of 38.214 which is appropriate to determine a limit for CSI report size when transmitted on PUCCH fromat 4 for potential dropping. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the description for the maximum number of UCI bits for PUCCH format 4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect limit for dropping UCI bits. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact The normal UE behavior is to include CRC bits for large number of UCI bits. In addiiton, Sublcause 5.2.4 in 38.214 ireflects this behavior. |  |  |  |  |  |  |  |  |

#### 9.2.5.2	UE procedure for multiplexing HARQ-ACK/SR/CSI in a PUCCH

< Unchanged parts are omitted >

A UE is configured by maxCodeRate a code rate for multiplexing HARQ-ACK, SR, and CSI report(s) in a PUCCH transmission using PUCCH format 2, PUCCH format 3, or PUCCH format 4.

< Unchanged parts are omitted >