3GPP TSG-RAN WG1 #96	R1-1901786

Athens, Greece, February 25th – March 1st, 2019

Agenda Item:	7.1.3

Souce:	MediaTek Inc.

Title:	Maintenance for rate matching

Document for:	Discussion and decision

# 1. Introduction

In this paper, the relation between “the resources of PDSCH DM-RS” and “the resources not available for PDSCH” is discussed.

# 2. Discussion

In sub-clause 5.1.4 PDSCH resource mapping of TS 38.214, the relation between “the REs for PDSCH DM-RS” and “the REs not available for PDSCH” is stated in the yellow part below.

### 5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SystemInformationBlockType1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCH with SPS, the REs corresponding to the configured or dynamically indicated resources in Subclauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

Our understanding is “the REs for PDSCH DM-RS” and “the REs not available for PDSCH” cannot overlap. Therefore, the following interpretation is true.

Interpretation 1: According to the PDSCH DM-RS physical resource mapping defined in sub-clause 7.4.1.1.2 of TS 38.211, the PDSCH DM-RS REs never collide with resources not available for PDSCH.

However, the description in the yellow part is ambiguous and may cause another incorrect interpretation as below.

Interpretation 2: According to the PDSCH DM-RS physical resource mapping defined in sub-clause 7.4.1.1.2 of TS 38.211, the PDSCH DM-RS REs may collide with resources not available for PDSCH. When this happens, those PDSCH DM-RS REs collided with the REs not available for PDSCH are punctured at the gNB side. In this case, since the gNB has punctured the PDSCH DM-RS REs, the UE does not need to handle the case where PDSCH DM-RS REs are overlapping with any RE(s) not available for PDSCH.

The yellow-part description is even more likely to lead to the incorrect interpretation especially because it is placed at the paragraph about the PDSCH resource mapping at PRBs containing SS/PBCH block transmission. An invalid example about the PDSCH resource mapping is given in Figure 1. According to Interpretation 1, the PDSCH resource mapping in Figure 1 is not valid.

If Interpretation 2 was true, then the PDSCH resource mapping in Figure 1 would become valid. Since the yellow-part description is placed just after the description about the PDSCH resource mapping at PRBs containing SS/PBCH block transmission, the readers may put them together and think the yellow-part description applies only to the scenario of PDSCH and SS/PBCH block co-located at the same PRBs. This additionally may lead readers to incorrectly think the PDSCH DM-RS is punctured at the SSB resource as shown in Figure 1.

Figure 1. An invalid example of PDSCH resource mapping at PRBs containing SS/PBCH block.

Therefore, we propose to move the yellow-part description to a new paragraph (since it is a general rule applicable to all scenarios) and have some changes in the wording.

Proposal: The following changes are made in TS 38.214.

### 5.1.4	PDSCH resource mapping

----- Start -----

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SystemInformationBlockType1.

When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCH with SPS, the REs corresponding to the configured or dynamically indicated resources in Subclauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

A UE is not expected to handle the case where the physical resources mapping of PDSCH DM-RS defined in sub-clause 7.4.1.1.2 of [4, TS 38.211] is overlapping, even partially, with any RE(s) not available for PDSCH.

----- End -----