3GPP TSG RAN WG1 #96bis		R1-1904062

Xi’an, China, 8th – 12th April, 2019

Source:	vivo

Title:	Discussion on initial access signals and channels

Agenda Item:	7.2.2.1.1

Document for:	Discussion and Decision

- Introduction

According to WID on NR-based access to unlicensed spectrum [1], the following parts should be specified:

-	Physical layer aspects including [RAN1]:

-	NR-U Discovery Reference Signal (DRS) containing at least SS/PBCH block burst set transmission and possibly CSI-RS, RMSI-CORESET(s)+PDSCH(s), OSI and paging with properties and extensions from NR Rel-15 in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2). 60kHz based SSB/PBCH block is outside the scope of the WI.

-	PRACH including possible extension of PRACH format(s) in line with agreements during the SI phase (TR 38.889, Section 7.2.1.2) to support minimum bandwidth requirement given by regulation. Determine the applicability of Rel-15 NR formats to NR-U operation. RAN1 should decide whether 60 kHz subcarrier spacing for PRACH is supported, based on a unified design with 15 kHz and 30 kHz PRACH for meeting occupied channel bandwidth (OCB) requirements.

In RAN1 Adhoc 1901 and RAN1 #95, the following agreements are made:

Agreement:

- UE assumes 30KHz SCS for SS/PBCH block for 5GHz band and 6GHz band if the SCS is not indicated by higher layers.

- Support configuration by higher layers of 15KHz or 30KHz SCS for SS/PBCH block

- Include this agreement in a LS to RAN4 (cc RAN2) for inclusion in specs managed by RAN4

Conclusion:

No changes are required to the time and frequency position of the PSS/SSS/PBCH relative to each other in one PSS/SSS/PBCH block.

Agreement:

The Type0-PDCCH monitoring configuration for NR-U should satisfy at least the following properties:

- TDM of Type0-PDCCH and SSB similar to existing pattern 1 (already agreed)

- Support the monitoring of Type0 PDCCH of the 2nd SSB position in a slot in the gap between 1st and 2nd SSB within the slot

  - FFS start at symbol #6 of #7 or both

- FFS: The Type0-PDCCH candidates associated with an SSB are confined within a slot carrying the associated SSB (with the same QCL assumptions)

Agreement:

Companies are encouraged to provide results comparing the different alternatives using the following simulation assumptions to select between alternative PRACH designs.

- The Rel-15 PRACH design should be simulated as a baseline

Details put in Appendix

Agreement:

- Down-select from the following options for SSB pattern (symbol index starts at 0)

  - Option 1: SSBs are at symbols (2,3,4,5) and (8,9,10,11) in the slot

  - Option 2: SSBs are at symbols (2,3,4,5) and (9,10,11,12) in the slot

- The down-selected pattern applies no matter if SSB SCS is indicated by higher layer or not, and no matter if RMSI is transmitted or not.

Agreement:

- The SCS for all SSBs and Coreset #0 on a carrier is always the same for operation of NR in unlicensed spectrum.

- CORESET #0 frequency domain resource configuration should be 48 RBs for 30KHz SCS and 96 RBs for 15KHz SCS.

In this paper, NR-U DRS design and PRACH based on WID guidance and the above agreements are discussed in the following sections.

- Discussion

Discovery reference signal (DRS) design

SSB transmission pattern

In NR Rel 15, several SSB patterns in time domain are defined for different SCS and frequency range. Regarding sub-7 (e.g. 5GHz), the following 3 patterns could be used as illustrated in Figure 1:

Figure 1		SSB patterns for sub-7 in NR Rel 15

To support 7-symbol DRS unit as described below, Case B is not feasible since there is no gap between some successive SSBs for placing RMSI-CORESET. Therefore, Case A and Case C in NR Rel15 are reused for NRU in all FR1 unlicensed bands, i.e. SSBs are at symbols (2,3,4,5) and (8,9,10,11) in the slot.

Proposal 1: NR-U supports SSBs are at symbols (2,3,4,5) and (8,9,10,11) in the slot as NR Rel15.

Regarding frequency domain aspect, NR Rel15 SSB itself couldn’t meet OCB requirement in unlicensed band. It is already agreed that inclusion of the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) associated with SS/PBCH block(s) is beneficial. In SA/DC mode, RMSI is always transmitted together with SSB and FDM with RMSI (potentially configured CSI-RS) could meet OCB. In CA mode, configured cell specific CSI-RS with specific design (i.e. spanning the whole channel) together transmitting with SSB could also meet OCB requirement. Therefore, there is no need to have frequency domain repetition for SSB in either mode.

Proposal 2: NR-U supports RMSI and/or CSI-RS is FDM with SSB in SA/DC/CA mode to meet OCB requirements.

DRS component design

In NR Release 15, multiple SS/PBCH blocks with different beamforming directions are supported to enhance the coverage and system data rate. Due to EIRP limit in unlicensed band and beamforming gain in EIRP calculation, the coverage couldn’t be improved by simply beamformed transmission. However, the beamformed transmission could reduce the interference and thus increase system data rate. In this case, beamformed DRS could be used for beam measurement and reference of beamformed data transmission (e.g. determine precoder). By considering this, there is no need to remove this possibility from spec aspect, i.e. the design should embrace more flexibility on omni-transmission with single/multiple SSBs or beamforming with multiple SSBs, which is up to implementation. To achieve this, it is natural that one DRS could include one or more DRS units where each DRS unit comprises of at least one SSB, RMSI-CORESET+PDSCH in the same direction (omni or one beamforming direction). Note that the actual number of transmission DRS units is up to gNB’s implementation. One example is illustrated in Figure 2 (see next page): DRS includes 4 DRS units, where each DRS unit is 7 symbols long including one SSB and RMSI-CORESET+RMSI PDSCH.

Proposal 3: For NR-U, DRS includes one or more DRS units where each one comprises of at least one SSB, RMSI-CORESET+RMSI PDSCH in the same direction (omni or one beamforming direction).

For one DRS unit in frequency domain, the bandwidth is limited by the RMSI-CORESET setting since the scheduled PDSCH carrying RMSI should be limited within RMSI-CORESET according to NR Rel 15. In order to meet OCB requirement, CORESET bandwidth is better to span the initial BWP as much as possible, e.g. 48 PRBs assuming 30KHz SCS. Another alternative is including or multiplexing other signals (CSI-RS, OSI, Paging etc.) to meet the OCB requirements. NR Rel 15 already supports such flexibility and no change needed for NRU DRS design in frequency domain. In time domain, there are two choices to favor this DRS unit concept, i.e. 7 symbols (see Figure 2) or 14 symbols (see Figure 3). The comparison of these two choices are given in the following table:

Table 1	Comparison of 7 symbol and 14 symbol DRS unit (assuming 30KHz SCS)

|  | No. of LBT opportunities in 5ms | No. of DRS unit in 1ms | Maximum schedulable RMSI REs |
| --- | --- | --- | --- |
| 7 symbols | 20 | 4 | 2496 REs |
| 14 symbols | 10 | 2 | 5568 REs |

Figure 2		Illustration of DRS with multiple DRS units (7-symbol DRS unit)

Figure 3		Illustration of DRS with multiple DRS units (14-symbol DRS unit)

It is obvious that 7 symbol DRS unit has more LBT opportunities and support more beam directions within the same time duration. However, the available resource for scheduling RMSI is limited in this case. Assuming MCS0 is used, only 2496*0.2344=585 bits could be scheduled for RMSI. From RAN1 aspect in NR Rel 15, the UE is not expected to receive a PDSCH assigned by a PDCCH with CRC scrambled by SI-RNTI with a TBS exceeding 2976 bits. So RAN2 designs the RMSI size targeting 2976 bits at maximum. Besides, RMSI size is variable depending on different deployment scenarios, especially the number of supporting PLMNs. For NRU, 3rd party deployment shared by multiple operators is one typical scenario which requires broadcasting multiple PLMNs and respective tracking area information. In this case, the resource in 7-symbol DRS unit is very probably not enough for carrying RMSI. Therefore, we make the following proposal:

Proposal 4: NR-U supports flexible configuration for time domain length of DRS unit, e.g. between 7 symbols and 14 symbols.

DRS period and transmission window

NR Rel 15 supports the following SS/PBCH Block periods {5, 10, 20, 40, 80, 160} ms while LAA Rel 13 supports DRS period {40, 80, 160}ms. Naturally NRU will support flexible configuration of DRS period. By considering the overhead, NRU DRS is better to be transmitted in large period, i.e. {40, 80, 160} ms. To increase the transmission opportunity, it is better to allow DRS sliding within one specific window, e.g. 6ms window. Larger DRS window could increase the transmission probability of DRS. On the other hand, it will result in more power consumption since UE needs to search SSB within a larger DRS window. So DRS window duration should be flexible and could be configured in different cases. For example, in low interference case, smaller DRS window duration is better to be configured. Besides, for 7-symbol DRS unit case, the DRS window duration could be less than that in 14-symbol DRS unit case since it has more opportunities to access the channel. Thus, the following proposal is made:

Proposal 5: NR-U supports flexible configuration of DRS period (e.g. {40ms, 80ms, 160ms}) and DRS window duration (e.g. {2ms, 4ms, 6ms, 8ms}).

Referring to section 7.2.1.2 in TR 38.889 [2], Pattern 1 is feasible for multiplexing of SSB and RMSI Coreset where Pattern 1 is understood as RMSI CORESET and an SS/PBCH block occurring in different time instances, and the RMSI CORESET bandwidth overlapping with the transmission bandwidth of the SSB. In NR Rel 15, the configuration for Type0-PDCCH common search space in Table 13-11 in TS 38.213 [3] is not feasible for NR-U because SSB and its related Type0-PDCCH is better to be contaminated in the same slot in NRU as we proposed. First, most of the configurations are not needed since they configure SSB and the related Type0-PDCCH in different slots. Second, some configurations (i.e. configuration index 0 and 1) need to be updated to meet the 14-symbol and 7-symbol DRS unit. Third, the Type0-PDCCH common search space is variable in multiple specific positions within one DRS window instead of fixed subframe in one DRS occasion.

Proposal 6: The Type0-PDCCH candidates associated with an SSB should be confined within a slot carrying the associated SSB and the Type0-PDCCH common search space configuration should be re-designed with more NRU related information conveyed.

RMSI rate matching

In NR Rel15, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the RMSI PDSCH. This is because RMSI is decoupling with SSB in most of configured cases. However, this is not the case for NRU which always confine SSB and RMSI in the same slot as proposed in Section 2.1. Thus, the NRU UE shall assume that SS/PBCH block is transmitted in the same slot of RMSI transmission. Particularly the UE shall assume that REs scheduled for the RMSI PDSCH reception and overlapping with SSB transmission are not used.

Proposal 7: NR-U UEs shall assume RMSI rate matching around SSB, i.e. REs scheduled for the RMSI PDSCH reception and overlapping with SSB transmission are not used.

In the licensed spectrum, the CSI-RS can only be configured to UEs in connected state, the UEs in the initial stage assume no CSI-RS in the PDSCH resources when receiving the broadcast PDSCH. While in the unlicensed spectrum, cell specific CSI-RS is better to be multiplexed with RMSI in a DRS unit to help RLM/RRM. Therefore, RMSI should be rate matching around the CSI-RS based on predefined pattern or configuration in PBCH, i.e. REs scheduled for the RMSI PDSCH reception and overlapping with CSI-RS transmission are not used.

Proposal 8: NR-U UEs shall consider RMSI rate matching around CSI-RS based on configuration or predefined pattern, i.e. REs scheduled for the RMSI PDSCH reception and overlapping with CSI-RS transmission are not used.

PRACH design

Rel-15 NR supports 13 preamble formats with different sequence lengths and configurable SCSs. The SCS is either 1.25 kHz or 5 kHz for long formats with sequence L=839 while 15/30/60/120 kHz are supported for short ones with L=139. One regulatory requirement commonly considered in unlicensed spectrum is OCB, which should be larger than 80% of the Nominal Channel Bandwidth for sub-7 GHz NR-U frequency. Apparently, the PRACH mechanism supported in the licensed band cannot fulfill the OCB requirement.

In the following sections, we discuss different PRACH transmission schemes and provide some simulation results.

PRACH format in unlicensed spectrum

If RAN1 agrees that UE is not forced to meet OCB requirement when transmitting PRACH, given that during a COT device may operate temporarily with an Occupied Channel Bandwidth of less than 80 % of its Nominal Channel Bandwidth with a minimum of 2 MHz during a COT, PRACH transmission occupying frequency range wider than 2 MHz bandwidth can be transmitted directly without further enhancement.

In Table 2, we summarize the bandwidth for each PRACH format. It can be noted that if multiple Msg.1 transmissions in frequency domain are not supported, format 0,1,2 can be excluded in NR-U. Furthermore, format 3 can also be excluded if it is necessary to ensure the same SCS for PRACH and other UL transmission.

Table 2. Bandwidth of RO in FR1

|  | length | SCS | Bandwidth of an RO |
| --- | --- | --- | --- |
| Long: format 0,1,2 | 839 | 1.25kHz | 1.08MHz |
| Long: format 3 | 839 | 5kHz | 4.32MHz |
| Short: A,B,C | 139 | 15kHz | 2.16MHz |
| Short: A,B,C | 139 | 30kHz | 4.32MHz |

If the OCB requirement is mandatory for PRACH transmission, several options can be considered:

- Opt.1 Multiple msg1 transmissions in frequency domain

- Opt.2 Interlace based PRACH transmission

On top of the legacy single Msg1 scheme, opt.1 allows a specific number of simultaneous Msg1 instances which occupy a wider frequency range to meet the OCB requirement. Since FDMed RO and 1-to-multiple SSB-RO mapping have already been supported in NR Rel.15, the multiple FDMed msg1 transmissions mechanism can be easily supported in the physical layer by using Rel.15 PRACH configuration.

One potential concern arising from the FDMed Msg1 repetition would be a lower PRACH capacity. Another concern is that in some conditions the largest gap between two ROs in legacy PRACH configuration still cannot meet the OCB requirement. For example, if prach-FDM=8, preamble length=139 and preamble SCS =15kHz, the total bandwidth of 8 RO is 8.64MHz, less than 80% of the nominal channel bandwidths. As mentioned in [2], in order to alleviate the impact of LBT failures further, additional opportunities for the RACH messages may be introduced, e.g. in time or frequency domain, for both 4-step and 2-step RACH. In this case, these issues can be solved by configuring denser RACH time instances and extending frequency range of FDMed ROs.

For example, in the unlicensed band, wider gap between two neighboring ROs in the frequency domain can be introduced. The gap should be carefully chosen to make sure that the OCB requirement can be sufficiently fulfilled. In opt.2, one leading idea is that RB level interlace structure is a useful enhancement that eases the UE multiplexing and power boosting for UL transmission. However, as is shown in in 2.2.2, PRACH detection performance of B-IFDMA severely deteriorates when suffering from interference.

Based on the above analysis, opt.1 is straightforward and requires minor changes to the RAN1 spec.

Observation 1: Msg1 repetition in frequency domain is a straightforward way to meet OCB requirement and requires minor RAN1 spec efforts.

Observation 2: Long PRACH formats are needed only if a few tens of kilometres coverage is expected in unlicensed band.

Proposal 9: If it is allowed that PRACH can be transmitted without meeting the OCB regulatory, some PRACH formats can be excluded, e.g., format 0,1,2,3.

Besides, as described in [2], the interlaced PRACH design are also proposed by some companies, the potential alternatives are given as follows.

- Contiguous PRB mapping in RO resource

  - Rel-15(baseline)

  - Repeated ROs in frequency domain

- PRB-level interlace mapping

  - Uniformed interlace

  - Non-Uniformed interlace

According to the simulation assumptions in [4] which are also listed in Appendix, we will discuss the above alternatives based on simulation.

For repeated ROs in frequency domain, the two preambles are mapped to {1, …, 12}-th RBs and {39, …, 50}-th RB, respectively. Note that, to reduce the PAPR, the preambles with different root/cyclic shift are mapped to the two ROs, respectively.

For Non-Uniformed PRB level interlaced PRACH, the preamble is mapped to {1 4 10 13 19 22 28 31 37 40 46 49}-th RB sequentially, as proposed in [5]. For uniformed PRB level interlaced PRACH, the zero correlation zone is greatly compressed, which will lead to severe performance degradation in TA estimation, it is down-prioritized in this contribution.

Without loss of generality, PRACH format A1 with 30 kHz subcarrier spacing is used for the following evaluation and other evaluation details are provided below:

- Detailed simulation assumptions

In addition to the simulation assumptions in [4], the same Ncs is set to 10 (zeroCorrelationZoneConfig=5), based on the following equation.

Ncs=2rc+τ×LRATseq×Ts

Where r=300/3 is the cell radius, c is the speed of light, τ=0.865 μs for TDL-C with 100ns delay spread, LRA=139, Tseq=2048, and Ts=12048×30000. The same Ncs is assumed for all PRACH design alternatives.

The SNR in the following evaluation is the signal to noise power ratio in each RE where preamble symbol is mapped. Thus the total transmission power is the same for contiguous and PRB level interlaced mapping, and for repeated preamble mapping, the total transmission power is twice of the transmission power of single RO transmission.

- Detection algorithm of PRACH

The PRACH detection algorithm is described as follows.

Step 1: the receiving signal is transformed to frequency domain through FFT.

Step 2: the Receiving signal is correlated with the 64 hypothetical preamble sequences to get the channel estimation in frequency domain.

Step 3: the frequency domain channel estimation coefficients for each preamble sequence hypothesis are transferred to time domain through IFFT to get the power delay profile.

Step 4: for each preamble sequence hypothesis, subtract the PDP samples in a window duration with different start position related to Ncs and the index of candidate sequence with the same root index, the number of samples depends on Ncs and IFFT length. The sample with the maximum power in window duration is compared with a certain threshold to make sure the false alarm rate of PRACH below 0.001 in noise only scenario.

  - If there is at least one sample in the window is greater than the threshold, TA is estimated based on the location of the maximum sample, and the TA estimation error is less than half of the normal CP, the preamble is determined as transmitted.

  - If there is no sample in the window greater than the threshold, or the TA estimation error is greater than half of the normal CP, the preamble is determined as not transmitted.

Based on the assumptions and algorithms described above, the evaluation results are provided in the following different aspects:

- Cubic metric

The PAPR and CM in dB are provided in Table 3, where the cubic metric is calculated using the following equation, as stated in [6].

CM=20log10rmsvnorm3(t)-1.521.56dB

where 20log10rmsvnorm3(t) is the raw cubic metric (in dB) of a signal, and rmsx=(x'x)/N, vnormt=vt/rmsv(t).

Table 3. Cubic metric for each PRACH design alternative

|  | Contiguous PRB mapping: (Rel-15) | Contiguous PRB mapping: 2 ROs repeated in frequency domain | Non Uniformed PRB level interlaced mapping |
| --- | --- | --- | --- |
| 95% cubic metric(dB) | -0.2 | 1.5 | 1.7 |

- Miss-detection and false alarm probability

a)  miss-detection                                                               b) false-alarm

Figure 4 Performance of different PRACH design-without interference

As shown in Figure 4, the repeated RO can achieve the best performance. For the interlaced structure, the correlation property can not be guaranteed, especially in wireless channel with high frequency selectivity, the peak of PDP will be lower than that in narrower contiguous bandwidth, thus lead to a deteriorated performance for detection performance, even if in a relatively high SNR region.

- Overall comparison of different PRACH design

Table 4. Cubic metric for each PRACH design alternative

| Parameter | Value |  |  | Notes |
| --- | --- | --- | --- | --- |
| Scheme | Alt1-ZC139: Rel-15 | Alt2-ZC139: Non-uniform PRB-level interlace mapping | Alt4-ZC139X2: 2 ROs repeated in frequency domain | Eg. Alt4-ZC139x2 |
| SCS | 30KHz | 30KHz | 30KHz | 15KHz or 30KHz |
| PRACH sequence length (L_RA) | 139 | 139 | 139 | Eg. 139, |
| # of repetition (R) | 1 | 1 | 2 | If repetition of sequence is used in freq domain |
| N_cs | 10 | 10 | 10 | Eg. 11 |
| # of RBs used for one RO (N_RB) | 12 | 12 | 12 | # of RBs occupied by PRACH. Eg. 12 for ZC139 design |
| # of interlaces used by one RO (N_interlace) | 5 | 4 | 5 | # of uniform interlaces (M=5 for 30KHz and M=10 for 15KHz) with RBs used for one PRACH RO |
| RACH frequency occupancy (MHz) | 4.32 | 4.32 | 8.64 | The actually used bandwidth with one RO, SCS*L_RA*R |
| Noise level, Np (dBm) | -112.8 | -112.8 | -109.8 | Np= -174+10*log10(SCS*L_RA*R)+NF NF=-5dB |
| SNR (dB) | -4.4 | 0.72 | -9.1 | SNR needed at 1% misdetection, read from simulation curve |
| P_max (dBm) | 16.355 | 19.355 | 20.79 | Maximum allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used by the proposed scheme |
| Backoff (dB) | 0dB* | 1.7dB | 1.9dB | Backoff is computed as 95% percentile of CCDF of [cubic metric] over the preambles in the RO. Note: If cubic metric is not used, information on the backoff metric used should be provided. |
| P_TX (dBm) | 16.355 | 19.355 | 20.79 | P_TX=min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff |
| MCL (dB) | 133.555 | 131.435 | 139.69 | MCL = P_TX-SNR-Np |
| N_FDM | 4 | 4 | 1 | # of ROs in 20MHz |
| Capacity (preambles) | 7617 | 7617 | 1904 | Across all ROs in 20MHz. Should report any constraints on ISD for the scheme evaluated. |
| Note: The cubic metric of Rel-15 PRACH format is considered as the reference, the power back-off for other PRACH design is computed based on the cubic metric deviation calculation at 95% CCDF. |  |  |  |  |

Based on above table, it can be observed that, although uniform and non-uniform interlaced PRACH may be transmitted with higher max transmission power. However, the MCL of the interlaced PRACH design is still lower than that of contiguous scheme, since the advantage of higher maximum transmission power of interlaced design is neutralized by its inferior detection performance.

- Detection performance with interference

- In unsilenced spectrum, the transmission resource is obtained through contention procedure, where hidden node issue is inevitable. Therefore, the interference should be considered in physical channel design. In this section, the detection performance of different PRACH design is evaluated, where the waveform of interference is CP-OFDM and 16QAM modulated, the interference is appeared in whole sub band. The power of the interference is -3/0/3dB compared with the target PRACH signal.

  - PRB-level interlaced PRACH

a)  miss-detection                                                               b) false-alarm

Figure 5	Miss-detection and false alarm performance for Non-Uniformed PRB level interlace PRACH

  - Contiguous PRACH mapping(Rel-15)

a)  miss-detection                                                               b) false-alarm

Figure 6	Miss-detection and false alarm performance for Contiguous PRACH mapping

  - Repeated preamble in frequency domain

a)  miss-detection                                                               b) false-alarm

Figure 7	Miss-detection and false alarm performance for repeated preamble mapping in frequency domain

As shown in Figure 5-7, the miss detection rate of interlaced PRACH design is much worse than the contiguous PRACH. With the increment of the interference, the detection performance will degrade obviously, and the miss-detection rate is too high to be acceptable, which may severely degrade MSG1 receiving performance and delay the RACH process.

- Summary of the evaluation

Based on the simulation and analysis above, we can conclude that, contiguous PRACH mapping and PRACH repetition in frequency domain can achieve robust detection performance and timing accuracy compared with interlaced PRACH. Meanwhile, lower MCL can be identified even if the CM and the maximum transmission power due to PSD limitation are taken into consideration.

Besides, due to the non-uniformed PRB level interlace structure, it is difficult to multiplex PRACH and PUSCH, which is mapped in uniformed manner, in the same time resource. Therefore, the necessity to introduce the non-uniformed PRB level interlace is doubtful.

Observation 3: Contiguous PRACH mapping and PRACH repetition in frequency domain can achieve better detection performance and higher MCL compared with PRB level interlaced PRACH.

Proposal 10: Contiguous PRACH mapping and PRACH repetition in frequency domain based on Rel-15 preamble should be supported to ensure the detection performance of PRACH in NRU.

LBT Gap between RACH occasions

In NR Rel-15 back to back RACH Occasions in time can be configured. In NR-U, a UE has to pass LBT before transmitting RACH preamble. The UE may get blocked by a RACH transmission of another UE in the previous RACH occasion. To reduce the PRACH delay caused by the blockage issue, LBT Gap between RACH occasions is preferred. Two alternatives can be considered.

- Alt.1: Truncated PRACH format in a RO

As defined in Rel-15, PRACH is composed by CP and several times of repeated preamble in time domain, when L=139. More times of preamble repetition means lower miss detection rate and higher timing accuracy. However, considering the limited coverage in NRU scenario, the length of sequence part of PRACH, i.e. Nu, is not necessarily to be the same as that in Rel-15, therefore the PRACH in NRU can be truncated based on Rel-15 design. As shown in Figure 8, after the PRACH part truncated, LBT Gap is created between the PRACH transmissions in consecutive ROs. Assuming the SCS is 30 kHz, and 1 repetition of time domain sequence is truncated, 33.3us Gap can be created, the blockage rate can be lowered when CAT2 LBT is performed in a gNB initiated COT.

Figure 8	Truncated PRACH format

- Alt.2: Sparser RO in a RACH slot

Alternatively, LBT gap can be realized by mapping the ROs non-contiguously in time without truncating the PRACH transmission, sparser RO mapping pattern can be defined additionally. In Rel-15, the RACH occasions in a RACH slot/frame and periodicity of the RACH frame is indicated by the higher layer parameter prach-ConfigurationIndex, 255 configurations can be indicated, no extra entries is left for additional patterns. To avoid significant specification work, the sparser RO mapping pattern can be designed based on the existing configuration by muting some of the RACH instance. In other words, some resources in time-domain are defined as invalid. As shown in Figure 9, only the odd/even numbered RACH instances configured in a RACH slot are valid, LBT gap with at least 2 symbols can be left to perform LBT before PRACH transmission in each valid RACH instance. Both CAT2 and CAT4 LBT can be performed in this time interval.

Figure 9	Sparser RO in PRACH slot based on existing configuration

Proposal 11: To reduce the PRACH delay caused by the blockage issue, LBT Gap between RACH occasions should be supported. Following alternatives can be considered.

  - Alt.1: Truncated PRACH format in a RO.

  - Alt.2: Sparser RO in a RACH slot based on existing PRACH configuration.

- Conclusion

In this contribution, we focus on the design of DRS and PRACH in NR-U spectrum, and have the following proposals and observations:

Proposal 1: NR-U supports SSBs are at symbols (2,3,4,5) and (8,9,10,11) in the slot as NR Rel15.

Proposal 2: NR-U supports RMSI and/or CSI-RS is FDM with SSB in SA/DC/CA mode to meet OCB requirements.

Proposal 3: For NR-U, DRS includes one or more DRS units where each one comprises of at least one SSB, RMSI-CORESET+RMSI PDSCH in the same direction (omni or one beamforming direction).

Proposal 4: NR-U supports flexible configuration for time domain length of DRS unit, e.g. between 7 symbols and 14 symbols.

Proposal 5: NR-U supports flexible configuration of DRS period (e.g. {40ms, 80ms, 160ms}) and DRS window duration (e.g. {2ms, 4ms, 6ms, 8ms}).

Proposal 6: The Type0-PDCCH candidates associated with an SSB should be confined within a slot carrying the associated SSB and the Type0-PDCCH common search space configuration should be re-designed with more NRU related information conveyed.

Proposal 7: NR-U UEs shall assume RMSI rate matching around SSB, i.e. REs scheduled for the RMSI PDSCH reception and overlapping with SSB transmission are not used.

Proposal 8: NR-U UEs shall consider RMSI rate matching around CSI-RS based on configuration or predefined pattern, i.e. REs scheduled for the RMSI PDSCH reception and overlapping with CSI-RS transmission are not used.

Observation 1: Msg1 repetition in frequency domain is a straightforward way to meet OCB requirement and requires minor RAN1 spec efforts.

Observation 2: Long PRACH formats are needed only if a few tens of kilometres coverage is expected in unlicensed band.

Proposal 9: If it is allowed that PRACH can be transmitted without meeting the OCB regulatory, some PRACH formats can be excluded, e.g., format 0,1,2,3.

Observation 3: Contiguous PRACH mapping and PRACH repetition in frequency domain can achieve better detection performance and higher MCL compared with PRB level interlaced PRACH.

Proposal 10: Contiguous PRACH mapping and PRACH repetition in frequency domain based on Rel-15 preamble should be supported to ensure the detection performance of PRACH in NRU.

Proposal 11: To reduce the PRACH delay caused by the blockage issue, LBT Gap between RACH occasions should be supported. Following alternatives can be considered.

  - Alt.1: Truncated PRACH format in a RO.

  - Alt.2: Sparser RO in a RACH slot based on existing PRACH configuration.

- References

- RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Dec. 2018.

- TR 38.889, Study on NR-based Access to Unlicensed Spectrum, Dec. 2018.

- TS 38.213-f40, Physical layer procedures for control, Sep. 2018.

- RAN1 chairman notes, 3GPP TSG RAN WG1 #19AH1.

- R1-1900995, “Initial access signals and channels for NR-U”, Ericsson, RAN1 19AH1.

- R1-060385, “Cubic metric in 3GPP-LTE”, Motorola, RAN1#44, Denver, USA, Feb. 2006.

- Appendix

Agreement:

Companies are encouraged to provide results comparing the different alternatives using the following simulation assumptions to select between alternative PRACH designs.

- The Rel-15 PRACH design should be simulated as a baseline

| Property | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| Channel model | TDL-C |
| Delay scaling | 10ns, 100 ns |
| Antenna configuration at BS(1) | (M,N,P) = (1,1,2) with omni-directional antenna element |
| Antenna configuration at UE | Single omni-directional antenna element |
| Antenna port virtualization | No beamforming and no beam selection |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |
| UE speed | 3 km/h |
| Initial timing offset | Uniformly distributed in [0, 1.2 µs (corresponding to 300 m ISD)] Optional: Uniformly distributed in [0, 2 µs (corresponding to 500 m ISD)] |
| PRACH format | A1 with other formats optional |
| Subcarrier spacing | 15/30 kHz. (with other SCS optional) |
| PRACH sequence and frequency resource allocation | For evaluation purpose, the Rel-15 PRACH ZC sequence (with possible length change) should be simulated. Additional/new sequences can be simulated. Each company should provide details on the sequence (type and length) and the resource allocation (e.g., Alt1~Alt4 and detailed mapping). |
| Total number of preambles per cell | 64, each company should provide details on how these 64 preambles are generated |
| Preamble detector | Each company should provide details on used algorithm |
| Interference assumption | No interference. Optional: -3/0/3dB interference power compared with target PRACH |
| Detection Criteria | 1% maximum mis-detection probability(2) |
|  | 0.1% maximum false alarm probability(3) |
|  | maximum timing estimation error being 50% of the normal CP length |
| Formatting of results (please also reference Section 8 of R1-1704144 for reporting formats) | Mis-detection probability vs. SNR |
|  | False alarm probability vs. SNR(4) |
|  | CDF of timing estimation error |
|  | PRACH capacity (maximum number of preambles) |
|  | Peak-to-average power ratio and cubic metric |
|  | MCL(5) |
| (1) See Table 7-1 of R1-1704144 (2) The missed detection probability is defined as the ratio between the total number of transmitted preambles that are either not detected, or detected as a different preamble, or detected but with timing error greater than the maximum value (i.e., 50% of normal CP length), and the total number of transmitted preambles within an observation interval. (3) Maximum false alarm probability refers to the case when input at receiver is noise only (considering 64 preamble detectors as in 3GPP TS 36.104, section 8.4.1). (4) False alarm probability is defined as the ratio of total number detected but not transmitted preambles, and the total number of possible detection occurrences, where each occurrence (occurrence refers to 64 detections, one for each of the 64 preambles in a cell) is one potential preamble transmission in a RO. (5) In the MCL calculation, needs to consider the maximum transmit power supported by the PRACH design under PSD limitation and PAPR/EVM characteristic of the design. Note: Assumptions on the following should be stated - use of a guard band (if any) - definition of SNR - signal bandwidth used |  |