3GPP TSG RAN WG1 #96bis		R1-1904091

Xi’an, China, 8th – 12th April, 2019

Source:	vivo

Title:	Evaluation on spectral efficiency in Dense Urban-eMBB for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our evaluation results on spectral efficiency and user experienced data rate in Dense Urban-eMBB scenario for IMT-2020 self-evaluation.

- Evaluation on spectral efficiency

According to [1][2], evaluation on spectral efficiency involves 5th percentile user spectral efficiency and average spectral efficiency. The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. While average spectral efficiency is the aggregate throughput of all users divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP. The minimum requirements for 5th percentile user spectral efficiency and average spectral efficiency for various test environments are summarized in Table1 and Table2 respectively.

Table1. 5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 0.3 | 0.21 |
| Dense Urban – eMBB | 0.225 | 0.15 |
| Rural – eMBB | 0.12 | 0.045 |

Table2. Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 9 | 6.75 |
| Dense Urban – eMBB | 7.8 | 5.4 |
| Rural – eMBB | 3.3 | 1.6 |

In addition, user experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. In single band case, the user experienced data rate Ruser is given by:

Ruser = W × SEuser

Where W denotes the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency.

In band aggregation case, the user experienced data rate will be summed over the bands.

The requirements of the user experienced data rate in the Dense Urban – eMBB test environment are as below:

- Downlink user experienced data rate is 100 Mbit/s.

- Uplink user experienced data rate is 50 Mbit/s.

- Evaluation results

In this section, the initial evaluation results on downlink 5th percentile user spectral efficiency and average spectral efficiency are summarized in Table3 and Table4. The detailed evaluation assumptions are listed on Annex A.

Table3. Downlink 5th percentile user spectral efficiency for Dense Urban-eMBB(4GHz, FDD, channel model B)

| Test environment | Antenna configuration | 5th percentile user spectral efficiency (bit/s/Hz) | Requirement (bit/s/Hz) |
| --- | --- | --- | --- |
| Dense Urban – eMBB | 32Tx/4Rx | 0.3905 | 0.225 |

Table4. Downlink average spectral efficiency for Dense Urban-eMBB(4GHz, FDD, channel model B)

| Test environment | Antenna configuration | Average spectral efficiency (bit/s/Hz/TRxP) | Requirement (bit/s/Hz/TRxP) |
| --- | --- | --- | --- |
| Dense Urban – eMBB | 32Tx/4Rx | 13.1628 | 7.8 |

Observision1:

- For downlink Dense Urban-eMBB scenario, antenna configuration with 32Tx/4Rx can achieve the minimum requirements of 5th percentile user spectral efficiency and average spectral efficiency of IMT-2020.

Based on Table3, the minimum required bandwidth to achieve downlink target user experienced data rate is listed in Table5.

Table5. Downlink user experienced data rate for Dense Urban-eMBB(4GHz, FDD, channel model B)

| Test environment | Antenna configuration | Bandwidth (MHz) | Ruser (Mbps) | Requirement (Mbps) |
| --- | --- | --- | --- | --- |
| Dense Urban – eMBB | 32Tx/4Rx | 250 | 97.625 | 100 |
| Dense Urban – eMBB | 32Tx/4Rx | 300 | 117.15 |  |

Observision2:

- For downlink Dense Urban-eMBB scenario, when bandwidth larger than or equal to 300MHz, antenna configuration with 32Tx/4Rx can achieve the minimum requirement of user experienced data rate of IMT-2020.

The initial evaluation results on uplink 5th percentile user spectral efficiency and average spectral efficiency are summarized in Table6 and Table7. The detailed evaluation assumptions are listed on Annex B.

Table6. Uplink 5th percentile user spectral efficiency for Dense Urban-eMBB(4GHz, FDD, channel model B)

| Test environment | Antenna configuration | 5th percentile user spectral efficiency (bit//Hz) | Requirement (bit/s/Hz) |
| --- | --- | --- | --- |
| Dense Urban – eMBB | 2Tx/32Rx | 0.3651 | 0.15 |

Table7. Uplink average spectral efficiency for Dense Urban-eMBB(4GHz, FDD, channel model B)

| Test environment | Antenna configuration | Average spectral efficiency (bit/s/Hz/TRxP) | Requirement (bit/s/Hz/TRxP) |
| --- | --- | --- | --- |
| Dense Urban – eMBB | 2Tx/32Rx | 7.86281 | 5.4 |

Observision3:

- For uplink Dense Urban-eMBB scenario, antenna configuration with 2Tx/32Rx can achieve the minimum requirements of 5th percentile user spectral efficiency and average spectral efficiency of IMT-2020.

Based on Table6, the minimum required bandwidth to achieve uplink target user experienced data rate is listed in Table8.

Table8. Uplink user experienced data rate for Dense Urban-eMBB(4GHz, FDD, channel model B)

| Test environment | Antenna configuration | Bandwidth (MHz) | Ruser (Mbps) | Requirement (Mbps) |
| --- | --- | --- | --- | --- |
| Dense Urban – eMBB | 2Tx/32Rx | 100 | 36.51 | 50 |
| Dense Urban – eMBB | 2Tx/32Rx | 150 | 54.765 |  |

Observision4:

- For uplink Dense Urban-eMBB scenario, when bandwidth larger than or equal to 150MHz, antenna configuration with 2Tx/32Rx can achieve the minimum requirement of user experienced data rate of IMT-2020.

- Conclusion

In this contribution, we provide our evaluation results on spectral efficiency and user experienced data rate in Dense Urban-eMBB scenario for IMT-2020 self-evaluation with the following observations:

Observision1:

- For downlink Dense Urban-eMBB scenario, antenna configuration with 32Tx/4Rx can achieve the minimum requirements of 5th percentile user spectral efficiency and average spectral efficiency of IMT-2020.

Observision2:

- For downlink Dense Urban-eMBB scenario, when bandwidth larger than or equal to 300MHz, antenna configuration with 32Tx/4Rx can achieve the minimum requirement of user experienced data rate of IMT-2020.

Observision3:

- For uplink Dense Urban-eMBB scenario, antenna configuration with 2Tx/32Rx can achieve the minimum requirements of 5th percentile user spectral efficiency and average spectral efficiency of IMT-2020.

Observision4:

- For uplink Dense Urban-eMBB scenario, when bandwidth larger than or equal to 150MHz, antenna configuration with 2Tx/32Rx can achieve the minimum requirement of user experienced data rate of IMT-2020.

- References

- Report ITU-R M.2410-0, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, 2017/11

- Report ITU-R M.2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”, 2017/10

- Annex A: Downlink evaluation assumptions

Table9.Downlink evaluation assumptions

| Parameter | Value |
| --- | --- |
| Scenarios | Dense Urban (4GHz) |
| BS antenna configurations (M,N,P,Mg,Ng,Mp,Np) | (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) (dH, dV)=(0.5, 0.8)λ |
| MS antenna configurations | (M,N,P,Mg,Ng; Mp,Np)= (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ |
| Etilt angle | 105 degree |
| System bandwidth | 10MHz(52RBs) |
| UE attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Duplex | FDD |
| UE speed | 3km/h for indoor, 30km/h for out door |
| Traffic model | Full buffer |
| Receiver | Non-ideal channel estimation and interference modeling MMSE-IRC receiver |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | CQI, PMI and RI reporting triggered per 5ms Type II codebook,4 beams, WB+SB amplitude quantization, 8 PSK phase quantization Max 2 layers per UE Feedback delay is 2 ms |
| Transmission scheme | MU-MIMO with rank adaptation, MU dimension Up to 12 layers |
| Wrapping method | Geographical distance based |
| Metrics | Average UE throughput, 5% UE throughput. |
| Overhead | PDCCH 2 symbol, TRS (20ms period), DMRS Type 2, NZP CSI-RS for CM, ZP CSI-RS (4Port) for IM, 1 SSB / 20ms |

- Annex B: Uplink evaluation assumptions

Table10.Uplink evaluation assumptions

| Parameter | Value |
| --- | --- |
| Scenarios | Dense Urban (4GHz) |
| BS antenna configurations (M,N,P,Mg,Ng,Mp,Np) | (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) (dH, dV)=(0.5, 0.8)λ |
| MS antenna configurations | (M,N,P,Mg,Ng; Mp,Np)= (1,1,2,1,1; 1,1) |
| Etilt angle | 105 degree |
| System bandwidth | 10MHz(52RBs) |
| UE attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Duplex | FDD |
| UE speed | 3km/h for indoor, 30km/h for out door |
| Traffic model | Full buffer |
| Receiver | Non-ideal channel estimation and interference modeling MMSE-IRC receiver |
| Hybrid ARQ | Maximum 4 transmissions |
| Transmission scheme | UL SU-MIMO with rank adaptation |
| Wrapping method | Geographical distance based |
| Metrics | Average UE throughput, 5% UE throughput. |
| Overhead | 2 PRBs and 14 OS for the slots, DMRS Type 2, SRS 2 symbol per 5ms |