73GPP TSG RAN WG1 #96bis		R1-1904092

Xi’an, China, 8th – 12th April, 2019

Source:	vivo

Title:	Evaluation on spectral efficiency Indoor Hotspot-eMBB for IMT-2020 self-evaluation.docx

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our evaluation results on spectral efficiency and area traffic capacity in Indoor Hotspot-eMBB scenario for IMT-2020 self-evaluation.

- Evaluation on spectral efficiency

According to [1][2], evaluation on spectral efficiency involves 5th percentile user spectral efficiency and average spectral efficiency. The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. While average spectral efficiency is the aggregate throughput of all users divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP. The minimum requirements for 5th percentile user spectral efficiency and average spectral efficiency for various test environments are summarized in Table1 and Table2 respectively.

Table1. 5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 0.3 | 0.21 |
| Dense Urban – eMBB | 0.225 | 0.15 |
| Rural – eMBB | 0.12 | 0.045 |

Table2. Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 9 | 6.75 |
| Dense Urban – eMBB | 7.8 | 5.4 |
| Rural – eMBB | 3.3 | 1.6 |

In addition, area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through equation below.

Carea = ρ × W × SEavg

In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands.

This requirement is defined for the purpose of evaluation in the related eMBB test environment.

The target value for Area traffic capacity in downlink is 10 Mbit/s/m2 in the Indoor Hotspot – eMBB test environment.

- Evaluation results

In this section, the initial evaluation results on downlink 5th percentile user spectral efficiency and average spectral efficiency are summarized in Table3 and Table4. The detailed evaluation assumptions are listed on Annex A.

Table3. Downlink 5th percentile user spectral efficiency for Indoor Hotspot-eMBB(4GHz, FDD, channel model B)

| Test environment | Antenna configuration | 5th percentile user spectral efficiency (bit/s/Hz) | Requirement (bit/s/Hz) |
| --- | --- | --- | --- |
| Indoor Hotspot – eMBB (12 site) | 32Tx/4Rx | 0.4095 | 0.3 |

Table4. Downlink average spectral efficiency for Indoor Hotspot-eMBB(4GHz, FDD, channel model B)

| Test environment | Antenna configuration | Average spectral efficiency (bit/s/Hz/TRxP) | Requirement (bit/s/Hz/TRxP) |
| --- | --- | --- | --- |
| Indoor Hotspot – eMBB (12 site) | 32Tx/4Rx | 11.6546 | 9 |

Observision1:

- For downlink Indoor Hotspot-eMBB with 12 TRxP scenario, antenna configuration with 32Tx/4Rx can achieve the minimum requirements of 5th percentile user spectral efficiency and average spectral efficiency of IMT-2020.

Based on Table 4, the minimum required bandwidth to achieve downlink target area traffic capacity is listed in Table5.

Table5. Downlink area traffic capacity for Indoor Hotspot-eMBB(4GHz, FDD, channel model B)

| Test environment | Antenna configuration | Bandwidth (MHz) | area traffic capacity (Mbps/s/m2) | Requirement (Mbps/s/m2) |
| --- | --- | --- | --- | --- |
| Indoor Hotspot – eMBB (12 site) | 32Tx/4Rx | 400 | 9.32368 | 10 |
| Indoor Hotspot – eMBB (12 site) | 32Tx/4Rx | 450 | 10.48914 |  |

Observision2:

- For downlink Indoor Hotspot-eMBB with 12 TRxP scenario, when bandwidth larger than or equal to 450MHz, antenna configuration with 32Tx/4Rx can achieve the minimum requirement of area traffic capacity of IMT-2020.

The initial evaluation results on uplink 5th percentile user spectral efficiency and average spectral efficiency are summarized in Table6 and Table7. The detailed evaluation assumptions are listed on Annex B.

Table6. Uplink 5th percentile user spectral efficiency for Indoor Hotspot-eMBB(4GHz, FDD, channel model B)

| Test environment | Antenna configuration | 5th percentile user spectral efficiency (bit/s/Hz) | Requirement (bit/s/Hz) |
| --- | --- | --- | --- |
| Indoor Hotspot – eMBB (12 site) | 4Tx/32Rx | 0.5095 | 0.21 |

Table7. Uplink average spectral efficiency for Indoor Hotspot-eMBB(4GHz, FDD, channel model B)

| Test environment | Antenna configuration | Average spectral efficiency (bit/s/Hz/TRxP) | Requirement (bit/s/Hz/TRxP) |
| --- | --- | --- | --- |
| Indoor Hotspot – eMBB (12 site) | 4Tx/32Rx | 8.36715 | 6.75 |

Observision3:

- For uplink Indoor Hotspot-eMBB with 12 TRxP scenario, antenna configuration with 4Tx/32Rx can achieve the minimum requirements of 5th percentile user spectral efficiency and average spectral efficiency of IMT-2020.

- Conclusion

In this contribution, we provide our evaluation results on spectral efficiency and area traffic capacity in Indoor Hotspot-eMBB scenario for IMT-2020 self-evaluation with the following observations:

Observision1:

- For downlink Indoor Hotspot-eMBB with 12 TRxP scenario, antenna configuration with 32Tx/4Rx can achieve the minimum requirements of 5th percentile user spectral efficiency and average spectral efficiency of IMT-2020.

Observision2:

- For downlink Indoor Hotspot-eMBB with 12 TRxP scenario, when bandwidth larger than or equal to 450MHz, antenna configuration with 32Tx/4Rx can achieve the minimum requirement of area traffic capacity of IMT-2020.

Observision3:

- For uplink Indoor Hotspot-eMBB with 12 TRxP scenario, antenna configuration with 4Tx/32Rx can achieve the minimum requirements of 5th percentile user spectral efficiency and average spectral efficiency of IMT-2020.

- References

- Report ITU-R M.2410-0, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, 2017/11

- Report ITU-R M.2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”, 2017/10

- Annex A: Downlink evaluation assumptions

Table8.Downlink evaluation assumptions

| Parameter | Value |
| --- | --- |
| Scenarios | Indoor Hotspot (4GHz) |
| BS antenna configurations (M,N,P,Mg,Ng,Mp,Np) | Indoor Hotspot (12Site): 32TRxP = (4,4,2,1,1,4,4), (dH,dV) = (0.5, 0.5)λ |
| MS antenna configurations | (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2) (dH,dV) = (0.5, 0.5)λ |
| System bandwidth | 10MHz (52RBs) |
| UE attachment | Based on RSRP (formula as shown in Appendix 3 of RP-180524) from port 0 |
| Duplex | FDD |
| UE speed | 3km/h for indoor |
| Traffic model | Full buffer |
| Receiver | Non-ideal channel estimation and interference modeling MMSE-IRC receiver |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | CQI, PMI and RI reporting triggered per 5ms Type II CSI Codebook Mode 1 based, (L=4, 3bit WB Amp., 3bit SB Phase) Max 2 layers per UE Feedback delay is 2 ms |
| Transmission scheme | MU-MIMO with rank adaptation |
| Wrapping method | Geographical distance based |
| Metrics | Average UE throughput, 5% UE throughput. |
| Overhead | PDCCH 2 symbol, TRS (20ms period), DMRS Type 2, NZP CSI-RS for CM, ZP CSI-RS (4Port) for IM, 1 SSB / 20ms |

- Annex B: Uplink evaluation assumptions

Table9.Uplink evaluation assumptions

| Parameter | Value |
| --- | --- |
| Scenarios | Indoor Hotspot (4GHz) |
| BS antenna configurations (M,N,P,Mg,Ng,Mp,Np) | (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1; 2,8) (dH,dV) = (0.5, 0.5)λ |
| MS antenna configurations | (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2) (dH,dV) = (0.5, 0.5)λ |
| System bandwidth | 10MHz (52RBs) |
| UE attachment | Based on RSRP (formula as shown in Appendix 3 of RP-180524) from port 0 |
| Duplex | FDD |
| UE speed | 3km/h for indoor |
| Traffic model | Full buffer |
| Receiver | Non-ideal channel estimation and interference modeling MMSE-IRC receiver |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | CQI, PMI and RI reporting triggered per 5ms NR 4Tx codebook Max 2 layers per UE Feedback delay is 2 ms |
| Transmission scheme | UL SU-MIMO with rank adaptation |
| Wrapping method | Geographical distance based |
| Metrics | Average UE throughput, 5% UE throughput. |
| Overhead | 2 PRBs and 14 OS for the slots, DMRS Type 2, SRS 2 symbol per 5ms |