3GPP TSG RAN WG1 #96bis		                                                      R1-1904094

Xian, China, 8th April – 12th April, 2019

Source:	vivo

Title:	Evaluation on mobility in Indoor Hotspot-eMBB for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our mobility evaluation results in Indoor Hotspot-eMBB scenario for IMT-2020 self-evaluation. The CDF of UL pre-processing SINR and UL spectral efficiency results are provided based on [1][2].

- Evaluation methodology

The mobility requirements of IMT-2020 [1] on spectral efficiency in Indoor Hotspot-eMBB scenario are listed in Table 1. The evaluation methodology in [2] involves first generating median (50th percentile) uplink SINR from system-level simulations. Then, link-level simulations are used to verify that the required normalized traffic channel link data rates can be reached at the median SINR values.

Table 1. Traffic channel link data rates normalized by bandwidth

| Test environment | Normalized traffic channel link date rate (Bit/s/Hz) | Mobility(km/h) |
| --- | --- | --- |
| Indoor Hotspot-eMBB | 1.5 | 10 |

- Evaluation results

  - Pre-processing SINR CDF and median SINR

This section presents the pre-processing SINR CDF and the median SINR value in Indoor Hotspot-eMBB scenario.

The preliminary UL pre-processing SINR results for indoor Hotspot-eMBB scenario are shown in Figure 1, the median pre-processing SINR value is given in Table 2. System level simulation assumptions are listed in Appendix I.

Figure 1: UL pre-processing SINR distribution for Indoor Hotspot-eMBB with channel model B

Table 2. UL median pre-processing SINR

| Test environment | Configuration | Channel Model | 50%-tile UL pre-processing SINR(dB) |
| --- | --- | --- | --- |
| Indoor Hotspot-eMBB | Config A | Model B | 4.33 |

  - UL spectral efficiency

By using the pre-processing SINR provided in the Table 2, the preliminary UL spectral efficiency results for mobility in Indoor Hotspot-eMBB scenario can be achieved. The results are list in Table 3.

Table 3. Mobility traffic channel link data rates

| Test environment | Configuration | Channel Model | Median UL SINR[dB] | ITU Requirement [bps/Hz] | Spectrum efficiency of NLOS | Spectrum efficiency of LOS | Fulfil the requirements |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | FDD |  |  |
| Indoor Hotspot-eMBB | Config A 4GHz | Model B | 4.33 | 1.5 | 1.88 | 2.18 | Y |

Observision1:

- In Indoor Hotspot-eMBB scenario, NR can achieve the minimum requirements of mobility traffic channel link data rates of IMT-2020.

- Conclusion

In this contribution, we provide our evaluation results on mobility traffic channel link data rates in Indoor Hotspot-eMBB for IMT-2020 self-evaluation with the following observations:

Observision1:

- In Indoor Hotspot-eMBB scenario, NR can achieve the minimum requirements of mobility traffic channel link data rates of IMT-2020.

- References

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- Appendix I:  Simulation assumption of SLS part for mobility

The simulation assumption of system level part for mobility evaluation is listed in Table A1-1.

Table A1-1. Simulation assumptions in SLS

| Parameters | Value |
| --- | --- |
| Test environment | Indoor Hotspot-eMBB |
| Evaluation configuration | Configuration A |
| Carrier frequency for evaluation | 4 GHz |
| Multiple access | OFDMA |
| Duplexing | FDD |
| Transmission scheme | SU-MIMO with rank 1 |
| BS antenna height | 3 m |
| BS noise figure | 5 dB |
| UE noise figure | 7 dB |
| Total transmit power per TRxP | 21 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm |
| Inter-site distance | 20 m |
| Number of antenna elements per TRxP | 32 Tx/Rx, (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) |
| Number of UE antenna elements | 1Tx/Rx, (M,N,P,Mg,Ng) = (1,1,1,1,1) |
| Device deployment | 100% indoor, Randomly and uniformly distributed over the area |
| UE speeds of interest | 10 km/h |
| Traffic model | Full buffer |
| Simulation bandwidth | 10 MHz |
| UE density | 10 UEs per TRxP |
| UE antenna height | 1.5 m |
| Channel model variant | Channel model B |
| TRxP number per site | 1 |
| Mechanic tilt | 180° in GCS (pointing to the ground) |
| Electronic tilt | 90° in LCS |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | No wrapping around |
| Polarized antenna model | Model-2 in TR36.873 |
| Power control parameters | a = 0.6, P0 = -60 dBm |
| Numerology | 15 kHz SCS |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Pre-processing SINR calculation | Aligned with Section 2.1.1 in R1-1805643 |

- Appendix Ⅱ: Additional LLS assumption for Indoor Hotspot-emBB scenario

Table A1-2. The additional link level simulation assumption for indoor Hotspot-eMBB environment

| Parameters | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Waveform | CP-OFDM |
| Duplexing | FDD |
| Simulation bandwidth | 10 MHz |
| Link-level Channel model | CDL-i |
|  | CDL-iv |
| UE speed | 10 km/h |
| Subcarrier spacing | 15kHz |
| Symbols number per slot | 14 |
| Antenna configuration at TRxP | 8R,(4,4,2,1,1;1,4) |
| Antenna configuration at UE | 1T, (1,1,1,1,1;1,1) |
| Transmission rank | 1 |
| TRxP receiver type | MMSE-IRC |
| Channel estimation | Real |
| Number of subcarriers per PRB | 12 |
| Data allocation | 14 symbol slots, with 12 RB allocated |
| Channel coding scheme | LDPC |
| Link adaptation | Link adaptation |
| HARQ | Max 4 HARQ transmissions |
| DMRS configuration | 2 symbol DMRS(front loaded and one additional) |
| Other overhead | SRS: 2 symbols per 5 slots PUCCH:2 RBs in 10 MHz bandwidth |