3GPP TSG RAN WG1 Meeting #96b                                                         	          R1-1904286

Xi'an, China, April 8th – April 12th, 2019

Agenda Item:	7.2.2.2.2

Source: 	Intel Corporation

Title:	Enhancements to initial access and mobility for NR-unlicensed

Document for: 	Discussion/Decision

# Introduction

In this contribution, the following aspects related to initial access and mobility procedures are addressed:

- Frame timing and QCL determination

  - PBCH decoding for RRM

  - UE complexity for Alt-1b

- Shift granularity

- RLM measurements

  - RLM measurements based on SSB

  - RLM measurements based on CSI-RS

  - Addressing LBT failure for RLM

- RRM

- Enhancements to 4-step random access procedure

# Frame timing and QCL determination

Agreement: - For a given cell, UE may assume SS/PBCH blocks in the same candidate position within the DRS transmission window are QCL across DRS transmission windows - Alt1: The PBCH DMRS sequence index is also the same - Alt2: The PBCH DMRS sequence index may be different - Note: The first candidate position of the DRS transmission window is located at the first half slot of a half frame - FFS: QCL assumption for SSBs in different candidate positions within a DRS transmission window and across DRS transmission windows

We introduce the following notation that was generally used in discussions pre and post RAN1#96.

Notation

|  |  | Values (example) |
| --- | --- | --- |
| t or y | Time-index or SSB candidate positions within 5 ms | 0,…,19 |
| c | Cycle index | 0, 1, 2, 3 |
| s | PBCH-DMRS index | 0, 1, …, 7 |
| b | Beam index | 0, 1, …, 7 |
| Q | Number of beams used in the cell | 1, 2,…, 8 |
| Stx | Total number of DMRS sequences used in the cell | 8, 5, 6, 7 |

Based on the above notation, the following describes some of alternatives from RAN1#96:

| Alt-1a | Time index t=8*c+s, c = 0, 1, 2 cycle index in PBCH, s = 0, …,7 is PBCH-DMRS index Beam index b = mod(8*c+s, Q), Q is number of beams, Q is carried in PBCH or RMSI FFS: inclusion of half-frame indicator bit. |
| --- | --- |
| Alt-1b | Time index t=Stx*c+s, Beam index b = mod(s, Q) Q; Stx; c. 1, 2, 4, 8; 8; 0, 1, 2. 3, 6; 6; 0, 1, 2, 3. 5; 5; 0, 1, 2, 3. 7; 7; 0, 1, 2. |
| Alt-2 | Timing information of the potential SS/PBCH block location in a DRS window shall be carried by the DMRS of PBCH in the corresponding SS/PBCH block. |

Comments:

- For Alt-1a, determination of beam index requires cycle index from PBCH that requires PBCH decoding. This was discussed as a drawback for RRM measurements

- For Alt-1b, beam index can be derived from PBCH-DMRS ‘s’ and Q that is made available to UE. This was discussed as an advantage for RRM measurements.

- For Alt-2, the motivation is to reduce PBCH soft-combining hypotheses – however, it limits to 16 SSB candidate positions instead of 20 and PBCH-DMRS detection performance is not available.

- Alt-1a and Alt-1b are equivalent for Q= 1, 2, 4, 8. From Table 1 (Appendix) we note that the relation between beam index ‘b’ and PBCH-DMRS ‘s’ does not depend on ‘c’. Therefore we can simply use b = mod(s, Q) for beam index determination in this case.

## PBCH decoding for RRM

According to Rel-15 NR, for intra-frequency measurements a UE shall be able to identify a new detectable intra-frequency cell without an explicit time allowance for PBCH detection if UE is not indicated to report SSB based RRM measurement result or UE has been indicated neighbour cell is synchronous with serving cell. For inter-frequency measurements a UE shall be able to identify a new detectable inter-frequency cell without an explicit time allowance for PBCH detection if the UE is not indicated to report SSB based RRM measurement result.

Note that for intra-frequency synchronous RRM, a UE can determine beam index since timing is known. Also synchronous RRM is assumed for licensed TDD FR1 bands. Also note that for asynchronous RRM there is no requirement for the UE to determine beam-index b. It is sufficient if the UE is able to average measurements across SSB instances that are QCL.

From Table 2 and Table 3 (Appendix) we can show that for all Q, if PBCH-DMRS st1, st2 are QCL, they are related by:

st2 = (st1 + k*Q) mod 8, where k is an integer and k=(t2-t1)/Q. The time difference between measurements (t2-t1) is known at the UE. Therefore if Q is known, this allows the UE to average SSBs that are QCL from PBCH-DMRS detection only (although the beam index b is not determined). This is shown in Figure 1 below.

Figure 1: Shows that in Alt1a for Q=3, the PBCH-DMRS index can be determined based on the difference in the time-index of the measured SSBs between two SMTC windows.

Therefore we have the following:

Proposal-1: Frame timing determination (Alt-1a): Time index t=8*c+s, Beam index determination: Beam index b = mod(8*c+s, Q), Q = 1, 2, …, 8.

Observation-2: In licensed band FR1, timing is assumed to be aligned in TDD. For asynchronous RRM measurements beam-index is not required to be reported without PBCH decoding.

Observation-3: Alt-1a allows RRM measurement and filtering within the same beam –index without PBCH decoding for all Q values and if Q is known.

## UE complexity for Alt-1b

Figure 11: (a) For Alt1-b when Q is unknown, hypotheses for PBCH-DMRS increases for a given time-index t determined using PSS/SSS detection (from 1 to 4 in certain cases as highlighted above) which can be challenging for RRM measurements/initial access for certain implementations. (b) for certain UE implementations following Rel-15 NR soft-combining principles where PBCH soft combining is feasible within the same cycle index c, Alt-b (for example Stx=5) can reduce opportunities for PBCH soft-combining.

Assuming PBCH-DMRS detection based on accumulation corresponding to 2 PSS/SSS peaks and as an example considering PSS/SSS peaks separated by 8 slots, we have 8 hypotheses for Alt-1a. We can observe for Alt-1b, for the same case, the number of PBCH-DMRS detection hypotheses is 34. Note that this accumulation is applicable to both intra-DRS and inter-DRS transmission window case.

Table 1: For Alt-1a PBCH-DMRS hypotheses for corresponding to two PSS/SSS peaks separated by 8 slots

| First PSS/SSS peak | s=0 | s=1 | s=2 | s=3 | s=4 | s=5 | s=6 | s=7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Second PSS/SSS peak | s=0 | s=1 | s=2 | s=3 | s=4 | s=5 | s=6 | s=7 |

Table 2: For Alt-1b PBCH-DMRS hypotheses for corresponding to two PSS/SSS peaks separated by 8 slots

| First PSS/SSS peak | s=0 | s=1 | s=2 | s=3 | s=4 | s=5 | s=6 | s=7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Second PSS/SSS peak | s=0,1,2,3,4 | s=1,2,3,4,5 | s=0,2,3,4 | s=1,3,4,5 | s=0,2,4,5 | s=1,3,5,6 | s=0,2,4,6 | s=0,1,3,7 |

# Shift granularity

Figure 9: Top figure shows sample time hypotheses for intra-DRS accumulation of 2 SSBs where shift granularity is 1 slot. Bottom figure shows that an additional hypotheses should be considered for the case if shift granularity is 1/2 slot.

Note that sample time hypotheses is a function of the number of OFDM symbols as well as symbol length. For 30 kHz, symbol 0 in the slot is a long symbol. Considering intra-DRS accumulation of 2 SSBs:

Sample time hypotheses for 1-slot granularity  t1, t1’, t2, t3

Sample time hypotheses for 1/2-slot granularity  t1, t1’, t2, t3, t3’

Similar increase is expected considering accumulation of 3 or 4 SSBs.

Observation-4: A shift granularity of ~0.5 slot increases sample time hypotheses considered at the UE for intra-DRS accumulation. A shift granularity of ~0.5 also implies that RMSI resources are limited to 0.5 slot.

# RLM

Agreement: - An RLM measurement window for serving cell RLM measurements based on SSBs in the DRS is supported for in-sync and out-of-sync evaluations. - FFS: How RLM measurement window is indicated or determined and relation to DRS transmission window - FFS: Whether or not an SSB can fall outside the measurement window and, if so, whether it can be used for in-sync and out-of-sync evaluations. - FFS: Any relationship of RLM measurements based on CSI-RS to the measurement window. - FFS: Mechanism to handle missing RLM-RS due to LBT failure

## RLM measurements based on SSB

According to Rel-15 NR UE behaviour, RLM measurements based on SSB is configured by RadioLinkMonitoringRS. Assuming the same mechanism is applicable to NR-U, in this case RLM measurement window for serving cell can be determined by the UE from frame timing and half-frame bit. Therefore a window definition is not essential.

In case the serving cell is coordinating with other cells regarding DRS transmission it is possible that a shorter DRS transmission window may be applicable – this can be applied at the gNB without specification support. Indicating this window to the UE may allow the UE to not monitor/search for CORESET#0/SSBs outside this window.

Proposal-5: For RLM based on SSB, DRS transmission window is determined at the UE from frame-timing and half-frame bit. A RLM measurement window definition is not necessary. The benefit of defining an RLM measurement window that is smaller than the DRS transmission window seems to be relatively small.

## RLM measurements based on CSI-RS

According to Rel-15 NR UE behaviour, RLM measurements based on CSI-RS can be configured using RadioLinkMonitoringRS. If RadioLinkMonitoringRS is not configured and TCI states are configured for PDCCH CORESETs, RLM measurements are based on CSI-RS-TRS or CSI-RS-CSI that are configured in those TCI states. Such CSI-RS is periodic and no window is configured for RLM measurements.

Using the same mechanism in NR-U is not as efficient because periodic RS cannot be guaranteed due to LBT requirements. Also note that TRS or CSI-RS-CSI is expected to be configured for PDSCH/PDCCH reception in Rel-15 NR for fine time-frequency-tracking purposes (in the absence of CRS as in LTE). Therefore we believe AP-TRS is essential and should be used as part of DRS transmission that will also be beneficial for meeting OCB requirements and to guarantee time-continuity of DRS transmission in certain cases. However, AP-TRS in Rel-15 NR can be indicated in a UE-specific DCI. This is inefficient in NR-U due to the limited capacity of CORESET#0 in DRS. Therefore we have the following proposal:

Proposal-6: Consider mechanisms of aperiodic TRS triggering as part of DRS with low DCI overhead. This can be achieved by a broadcast or group-cast TRS trigger.

## Addressing LBT failure for RLM

Radio link monitoring (RLM) in NR Rel-15 relies on in –sync (IS)/out-of-sync (OOS) indications based on RLM-RS based link quality measurements where RLM-RS may take the form of SSB and/or CSI-RS. The physical layer at a UE indicates an OOS indication to higher layers if the radio link quality of all the configured RLM-RS resources is below a pre-defined threshold (Thout-of-sync). On the other hand, L1 indicates an IS indication to higher layers if the radio link quality is better than another pre-defined threshold (Thin-sync) for any resource in the set of RLM-RS resources configured for radio link monitoring.

In the case of standalone NR-U operation, RLM/RLF (radio link failure) mechanism should consider the uncertainty of channel access by LBT operation. In particular, a determination of link quality below Thout-of-sync may occur either due to the absence of RLM-RS (LBT failure) or due to poor radio link quality. Therefore, following the RLM procedure of a licensed carrier will lead to OOS indications when an RLM-RS is dropped due to LBT failure.

One way to mitigate the aforementioned problem is to enable a mechanism that would reduce LBT failure probability of RLM-RS.

In order to improve the probability of successful LBT, RLM-RS may be included as part of NR-U DRS. Additionally RLM measurements can be restricted within a certain window (RLM-DMTC) such that the probability of false OOS indications is reduced assuming that the RS transmission inside DRS transmission window can utilize Cat-2 LBT. Note, however, that the cost of always restricting RLM OOS measurement to DRS occasions can lead to a slower RLM process when the medium is not congested. The gNB should be able to configure a window RLM OOS window for a UE.

The main issue with respect to RLM is that dropped RLM-RS transmission occasions due to LBT failure may trigger RLF that is undesired. In this case, for an evaluation period, a UE may determine whether IS or OOS indication can be determined with sufficient confidence and accuracy. For example, if RLM-RS is defined as a SS/PBCH block index, a UE may perform PSS/SSS/PBCH detection to determine successful RLM-RS transmission for a particular transmission instance. A UE may also consider information such as remaining COT information to determine RLM-RS presence. If the number of successful RLM-RS presence detection within an evaluation period is 0 or small, then a UE may indicate a third indicator type (say failed to detect – FTD indication) to higher layers. This is feasible if the required SINR for OOS indication is much higher than the required SINR for PSS/SSS/PBCH detection which is generally true.

Figure 7: Detection of RLM-RS for usage in OOS/IS evaluation is possible if RLM-RS can be detected at a lower SINR than OOS detection or from acquired side information (like remaining COT time etc.)

Proposal-7: Consider indicating a third indicator type (e.g. failed to detect – FTD indication) to higher layers in addition to IS and OOS indications.

# RRM

In Rel-15 NR, CSI-RS can be configured for RRM measurements. In order to ensure the same functionality in NR-U it is beneficial to allow CSI-RS transmission in DRS for RRM measurements. This will allow such CSI-RS transmission with high priority and will also be beneficial for meeting OCB requirements and to guarantee time-continuity of DRS transmission in certain cases. From a UE perspective, it can measure such CSI-RS using a SMTC window (or something similar). Therefore we have the following proposal:

Proposal-8: Consider configuration of CSI-RS based RRM measurements where CSI-RS is transmitted as part of DRS transmission.

Also, note that the RMTC measurement framework introduced in LTE was motivated by the need to capture load conditions for neighbouring cells, capture interference outside the downlink burst and capture impact of hidden nodes. The same motivations apply for NR and we may consider using the same RMTC measurement framework as in LTE.

Proposal-9: Consider re-using RMTC measurement framework as in LTE for average RSSI measurement. Consider re-using channel occupancy reporting as for LTE in NR-U.

# Enhancements to the 4-step random access procedure

In previous RAN1 meetings during the study on NR-based access to unlicensed spectrum the following are some of the agreements were made related to initial access and mobility:

Agreement (RAN1#94bis):

Following options have been identified for potential RACH resource enhancements in NR-U beyond the flexibility already available in Rel-15 NR:

- Frequency-domain enhancement

  - Multiple PRACH resources across multiple LBT sub-bands/carriers for both contention-free and contention-based RA

- Time-domain enhancements

  - For connected mode UE, scheduling of PRACH resources via DCI.

    - Triggered PRACH within TXOP can use a new resource

  - For idle mode UE, scheduling of PRACH resources via paging

    - Note: potential inefficiency in network resource due to paging across multiple cells

  - Additional, new RACH resources are used immediately following detection of DRS transmission

  - Multiple PRACH transmissions before Msg2 reception in RAR window for initial access

    - Number of allowed transmissions is pre-defined or indicated, e.g., in RMSI

    - FFS: How to handle potential multiple RARs to same UE

  - Group wise SSB-to-RO mapping by frequency first-time second manner, where grouping is in time domain

Release 15 NR uses 4-step random access procedure like LTE. The 4-step random access procedure consists of Msg-1 (PRACH), Msg-2 (RAR), Msg-3 (RRC connection request) and Msg-4 (contention resolution) transmissions. As per TR38.889, 4 –step RACH procedure can be considered as the baseline for NR-unlicensed and mechanisms to handle LBT related aspects (e.g. reduce transmission opportunities for Msg. 1/2/3/4) can be further studied. Further, several frequency and time domain enhancements were identified in TR38.889 as potential options to RACH resource enhancements for unlicensed operation, as mentioned in the above agreement. In this regard, the following enhancements are envisioned for 4-step random access procedure.

  - Multiple PRACH resource configurations over multiple configured initial UL BWPs

The following agreement was made in RAN1 in the past regarding channel access mechanism during initial access:

Agreement:

- Initial active DL/UL BWP is approximately 20MHz for 5GHz band

  - The final value will be quantized to number of PRBs

- Initial active DL/UL BWP is approximately 20MHz for 6GHz band if similar channelization as 5GHz band is used for 6GHz band

As per the above agreement, the initial active UL BWP for 5 GHz band is subject to one sub-band LBT (20 MHz). Consider that a single PRACH resource is configured within one configured initial UL BWP to an idle mode UE for initial access. If 20 MHz spectrum of the UL BWP that includes the PRACH resource for CBRA is occupied by other neighbouring network, the preamble transmission has to be delayed to the next occasion and, thereby, the overall initial access delay will increase.

In order to overcome the abovementioned issue, it can be considered that gNB can configure multiple initial BWPs, each with a PRACH resource configuration, as illustrated in the figure below. The configured BWPs may be contiguous or non-contiguous across frequency. Out of these multiple configured BWPs, one single UL BWP is activated based on LBT outcome (each BWP being subject to independent 20 MHz sub-band LBT) at the UE side and UE transmits PRACH on this single activated BWP. In case CCA is successful on multiple configured BWPs, UE selects one BWP out of these multiple candidates as the initial active UL BWP and use the corresponding PRACH resource to transmit the preamble.

Figure 6. Multiple PRACH resource configurations in frequency domain on multiple configured initial UL BWPs

Proposal-10: NR-unlicensed supports multiple PRACH resource configurations in the frequency domain on multiple configured UL BWPs.

# Conclusions

In this contribution, we made the following proposals/observations:

Proposal-1: Frame timing determination (Alt-1a): Time index t=8*c+s, Beam index determination: Beam index b = mod(8*c+s, Q), Q = 1, 2, …, 8.

Observation-2: In licensed band FR1, timing is assumed to be aligned in TDD. For asynchronous RRM measurements beam-index is not required to be reported without PBCH decoding.

Observation-3: Alt-1a allows RRM measurement and filtering within the same beam –index without PBCH decoding for all Q values and if Q is known.

Observation-4: A shift granularity of ~0.5 slot increases sample time hypotheses considered at the UE for intra-DRS accumulation. A shift granularity of ~0.5 also implies that RMSI resources are limited to 0.5 slot.

Proposal-5: For RLM based on SSB, DRS transmission window is determined at the UE from frame-timing and half-frame bit. A RLM measurement window definition is not necessary. The benefit of defining an RLM measurement window that is smaller than the DRS transmission window seems to be relatively small.

Proposal-6: Consider mechanisms of aperiodic TRS triggering as part of DRS with low DCI overhead. This can be achieved by a broadcast or group-cast TRS trigger.

Proposal-7: Consider indicating a third indicator type (e.g. failed to detect – FTD indication) to higher layers in addition to IS and OOS indications.

Proposal-8: Consider configuration of CSI-RS based RRM measurements where CSI-RS is transmitted as part of DRS transmission.

Proposal-9: Consider re-using RMTC measurement framework as in LTE for average RSSI measurement. Consider re-using channel occupancy reporting as for LTE in NR-U.

Proposal-10: NR-unlicensed supports multiple PRACH resource configurations in the frequency domain on multiple configured UL BWPs.

# References

[1] RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”, Qualcomm Inc, 3GPP TSG RAN Meeting #82, Dec. 2018.

[2] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #96, February, 2019.

# Appendix

Table 3: For Alt-1a or Alt-1b, values of c, s, b for Q=1, 2, 4 and 8

| c | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| t | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| s | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 |
| Q=8, b | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 |
| Q=4, b | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| Q=2, b | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| Q=1, b | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 4: For Alt-1a values of c, s, b for Q=3, 5, 6 and 7

| c | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| t | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| s | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 |
| Q=3, b | 0 | 1 | 2 | 0 | 1 | 2 | 0 | 1 | 2 | 0 | 1 | 2 | 0 | 1 | 2 | 0 | 1 | 2 | 0 | 1 |
| Q=5, b | 0 | 1 | 2 | 3 | 4 | 0 | 1 | 2 | 3 | 4 | 0 | 1 | 2 | 3 | 4 | 0 | 1 | 2 | 3 | 4 |
| Q=6, b | 0 | 1 | 2 | 3 | 4 | 5 | 0 | 1 | 2 | 3 | 4 | 5 | 0 | 1 | 2 | 3 | 4 | 5 | 0 | 1 |
| Q=7, b | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 0 | 1 | 2 | 3 | 4 | 5 |

Table 5: For Alt-1a list of s values observed for RRM measurements for Q = 3, 5, 6 and 7

|  | st1 | st2 | st3 | st4 | st5 | st6 | st7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| b=0, Q=3 | 0 | 3 | 6 | 1 | 4 | 7 | 2 |
| b=1, Q=3 | 1 | 4 | 7 | 2 | 5 | 0 | 3 |
| b=2, Q=3 | 2 | 5 | 0 | 3 | 6 | 1 |  |
| b=0, Q=5 | 0 | 5 | 2 | 7 |  |  |  |
| b=1, Q=5 | 1 | 6 | 3 | 0 |  |  |  |
| b=2, Q=5 | 2 | 7 | 4 | 1 |  |  |  |
| b=3, Q=5 | 3 | 0 | 5 | 2 |  |  |  |
| b=4, Q=5 | 4 | 1 | 6 | 3 |  |  |  |
| b=0, Q=6 | 0 | 6 | 4 | 2 |  |  |  |
| b=1, Q=6 | 1 | 7 | 5 | 3 |  |  |  |
| b=2, Q=6 | 2 | 0 | 6 |  |  |  |  |
| b=3, Q=6 | 3 | 1 | 7 |  |  |  |  |
| b=4, Q=6 | 4 | 2 | 0 |  |  |  |  |
| b=5, Q=6 | 5 | 3 | 1 |  |  |  |  |
| b=0, Q=7 | 0 | 7 | 6 |  |  |  |  |
| b=1, Q=7 | 1 | 0 | 7 |  |  |  |  |
| b=2, Q=7 | 2 | 1 | 0 |  |  |  |  |
| b=3, Q=7 | 3 | 2 | 1 |  |  |  |  |
| b=4, Q=7 | 4 | 3 | 2 |  |  |  |  |
| b=5, Q=7 | 5 | 4 | 3 |  |  |  |  |
| b=6, Q=7 | 6 | 5 |  |  |  |  |  |