3GPP TSG RAN WG1 #96b                                                                  R1-1904303

Xi'an, China, 8th – 12th April, 2019

Source: 	Intel Corporation

Title:	Discussion on control procedures for NTN

Agenda item:	7.2.5.2

Document for:	Discussion and Decision

# Introduction

At the RAN#80 meeting the new study item on Non Terrestrial Networks (NTN) was agreed [1], the latest study item description can be found in [2]. Objectives of the study item related to physical layer are copied below.

Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed [RAN1]: - Physical layer control procedures (e.g. CSI feedback, power control) - Uplink Timing advance/RACH procedure including PRACH sequence/format/message - Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms. Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1]

In this contribution physical layer control procedures for NTN are discussed including UL power control and CSI feedback procedure.

# Discussion

## 2.1. UL power control

Considering large cell size for NTN (up to 1000 km for GEO satellites), difference in altitude for NTN terminals (e.g. sea level altitude for a high speed train and 10000 km altitude for an airplane) and different antenna gain for different UE positions within the cell, UL signal power at the receiver onboard of spaceborne vehicle can differ significantly for different UEs. In order to equalize UL signal power for different UEs open-loop power control should be used for NTN. Open-loop power control allows to compensate path loss by adjusting UE transmission power according to path loss measured from DL reference signals configured by the gNB. Also open-loop power control allows to adjust UL signal power to achieve target SNR level (operation point) using higher-layer configuration. Since DL and UL carrier frequency may be significantly different for NTN (e.g. around 20 GHz for DL around 30 GHz for UL in Ka band) the difference of path loss should be also compensated by higher layer open-loop power control parameters considering possibility of additional attenuation due to atmospheric effects for different carrier frequencies.

Proposal 1:

- Open-loop power control should be used for NTN to compensate path loss and adjust signal power to target level (operation point) via higher-layer configuration

In addition to open-loop power control, closed loop power control can be used to dynamically adjust UL transmission power to track channel variation or adjust UL interference level. Due to specific factors such as long propagation delay and/or high mobility of NTN terminal and spaceborne vehicle, closed loop power control may require enhancements. For example, extension of range of UL transmission power variation indicated by one TPC command can be considered to reduce number of TPC command transmissions.

Observation 1:

- Enhancements of closed-loop power control may be needed considering long propagation delay and/or high mobility of NTN terminal and spaceborne vehicle

  - Extension of range of UL transmission power variation indicated by one TPC command can be considered to reduce number of TPC command transmissions

## 2.2. CSI feedback

CSI is used in cellular system in order to assist precoding, link adaptation and scheduling operations. While for cellular systems CQI provided by the UE via CSI feedback allows to properly select modulation and coding scheme (MCS) for DL transmission with a given target BLER, for NTN CQI received by the spaceborne vehicle may be outdated due to channel aging considering high propagation delays and high mobility of NTN terminal and spaceborne vehicle. Considering the above, reliability of CSI feedback should be further studied by RAN1.

Proposal 2:

- Study the reliability of CSI feedback for NTN considering large delays and high mobility of NTN terminal and spaceborne vehicle

In some cases variation of signal power at the receiver of NTN terminal may be predicted based on information of NTN terminal mobility and mobility of spaceborne vehicle. In case such information is available at the gNB, MCS for DL transmission can be predicted based on the previous CQI feedback from the NTN terminal.

Proposal 3:

- Consider MCS prediction at the gNB using information on mobility of NTN terminal and spaceborne vehicle

# Conclusion

In this contribution physical layer control procedures for NTN are discussed including UL power control and CSI feedback. The following proposals and observations were made.

Proposal 1:

- Open-loop power control should be used for NTN to compensate path loss and adjust signal power to target level (operation point) via higher-layer configuration

Observation 1:

- Enhancements of closed-loop power control may be needed considering long propagation delay and/or high mobility of NTN terminal and spaceborne vehicle

  - Extension of range of UL transmission power variation indicated by one TPC command can be considered to reduce number of TPC command transmissions

Proposal 2:

- Study the reliability of CSI feedback for NTN considering large delays and high mobility of NTN terminal and spaceborne vehicle

Proposal 3:

- Consider MCS prediction at the gNB using information on mobility of NTN terminal and spaceborne vehicle

# References

- RP-181999 Report of 3GPP TSG RAN meeting #80, La Jolla, USA, June 11 – 14, 2018

- RP-182880 Study on solutions for NR to support non-terrestrial networks (NTN), Thales

- 3GPP TS 38.213 V15.4.0 (2018-12) 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical layer procedures for control (Release 15)

- 3GPP TR 38.811 V15.0.0 (2018-06) 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on New Radio (NR) to support non terrestrial networks (Release 15)