3GPP TSG RAN WG1 #96	R1-1904465

Xi’an, China, 8th – 12th April, 2019

Agenda item:	7.2.7.1

Source:	Samsung

Title:	Discussion on evaluation methodology for Dense Urban eMBB scenario

Document for:	Discussion and Decision

# Introduction

This contribution discusses about the evaluation methodology for Dense-Urban eMBB scenario, especially for spectral efficiency evaluation with frequency range 2. This contribution is revision of R1-1902320.

# Discussion

As required by Report ITU-R M.2412, average spectral efficiency and 5th percentile user spectral efficiency were evaluated for both NR FDD and TDD in three different scenarios (e.g., Indoor Hotspot-eMBB, Dense-Urban-eMBB, Rural-eMBB). For Indoor Hotsport-eMBB and Dense-Urban-eMBB scenarios, average spectral efficiency and 5th percentile user spectral efficiency were evaluated for both frequency range 1 and 2 while average spectral efficiency and 5th percentile user spectral efficiency for frequency range 1 is evaluated for Rural-eMBB. Detailed evaluation assumptions and results can be found in TR37.910. As a conclusion seen in TR37.910, it was observed that both NR FDD and TDD fufill the DL spectral efficiency requirement for all configurations, except evaluation configuration B (CF=30GHz) for Dense-Urban-eMBB scenario. It is noted that evaluation results for evaluation configuration B for Dense-Urban-eMBB scenario were not captured in TR37.910, which will be updated into the TR for final submission.

Observation 1: Evaluation results for evaluation configuration B for Dense-Urban-eMBB scenario were not captured in TR37.910.

Although evaluation results for higher frequency range for indoor scenario were captured in the TR, deploying outdoor scenario with higher frequency range is also important. Therefore, it is beneficial to include evaluation results for higher frequency range in both indoor and outdoor scenario into the TR.

Proposal 1: Evaluation results for evaluation configuration B for Dense-Urban-eMBB scenario should be captured in TR37.910 for final submission.

Therefore, it is recommended that companies are encouraged to evaluate and provide evaluation results for evaluation configuration B for Dense-Urban-eMBB scenario for final submission.

Proposal 2: Companies are encouraged to evaluate and provide evaluation results for evaluation configuration B especially for Dense-Urban-eMBB scenario for final submission.

With the observation from preliminary results submitted to the evaluation campaign, it was observed that it is difficult to fulfill the spectral efficiency requirement for evaluation configuration B for Dense-Urban-eMBB scenario under existing evaluation methodologies. One of difficulties to obtain higher spectral efficiency for higher frequency range may be due to a limited coverage from the characteristics of mmwave, which would result in that the UE may be attached with the strongest gNB but its signal strength from gNB is too weak for communications. Another aspect is the limitation of the number of antenna elements per TRxP. According to the current evaluation methodology, the number of antenna elements per TRxP for indoor and outdoor scenario is identical as up to 256Tx/Rx, which can be increased for outdoor scenario. Following table summarizes evaluation configurations for Dense Urban-eMBB test environment.

b) Evaluation configurations for Dense Urban-eMBB test environment

| Parameters | Dense Urban-eMBB |
| --- | --- |
|  | Spectral Efficiency and Mobility Evaluations |
|  | Configuration B |
| Carrier frequency for evaluation | 1 layer (Macro) with 30 GHz |
| BS antenna height | 25 m |
| Total transmit power per TRxP | 40 dBm for 80 MHz bandwidth 37 dBm for 40 MHz bandwidth e.i.r.p. should not exceed 73 dBm |
| UE power class | 23 dBm, e.i.r.p. should not exceed 43 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss |
| Inter-site distance | 200 m |
| Number of antenna elements per TRxP | Up to 256 Tx/Rx |
| Number of UE antenna elements | Up to 32 Tx/Rx |
| Device deployment | 80% indoor, 20% outdoor (in-car) Randomly and uniformly distributed over the area under Macro layer |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. |
| UE speeds of interest | Indoor users: 3 km/h Outdoor users (in-car): 30 km/h |
| Inter-site interference modeling | Explicitly modelled |
| BS noise figure | 7 dB |
| UE noise figure | 10 dB |
| BS antenna element gain | 8 dBi |
| UE antenna element gain | 5 dBi |
| Thermal noise level | -174 dBm/Hz |
| Traffic model | Full buffer |
| Simulation bandwidth | 80 MHz for TDD, 40 MHz+40 MHz for FDD |
| UE density | 10 UEs per TRxP Randomly and uniformly distributed over the area under Macro layer |
| UE antenna height | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |

Therefore, it is recommended that RAN1 studies and identifies proper evaluation methodologies especially for evaluation configuration B for Dense-Urban-eMBB scenario, with consideration of various characteristics of mmwave. For example, reduce inter-site distance and/or indoor UE ratio in device deployment can be considered as candidates, increase number of antenna elements per TRxP and/or number of UE antenna elements, etc. If additional evaluation results are provided by companies with the modification on some of evaluation methodology, it should also be captured in the final evaluation results with the proper explanation on the modification. Following table shows a primitive evaluation result for DL with some modified parameters (e.g., UE distribution). As shown in the table, the requirement can be met with some parameter modifications.

| DL | ISD200m, all outdoor, pedestrian100% | Requirement |
| --- | --- | --- |
| Average [bits/Hz] | 9.4 | 7.8 |
| 5% edge [bits/Hz] | 0.27 | 0.225 |

Proposal 3: Companies are encouraged to provide additional evaluation results with modification on some of evaluation methodology especially for evaluation configuration B for Dense-Urban-eMBB scenario.

Proposal 4: If additional evaluation results with some modifications on evaluation methodologies, it should also be captured in the final submission with the proper explanation on the modification.

# Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: Evaluation results for evaluation configuration B for Dense-Urban-eMBB scenario should be captured in TR37.910 for final submission.

Proposal 2: Companies are encouraged to evaluate and provide evaluation results for evaluation configuration B especially for Dense-Urban-eMBB scenario for final submission.

Proposal 3: Companies are encouraged to provide additional evaluation results with modification on some of evaluation methodology especially for evaluation configuration B for Dense-Urban-eMBB scenario.

Proposal 4: If additional evaluation results with some modifications on evaluation methodologies, it should also be captured in the final submission with the proper explanation on the modification.