3GPP TSG RAN WG1 #96bis                                                       R1-1904477

Xi’an, China, 8th – 12th April, 2019

Source: MediaTek Inc.

Title: Full Tx power UL transmission

Agenda Item: 7.2.8.4

Document for:  Discussion

# Introduction

A number of agreements were reached in previous meetings:

From RAN1 #94b

Agreement

Consider the following potential solutions and other solutions (such as combination of the solutions below) for UL full power transmission. Decision will be made in RAN1#95:

- Option 1: Refinement/adjustment of UL codebook is supported

  - 1-1: Support a new codebookSubset for non-coherent and partial-coherent transmission capable UEs

  - 1-2: Introduce additional scaling factor for uplink codebook

- Option 2: UE transparently apply a small cyclic or linear delay

- Option 3: Power control mechanism to be modified to support UL full power transmission without precluding the use of full rated PA(s)

  - Note: Full rated PA refers to a PA having power not lower than that of the power class

- Option 4: Up to UE implementation (no specification impact)

From RAN1 #95

Agreement

Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs

- This specification support is a UE optional feature

- FFS: Whether this applies for the entire codebook or subset of codebook

Agreement

Full TX power UL transmission, one additional option (option 5) is added

Option5: For the precoders with 0 entries, the linear value PPUSCH,b,f,c(i,j,qd,l) of a PUSCH transmission power is scaled by a ratio Rel-16.  The value of Rel-16 is selected up to UE implementation within the range of [Rel-15, 1],  where Rel-15 is the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the number of configured antenna ports for the PUSCH transmission scheme as defined in NR Rel-15 specification.

- UE is required to maintain consistent Rel-16αRel-16 value on different occasions of PUSCH transmissions with the same precoder for PUSCH

Agreement

Full TX power UL transmission, option 4 is updated as follows

Option 4: Up to UE implementation with UE capability signalling of full power transmission in UL (no specification impact)

From RAN1 #AdHoc 1901

Agreement

Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs. The support of this feature is indicated by the UE as part of UE capability signalling. For power class 3:

- UE capability 1: for the UE to support full Tx power in UL transmission, full rated PAs on each Tx chain is supported with a new UE capability

  - FFS: detailed power scaling description

  - Note: Full Tx power means UE delivers total power of 23dBm for PC3

- UE capability 2: for the UE to support full Tx power in UL transmission, no Tx chain is assumed to deliver full power with the new UE capability

  - FFS: detailed design

- UE capability 3: for the UE to support full Tx power in UL transmission, subset of Tx chains with full rated PAs is supported with a new UE capability

FFS: Whether all three capabilities will be specified or a subset will be specified

FFS: UE capability signalling/reporting details

Note: Two or more of the above capabilities could be merged depending on the further details

From RAN1 #96

Agreement

Note: UE capability 1, 2, 3 agreed in RAN1#AH1901 mean the PA architectures.

At least for PC3, UE capability 1, 3 can support full power transmission.

Working assumption: For PC3, UE capability 2 can support full power transmission.

- Companies to check for any implementation issues and/or performance of Rel-16 full power transmission compared to Rel-15 non-coherent codebook subset uplink transmission)

Agreement

Down select among the following two alternatives by RAN1#96bis. As part of UE capabilities signalled the following is included:

Alt1: UE capability signaling of supported one or group of TPMI precoder(s)

Alt3: UE capability signaling of power scaling schemes for full uplink power transmission

- Note: This does not imply any restriction on UE antenna virtualization

- FFS: Whether full uplink power transmission needs to be supported for all precoders

In this contribution, we provide our view and simulation results for this topic.

# Discussion of full Tx power UL

The Rel-15 codebook has the following allocation of number of codewords according to the UE capability (non-coherent, partially coherent, coherent transmission) and rank:

| Rank | Full Coherence | Partially Coherent | Non Coherent |
| --- | --- | --- | --- |
| Rank 1 | 16 | 8 | 4 |
| Rank 2 | 8 | 8 | 6 |
| Rank 3 | 4 | 2 | 1 |
| Rank 4 | 2 | 2 | 1 |

Table 1 Allocation of number of codewords according to UE capability and rank

For a UE reporting non-coherence transmission capability, only 4, 6, 1 and 1 codewords at rank 1, 2, 3 and 4 respectively for non-coherent transmission are available.

For a UE reporting partial coherence transmission capability, only 8+4, 8+6, 2+1, and 2+1 codewords at rank 1, 2, 3 and 4 respectively for partial-coherence and non-coherence transmissions are available.

For a UE reporting full coherence transmission capability, 16+8+4, 8+8+6, 4+2+1 and 2+2+1 codewords at rank 1, 2, 3 and 4 respectively for full-coherence, partial coherence and non-coherence are available.

It is noted that a codeword under partially coherent and non-coherent transmissions does not utilize fully the Tx power available at a UE.

Figure 1 Implementation with four 17 dBm power amplifiers.

- With the architecture in Figure 1, four 17 dBm PAs are used at a UE with Pc,max=23 dBm. Note when a UE reports non-coherence transmission capability, the maximum transmission power at rank 1 is limited to 17 dBm, which may lead to link budget issues in uplink.

## UE Capability 1

Figure 2 Implementation with four 23 dBm power amplifiers (PAs) for a UE with Pc,max=23 dBm

In Figure 2, full rated PAs on each Tx chain (UE Capability 1) is conceptually straight-forward, yet may not be so straightforward in reality,  with four 23 dBm PAs can be used to support all codewords in the Rel-15 specification at a UE’s maximum power (e.g. Pc,max=23 dBm). Some companies have proposed to modify the power control formula in NR so the architecture in Figure 2 or some other architectures providing the same output power for all scenarios are required. Note the solution in Figure 2 can be rather costly from UE implementation point of view, and also wasteful, as all the full power PAs are not used all the time.

## UE Capability 3

For UE Capability 3, subset of Tx chains with full rated PAs is supported. In this situation,

power amplifiers and switch(es) are used.

Figure 3 Alternate implementation with two 23 dBm, two 17 dBm PAs for a UE with Pc,max=23 dBm

In Figure 3,   when 23 dBm power is required at Antenna 2, PA 1 is switched to Antenna 2 (shown with a dashed line), and PA 2 is turned off. When 23 dBm power is required at Antenna 4, PA 3 is switched to Antenna 4 (shown with a dashed line), and PA 4 is turned off.

If the antenna switching based approach is utilized, it is expected that non-zero switching time needs to be budgeted in the case PA 1 is switched to Antenna 2, that can be due to either rank switching or precoder switching for example from using 1/21 0Tto using 1/20 1T. As the decision of antenna switching may be dependent on UE implementation (e.g. when to start the antenna switching), the PUSCH scheduling latency may be impacted as the network may have to assume the longest switching time.

Observation 1: If the antenna switching based approach is utilized, the PUSCH scheduling latency may be impacted as the network may have to assume the longest switching time.

## UE Capability 2

Here, we propose detailed support for capability 2.

In the Rel-15 specification, the DCI fields are budgeted according to Table 1. To reduce changes to the specification in Rel-16, we can consider the following scheme:

- For Rank 1, the codebook consists of 16 + 8 + 4 codewords coming from the codewords chosen for full coherence, partial coherence, and non-coherence as defined in TS 38.211 Table 6.3.1.5-2/ Table 6.3.1.5-3.

- For Rank 2, the codebook consists of 8 + 8 + 6 codewords coming from the codewords chosen for full coherence, partial coherence, and non-coherence as defined in TS 38.211 Table 6.3.1.5-5.

- For Rank 3, the codebook consists of 4+2+1 codewords coming from the codewords chosen for full coherence, partial coherence, and non-coherence as defined in TS 38.211 Table 6.3.1.5.4-6.

- For Rank 4, the codebook consists of 2+2+1 codewords coming from the codewords chosen for full coherence, partial coherence, and non-coherence as defined in TS 38.211 Table 6.3.1.5-7.

We propose to revisit the agreement concerning codebook subset restriction reached in Rel-15. In Rel-15 codebook subset restriction is per UE capability in terms of coherent transmission/partially coherent transmission/non-coherent transmission. With that agreement, when a UE reporting non-coherence/partial coherence transmission capability, no codeword utilizing all 4 Tx chains (i.e. codewords designed for coherent transmission) is available at the UE.

One could envision if codebook subset restriction be under the discretion of the gNB, and codebook subset restriction   be decoupled from a UE’s capability signaling, then even when a UE reports non-coherent transmission, the gNB could still configure codewords utilizing all Tx chains, e.g. a codeword designed for coherent transmission, so all the Tx chains could be actively involved in the UL transmission and full power utilization could be achieved. Of course, in this case it is necessary to clarify how a non-coherent transmission only capable UE should utilize a codeword originally design for coherent transmission.

Depending on the UE capability in terms of coherent transmission (which enforces restriction on the signaling overhead by a design choice taken in Rel-15), through codebook subset restriction (e.g. a bitmap), the gNB signal the selection of codewords at each rank. And the number of chosen codewords at each rank at a reported UE capability follows the Table 2. Note a gNB should be allowed to select any codeword at a given rank, i.e. the allowed codewords are decoupled from the UE capability reporting concerning coherence transmission. As the number of available codewords at each rank and each reported UE capability is the same as in Rel-15, modification in dynamic control signaling design is avoided.

| Rank | Full Coherence | Partially Coherent | Non Coherent |
| --- | --- | --- | --- |
| Rank 1 | 28 | 12 | 4 |
| Rank 2 | 22 | 14 | 6 |
| Rank 3 | 7 | 3 | 1 |
| Rank 4 | 5 | 3 | 1 |

Table 1 Allowed number of codewords according to UE capability and rank

One example is given below for a case where a UE reports non-coherence transmission capability, the codewords marked in yellow from Tables from TS 38.211 are selected by a gNB (e.g. through a bitmap or indexing scheme).

The selection can be through a bitmap, e.g. [0000 0000 0000 1010 0000 1010 0000] for the selected words (in yellow) for rank 1, similarly for selected codewords for other ranks.

Note in Rel-15, the codewords marked in purple are the only codewords available for a UE reporting non-coherence transmission capability; with the proposed scheme then the gNB is free to choose any codewords while meeting the restriction on the number of codewords at each rank as given in Table 2.

Table 6.3.1.5-3: Precoding matrix  for single-layer transmission using four antenna ports with transform precoding disabled.

| TPMI index | (ordered from left to right in increasing order of TPMI index) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 7 |  |  |  |  |  |  |  |  |
| 8 – 15 |  |  |  |  |  |  |  |  |
| 16 – 23 |  |  |  |  |  |  |  |  |
| 24 – 27 |  |  |  |  | - | - | - | - |

Table 6.3.1.5-5: Precoding matrix  for two-layer transmission using four antenna ports with transform precoding disabled.

| TPMI index | (ordered from left to right in increasing order of TPMI index) |  |  |  |
| --- | --- | --- | --- | --- |
| 0 – 3 |  |  |  |  |
| 4 – 7 |  |  |  |  |
| 8 – 11 |  |  |  |  |
| 12 – 15 |  |  |  |  |
| 16 – 19 |  |  |  |  |
| 20 – 21 |  |  | - | - |

Table 6.3.1.5.4-6: Precoding matrix  for three-layer transmission using four antenna ports with transform precoding disabled.

| TPMI index | (ordered from left to right in increasing order of TPMI index) |  |  |  |
| --- | --- | --- | --- | --- |
| 0 – 3 |  |  |  |  |
| 4 – 6 |  |  |  | - |

Table 6.3.1.5-7: Precoding matrix  for four-layer transmission using four antenna ports with transform precoding disabled.

| TPMI index | (ordered from left to right in increasing order of TPMI index) |  |  |  |
| --- | --- | --- | --- | --- |
| 0 – 3 |  |  |  |  |
| 4 |  | - | - | - |

To avoid the situation that the phase discontinuity is so severe and then signals from two chains may cancel each other instead add up; then robust transmission schemes such as CDD can be considered as shown below.

Figure 4 transmission scheme for a non-coherence capable UE to utilize a codeword originally designed for coherent transmission

When a codeword which is for partially coherent transmission/coherent transmission in Rel-15 is used for a Rel-16 UE capable of non-coherent transmission only, a UE can be allowed to choose/use a power combination scheme, e.g. small delays can be introduced among Tx chains. In the case a codeword which is for coherent transmission in Rel-15 is used at a Rel-16 UE capable of non-coherent transmission only, for each spatial layer, a first set of entries, e.g. the entries corresponding to the same coherence group (e.g. Tx 1 and Tx 3) are transmitted from relevant Tx chains at the same time, while other entries (e.g. for Tx 2 and Tx 4) are transmitted at a different timing from the first set of entries. More specifically

τk,1≤k≤4 is the small delay introduced at Tx chains, and for a UE reporting non-coherent transmission capability,  τm≠τn,1≤m,n≤4, m≠n, and for a UE reporting partial-coherent transmission capability, τ1=τ3≠τ2=τ4.

In summary, we propose the following procedure:

- A UE reports its coherence transmission capability to the network (non-coherence, partial coherence, full coherence);

- The allowed number of codewords at each rank is looked up from Table 2 by the gNB. And according to the reported coherence transmission capability, the gNB selects a corresponding number of codewords out of all the codewords which may be originally designed for non-coherence, partial coherence, full coherence, at a given rank (e.g. through a bitmap for each rank) Such selection by The gNB with a codebook subset restriction (i.e. selection above) is conveyed as in a RRC signaling to a UE. The gNB can also configure multiple sets of codebook subset restriction to a UE, then by using MAC CE, the active codebook subset restriction is selected at a UE.

- The UE receive the RRC signaling for the configuration of one or multiple codebook subset restriction(s) and potentially MAC CE activation/selection, and the selected codewords at each rank are indexed sequentially as the “1”’s positions in the bitmap.

When The UE receives an uplink DCI, the relevant fields concerning TMPI are construed according to the above steps. CDD may be adopted for the transmission from a non-coherence transmission capable UE with a codeword originally designed for partial coherence transmission or full coherence transmission; and CDD may be adopted for the transmission from a partial-coherence transmission capable UE with a codeword originally designed for full coherence transmission.

Simulation comparing coherent precoders, non-coherent precoders, codebook subset to utilize full Tx power with/without small delay CDD are provided in the next section. The results show that with codebook subset utilizing full UE’s power, the performance gain over non-coherent precoders is observed without small delay CDD under medium range of relative phase error. Furthermore, small delay CDD provides robust and good performance gain over non-coherent precoders even with severe relative phase error.

To enable UE capability 2, the UE capability signal should include one or group of TPMI precoder(s) to indicate the UE can support full Tx power via UE capability 2.

Proposal:

- A non-coherence capable UE with capability 2 can be signalled to use a precoder originally designed for partial coherent transmission/full coherent transmission in a Rel-15 UL MIMO codebook; a partial coherence capable UE can be signalled to use a precoder originally designed for full coherent transmission in a Rel-15 UL MIMO codebook.

- Small delay CDD can be used for the situation with severe phase discontinuity.

- Confirm the working assumption: For PC3, UE capability 2 can support full power transmission.

- UE capability signalling indicates supported one or group of TPMI precoder(s) to enable UE capability 2.

# Simulation evaluations

In this section, we provide simulation results for a number of considered schemes for 2Tx and 4Tx. CDL-C with 50ns for RMS delay spread, and ideal channel estimation is used.

## Notations

We first establish notations used in reported simulation results.

### 2Tx

Case 1 (“CWs 0-5”)

For Case 1, for a UE reporting “nonCoherent” in pusch-TransCoherence (UE capability), the network signals “fullyAndPartialAndNonCoherent” for codebookSubset within PUSCH-Config, and the DCI fields in Format 0_1 follow those for  “fullyAndPartialAndNonCoherent” . At rank 1, depending the maxRank=1 or maxRank=2, the codewords marked in yellow are available (in another word, the PMI takes a value from 0…5).

Table 7.3.1.1.2-4: Precoding information and number of layers, for 2 antenna ports, if transformPrecoder=disabled and maxRank = 2

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 2 layers: TPMI=0 | 2 | 2 layers: TPMI=0 |
| 3 | 1 layer: TPMI=2 | 3 | reserved |
| 4 | 1 layer: TPMI=3 |  |  |
| 5 | 1 layer: TPMI=4 |  |  |
| 6 | 1 layer: TPMI=5 |  |  |
| 7 | 2 layers: TPMI=1 |  |  |
| 8 | 2 layers: TPMI=2 |  |  |
| 9-15 | reserved |  |  |

Table 7.3.1.1.2-5: Precoding information and number of layers, for 2 antenna ports, if transformPrecoder= enabled, or if transformPrecoder= disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |  |  |
| 3 | 1 layer: TPMI=3 |  |  |
| 4 | 1 layer: TPMI=4 |  |  |
| 5 | 1 layer: TPMI=5 |  |  |
| 6-7 | reserved |  |  |

Case 2 (“CWs 0-1”)

For Case 2 a UE reporting “nonCoherent” in pusch-TransCoherence (UE capability), the TPMI is restricted to be 0 or 1 (Rel-15 behaviour).

Case 3 (“CWs 2-3”)

For Case 3, for a UE reporting “nonCoherent” in pusch-TransCoherence (UE capability), the network signals a codebook subset restriction with 2 allowed codwords at rank 1. The DCI fields in Format 0_1 follow those for  “NonCoherent”  with updated tables shown below.  At rank 1, depending the maxRank=1 or maxRank=2, the codewords marked in red are available (in another word, the PMI takes a value from 2 and 3).

Table 7.3.1.1.2-4: Precoding information and number of layers, for 2 antenna ports, if transformPrecoder=disabled and maxRank = 2

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=2 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=3 |
| 2 | 2 layers: TPMI=0 | 2 | 2 layers: TPMI=0 |
| 3 | 1 layer: TPMI=2 | 3 | reserved |
| 4 | 1 layer: TPMI=3 |  |  |
| 5 | 1 layer: TPMI=4 |  |  |
| 6 | 1 layer: TPMI=5 |  |  |
| 7 | 2 layers: TPMI=1 |  |  |
| 8 | 2 layers: TPMI=2 |  |  |
| 9-15 | reserved |  |  |

Table 7.3.1.1.2-5: Precoding information and number of layers, for 2 antenna ports, if transformPrecoder= enabled, or if transformPrecoder= disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=2 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=3 |
| 2 | 1 layer: TPMI=2 |  |  |
| 3 | 1 layer: TPMI=3 |  |  |
| 4 | 1 layer: TPMI=4 |  |  |
| 5 | 1 layer: TPMI=5 |  |  |
| 6-7 | Reserved |  |  |

Case 4 (“CWs 2-3” with CDD)

For Case 4, for a UE reporting “nonCoherent” in pusch-TransCoherence (UE capability), the network signals a codebook subset restriction with 2 allowed codwords at rank 1. The DCI fields in Format 0_1 follow those for  “NonCoherent”  with updated tables as same as in Case 3.  At rank 1, depending the maxRank=1 or maxRank=2, the codewords marked in red are available (in another word, the PMI takes a value from 2 and 3). Note a small delay CDD is applied as illustrated in Figure 5. The small delay CDD amount can be 1/4, 1/8, 1/32, etc of the CP duration.

For 2Tx, we consider cross polarization and co-polarization.

### 4Tx

Case 1 (“CWs 0-27”)

For Case 1, for a UE reporting “nonCoherent” in pusch-TransCoherence (UE capability), the network signals “fullyAndPartialAndNonCoherent” for codebookSubset within PUSCH-Config, and the DCI fields in Format 0_1 follow those for  “fullyAndPartialAndNonCoherent” . At rank 1, depending the maxRank=1 or maxRank=2~4, the codewords marked in yellow are available (in another word, the PMI takes a value from 0:3, 12:19, 32:47 from Table 7.3.1.1.2-2 or 0:27 from Table 7.3.1.1.2-3).

Table 7.3.1.1.2-2: Precoding information and number of layers, for 4 antenna ports, if transform precoder is disabled and maxRank = 2 or 3 or 4

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 |
| 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 |
| 12 | 1 layer: TPMI=4 | 12 | 1 layer: TPMI=4 | 12-15 | reserved |
| … | … | … | … |  |  |
| 19 | 1 layer: TPMI=11 | 19 | 1 layer: TPMI=11 |  |  |
| 20 | 2 layers: TPMI=6 | 20 | 2 layers: TPMI=6 |  |  |
| … | … | … | … |  |  |
| 27 | 2 layers: TPMI=13 | 27 | 2 layers: TPMI=13 |  |  |
| 28 | 3 layers: TPMI=1 | 28 | 3 layers: TPMI=1 |  |  |
| 29 | 3 layers: TPMI=2 | 29 | 3 layers: TPMI=2 |  |  |
| 30 | 4 layers: TPMI=1 | 30 | 4 layers: TPMI=1 |  |  |
| 31 | 4 layers: TPMI=2 | 31 | 4 layers: TPMI=2 |  |  |
| 32 | 1 layers: TPMI=12 |  |  |  |  |
| … | … |  |  |  |  |
| 47 | 1 layers: TPMI=27 |  |  |  |  |
| 48 | 2 layers: TPMI=14 |  |  |  |  |
| … | … |  |  |  |  |
| 55 | 2 layers: TPMI=21 |  |  |  |  |
| 56 | 3 layers: TPMI=3 |  |  |  |  |
| … | … |  |  |  |  |
| 59 | 3 layers: TPMI=6 |  |  |  |  |
| 60 | 4 layers: TPMI=3 |  |  |  |  |
| 61 | 4 layers: TPMI=4 |  |  |  |  |
| 62-63 | reserved |  |  |  |  |

Table 7.3.1.1.2-3: Precoding information and number of layers for 4 antenna ports, if transform precoder is enabled, or if transform precoder is disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset= partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 1 layer: TPMI=4 | 4 | 1 layer: TPMI=4 |  |  |
| … | … | … | … |  |  |
| 11 | 1 layer: TPMI=11 | 11 | 1 layer: TPMI=11 |  |  |
| 12 | 1 layers: TPMI=12 | 12-15 | reserved |  |  |
| … | … |  |  |  |  |
| 27 | 1 layers: TPMI=27 |  |  |  |  |
| 28-31 | reserved |  |  |  |  |

Case 2 (“CWs 0-3”)

For Case 2 a UE reporting “nonCoherent” in pusch-TransCoherence (UE capability), the TPMI is restricted to be 0 or 3 (Rel-15 behaviour).

Case 3 (“CWs 12, 14, 20, 22”)

For Case 3, for a UE reporting “nonCoherent” in pusch-TransCoherence (UE capability), the network signals a codebook subset restriction with 4 allowed codwords at rank 1. The DCI fields in Format 0_1 follow those for  “NonCoherent”  with updated tables shown below.  At rank 1, depending the maxRank=1 or maxRank=2~4, the codewords marked in red are available (in another word, the PMI takes a value from 12, 14, 20 and 22).

Case 4 (“CWs 12, 14, 20, 22” with CDD)

For Case 4, for a UE reporting “nonCoherent” in pusch-TransCoherence (UE capability), the network signals a codebook subset restriction with 4 allowed codwords at rank 1. The DCI fields in Format 0_1 follow those for  “NonCoherent”  with updated tables as same as in Case 3.  At rank 1, depending the maxRank=1 or maxRank=2~4, the codewords marked in red are available (in another word, the PMI takes a value from 12, 14, 20 and 22). Note a small delay CDD is applied as illustrated in Figure 4. The small delay CDD amount can be 1/4, 1/8, 1/16, etc of the CP duration.

Table 7.3.1.1.2-2: Precoding information and number of layers, for 4 antenna ports, if transform precoder is disabled and maxRank = 2 or 3 or 4

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=12 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=14 |
| … | … | … | … | 2 | 1 layer: TPMI=20 |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=22 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 |
| 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 |
| 12 | 1 layer: TPMI=4 | 12 | 1 layer: TPMI=4 | 12-15 | reserved |
| … | … | … | … |  |  |
| 19 | 1 layer: TPMI=11 | 19 | 1 layer: TPMI=11 |  |  |
| 20 | 2 layers: TPMI=6 | 20 | 2 layers: TPMI=6 |  |  |
| … | … | … | … |  |  |
| 27 | 2 layers: TPMI=13 | 27 | 2 layers: TPMI=13 |  |  |
| 28 | 3 layers: TPMI=1 | 28 | 3 layers: TPMI=1 |  |  |
| 29 | 3 layers: TPMI=2 | 29 | 3 layers: TPMI=2 |  |  |
| 30 | 4 layers: TPMI=1 | 30 | 4 layers: TPMI=1 |  |  |
| 31 | 4 layers: TPMI=2 | 31 | 4 layers: TPMI=2 |  |  |
| 32 | 1 layers: TPMI=12 |  |  |  |  |
| … | … |  |  |  |  |
| 47 | 1 layers: TPMI=27 |  |  |  |  |
| 48 | 2 layers: TPMI=14 |  |  |  |  |
| … | … |  |  |  |  |
| 55 | 2 layers: TPMI=21 |  |  |  |  |
| 56 | 3 layers: TPMI=3 |  |  |  |  |
| … | … |  |  |  |  |
| 59 | 3 layers: TPMI=6 |  |  |  |  |
| 60 | 4 layers: TPMI=3 |  |  |  |  |
| 61 | 4 layers: TPMI=4 |  |  |  |  |
| 62-63 | reserved |  |  |  |  |

Table 7.3.1.1.2-3: Precoding information and number of layers for 4 antenna ports, if transform precoder is enabled, or if transform precoder is disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset= partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=12 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=14 |
| … | … | … | … | 2 | 1 layer: TPMI=20 |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=22 |
| 4 | 1 layer: TPMI=4 | 4 | 1 layer: TPMI=4 |  |  |
| … | … | … | … |  |  |
| 11 | 1 layer: TPMI=11 | 11 | 1 layer: TPMI=11 |  |  |
| 12 | 1 layers: TPMI=12 | 12-15 | reserved |  |  |
| … | … |  |  |  |  |
| 27 | 1 layers: TPMI=27 |  |  |  |  |
| 28-31 | reserved |  |  |  |  |

## Evaluation results

We show the results of BLER with fixed MCS for 2Tx and 4Tx; and show throughput for 2Tx when link adaption is ON.

The RAN4 requirements for full coherence transmission can be found in Appendix. To be considered meeting the full coherence requirement, for error in relative phase difference, a UE needs to keep the error below 40 degrees within a 20 millisecond time window.

Figure 5 With Phase discontinuity, relative phase error between each Tx for PUSCH follows a uniform distribution [-80o, 80 o].

In Figure 5, the relative phase error for PUSCH (using SRS transmissions as reference) between each Tx chains follow a uniform distribution [-80o, 80 o], and the UE report “nonCoherent” in UE capability signalling. Several transmission schemes as explained in Section 2.1 are compared. Among them, making all the codewords available to the UE (“CWs 0-5” for 2Tx (top-left and bottom figure), “CWs 0-27” for 4Tx (top-right figure)) helps, and a small number of codewords with small delay CDD (with τ=1/8) performs well. On the other hand, the Rel-15 non-coherent transmission scheme (“CWs 0-1” for 2Tx (left figure) and “CWs 0-3” for 4Tx (right figure)) performs worst. The gap between the best performing scheme and Rel-15 non-coherent transmission scheme is about 2 dB at 10% BLER for 2Tx scheme and 4.5 dB at 10% BLER for 4Tx scheme.

Figure 6 With Phase discontinuity, a fixed 80 degree relative phase error between each Tx for PUSCH

In Figure 6, the relative phase error for PUSCH (using SRS transmissions as reference) between each Tx chains are fixed to 80 o, and the UE report “nonCoherent” in UE capability signalling. Several transmission schemes as explained in Section 2.1 are compared. Among them, making all the codewords available to the UE helps, and  small delay CDD (with τ=1/8) performs well. On the other hand, the Rel-15 transmission scheme performs worst. The gap between the best performing scheme and Rel-15 non-coherent transmission scheme is about 2 dB at 10% BLER for 2Tx scheme and 4.5 dB at 10% BLER for 4Tx scheme.

Figure 7 With Phase discontinuity, a fixed 180 degree relative phase error between each Tx for PUSCH

In Figure 7, the relative phase error for PUSCH (using SRS transmissions as reference) are fixed to 180 o, and the UE report “nonCoherent” in UE capability signalling. Several transmission schemes as explained in Section 2.1 are compared. Among them,

- For 2Tx, “CWs 0-5” performs the worst, as the uplink CSI acquired from SRS provides the worst possible precoder for PUSCH due to the 180 o  relative phase error. In this case, small delay CDD (“CWs 2-3” with τ=1/8) performs well.  The gap between the best performing scheme and Rel-15 non-coherent transmission scheme is about 2 dB at 10% BLER. We note it may be possible to calibrate out the phase error on the network side by requiring SRS transmissions at different power levels and compensate the phase error for PUSCH transmission [1].

- For 4Tx, as the relative phase error between each antenna are fixed to 180 o, so the phase errors at 4 Tx chains are 0, 180 o , 360 o , 540 o  (or 0, 180 o , 0, 180 o ) respectively. The Rel-15 non-coherent transmission performs the worst. In this case, small delay CDD (“CWs 12, 14, 20, 22” with τ=1/8) performs well. The gap between the best performing scheme and Rel-15 non-coherent transmission scheme is about 4.5 dB at 10% BLER.

We have

Observation 2:  Small delay CDD provides robust performance and good performance gain over the Rel-15 non-coherence transmission scheme; depending on the relative phase error characteristics, allowing the use of coherent codewords without the additional small delay CDD operation can be also helpful.

# Conclusion

In this contribution, we provide our views on full power utilization in UL MIMO. We have

Observation 1: If the antenna switching based approach is utilized, the PUSCH scheduling latency may be impacted as the network may have to assume the longest switching time.

Observation 2:  Small delay CDD provides robust performance and good performance gain over the Rel-15 non-coherence transmission scheme; depending on the relative phase error characteristics, allowing the use of coherent codewords without the additional small delay CDD operation can be also helpful.

Proposal:

- A non-coherence capable UE with capability 2 can be signalled to use a precoder originally designed for partial coherent transmission/full coherent transmission in a Rel-15 UL MIMO codebook; a partial coherence capable UE can be signalled to use a precoder originally designed for full coherent transmission in a Rel-15 UL MIMO codebook.

- Small delay CDD can be used for the situation with severe phase discontinuity.

- Confirm the working assumption: For PC3, UE capability 2 can support full power transmission.

- UE capability signalling indicates supported one or group of TPMI precoder(s) to enable UE capability 2.

# References

- R1-1718338,  Remaining details on SRS, MediaTek, #90bis, October 2017

# Appendix RAN4 requirements on UL MIMO coherence

The RAN4 requirements for UL MIMO coherence can be found in TS 38.101-1, v15.3.0:

### 6.4D.4	Requirements for coherent UL MIMO

For coherent UL MIMO, Table 6.4D.4-1 lists the maximum allowable difference between the measured relative power and phase errors between different antenna ports in any slot within the specified time window from the last transmitted SRS on the same antenna ports, for the purpose of uplink transmission (codebook or non-codebook usage) and those measured at that last SRS. The requirements in Table 6.4D.4-1 apply when the UL transmission power at each antenna port is larger than 0 dBm for SRS transmission and for the duration of time window.

Table 6.4D.4-1: Maximum allowable difference of relative phase and power errors in a given slot compared to those measured at last SRS transmitted

| Difference of relative phase error | Difference of relative power error | Time window |
| --- | --- | --- |
| 40 degrees | 4 dB | 20 msec |