3GPP TSG RAN WG1 #96bis		R1-1904576

Xi’an, China, 8th – 12th April, 2019

Agenda Item:	7.2.4.2.1

Source:	Lenovo, Motorola Mobility

Title:	Discussion on resource allocation for NR sidelink Mode 1

Document for:	Discussion

Introduction

In RAN#83 Plenary, a new WID on 5G V2X with NR sidelink was approved with the following resource allocation Mode 1 related objectives [1]:

- Mode 1 - NR sidelink scheduling by NR Uu and LTE Uu as per the study outcome

In this contribution we discuss the mechanisms of resource allocation for NR-sidelink based on LTE/NR Uu.

Discussion

LTE-V2X mode-3 is used for sidelink resource allocation for in-coverage scenario. In this mode, eNB transmits DCI format-5A to indicate the resource allocation for sidelink transmission. It is beneficial to resource collision avoidance. When there are multiple broadcast sessions to be transmitted, the sidelink TX UE can determine which broadcast session will be transmitted using the resource indicated by DCI format-5A. However for NR V2X, besides broadcast, unicast and groupcast are also supported. For unicast and groupcast, the sidelink transmission is only towards a certain UE or some certain UEs.

Compared with LTE V2X, NR V2X has the following enhancements:

- Support different transmission types, e.g., unicast, groupcast and broadcast.

- Support high frequency, e.g., FR2 for NR sidelink, and the associated beamforming scheme.

If LTE V2X mode-3 is reused in NR-V2X, some issues need to be considered:

- Half-duplex

As shown in Figure 1, UE#1 has three unicast sessions towards UE#2, UE#3 and UE#5 respectively, and UE#2 has one unicast session towards UE#4. gNB can assign resources for UE#1 and UE#2 for sidelink transmission. For UE#1, if it transmits the unicast session to UE#2 on the assigned resource, there will be half duplex issue for UE#2 because at the same time UE#2 is transmitting a unicast session to UE#4.

Unicast

DCI

gNB

UE#1

UE#2

Unicast

UE#5

Unicast

UE#3

DCI

UE#4

Unicast

Figure 1. Sidelink transmission for NR mode-1

- Multiple beam

NR sidelink should support FR2, and this means that beamforming will be used for sidelink communication. For example, as shown in Figure 2, there are three sidelink sessions to be transmitted for UE#1. If UE#1 determines the actually transmitted session on the allocated resource, it is possible that UE#1 and UE#4 perform sidelink transmission towards UE#2 at the same time. If UE#2 only has one RX beam, UE#2 may miss the transmission from UE#1 or UE#4. Hence, the network should avoid this situation.

TX beam

TX beam

RX beam

RX beam

DCI

gNB

UE#1

UE#2

UE#5

UE#3

DCI

UE#4

TX

Figure 2. Sidelink transmission under multi-beam

Based on the aforementioned cases, a straightforward solution is to indicate the information of the Rx UE in DCI so that the network can assign sidelink resource for a specific Rx UE. In this way, the system performance can be enhanced.

Proposal 1: For NR sidelink mode-1, the DCI for sidelink scheduling grant should indicate the RX UE(s) for the allocated resource.

To indicate RX UE(s) for mode-1 resource allocation, an obvious solution is the explicit indication: the destination ID of the reception UE(s) is included in the DCI for sidelink resource allocation. Each sidelink UE has destination ID for unicast, destination Group ID for groupcast and destination ID for each broadcast service. However in LTE sidelink the L2 destination ID is 24 bits. If the L2 destination ID is transmitted within the DCI, the overhead is significant. To reduce the size of DCI, implicit indication should be studied, e.g., using the reported list of destination ID(s) or reported BSR MAC-CE.

Proposal 2: Design implicit scheme to indicate the RX UE(s) for mode-1 resource allocation to save the DCI payload size.

Conclusion

In this contribution, we discuss the mechanisms for NR sidelink resource allocation mode-1. We have the following proposals:

Proposal 1: For NR sidelink mode-1, the DCI for sidelink scheduling grant should indicate the RX UE(s) for the allocated resource.

Proposal 2: Design implicit scheme to indicate the RX UE(s) for mode-1 resource allocation to save the DCI payload size.

- References

- RP-190766, New WID on 5G V2X with NR sidelink