3GPP TSG RAN WG1 Meeting #96bis	R1-1904698

Xi’an, China, April 8th – 12th, 2019

Agenda Item:	7.2.7.2

Source:	Huawei, HiSilicon

Title:	Considerations and evaluation results for IMT-2020 for Dense Urban-eMBB spectral efficiency in LTE

Document for:	Discussion and decision

Introduction

According to 3GPP work plan on IMT-2020 submission, the final submission will be made in June 2019. More test environments or configurations for LTE can be evaluated for the final submission. In this contribution, the evaluation results for eMBB spectral efficiency (average and the 5th percentile user spectral efficiency) for LTE in Dense Urban-eMBB test environment with configuration A are provided.

Evaluated technical features

The IMT-2020 eMBB spectral efficiency requirement is three times higher compared to IMT-Advanced. Therefore, it is a challenging requirement and the LTE features that could contribute to the improvement of spectral efficiency should be selected.

On the other hand, it is recalled that for LTE-Advanced evaluation towards IMT-Advanced submission, the evaluation is basically applied based on duplexing schemes, i.e., to FDD and TDD, respectively. This is due to the fact that, duplexing scheme is one of the fundamental features among the other features that impact spectral efficiency performance.

Spectral efficiency calculation for duplexing scheme

The spectral efficiency of different duplexing schemes can be calculated according to Report ITU-R M.2412.

For DL average spectral efficiency and 5th percentile spectral efficiency,

- In case of FDD, the simulation bandwidth is 10 MHz for DL and 10 MHz for UL. The DL average spectral efficiency is given by

(1)

where W is the DL bandwidth of 10 MHz; Ri (T) denotes the number of correctly received bits of user i, and the overhead of DL control and DL reference signals on the DL bandwidth of 10 MHz is taken into account when deriving Ri (T); and T is the simulation time. Similar notations are applied to 5th percentile user spectral efficiency.

- For TDD, the simulation bandwidth is 20 MHz for DL and UL. The DL average spectral efficiency is given by (1), where W is the effective DL bandwidth that accounts for the time-frequency resource used for DL transmission (including GP symbols); Ri (T) denotes the number of correctly received bits of user i, and the overhead of DL control, DL reference signal on the DL effective bandwidth is taken into account; and T is the simulation time. Similar notations are applied to 5th percentile user spectral efficiency.

For UL average spectral efficiency and 5th percentile spectral efficiency, similar way is employed to derive the evaluation results for these two metrics.

Evaluation results

Based on the above discussion, evaluation results for average spectral efficiency and 5th percentile spectral efficiency is evaluated in Dense Urban-eMBB test environment with configuration A. The detailed evaluation assumptions can be found in Appendix.

Spectral efficiency evaluation results

The detailed evaluation assumptions for LTE DL and UL spectral efficiency evaluation can be found in Appendix A.

DL spectral efficiency evaluation results

The DL spectral efficiency evaluation results for LTE are given in Table 1.

TABLE 1 TDD DL spectral efficiency evaluation results

| Test env. | Evaluation config. | Channel model | Average spectral efficiency (bit/s/Hz/TRxP) |  | 5th percentile spectral efficiency (bit/s/Hz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BW=20 MHz | Req. | BW=20 MHz | Req. |
| Dense Urban | Config. A (4 GHz); 64T2R | A | 8.776 | 7.8 | 0.25 | 0.225 |
|  | Config. A (4 GHz); 64T2R | B | 9.048 |  | 0.256 |  |

Observation 1: LTE can fulfill DL spectral efficiency requirements in Dense Urban-eMBB test environment with configuration A.

UL spectral efficiency evaluation results

The UL spectral efficiency evaluation results for LTE are given in Table 2.

TABLE 2 TDD UL spectral efficiency evaluation results

| Test env. | Evaluation config. | Channel model | Average spectral efficiency (bit/s/Hz/TRxP) |  | 5th percentile spectral efficiency (bit/s/Hz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BW=20 MHz | Req. | BW=20 MHz | Req. |
| Dense Urban | Config. A (4 GHz); 64T2R | A | 6.648 | 5.4 | 0.325 | 0.15 |
|  | Config. A (4 GHz); 64T2R | B | 6.594 |  | 0.299 |  |

Observation 2: LTE can fulfill UL spectral efficiency requirements in Dense Urban-eMBB test environment with configuration A.

Conclusion

In this document, the evaluation of eMBB spectral efficiency for LTE is provided. Based on the evaluation, the following observations are made.

Observation 1: LTE can fulfill DL spectral efficiency requirements in Dense Urban-eMBB test environment with configuration A.

Observation 2: LTE can fulfill UL spectral efficiency requirements in Dense Urban-eMBB test environment with configuration A.

- Reference

- R1-1803386, “Summary of offline discussion for IMT-2020 self evaluation”, Huawei, February 2018.

- Appendix A: Evaluation assumptions

Table A-1 Evaluation assumptions for LTE DL

| Parameter |  | Value |
| --- | --- | --- |
| Test environment |  | Dense Urban-eMBB |
| Evaluation configuration |  | Configuration A |
| Channel model |  | UMa_A/UMa_B |
| ISD |  | 200 m |
| TDD frame structure |  | DSUDD, S(10DL:2GP:2UL) |
| Carrier Frequency |  | 4 GHz |
| System bandwidth |  | 20 MHz |
| Subcarrier spacing |  | 15 kHz |
| Symbols number per slot |  | 14 |
| Number of antenna elements per TRxP |  | 256Tx cross-polarized antennas (M,N,P,Mg,Ng) = (12,8,2,1,1; 4,8); |
| Number of TXRU per TRxP |  | 64TXRU: Vertical 1-to-3, Horizontal 1-to-1 |
| Number of antenna elements per UE |  | 2Rx with 0°,90° polarization |
| Transmit power per TRxP |  | 44 dBm |
| TRxP number per site |  | 3 |
| Mechanic tilt |  | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | 105deg in LCS |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling |  | MU-PF |
| ACK/NACK delay |  | At least N+4 |
| MIMO mode |  | MU-MIMO with rank 2 adaptation per user; Maximum MU layer = 8 |
| Guard band ratio |  | 10% |
| BS receiver type |  | MMSE-IRC |
| CSI feedback |  | 5 slots period based on beamformed CSI-RS with delay |
| Overhead | PDCCH | 2 complete symbols |
|  | DMRS | 12 RE/PRB |
|  | CSI-RS | 4 ports per UE per 5 slots |
|  | CSI-RS for IM | ZP CSI-RS with 5 slots period; 4 RE/PRB/5 slots |
|  | CRS | 2 ports |
|  | PSS/SSS | 288 REs per 10 ms |
|  | PBCH | 240 REs per 10 ms |
|  | MBSFN | 4 MBSFN in 10 subframes |
|  | GP | 50% length of GP symbols |
| Channel estimation |  | Non-ideal |
| Waveform |  | OFDM |

Table A-2 Evaluation assumptions for LTE UL

| Parameter |  | Value |
| --- | --- | --- |
| Test environment |  | Dense Urban-eMBB |
| Evaluation configuration |  | Configuration A |
| Channel model |  | UMa_A/UMa_B |
| ISD |  | 200 m |
| Subcarrier spacing |  | 15 kHz |
| TDD frame structure |  | DSUDD, S(10DL:2GP:2UL) |
| Symbols number per slot |  | 14 |
| Number of antenna elements per TRxP |  | 256Tx cross-polarized antennas (M,N,P,Mg,Ng) = (12,8,2,1,1; 4,8); |
| Number of TXRU per TRxP |  | 64TXRU: Vertical 1-to-3, Horizontal 1-to-1 |
| Number of antenna elements per UE |  | 2Tx with 0°,90° polarization |
| UE power class |  | 23 dBm |
| TRxP number per site |  | 3 |
| Mechanic tilt |  | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | 105deg in LCS |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 |
| Scheduling |  | SU-PF |
| MIMO mode |  | SU-MIMO with rank 2 adaptation; |
| Guard band ratio |  | 10% |
| BS receiver type |  | MMSE-IRC |
| UE precoder scheme |  | Codebook based |
| UL CSI derivation |  | Non-precoded SRS based, with delay |
| Power control |  | α=0.6， P0=-60 dBm |
| Power backoff model |  | Continuous RB allocation: follow TS 36.101 in Section 6.2.3 |
| Overhead | PUCCH | 4 RB, 14 OFDM symbols |
|  | DMRS | 2 complete symbols |
|  | SRS | 2 symbols per 5 slots |
|  | GP | 50% length of GP symbols |
| Channel estimation |  | Non-ideal |
| Waveform |  | DFT-S-OFDM |