3GPP TSG RAN WG1 Meeting #96bis 	R1-1904864

Xi’an, China, April 8th – 12th, 2019

Source:	Sharp

Title:	Consideration on UE-group wake-up signal for Rel-16 MTC

Agenda Item:	6.2.1.1

Document for:	Discussion and Decision

Introduction

In the RAN1#96 meeting, the following agreements were made on UE-group wake-up signal for MTC [1].

Agreement - A UE is required to monitor WUS(s) in only one WUS (time/frequency) resource location Agreement - Single-sequence CDM is used in the same WUS resource if more than one UE group is configured to monitor the WUS resource. - Select one among the following: - Up to 2 orthogonal WUS resources may be configured in time domain - Up to 2 orthogonal WUS resources may be configured in frequency domain - Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total) - Up to 2 orthogonal WUS resources may be configured either in time or frequency domain (only one of the two can be configured) - FFS: whether one of the WUS resources is configured to be shared with legacy WUS Agreement If configured, a common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource. - FFS: Whether the above is also applicable for Rel-15 WUS UEs - FFS: Whether to additionally support the feature of waking up a subset of all WUS UE groups

In this contribution, we share our considerations on UE-group wake-up signal for Rel-16 MTC. This contribution is revised from R1-1902641 [2]

Discussions

In the RAN1#96 meeting, it was agreed a common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource and FFS: Whether to support waking up a subset of all WUS UE groups.

For the FFS, if more than one UE group occupies the same WUS resource, considering single-seq. CDM is applied for multiplexing the UE groups in the same WUS resource, a longer time is needed to wake up all the UEs in the different WUS UE groups if not support waking up a subset of all WUS UE groups, and power consumption will also be increased. So it needs to support waking up a subset of all WUS UE groups.

Proposal 1:

- Waking up a subset of all WUS UE groups is supported.

In the RAN1#96 meeting, the following agreements were made:

- Select one among the following:

  - Up to 2 orthogonal WUS resources may be configured in time domain

  - Up to 2 orthogonal WUS resources may be configured in frequency domain

  - Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total)

  - Up to 2 orthogonal WUS resources may be configured either in time or frequency domain (only one of the two can be configured)

- FFS: whether one of the WUS resources is configured to be shared with legacy WUS

In a MTC NB, there are 6 PRBs; legacy WUS occupies 2 PRBs; so there are 4 residual PRBs. It’s a straightforward to use the residual PRBs for group WUS by FDM. Therefore, we propose up to 2 orthogonal WUS resources may be configured in frequency domain.

Proposal 2:

- Up to 2 orthogonal WUS resources may be configured in frequency domain.

For the FFS, we think it depends on whether only 1 WUS resource can be configured. If yes, then a group WUS resource should be shared with legacy WUS on the same WUS resource. If 2 orthogonal WUS resources are configured, it is better to configure whether a group WUS resource is shared with legacy WUS or not by eNB depending on the number of legacy UEs.

Proposal 3:

- A group WUS resource should be shared with legacy WUS if only 1 WUS resource can be configured.

- Whether a group WUS resource is shared with legacy WUS or not can be configured by eNB if 2 orthogonal WUS resources are configured.

In the RAN1#94 meeting, it was agreed to study the RAN1 consequence of UE-grouping on the following basis [3]:

  - UE ID

  - Coverage

  - DRX/eDRX

  - Gap configuration

  - Services

In the RAN1#94bis meeting, it was agreed UE grouping is based on at least UE ID or some function of UE ID [4].

UE grouping based on DRX/eDRX and gap configuration are already supported in Rel-15 [5]. Rel-16 MTC should continue to support them for UE grouping.

For coverage base UE grouping, eNB and UE may have a different understanding of UE’s coverage information because of UE’s mobility or environmental change. So coverage is not appropriate for UE grouping.

For service base UE grouping, the physical layer has no information of the services. So it is hard to use service for UE grouping.

Proposal 4:

- UE grouping is based on UE ID in addition to DRX/eDRX and gap configuration.

Conclusion

In this contribution, we have the following proposals:

Proposal 1:

- Waking up a subset of all WUS UE groups is supported.

Proposal 2:

- Up to 2 orthogonal WUS resources may be configured in frequency domain.

Proposal 3:

- A group WUS resource should be shared with legacy WUS if only 1 WUS resource can be configured.

- Whether a group WUS resource is shared with legacy WUS or not can be configured by eNB if 2 orthogonal WUS resources are configured.

Proposal 4:

UE grouping is based on UE ID in addition to DRX/eDRX and gap configuration.

References

Chairman’s notes in RAN1#96, Athens, Greece, February 25 – March 1, 2019.

R1-1902641, “Consideration on UE-group wake-up signal for Rel-16 MTC”, Sharp, RAN1#96, Athens, Greece, February 25 – March 1, 2019.

Chairman’s notes in RAN1#94, Gothenburg, Sweden, August 20 – 24, 2018.

Chairman’s notes in RAN1#94bis, Chengdu, China, October 8 – 12, 2018.

3GPP TS 36.304 V15.2.0 (2018-12)