3GPP TSG RAN WG1 Meeting #96Bis		R1-1904875

Xi’an, China, Apr 8th – 12th, 2019

Source:	Sharp

Title:	HARQ enhancement for NR unlicensed operation

Agenda Item:	7.2.2.2.3

Document for:	Discussion and Decision

Background

The following agreements have been achieved in RAN1 meetings AH1091 [2] and #96 [3].

Agreement: - RRC parameter dl-DataToUL-ACK supports a value that can be signaled by PDSCH-to-HARQ-timing-indicator, which indicates that the UE needs to store the HARQ A/N feedback result for the corresponding PDSCH, and which does not provide any timing for the transmission of this HARQ A/N feedback result Conclusion: - No additional value of K1 signaled by PDSCH-to-HARQ-timing-indicator needs to be introduced for the purpose of requesting feedback outside the COT (i.e. Alt3 in TR38.889 section 7.2.1.3.3) - Further discuss the details for determining the LBT type for the UE transmission of HARQ A/N feedback for this case - Further discuss whether additional values need to be introduced for the purpose of requesting feedback inside the COT - If additional values are introduced these can also be used for requesting feedback outside the COT - Note: This does not necessitate that the UE needs to know whether the feedback is inside or outside the COT at the time when the feedback is requested Agreement: For enabling multiple opportunities for HARQ A/N transmission and for cross-COT HARQ-ACK feedback, at least the following is supported: gNB requests/triggers feedback for PDSCH from earlier COT(s) or additional reporting of earlier HARQ feedback, where the exact HARQ feedback timing and resource is provided to the UE in another DCI (in the same or in another COT) Agreement: Cross-carrier HARQ re-transmissions will not be discussed for NR-U in Rel-16. Agreement: Scheduling PUSCH over multiple slots/mini-slots by single DCI supports at least multiple continuous PUSCHs with separate TBs - Each TB is mapped to at most one slot or one mini-slot

In this contribution we present our views on HARQ enhancement for NR-U operation.

Multiple HARQ-ACK feedback opportunities in the same shared COT

The issue of multiple HARQ-ACK feedback opportunities in the same shared COT is studied in NR-U SI phase and summarized as following in the TR38.889 [1].

Text from TR38.889 Transmission of HARQ A/N for the corresponding data in the same shared COT is identified as beneficial. For NR-U, the design strives to support transmitting all HARQ A/N for the corresponding data in the same shared COT, if possible, considering the current NR UE processing time required. A gap of up to 16 s should be allowed between the end of the DL transmission and the immediate transmission of feedback to accommodate for the hardware turnaround time. It is beneficial to be able to support transmissions (e.g. CSI reporting or SRS, or other PUSCH, or CSI-RS, or other PDSCH) in the time between one DL data transmission for a UE and the corresponding UL transmission of DL HARQ feedback for the same UE within a shared COT. Potential enhancements for such type of operation, e.g. by possibly pre-configured or pre-determined uplink transmissions for reducing signaling overhead for these transmissions, may be beneficial. Techniques to handle reduced HARQ A/N transmission opportunities for a given HARQ process due to LBT failure are identified as beneficial. Potential techniques include mechanisms to provide multiple and/or supplemental time and/or frequency domain transmission opportunities. … - Alt 4: preconfigured/pre-indicated multiple opportunities in frequency domain in different LBT subbands - Alt 5: preconfigured/pre-indicated multiple opportunities in time domain - Alt5a: Multiple candidate opportunities by providing multiple timings in PDSCH-to-HARQ-timing-indicator and/or other DCI fields - Alt5b: Multiple candidate slots in a window with size configured by RRC. There could be some activation/deactivation by DCI

In NR-U cells, HARQ-ACK feedback probably cannot be successfully delivered to gNB at the indicated timing, since the HARQ-ACK transmission has to be subjected to LBT. In order to increase the probability of successful delivery of HARQ-ACK feedback with low latency, it is beneficial to support method(s) of creating multiple HARQ-ACK feedback opportunities in the same shared COT, in time domain and/or frequency domain.

- Alt4: In order to use multiple HARQ-ACK feedback opportunities in frequency domain, there are still issues that have to be solved first. For example, multiple frequency resources (BWPs or LBT subbands or carriers) might be found available after UE performs LBT procedure. If multiple frequency resources are used for HARQ-ACK feedback transmission, then problems of transmission power or PAPR have to be further considered. While if only one frequency resource is used for HARQ-ACK feedback transmission, additional processing on determining which frequency opportunity to be used becomes necessary.

- Alt5: With multiple HARQ-ACK feedback opportunities in time domain, when UE misses a HARQ-ACK feedback opportunity due to LBT failure, UE can continue to try the next opportunity, rather than giving up the HARQ-ACK feedback transmission within the COT.

Proposal 1:

- Method(s) of creating multiple HARQ-ACK feedback opportunities in time domain within the same shared COT should be supported.

As illustrated in Figure 1, with Alt5a multiple timings are provided (reserved) by PDSCH-to-HARQ-timing-indicator and/or other DCI fields. For the HARQ-ACK feedback corresponding to PDSCH1, UE cannot use timing 1A due to LBT failure, then UE continues to perform LBT procedure and successfully transmit PUCCH1 at timing 1B. However, for the HARQ-ACK feedback corresponding to PDSCH2, UE directly transmits PUCCH2 at timing 2A without LBT failure. In this case, the reserved timing 2B is not used by this UE and cannot be used by other UEs, thus resulting in resource waste. In Alt5b as shown in Figure 2, UE is configured with time windows for HARQ-ACK feedback transmission. Similarly, the remained window size (the gray portion) after UE seizes the channel by LBT procedure and transmits the HARQ-ACK feedback is not efficiently utilized. Thus, it is natural to consider how to efficiently utilize each time window for multiple PDSCHs (rather than reserved for only one PDSCH) in order to alleviate the resource waste.

Figure 1: Illustration of multiple candidate opportunities by providing multiple timings (Alt5a)

Figure 2: Illustration of multiple opportunities by configuring a time window (Alt5b)

Before introducing our proposal on multiple HARQ-ACK opportunities in each time window, the mechanism of semi-static HARQ-ACK is explained with the following example first for better understanding. As shown in Figure 3, PUCCH1 carrying HARQ-ACK codebook1 is indicated by the DCI scheduling PDSCH1, while PUCCH2 carrying HARQ-ACK codebook2 is indicated by the DCI scheduling PDSCH2. With Release 15 semi-static HARQ-ACK method, the size of HARQ-ACK codebooks is firstly determined based on configurations of PDSCH-to-HARQ-timing-indicator K1 and PDSCH. When UE generates HARQ-ACK codebooks, the HARQ-ACK bit in HARQ-ACK codebook1 corresponding to PDSCH1 is enabled and the HARQ-ACK bit in HARQ-ACK codebook2 corresponding to PDSCH1 is disabled (i.e., set to ‘NACK’). Similarly, the HARQ-ACK bit in HARQ-ACK codebook2 corresponding to PDSCH2 is enabled and the HARQ-ACK bit in HARQ-ACK codebook1 corresponding to PDSCH2 is disabled.

Figure 3: Illustration of semi-static HARQ-ACK codebook

In order to solve the resource efficiency problem in Alt5a and Alt5b, we propose a method of creating multiple HARQ-ACK feedback opportunities in each time window during semi-static HARQ-ACK codebook generation. As shown in Figure 4, since potential multiple HARQ-ACK feedback opportunities already exist within the time window corresponding to each PDSCH, it is beneficial to exploit the disabled HARQ-ACK bits to create additional HARQ-ACK feedback opportunities. For example, enable the HARQ-ACK bit in HARQ-ACK codebook2 corresponding to PDSCH1 and enable the HARQ-ACK bit in HARQ-ACK codebook1 corresponding to PDSCH2. Since originally both PUCCH1 and PUCCH2 are supposed to be transmitted, it can be seen that additional HARQ-ACK feedback opportunities are created for both PDSCH1 and PDSCH2 without any additional resources. Consider that the transmission of HARQ-ACK codebook1 via PUCCH1 is not possible due to LBT failure but HARQ-ACK codebook2 via PUCCH2 can be transmitted, then the HARQ-ACK feedback for PDSCH1 is still delivered.

Figure 4: Illustration of multiple opportunities by enhancing semi-static HARQ-ACK codebook

Proposal 2:

A method of creating multiple HARQ-ACK feedback opportunities in time domain by enhancing the semi-static HARQ-ACK codebook should be supported.

Multi-TTI PUSCH scheduling

Agreement: Scheduling PUSCH over multiple slots/mini-slots by single DCI supports at least multiple continuous PUSCHs with separate TBs - Each TB is mapped to at most one slot or one mini-slot

Scheduling multi-TTI PUSCH with separate TB by a single DCI format has been agreed in the last RAN1 #96 meeting [3] as shown above. Compared to NR Rel-15 DCI indication scheduling one PUSCH at a time, some DCI fields need to be enhanced or redesigned to accommodate multi-TTI scheduling. Here we categorize the DCI fields into three types as the following.

- PUSCH-common: the DCI field is indicated once and commonly applied to all PUSCHs.

- PUSCH-specific: different PUSCHs may require different indications (explicitly or implicitly)

- PUSCH-irrelevant: the DCI field is applied to the whole multi-TTI scheduling, not to PUSCH level

Some major DCI fields are summarized in the following table. Since the DCI field design in eLAA might be a good starting point, we also list fields of DCI format 0B/4B in eLAA for reference.

| DCI field | NR-U | eLAA |
| --- | --- | --- |
| Identifier for DCI formats | PUSCH-common. | N/A |
| UL/SUL | PUSCH-common. | N/A |
| Carrier indicator | PUSCH-common. | PUSCH-common. |
| BWP indicator | PUSCH-common. | N/A |
| MCS | PUSCH-common. | PUSCH-common. |
| FDRA | Can be PUSCH-common or PUSCH-specific | PUSCH-common. |
| TDRA | PUSCH-specific. | N/A |
| NDI | PUSCH-specific. | PUSCH-specific. |
| RV | PUSCH-specific. | PUSCH-specific. |
| HARQ ID | PUSCH-specific. (Can inherit eLAA method) | PUSCH-specific. (Only indicate the first PUSCH) |
| SRS requests | PUSCH-irrelevant. | PUSCH-irrelevant. |
| CSI request | PUSCH-irrelevant. | PUSCH-irrelevant. |

Table 1: Categorization of DCI fields with respect to multi-TTI scheduling

For the PUSCH-common fields, the corresponding legacy DCI fields in NR Rel-15 can be reused without modification. For the PUSCH-specific fields, the corresponding legacy DCI fields in NR Rel-15 should be enhanced with consideration of the increased number of bits. In eLAA, some problems have been solved with elegant mechanisms. For instance, when multiple PUSCHs are scheduled, only the HARQ ID for the first PUSCH is indicated and HARQ IDs for the subsequent PUSCHs can be simply calculated. It is beneficial to inherit this mechanism in NR-U since the problem is almost the same. For the PUSCH-irrelevant fields, since the indication information is for the whole multi-TTI scheduling, the indication can be applied to one predefined TTI or multiple TTIs or all the scheduled TTIs.

The enhancement of TDRA field need to be carefully considered. When the TDRA field is PUSCH-common, the identical time domain resource is assigned to all the scheduled PUSCHs, which is problematic. If the TDRA is configured to implement partial starting (the first PUSCH) or partial ending (the last PUSCH), then it cannot be guaranteed that the middle PUSCHs are continuous. On the contrast, if the TDRA is configured for continuous PUSCH scheduling, then it is not applicable to either the first PUSCH with partial starting or the last PUSCH with partial ending. For the case of PUSCH-specific TDRA field, the main potential issue is how to efficiently indicate different time domain resource for different PUSCHs. Basically, the middle PUSCHs can be indicated by a PUSCH-common indication, while normally the first PUSCH and the last PUSCH require two different PUSCH-specific indications.

In eLAA, all the scheduled PUSCHs are assigned with an identical frequency domain resource. With respect to the time domain resource allocation, in order to implement multiple PUSCH scheduling with partial starting and/or partial ending, several new fields (e.g., timing offset, partial PUSCH Mode 1, 2, 3, etc.) had to be introduced. Like eLAA, at least DCI scheduling multiple PUSCHs should indicate 1) starting OFDM symbol index in the first PUSCH, 2) number of PUSCHs, and 3) ending OFDM symbol index in the last PUSCH. Fortunately, by efficiently exploiting the flexibility provided by TDRA field in NR, this problem might be solved by just enhancing the TDRA field indicating (1), (2), and (3) but without adding new DCI fields.

Observation 1:

- Similar as in eLAA, enhancement on the existing DCI fields and/or introduction of new DCI format(s) for multi-TTI PUSCH scheduling should be considered.

Proposal 3:

- With respect to TDRA field enhancement for NR-U, at least DCI scheduling multiple PUSCHs should indicate 1) starting OFDM symbol index in the first PUSCH, 2) number of PUSCHs, and 3) ending OFDM symbol index in the last PUSCH.

Conclusion

In this contribution we present our views on HARQ enhancement for NR-U operation, and we make the following proposals and observations:

Proposal 1:

- Method(s) of creating multiple HARQ-ACK feedback opportunities in the same shared COT should be supported.

Proposal 2:

- A method of creating multiple HARQ-ACK feedback opportunities in time domain by enhancing the semi-static HARQ-ACK codebook should be supported.

Observation 1:

- Similar as in eLAA, enhancement on the existing DCI fields and/or introduction of new DCI format(s) for multi-TTI PUSCH scheduling should be considered.

Proposal 3:

- With respect to TDRA field enhancement for NR-U, at least DCI scheduling multiple PUSCHs should indicate 1) starting OFDM symbol index in the first PUSCH, 2) number of PUSCHs, and 3) ending OFDM symbol index in the last PUSCH.

References

- RP-182767, 3GPP TR 38.889 V1.1.0 (2018-12), Study on NR-based access to unlicensed spectrum (Release 16)

- Chairman’s notes RAN1 AH #1901, Jan 2019.

- Chairman’s notes RAN1 #96, Feb 2019.