3GPP TSG-RAN WG1 Meeting #96bis	R1-1905477

Xi’an, China, April 8th – 12th 2019

Agenda Item:	7.2.4.2.2

Source:	Ericsson

Title:	Resource allocation for Mode-2 transmissions

Document for:	Discussion, Decision

# 1	Introduction

During RAN1#96, the following was agreed.

- Agreements: - Blind retransmissions of a TB are supported for SL by NR-V2X - Details are for the WI phase - Agreements: - NR V2X Mode-2 supports reservation of sidelink resources at least for blind retransmission of a TB - Whether reservation is supported for initial transmission of a TB is to be discussed in the WI phase - Whether reservation is supported for potential retransmissions based on HARQ feedback is for the WI phase - Agreements: - Mode-2 sensing procedure utilizes the following sidelink measurement - L1 SL-RSRP based on sidelink DMRS when the corresponding SCI is decoded - FFS whether/which measurement is used if the corresponding SCI is not decoded e.g. SL-RSRP after blind DMRS detection, SL-RSSI

In this contribution we discuss aspects related to resource allocation, including the issues left open in the preceding agreements.

# 2	Sensing procedure

The purpose of sensing is to extract information from current transmissions that is relevant for predicting future resource utilization. Information may be extracted from current transmissions in two different ways:

- Reading fields in control messages (e.g., SA in PSCCH, PSFCH, etc.).

- Performing energy/power measurements (e.g., RSRP, RSSI, etc.).

In the following, we discuss each of them separately.

## 2.1	Information contained in control signaling

RAN1 has already agreed to use information about the sidelink resources indicated by the UE transmitting the SCI. Such indication is in fact a reservation of resources. We distinguish two types of reservations:

- Reservations associated with a TB. This is the case in which the UE is reserving resources for a transmission of a TB that is already in the buffer of the UE. For example, as part of resource allocation, the UE may transmit an initial reservation message with a reservation of the resource. Another example is cross-scheduling across retransmissions of a TB (e.g., in LTE the first transmission of a TB reserves the resources for the retransmission of the same TB).

- Reservations not associated with any TB. This is the case in which the UE is reserving resources for a transmission of a TB that is, in general, not present in the buffer of the UE. For example, in LTE the UE has the possibility of reserving a resource for future transmission of a TB (in 20 ms, 50 ms, 100 ms, 200 ms, etc.) even if this TB is not present in the transmit buffer of the UE.

In our view, both types of reservations are necessary in NR.

The sensing procedure extracts from the decoded SCI indications of reservations of resources with and without an associated TB.

Although the corresponding features are not agreed at this point, we believe that the SCI will also carry other information that is directly related to resource utilization such as requests for HARQ feedback and pre-emption messages. The former allows the decoding UE to identify which resources are used for PSFCH transmission and avoid using them. The latter allows the pre-empted UE to stop transmission. In our view, such fields should be contained in SCI and extracting by the sensing procedure.

The sensing procedure extracts requests for HARQ feedback transmissions and pre-emption messages from the decoded SCI.

Depending on future RAN1 agreements, the SCI may contain other relevant information that may be extracted by the sensing procedure. For example, in LTE the PPPP value carried in SCI is extracted during sensing and used for resource selection.

The sensing procedure extracts QoS parameters from the decoded SCI.

## 2.2	Sidelink measurements

In RAN1#96, it was agreed to support the use of L1-RSRP measurements of sidelink DMRS but the specific channel on which the measurement would be performed was not decided. In our view, there are two alternatives:

- RSRP is measured on the DMRS in the PSCCH carrying the SCI.

- RSRP is measured on the DMRS in the PSSCH that accompanies the PSCCH carrying the SCI. This is behavior in LTE.

It is reasonable to expect that in all transmissions, the number of REs carrying PSSCH-DMRS will be larger than the number of REs carrying PSCCH-DMRS. Consequently, RSRP performed on PSSCH-DRMS will be more accurate than RSRP performed on PSCCH-DMRS. On the other hand, the use of PSSCH-DRMS for RSRP measurements places a constraint on the design of the PHY channels in future releases. The consequences derived from such an assumption were discussed for a long time in Rel-15 (LTE). In our view, it is crucial to avoid such problems in the future.

It is expected that the accuracy of RSRP measured on PSSCH-DMRS be higher than that of RSRP measured on PSCCH-DMRS.

Performing RSRP measurements on PSSCH-DMRS may create a backward compatibility problem for future releases.

For sensing purposes, RSRP measurements on PSCCH-DMRS are used if the impact on system level performance is small. Otherwise, RSRP is measured on PSSCH-DRMS.

- Decision to be made after the structure of PSCCH and PSSCH is decided.

In addition to measurements associated with a decoded SCI, in LTE RSSI measurements are performed when no SCI is detected. The measurements are averaged and projected into the future using a 100 ms periodicity. Thus, they allow for detecting periodic transmissions such as CAM or BSM. In contrast to LTE, NR targets a wider array of traffic patterns (periodic, aperiodic, in bursts, etc.). In our view, it is not desirable to make assumptions on the traffic of other UEs. Consequently, we propose not to use energy/power measurements of resources (e.g., based on blind DMRS detection, etc.) for the case in which the corresponding SCI is not decoded.

The use of measurements without the corresponding decoded information (e.g., carried in SCI) is not supported.

Regarding the duration of sensing, our view is that, like in LTE, it should be equal to the longest reservation that can be signaled in SCI (i.e., reservation, scheduling, pattern, etc.). This allows UEs to make the most out of the sensing procedure. For UEs mounted of vehicles, energy is not a concern and, consequently, sensing may be assumed to be always on.

Prior to selection of resources, the UE shall sense the channel during a window of length equal to the longest reservation that can be signaled in the SCI.

# 3	Resource selection procedure

In LTE, the resources are selected with a procedure consisting of the following steps:

- Exclusion of resources based on sensing

- Ranking of remaining resources based on received energy.

- Selection of the 20% resources with lowest received energy.

- Among the low-energy 20%, selection at random of one resource per transmission.

As we have discussed before, in NR the use of energy measurements is not desirable. Consequently steps 2 and 3 in the above procedure are not directly applicable.

Resources for transmission of PSSCH are selected at random from the list of candidate resources.

- FFS whether ranking based on measured RSRP is used.

# 4	Retransmissions and reservations

In this section, we first discuss the use of reservations for the first transmission, which we refer to as initial reservations. Then, we discuss the use of retransmissions and reservations and the relationship between them for broadcast, groupcast, and unicast communication.

## 4.1	Reservations for initial transmission

In RAN#96 it has been agreed to support reservation-based resource allocation for mode-2 at least for blind retransmissions. Moreover, it has been agreed to investigate during the WI phase whether reservation is supported for initial transmission of a TB and whether reservation is supported for potential retransmissions based on HARQ feedback.

Consider the following with reservations for the initial transmission:

Assume a TB arrives at the transmit buffer of a UE at time T = Tarr. - At T = Tarr: - The UE selects a resource for transmission of an initial reservation message. The resource is selected at random from the idle resources in the window (Tarr + T1, Tarr + T2). Let T = Tres denote the time of the selected resource. - In evaluating the state (idle/busy) of the resources in the window, sensing results obtained up to T = Tarr are used. If all resources are busy, then any of them may be selected at random. - At T = Tres: - The UE selects resources for transmission(s) of the TB. The resources are selected at random from the idle resources in the window (Tres + T2, Tres + T3). Let T = Ttx,i denote the time of the resource selected for the ith transmission of the TB. - In evaluating the state (idle/busy) of the resources in the window, sensing results obtained up to T = Tres are used. If all resources are busy, then any of them may be selected at random. - The UE transmits the initial reservation message indicating the resource(s) selected for the transmission(s) of the TB. - At T = Ttx,i: - The UE transmits the TB using the resources indicated in the initial reservation.

T1, T2, and T3 are configurable parameters. An example of channel access using this procedure is illustrated in Figure 1. Two users receive TBs in their TX buffer at time T = n. They both select resources in the window [n+T1, n+T2] for transmission of their reservation messages. The first UE (red) transmits its reservation message at time n+T1. This reservation message is decoded by the second UE (blue) before the transmission of its own reservation message at time n+T1+2. In this way, the second UE (blue) can select resources for transmission of the TB as to avoid a collision with the transmission by the first UE (red).

Figure 1. Illustration of channel access with initial reservations.

Initial reservation messages are transmitted using specific control signaling and only carry information about reserved time-frequency resources. Note that the reservation message is not a replacement for PSCCH, meaning that the initial reservation message does not contain enough information to decode PSSCH. Moreover, it is not necessary to decode the reservation message to be able to process PSSCH.  For very small payloads or very latency-critical messages, initial reservation messages may be skipped altogether.

The reservation message in not a replacement for PSCCH. Decoding initial reservation messages is not a requirement for decoding PSSCH.

For the sake of comparison, we describe a method for resource selection without initial reservation and illustrate it in Figure 2.

Assume a TB arrives at the transmit buffer of a UE at time T = Tarr. - At T = Tarr, the UE selects a resource for the transmission in the window (Tarr + T1, Tarr + T2) with T = Ttx,i the time of the selected resource for the ith transmission. - At T = Ttx,i, the UE transmits the TB using the selected resources.

Figure 2. Illustration of channel access without initial reservations.

In Figure 3, Figure 4, and Figure 5, we show the PRR performance with and without initial reservation messages for broadcast, groupcast, and unicast communications, respecitvely. We observe that the use of reservation for the first transmission greatly improves the performance.

Figure 3: PRR performance of the reservation-based channel access with and without pre-reservation in highway scenario with aperiodic and periodic broadcast traffic.

Figure 4: PRR performance of the reservation-based channel access with and without pre-reservation in highway scenario with aperiodic and periodic groupcast traffic.

Figure 5: PRR performance of the reservation-based channel access with and without pre-reservation in highway scenario with aperiodic and periodic unicast traffic.

In light of this results, we propose to support reservation for initial messages too.

NR sidelink supports the use of reservation messages for initial transmission.

## 4.2	Reservations for blind retransmissions in broadcast

In RAN1#96, it was agreed to support the use of blind retransmissions for broadcast transmissions. In our view, NR sidelink should allow by design for an arbitrary number of retransmissions (this is especially relevant for unicast and groupcast, but we believe that most of the signaling and PHY structures will be common with broadcast). This is different from LTE, where the maximum number of retransmissions was fixed to 2. For this purpose, it is necessary to include multiple fields in SCI that allow for HARQ combination of the retransmissions (e.g., source/destination IDs, HARQ process IDs, redundancy version, etc.). We discuss these aspects in [3] and [4]. In addition, it is necessary to define the behavior in terms of reservations of retransmissions. Consider the following two alternatives:

- Alternative 1: each transmission may reserve resources for a single (future) transmission.

- Alternative 2: each transmission may reserve resources for multiple (future) transmissions.

Alternative 1 requires the smallest number of bits for signaling reservations in SCI. On the other hand, to combine multiple transmissions, the corresponding SCI or a reservation must be decoded. On the other hand, alternative 2 requires a larger number of bits for signaling reservations but is more robust against failed PSCCH decoding. We illustrate both alternatives in Figure 5.

Figure 5. Illustration of blind retransmission schemes for broadcast with Nretr=4 for one reserved resource at the time (alternative 1, left) and with two reserved resources at the time (alternative 2, right).

In Table 1, we illustrate the HARQ combining possibilities of each of the two alternatives for four different situations, each of them consisting of four transmissions. For example, in situation 2, PSCCH is decoded correctly for the first two transmission. Consequently, alternative 1 allows for HARQ combining 3 transmissions of PSSCH, whereas alternative 2 allows for HARQ combining all 4 transmissions of PSSCH.

Table 1. Different decoding situations for four transmissions of a TB. X denotes a PSSCH transmission that can be used for decoding.

|  |  | Situation 1 |  | Situation 2 |  | Situation 3 |  | Situation 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Alt 1 | Alt 2 | Alt 1 | Alt 2 | Alt 1 | Alt 2 | Alt 1 | Alt 2 |
| 1st transmission | PSCCH | Decoded |  | Decoded |  | Decoded |  | Decoded |  |
|  | PSSCH | X | X | X | X | X | X | X | X |
| 2nd transmission | PSCCH | Not decoded |  | Decoded |  | Not decoded |  | Not decoded |  |
|  | PSSCH | X | X | X | X | X | X | X | X |
| 3rd transmission | PSCCH | Not decoded |  | Not decoded |  | Decoded |  | Not decoded |  |
|  | PSSCH |  | X | X | X | X | X |  | X |
| 4th transmission | PSCCH | Not decoded |  | Not decoded |  | Not decoded |  | Decoded |  |
|  | PSSCH |  |  |  | X | X | X | X | X |
| Number of combined PSSCH transmissions |  | 2 | 3 | 3 | 4 | 4 | 4 | 3 | 4 |

Regardless of the supported alternative, we believe that UEs should select resources to reserve for a PSSCH transmission as late as possible. Considering alternative 1 Figure 5, this means that the resources for the second transmission are selected right before starting the first transmission; the resources for the third transmission are selected right before starting the second transmission; and the resources for the fourth transmission are selected right before starting the third transmission.

Selection of reserved resources takes places immediately before sending the reservation.

In Figure 6, we show the PRR performance for broadcast communication with 3 blind transmissions per TB and different number of reservations signaled in SCI. We observe that the gain obtained by having an additional reservation conveyed in SCI is limited.

Figure 6. PRR performance for broadcast traffic for 3 blind transmissions with different number of reservations signaled in SCI.

A single reservation for retransmission of a TB is signaled in SCI.

## 4.3	Reservations for HARQ-based retransmissions in unicast and groupcast

For unicast and groupcast, NR sidelink supports the use of retransmission based on HARQ feedback. One aspect to discuss is the relationship between retransmissions and reservations. We consider two alternative behaviors:

- Behavior 1 (‘Reserved retransmission’). Each transmission of a TB reserves resources for one further retransmission of the same TB:

  - In case of a negative acknowledgement (i.e., reception of NACK) or no acknowledgement at all (for unicast or for groupcast with ACK+NACK), the reserved resources are used for retransmission of the TB.

  - In case of a positive acknowledgement (for unicast or for groupcast with ACK+NACK) or no acknowledgement (for groupcast with NACK-only), the resources are not used for retransmission of the TB.

- Behavior 2 (‘Non-reserved retransmission’). Each transmission of a TB is independently scheduled, without reservation between retransmissions:

  - In case of a negative acknowledgement (i.e., reception of NACK) or no acknowledgement at all (for unicast or for groupcast with ACK+NACK), a new resource is selected and used for retransmission of the TB.

  - In case of a positive acknowledgement (for unicast or for groupcast with ACK+NACK) or no acknowledgement (for groupcast with NACK-only), no further resources are used for retransmission of the TB.

We illustrate both behaviors in Figure 7.

Figure 7. Illustration of transmissions for unicast/groupcast with reservation of the retransmission (left) and without reservation (based on received NACK) in (right).

‘Reserved retransmission’ has the advantage that every retransmission used reserved resources and, consequently, experiences lower probability of collision. On the other hand, we observe two drawbacks. First, the reserved resources may remain unused if a positive acknowledgement is received. Second, link adaptation is constrained to use the reserved resource (e.g., big changes in MCS may not be possible). The situation is reversed for ‘Non-reserved retransmission’: retransmissions are subject to higher probability of collision but, on the other hand, reserved resources do not remain unused and link adaptation is not constrained to use a specific bandwidth.

In our view, ‘Non-reserved retransmission’ will be supported because from a receiver point of view it is not reasonable to assume that all reservations are received. In other words, the receiver must be able to process multiple transmission of a TB independently of whether they are transmitted on reserved resources or not. Moreover, from a receiver point of view it seems unreasonable to mandate reservation of resources. Thus, the question is whether ‘Reserved retransmission’ is supported or not.

In Figure 8 and Figure 9, we show the performance of both alternative behaviors in the highway scenario for unicast and groupcast communication, respectively. HARQ feedback for groupcast makes use of ACK+NACK from every receiver in range. Other simulation assumptions are summarized in the appendix. We observe that ‘Reserved retransmission’ yields significantly better PRR performance than ‘Non-reserved retransmission’. For comparison, we have also included the performance when the retransmissions are blind (‘Blind retransmission’). We observe that for the case of groupcast, the difference between ‘Reserved retransmission’ and ‘Blind retransmission’ is negligible, the reason being that the maximum number of retransmissions are always used due to the large size of the groups.

Figure 8. PRR performance for ‘Reserved retransmission’, ‘Non-reserved retransmission’, and ‘Blind retransmission’ with aperiodic unicast traffic.

Figure 9. PRR performance for ‘Reserved retransmission’, ‘Non-reserved retransmission’, and ‘Blind retransmission’ with aperiodic groupcast traffic.

Given these, results we propose to support ‘Reserved retransmission’.

For unicast and groupcast, it is supported that each transmission of a TB reserves resources for a future retransmission of the TB:

- The TB may only be retransmitted if a negative acknowledgement is received or if a positive acknowledgement is expected but not received.

- FFS whether the resources can be used for transmission of a different TB after receiving a negative acknowledgement.

# 5	Conclusion

In the previous sections we made the following observations:

Observation 1	It is expected that the accuracy of RSRP measured on PSSCH-DMRS be higher than that of RSRP measured on PSCCH-DMRS.

Observation 2	Performing RSRP measurements on PSSCH-DMRS may create a backward compatibility problem for future releases.

Observation 3	The reservation message in not a replacement for PSCCH. Decoding initial reservation messages is not a requirement for decoding PSSCH.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The sensing procedure extracts from the decoded SCI indications of reservations of resources with and without an associated TB.

Proposal 2	The sensing procedure extracts requests for HARQ feedback transmissions and pre-emption messages from the decoded SCI.

Proposal 3	The sensing procedure extracts QoS parameters from the decoded SCI.

Proposal 4	For sensing purposes, RSRP measurements on PSCCH-DMRS are used if the impact on system level performance is small. Otherwise, RSRP is measured on PSSCH-DRMS.

	Decision to be made after the structure of PSCCH and PSSCH is decided.

Proposal 5	The use of measurements without the corresponding decoded information (e.g., carried in SCI) is not supported.

Proposal 6	Prior to selection of resources, the UE shall sense the channel during a window of length equal to the longest reservation that can be signaled in the SCI.

Proposal 7	Resources for transmission of PSSCH are selected at random from the list of candidate resources.

	FFS whether ranking based on measured RSRP is used.

Proposal 8	NR sidelink supports the use of reservation messages for initial transmission.

Proposal 9	Selection of reserved resources takes places immediately before sending the reservation.

Proposal 10	A single reservation for retransmission of a TB is signaled in SCI.

Proposal 11	For unicast and groupcast, it is supported that each transmission of a TB reserves resources for a future retransmission of the TB:

	The TB may only be retransmitted if a negative acknowledgement is received or if a positive acknowledgement is expected but not received.

	FFS whether the resources can be used for transmission of a different TB after receiving a negative acknowledgement.

# 6	 References

TR 37.885, “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR (Release 15),” v15.2.0, 3GPP.

TR 38.885, “Study on NR Vehicle-to-Everything (V2X) (Release 16)”, v16.0.0., 3GPP

R1-1905480, “Physical layer procedures for sidelink,” Ericsson, RAN1#96bis, Apr 2019.

R1-1905490, “On SCI and SFCI formats,” Ericsson, RAN1#96bis, Apr 2019.

# Appendix: evaluation assumptions

Unless otherwise states, the simulation assumptions follow [1] and [2]. Simulator parameters are summarized in Table 1.

Table 2. Simulation parameters.

| Parameter |  | Value |
| --- | --- | --- |
| Carrier frequency |  | 6 GHz |
| Bandwidth |  | 20 MHz |
| Sub-channel size |  | 12 RBs |
| Sub-carrier spacing |  | 30 kHz |
| Scenario |  | Highway Option A |
| Traffic model |  | Medium intensity, according to the corresponding simulation profile (see [2]). |
| PSCCH | Allocation (including overhead) | 3 OFDM symbols |
|  | Modulation | QPSK |
| PSSCH | Allocation (including overhead) | 9 OFDM symbols (if PSCCH is transmitted in the same slot) 14 OFDM symbols (if PSCCH is not transmitted in the same slot) |
|  | Modulation | 16-QAM |
|  | Coding rate (CRtarget) | 0.85 |
| Overhead (for both PSSCH and PSCCH) |  | 4/14 (GP, DMRS, AGC) |
| Reservation-based channel access parameters | Reservation allocation | 1 slot and 5 RBs |
|  | T1 | 1 slots |
|  | T2 | 2 slots |
|  | T3 | 15 slots (unicast) / 25 slots (broadcast) |
| Sensing distance (dsensing) |  | 750 m (highway) |
| Unicast /groupcast session distance |  | 500 m |
| Maximum number of retransmissions |  | 2 |