3GPP TSG RAN WG1 #96bis	R1-1905510

Xi’an, China, 8th – 12th April, 2019

Agenda Item:	7.2.11.3

Source:	NTT DOCOMO, INC., Fraunhofer HHI and Rohde & Schwarz

Title:	Discussion on fast fading model for indoor industrial scenarios

Document for:	Discussion

1	Introduction

The study on channel model for frequencies from 0.5 to 100 GHz (3GPP TR 38.901) defines a wideband channel model and gives model parameters for certain scenarios. It is assumed, that the industrial indoor scenario has significantly different propagation characteristics compared to the scenarios covered in the study. Additionally, many of the given parameters need further support from measurement campaigns. Thus, there is strong demand for further channel sounding campaigns and channel investigations.

2	Methodology

A systematic channel sounding campaign at 28 GHz was performed within a factory environment. The investigated factory hall has dimensions of 100 m x 70 m and a height of around 7 m. The hall is used as an assembly space for electronic devices and it is densely equipped with assembly and production cells. The cells have a constant height of 2.5 m and consist mainly of tables, shelves and racks for instrumentation (see panoramic view below). Between the different production cells are aisles with a width of around 2 m and there is also a broader circular aisle along the outer walls. There are no walls within the hall.

For the measurement campaign an omnidirectional transmit antenna was placed on a balcony at one outer wall at a height of around 4.5 m over the floor of the main hall. The receiver was mounted on a cart, which was moved along certain tracks along three different aisles. The receiving antenna was 1.5 m above the floor. On these tracks every 0.6 m a measurement snapshot was taken. One aisle was under line-of-sight (LOS) conditions facing directly to the transmitter, whereas the other two aisles were under obstructed-line-of-sight (OLOS) conditions. In these aisles at about 50% of the positions, the LOS path was obstructed. An overall number of 50 snapshots was taken for the LOS case and a number of 130 snapshots was taken for the OLOS case.

The measurements were performed with a virtual circular array at the receiver side, embodying 1000 virtual omnidirectional array elements on a circle of 100 mm diameter. For each of the virtual array elements a channel sounding with a correlation sequence of 50 µs length was implemented, resulting in 50 ms measurement time for one snapshot. Thus, 1000 channel impulse responses were gathered from the virtual array, for one measurement position i.e. for one channel snapshot. The measurement bandwidth was 2 GHz, the carrier frequency was 28 GHz and the transmit power was 28 dBm.

The 1000 channel impulse responses were used to calculate an averaged power delay profile (APDP) to compensate for the small scale fading with the aperture of the virtual array. The figure below shows an exemplarily result for a measurement in OLOS conditions at a distance of around 45 m from the transmitter. Due to the high number of virtual array elements, a very high dynamic range and a low noise floor could be achieved.

From the APDP, peaks were detected which have a certain prominence (in this case 1.5 dB). At the position of each peak i.e. delay tap an estimation of contributing paths was performed resulting in a list of contributing paths including magnitude and angle of arrival (azimuth and elevation), whereas multiple paths per tap could be determined. The evaluation was done with respect to a constant threshold of 40 dB relatively to the strongest tap. Typically, a number of 100 delay taps and a number of 200 paths were found. The following pictures show the evaluated taps and the angular power spectrum in azimuth for the exemplarily given APDP.

From this list of paths, large scale channel modeling parameters could directly be calculated using the definitions in TR 38.901.

3	Observations

In the following RMS delay spread and RMS angle spread are discussed. Each spread was evaluated for the measurements under LOS and OLOS conditions separately. For these samples the logarithmic channel model parameters according to TR 38.901 were calculated.

2.1	Delay Spread

| Line-of-Sight | Obstructed-Line-of Sight |
| --- | --- |

|  | Line-of-Sight | Obstructed-Line-of-Sight |
| --- | --- | --- |
| µlgDS | -7.67 | -7.27 |
| lgDS | 0.09 | 0.16 |

The results for the delay spread are given in the table above. The parameters for OLOS are significantly larger than the parameter for LOS. Furthermore the normal distribution fits the OLOS samples not quite well.

2.2	AOA Spread

| Line-of-Sight | Obstructed-Line-of-Sight |
| --- | --- |

|  | Line-of-Sight | Obstructed-Line-of-Sight |
| --- | --- | --- |
| µlgASA | 1.38 | 1.77 |
| lgASA | 0.16 | 0.21 |

The results for the angle spread are given in the table above. The parameters for OLOS are larger than the parameter for LOS. The normal distribution (red curve) does not fit the OLOS samples so well.

References

3GPP TR 38.901 (V15.0.0): “Study on channel model for frequencies from 0.5 to 100 GHz".

J. Karedal, S. Wyne, P. Almers, F. Tufvesson and A. F. Molisch, "UWB channel measurements in an industrial environment," IEEE Global Telecommunications Conference, 2004. GLOBECOM '04., Dallas, TX, 2004, pp. 3511-3516 Vol.6.

E. Tanghe et al., "The industrial indoor channel: large-scale and temporal fading at 900, 2400, and 5200 MHz," in IEEE Transactions on Wireless Communications, vol. 7, no. 7, pp. 2740-2751, July 2008.