3GPP TSG RAN WG1 Meeting #96 bis 	R1-1905511

Xi'an, China, April 8 - 12, 2019

Agenda Item:	7.2.11.4

Source:	NTT DOCOMO, INC., Fraunhofer HHI and Rohde & Schwarz

Title:	Proposal for additional delay for absolute time arrival modeling in NLOS environment

Document for:	Discussion

Introduction

In email discussion on SI on Channel modeling for Indoor Industrial scenarios, way forward in which absolute time of arrival of multipath components is added as an additional modeling component was agreed [1, 2]. In this contribution, channel model for absolute additional delay in NLOS environment is proposed based on our measurements.

Measurement

Delay profile measurements were performed in our lab and absolute time of arrival of first wave in NLOS environment is studied．Measurement environment is same as the environment which is described in our contribution regarding delay spread at RAN1#96 [3, 4]. Figure 1 shows measurement route and Table 1 shows measurement parameters. Figure 2 shows the distribution of measured delay spreads in NLOS environment respectively.

Fig. 2 Distribution of measured delay spread in NLOS environment

Fig. 3 Distribution of Δτ in NLOS environment

Fig. 4 Histogram of Δτ in NLOS environment

In this contribution, channel model for absolute additional delay in NLOS environment is studied. As a starting point, the model is assumed as

τ0= d3Dc+ Δτ		                                                                                                                              (1)

τ0 is absolute time of arrival of first wave (s), d3D is 3D distance between BS and MS (m), c is speed of light (m/s) and Δτ is additional delay (s). To model Δτ, τ0 and are obtained from measurement and the distribution of Δτ is studied. Figure 3 and Fig. 4 shows CDF and histogram of Δτ respectively. From results, the distribution can be approximated as Gaussian distribution. And median is 55 (ns) and standard deviation is 28 (ns).

Observation: Additional delay Δτ can be approximated as Gaussian distribution.

Proposal: Absolute time of arrival in NLOS environment is given by

τ0=max⁡(d3Dc, τ0')                                                                                                                                               (2a)

τ0'= d3Dc+ Δτ                                                                                                                                        (2b)

pΔτ= 12πσΔπexp-(Δτ-Δτmed)22σΔπ2                                                                                                             (2c)

d3D is 3D distance between BS and MS (m), c is speed of light (m/s) and Δτ is additional delay (s), pΔτ is probability density function of Δτ, Δτmed is median of Δτ ,and σΔπ is standard deviation of Δτ. Value of Δτmed and σΔπ is FFS.

Conclusion

In this contribution, channel model for absolute additional delay in NLOS environment is studied. The followings are our observations and proposals based on the discussion.

Observation: Additional delay Δτ can be approximated as Gaussian distribution.

Proposal: Absolute time of arrival in NLOS environment is given by

τ0=max⁡(d3Dc, τ0')

τ0'= d3Dc+ Δτ

pΔτ= 12πσΔπexp-(Δτ-Δτmed)22σΔπ2

d3D is 3D distance between BS and MS (m), c is speed of light (m/s) and Δτ is additional delay (s), pΔτ is probability density function of Δτ, Δτmed is median of Δτ ,and σΔπ is standard deviation of Δτ. Value of Δτmed and σΔπ is FFS.

- References

RP-182138, SID on Channel Modeling for Indoor Industrial Scenarios, Ericsson, 3GPP TSG-RAN Meeting #81, Gold Coast, Australia, September 10th – 13th 2018.

R1-1903124, Email discussion on measurements and observations, Ericsson, 3GPP TSG-RAN Meeting #96, Athens, Greece, February 25th – March 1st, 2019

R1-1813337, On accuracy of InH path loss model of 38.901 for factory scenario, NTT DOCOMO, INC., Spokane, WA, USA, November 12th – 16th, 2018.

R1-1902817, Discussion on delay spread for factory scenario, NTT DOCOMO, INC., 3GPP TSG-RAN Meeting #96, Athens, Greece, February 25th – March 1st, 2019.