[Offline Agreement] Proposal 2:

At least for eMBB with multi-DCI based multi-TRP/panel transmission, different PDSCH scrambling sequences can be supported for PDSCHs, and down-select from following alternatives in RAN1 97:

- Alt 1: enhance c_init, FFS detailed design in RAN1 97

- Alt 2: enhance RRC configurations to support multiple dataScramblingIdentityPDSCH

FFS whether/how to support separated parameters for M-TRP, e.g. for scrambling ID, TCI state lists.

[Draft of Proposal]

- Study PDCCH/PDSCH processing timing for supporting multiple PDCCH for potential relaxation if needed, considering

  - overlapped scheduling PDCCH and scheduled PDSCH,

  - overlapped PDSCH and PDSCH,

  - PDSCH mapping scheduling types,

  - MIMO capability considering the implementation of CA/DC, for example

    - UE capability considering CA/DC relaxation

    - UE capability considering no CA/DC relaxation

[Offline Agreement]  Proposal 4:

  - For PDCCH monitoring and blind decoding  for multi-DCI based multi-TRP/panel transmission,

    - introduce a rule for prioritizing PDCCH candidates from different search spaces when they collide, conditioned on

      - Alt 1: CORESET ID per TRP

      - Alt 2: CORESET group ID per TRP

    - Increase the maximal number of CORESETs per “PDCCH-config” up to  N=[4,5 or 6],  subject to UE capability design

    - Increase the maximal number of BD/CCE per slot per serving cell, subject to UE capability design

- [Offline Agreement]  Proposal 5:

  - For separate ACK/NACK payload/feedback for received PDSCHs where multiple DCIs are used, [at least for eMBB with non-ideal backhaul],

    - Support TDMed PUCCH transmission within a slot to convey, at least separate ACK/NACK only feedback, with separated HARQ-ACK codebook for two TRPs,

      - Further restrictions for PUCCH transmission within the slot can be discussed

    - Study following alternative, for example

      - All PUCCH resource configuration within the first PUCCH resource group do not overlap in time with any PUCCH resource configuration within the second PUCCH resource group, considering

        - how to support PUCCH resource groups composed with resources or resource sets

      - Whether/how to support semi-static PUCCH resource groups configuration or [dynamic separation of PUCCH resources at time domain]

      - Whether/how to drop PUCCH resource(s) when PUCCH resources are overlapped.

      - UL TRP differentiation  can be identified by a CORESET group associated to the PUCCH resource group and down-select:

        - 5-1: introduce a group ID to explicitly define a CORESET group for UL TRP differentiation

        - 5-2: reuse NR Rel-15 TCI State and Spatial Relation frameworks to implicitly define a CORESET group for UL TRP differentiation

    - Support long-PUCCH + long-PUCCH formats within one slot

  - Study whether parameters in PUCCH-config and PUSCH-config shall be independently configured for M-TRP with non-ideal backhaul

  - Support joint HARQ-Ack feedback from received PDSCHs where multiple DCIs are used, at least for eMBB with ideal or close-ideal backhaul

- [Draft for offline] Proposal 6:

  - For multi-DCI based multi-TRP/Panel transmission, at least for eMBB, support to increase the  number of HARQ processes to 32,

    - FFS further details of how to increase explicitly or implicitly and associated number of HARQ entities  in RAN1 97

    - Note that the support is subject to UE MIMO and/or CA capability design in RAN1-98