3GPP TSG RAN WG1 Meeting #96bis															R1- 1905751

Xi’an, China, April 8th – 12th, 2019

Source:	Ericsson

Title:	Offline session1 notes for NR Positioning AI - 7.2.10.1 DL and UL Reference Signals for NR Positioning (UL Part)

Agenda item:	7.2.10.1

Document for:	Discussion and decision

Introduction

This document is the feature lead summary concerning the Uplink reference signal design discussion for NR positioning, based on the contributions provided in reference.

UL RS design for positioning

Scope of signals

- The NR positioning WID[1] states that the work item shall “identify whether and which Rel-15 NR reference signals can be used for different NR positioning techniques”. After completion of the study item phase, TR 38.855 stated:

- The following candidate reference signals were identified for UL positioning evaluation and are to left for future specification work - - NR PRACH - - NR SRS - - NR UL DMRS - - NR UL PTRS - - New UL positioning reference signals (UL PRS)

- The following proposal were identified regarding which signals to include in the specification support for positioning:

| Source | Proposal |
| --- | --- |
| - [9] | Proposal 17: Both SRS and PRACH should be supported for UL positioning and following principles should be considered for further enhancements (Samsung) - NR reference signal design and configuration already provide high level of flexibility so that further enhancements to support positioning functionality should be minimized. - Same reference signal should be chosen for joint measurement of both timing advance and angle when possible to reduce complexity, latency and storage size |
| - [17] | Proposal 9 Keep NR PRACH as a viable candidate for UL positioning (Ericsson) |
| - [17] | Proposal 10 PRACH for the purpose of positioning need to be distinguished from initial access PRACH (Ericsson) |
| [17] | Proposal 11 UL DMRS and UL PTRS are not considered for the purpose of positioning (Ericsson) |

Based on these proposals the following potential consensus are proposed:

UL DMRS and UL PTRS are not considered for the purpose of positioning

Both SRS and PRACH should be supported for UL positioning

- FFS resolving of ambiguity of PRACH use for positioning or initial access

| Qualcomm | - Support Proposal 2. |
| --- | --- |
| Fraunhofer | We prefer to focus on SRS enhancement. |

UL SRS for positioning

SRS usage

The following  proposals mention a new SRS usage for positioning

| source | proposal |
| --- | --- |
| [4] | Proposal 11: For UL positioning, the following enhancements on SRS configuration should be considered (Vivo), - SRS resource set with usage of ‘positioning’ is supported. |
| [11] | Proposal 17: Support a new set usage for SRS, “positioning” (Mitsubishi) |
| [15] | Proposal 18: Define a new usage of Rel-16 SRS for positioning purpose. (Nokia) |

Define a new usage of Rel-16 SRS for positioning purpose.

| Qualcomm | - We are fine with the intention, but we may need to be a bit more precise. The SRS resource set have a purpose in Rel-15, so I assume we don’t plan to change this approach right? - Then, is it a common understanding that the same SRS resource can be reused between a set of “communication purposes” and a set of “positioning purposes”? Or are we saying that we will always require a new different physical transmission from the UE to perform separate purposes? Maybe this can be left for further discussion at this point, but it would be best if it is captured. So what about the following proposal: - Define a new usage of Rel-16 SRS resource set for positioning purpose. - Discuss further the case of SRS resource reuse across different SRS resource sets. |
| --- | --- |
| Fraunhofer | We agree to Qualcomm on the usage definition for positioning purposes. |

Configuration of Periodicity

Support of periodic, aperiodic and semi persistent configuration is discussed by several papers, with the following proposal:

| Source | proposal |
| --- | --- |
| [18] | Proposal 3: Support a semi-persistent allocation of the ULP-slot. (Fraunhofer) |
| [16] | Proposal 20: The existing configurations for SRS transmission mode, transmission periodicity and transmission offset can be reused for NR positioning. (CATT) |
| [3] | Proposal 19: Reuse periodic and semi-persistent SRS resource set configuration from Rel. 15 NR for positioning purposes. (Huawei) |
| [17] | Proposal 12 The UE is configured with SRS for positioning by the serving cell. The location server configures the serving and non serving cells with the appropriate assistance data. (Ericsson) |
| [17] | Proposal 13 UL SRS for positioning can be configured to be periodic, aperiodic, semi-persistent (Ericsson) |
| [17] | Proposal 15: The legacy NR Rel-15 time-domain behaviors of SRS resources, i.e., periodic, semi-persistent, aperiodic, can be used as UL PRS. |

Support semi persistent configuration of SRS for positioning

Support periodic configuration of SRS for positioning

Support aperiodic configuration of SRS for positioning

| Qualcomm | - Support having all options. Note that this discussion can be related to the previous one: if an SRS resource is associated to two purposes, then since for the “communication purpose” we already support all three time-domain behaviors, it would mean it would make sense to be supported for positioning purposes also, to reduce SRS overhead. |
| --- | --- |
| Fraunhofer | - Support Proposal 4 |

Number of symbols

The following proposals discuss the number of SRS symbols in UL positioning, with different approaches

| Source | proposal |
| --- | --- |
| [8] | Proposal 12: For the purpose of NR positioning, increase number of symbols for UL SRS (PRS) transmission within single UL SRS (PRS) Resource to 8 and 14. (Intel) |
| [18] | Proposal 1: SRS can be allocated in any of up to the [12] of the 14 OFDM symbols of an ULP slot (slot shared by multiple UEs containing SRS signals for positioning use) (Fraunhofer) |
| [10] | Proposal 3b-2: Consider the increase of the SRS symbol number to 6 with staggering pattern, which is similar to the downlink RS structure (Mediatek) |
| [4] | Proposal 11: For UL positioning, the following enhancements on SRS configuration should be considered, (Vivo) - SRS can transmit on whole symbols within a slot. |

- Offline consensus: For positioning, regarding the SRS symbols per resource, downselect by RAN1#97 between the following alternatives:

- Alt. 1: 1, 2 or 4 SRS symbols per resource (legacy NR Rel-15 behavior),

- Alt. 2: Increase the set of allowable number of SRS symbols per resource compared to the NR Rel-15 allowable set {1,2,4}. FFS: Details

- Offline consensus: For positioning, regarding the SRS symbols per slot downselect by RAN1#97  between the following alternatives:

- Alt. 1: SRS can be configured only in the last 6 symbols of a slot (legacy),

- Alt. 2 : SRS can be configured only in the last N symbols of a slot with N>6

  - FFS: Value of N

  - Note: If N=14, then any symbol of the slot may be configured for SRS transmissions.

| Qualcomm | - We think that both proposals 7/8 may need some restructuring. There are 2 issues being addressed here: number of SRS symbols per resource, and number of SRS symbols per slot. - Also, the SRS is not used only for “UL positioning” but also for “DL&UL positioning”, so the expression “UL positioning” may create confusion in the future. - We propose to restructure the options as follows: - For positioning, regarding the SRS symbols per resource downselect between the following alternatives: - Alt. 1: 1, 2 or 4 SRS symbols per resource (legacy NR Rel-15 behavior), - Alt. 2: Increase the set of allowable number of SRS symbols per resource compared to the NR Rel-15 allowable set {1,2,4}. FFS: Details - For positioning, regarding the SRS symbols per slot downselect between the following alternatives: - Alt. 1: SRS can be configured only in the last 6 symbols of a slot (legacy), - Alt. 2 : SRS can be configured only in the last N symbols of a slot with N>6 - Note: If N=14, then any symbol of the slot may be configured for SRS transmissions. - FFS: Value of N |
| --- | --- |
| ericsson | - We agree with the proposed rewording. |
| Fraunhofer | - We also propose to have an option to define an ULP (UL slot for positioning). Defining an SRS with one ULP slot dedicated for positioning minimizes the standardization impact when it comes to interference coordination. - The assumptions on coordination of SRS resources within the positioning network should be clarified, so that the interference scenarios can be identified and taken into account in the simulation results. - The proposal can reformulated as: - For positioning, regarding the SRS symbols per slot downselect between the following alternatives: - Alt. 1: SRS can be configured only in the last 6 symbols of a slot (legacy), - Alt. 2 : SRS can be configured only in the last N symbols of a slot with N>6 - FFS: Value of N - FFS: UL slot for positioning for N values >6 - - Note: If N=14, then any symbol of the slot may be configured for SRS transmissions. |

Comb separation

Multiple contributions have given proposals regarding the comb design for SRS for positioning, several calling for a larger comb size, and also mentioning the use of staggering patterns in conjunction with combs.

| Source | Proposal |
| --- | --- |
| [18] | Proposal 2: For the SRS enhancements, the following shall be supported (Fraunhofer) • higher comb numbers in combination with staggering • code multiplex for the SRS for positioning use (e.g. by assigning individual Zadoff-Chu sequence root index to each UE sharing the same RE). |
| [8] | - For the purpose of NR positioning, additionally support comb-6 and comb-12 transmission patterns for UL SRS(PRS) configuration (Intel) |
| [15] | Proposal 14: The baseline SRS for UL positioning is NR SRS comb-2 or comb-4 with staggering allocation with adopting Rel-15 NR SRS configurations. (Nokia) Proposal 15: Consider comb-N SRS structure additionally for UL positioning with cases N>4 (i.e. N=6), which requires Rel-15 SRS specification changes. (Nokia) |
| [3] | Proposal 20: For positioning purposes, SRS resource mapping in time and frequency domains follows the SRS configuration in NR Rel. 15. (Huawei) |
| [16] | Proposal 19: Staggered SRS patterns can be considered for NR positioning. (CATT) |
| [4] | Proposal 12: The staggered pattern with 4 symbols of SRS should be adopted for uplink positioning.(Vivo) |

For positioning, downselect the options for supported SRS comb size between

    - Option 1: legacy (comb 2 and 4) with extension with comb-6

    - Option 2: legacy (comb 2 and 4) with extension with comb-6 and comb-12

    - Option 3: legacy (comb 2 and 4) with no extensions

Staggered patterns are supported for UL SRS

    - FFS details on how staggering is implemented

| Qualcomm | - For proposal 9, is the understanding that we are supposed to downselect one of the options? If yes, then it is best if it is phrased this way. |
| --- | --- |
| Fraunhofer | - Support Proposal 10. Clarification is needed on the extension meaning in Proposal 9. |

Alignment with DL signals / beam correspondence

The following proposals discuss the issue of the relation between the SRS beams and DL beams.

| Source | Proposal |
| --- | --- |
| [16] | Proposal 21: The beam information of the DL reference signals (e.g., SSB, CSI-RS tec.) and UL reference signals (e.g., SRS) should be included in the messges exchanged between UE, gNBs, and LMF for supporting UL positioning based on SRS signals. (CATT) |
| [3] | Proposal 15: Extend the existing UE configuration to receive reference signal for SRS spatial domain transmission filter from the neighboring gNBs.(Huawei) |
| [14] | Proposal 11: The spatial Relation info of an SRS resource can be a configured to be a DL Reference Signal (e.g., PRS resource, CSI-RS resource, SSB) from the serving or a neighboring BS. |
| [9] | Proposal 18: For AoA/AoD based positioning exploring beam information, the existing NR beam measurement, reporting and management mechanism is sufficiently flexibly to support positioning except a reference beam pointing to a pre-defined geographic direction needs to be defined. (Samsung) |
| [9] | Proposal 22: UL beam sweeping should be supported and the same mechanism for DL beam sweeping can be employed. Beam alignment is not supported. (Samsung) |
| [7] | Proposal 6: The uplink positioning reference signal (e.g. SRS) beam configuration are determined based on the UE reports on selected downlink measurement information. (Sony) |

Based on the variety of the proposals, it is suggested to continue discussions on the topic.

SRS TA for positioning

Some companies have issued proposals regarding TA procedure extensions for support of neighbour (i.e. non serving) cells. However, other have proposed to keep rel-15 procedure as they are.

| Source | Proposal |
| --- | --- |
| [16] | Proposal 22: NR should avoid defining new SRS timing advance procedure for supporting NR positioning unless it is demonstrated with the clear benefit for NR positioning without adverse impact on NR data communication. (CATT) |
| [3] | Proposal 17: For positioning purposes, the UE obtains gNB-specific TA using timing measurement from a configured DL reference signal. (Huawei) |
| [2] | The feasibility of Multi-cell TA estimation and UL PRS power control should be studied and corresponding procedure should be identified. (ZTE) |
| [7] | Proposal 21: The propagation delay difference between the serving cell and the neighbor cells can be handled by the CP and thus there is no need to change TA configuration.(Sony) |
| [14] | Proposal 13: In order to perform SRS transmission for positioning purposes, the TA value applied to the corresponding UL symbol is the same as the latest TA for regular UL symbols transmitting to the serving gNB.Qualcomm) |
| [15] | Proposal 16: Study TA control methods for SRS reception of neighbouring cells such as cell specific TA control or further enhancements. (Nokia) |
| [14] | Proposal 12: For an SRS resource set used for the legacy NR Rel-15 use-cases, no change in the UL power control, or UL timing advance should be considered even if these SRS resources are also used for positioning purposes. (Qualcomm) |

Based on the proposals, the following consensus is proposed:

The SRS TA for the purpose of positioning follows rel-15 legacy procedures, e.g using the latest configured TA value for the UL serving cell transmissions.

Further Discuss Supporting additional TA procedures specifically for positioning to e.g. support the SRS reception in the neighbouring cells.

Power control

Several paper propose extensions of power control procedures for positioning in order to address neighbour cell hearability.

| Source | proposal |
| --- | --- |
| [16] | Proposal 23: NR should avoid defining new SRS power control procedure for supporting NR positionin, unless it is demonstrated with the clear benefit for NR positioning without adverse impact on NR data communication. (CATT) |
| [3] | Proposal 16: For positioning purposes, support configuring the UE to measure the pathloss from neighboring gNBs based on received DL-RS sent from the neighboring gNBs. (Huawei) |
| [6] | Transmission of the additional UL PRS resources that have different transmission power could be considered for the enhancement of the hearability of UL PRS.(LG) |
| [15] | Proposal 17: Study further transmission power control methods for UEs nearby to their serving cell, to improve hearability of neighbouring cells. (Nokia) |
| [14] | Proposal 12: For an SRS resource set used for the legacy NR Rel-15 use-cases, no change in the UL power control, or UL timing advance should be considered even if these SRS resources are also used for positioning purposes. (Qualcomm) |
| [7] | Proposal 5: The transmit power of uplink positioning reference signal (e.g. SRS) is determined based on the downlink measurement information of one or more gNBs. (Sony) |
| [2] | The feasibility of Multi-cell TA estimation and UL PRS power control should be studied and corresponding procedure should be identified. (ZTE) |
| [4] | Proposal 13: For UL positioning, support power control mechanism enhancement to transmit SRS for all positioning cells. |
| [9] | Proposal 20: In order to improve the hearability of UL PRS, both open loop and closed loop power control can be considered (Samsung) · For open loop power control, UE can transmit based on the weakest link of surrounding TRPs/gNBs as long as it is lower than the maximum allowed transmit power per carrier; · For closed loop power control, UE can receive multiple TPCs from multiple TRPs/gNBs and adjust its transmission power based on the received TPCs, e.g., maximum power, average power. |

For power control, UL SRS for positioning follows

    - Option 1: legacy procedures from NR rel-15

    - Option 2: a to-be define new procedure, aimed at improving hearability for neighbor cells

SRS bandwidth for positioning

For bandwidth parts the majority of proposals aim at preserving the legacy configurations

| source | proposal |
| --- | --- |
| [16] | Proposal 24: No additional enhancement is needed in terms of the SRS transmission bandwidth for NR positioning. (CATT) |
| [3] | Proposal 21: Carry over the configuration of SRS for positioning per UL BWP from NR Rel. 15. (Huawei) |
| [3] | Proposal 22: Carry over SRS transmission in the active UL BWP of an active cell and SRS carrier switching from NR Rel. 15. (Huawei) |
|  | Proposal 14: Carry-over the association of an SRS with an UL BWP from NR Rel-15 to NR Rel-16. Use the legacy features of UL BWP switching and UL SRS carrier switching for transmitting SRS used for positioning in other BWPs or CCs. |
| [9] | Proposal 19: For the BWP configuration of NR UL PRS, e.g. SRS, both solutions as follows should be supported (Samsung) Alt1: SRS is configured within the active UL BWP; Alt2: SRS is configured in a dedicated BWP with the largest bandwidth or subcarrier spacing. |

Therefore the proposal for consensus is as follow

For positioning The SRS bandwidth is configured as in Rel-15 NR

SRS repetition

Two contribution discussed repetitions, with a proposal to extend the repetition factor beyond 4

| Source | proposal |
| --- | --- |
| [14] | Proposal 10: NR Rel-16 should support configuration of SRS resource with repetition factor with up to N consecutive symbols, where N is larger than 4. Study further the value of N. (Qualcomm) Proposal 9: NR Rel-16 should support configuration of SRS resource with repetition across N symbols with comb-N such that all N frequency domain offsets are used (i.e., frequency-domain staggering), where N = 2 or 4. (Qualcomm) |
| [4] | Proposal 11: For UL positioning, the following enhancements on SRS configuration should be considered, (Vivo) - SRS repetition on is supported. |

For UL positioning SRS repetitions is supported with repetition factor N

    - FFS value of N

Discuss the connection between repetition and staggering patterns

Others

Beside proposals common to several companies, many proposals were discussing issues only presented by one contribution. It is propose to discuss these proposals if time allow or at a later meeting.

Proposal 5:	Study group management and coordinate the related procedures with higher layers. (Fraunhofer)

Proposal 3a-1: Study the influence of FDSS to the timing measurement accuracy (Mediatek)

Proposal 3a-2: The backward compatibility should also be considered seriously for potential development of pi/2 BPSK based SRS (Mediatek)

Proposal 25: The configuration of UL PRS should be exchanged between serving gNB and neighboring gNBs via X2 interface so that the neighboring gNBs know when and where exactly to measure UL PRS..(Samsung)

Proposal 18: The bit width of parameter sequenceId of the SRS resource configuration is [16] bits. (Huawei)

Further consider to introduce time and frequency domain code multiplexing for UL SRS (PRS) in order to increase the UE multiplexing capacity for NR positioning in UL (Intel)

Proposal 4:	If several UEs share the same time/frequency, the selection of different ‘activity pattern’ (e.g. different transmit periods per UE) is an additional possibility along code multiplex to further increase the number of supported positioning UEs. .(Fraunhofer)

Proposal 3b-1: One or two SRS antenna ports can be considered. The two antenna ports transmission can be through different polarizations (Mediatek)

- Proposal 14: In addition to ZC sequence, support to reuse DMRS sequence for DFT-s-OFDM waveform based PUSCH for pi/2- BPSK modulation agreed in Rel-16 for SRS sequence generation for positioning.(Vivo)

| Fraunhofer | - User group management for UTDOA will reduce interference issues known as “near/far” when using code-multiplex. From RAN1 perspective the impact is minimal. The network coordination needed is a higher layer issue however in our opinion RAN1 should study the benefits of this or similar solutions. This should also include interference assumptions for the simulation to be carried when analyzing the UTDOA scenarios. |
| --- | --- |

Conclusions

This contribution attempted to summarises the proposals for UL positioning reference signals in AI 7.2.10.1. the follow proposals were made:

And the following proposals:

Proposal 1	UL DMRS and UL PTRS are not considered for the purpose of positioning

Proposal 2	Both SRS and PRACH should be supported for UL positioning

-	FFS resolving of ambiguity of PRACH use for positioning or initial access

Proposal 3	Define a new usage of Rel-16 SRS for positioning purpose.  (Nokia)

Proposal 4	Support semi persistent configuration of SRS for positioning

Proposal 5	Support periodic configuration of SRS for positioning

Proposal 6	Support aperiodic configuration of SRS for positioning

Proposal 7	For UL positioning, the SRS may be configured with the following number of symbols

	Option 1: follow legacy

	Option 2: increase the  maximum number of symbols up to a maximum value

	Option 3: add a selected number of symbols

Proposal 8	Proposal: the SRS may be allocated in any of the UL SRS symbols in the positioning transmission slot.

Proposal 9	For positioning, support SRS comb size of

	Option 1: legacy (comb 2 and 4) with extension with comb-6

	Option 2: legacy (comb 2 and 4) with extension with comb-6 and comb-12

	Option 3: legacy (comb 2 and 4) with no extensions

Proposal 10	Staggered patterns are supported for UL SRS

	FFS details on how staggering is implemented

Proposal 11	The SRS TA for the purpose of positioning follows rel-15 legacy procedures, e.g using the latest configured TA value for the UL serving cell transmissions.

Proposal 12	Further Discuss Supporting additional TA procedures specifically for positioning to e.g. support the SRS reception in the neighbouring cells.

Proposal 13	For power control, UL SRS for positioning follows

	Option 1: legacy procedures from NR rel-15

	Option 2: a to-be define new procedure, aimed at improving hearability for neighbor cells

Proposal 14	For positioning The SRS bandwidth is configured as in Rel-15 NR

Proposal 15	For UL positioning SRS repetitions is supported with repetition factor N

	FFS value of N

Proposal 16	Discuss the connection between repetition and staggering patterns

References

- RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson, Shenzhen, China, March, 2019

- R1-1903901	Discussion on DL&UL PRS	ZTE Corporation,Sanechips

- R1-1904004	DL and UL Reference Signals for NR Positioning	Huawei, HiSilicon

- R1-1904107	Discussion on DL and UL RS for NR positioning	Vivo

- R1-1904158	DL and UL Reference Signals for NR Positioning	BUPT, ZTE, CAICT

- R1-1904200	Discussion on DL and UL Reference Signals for NR Positioning	 LG Electronics

- R1-1904258	Considerations on DL and UL reference signals for NR positioning	Sony

- R1-1904320	Downlink and uplink reference signal design for NR positioning	Intel Corporation

- R1-1904394	DL and UL Reference Signals Design for NR Positioning	Samsung

- R1-1904500	Reference signal design for NR positioning	Mediatek Inc.

- R1-1904600	Views on reference signal designs for NR positioning	Mitsubishi Electric Corp

- R1-1904740	Discussion on reference signal design for NR positioning	CMCC

- R1-1904790	Discussion on DL reference signals for NR positioning	Spreadtrum Communications

- R1-1905033	DL & UL Reference Signals for NR Positioning	Qualcomm Incorporated

- R1-1905262	Views on DL and UL reference signals for NR Positioning	Nokia, Nokia Shanghai Bell

- R1-1905346	DL and UL Reference Signals for NR Positioning	CATT

- R1-1905461	DL and UL Reference Signals for NR Positioning	Ericsson

- R1-1905484	SRS Enhancements for NR positioning	Fraunhofer IIS, Fraunhofer HHI