3GPP TSG RAN WG1 Meeting #96bis	R1-1905856

Xi’an, China,  8th April – 12th April, 2019

Title:	[Draft] Reply LS on collision of RRM measurement and UL transmission

Response to:	R4-1904682/R1-1905706

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Huawei, HiSilicon, [RAN1]

To:	RAN4

Contact Person:

Name:	Xi Zhang

E-mail Address:	panda.zhang@huawei.com

Attachments:	None

Introduction

RAN1 would like to thank RAN4 for the LS in R4-1904682/R1-1905706 on collision of RRM measurement and UL transmission.

RAN1 discussed the RAN4 conclusion on scheduling availability of UL transmission over RSSI/SSB symbols (plus one symbol before/after consecutive SSBs) and the RAN4 decisions on postponing related requirements design for half-duplex CA and CSI-RS based neighbor cell measurements to Rel-16.

RAN1 assumes the SSBs mentioned above include SSBs in both serving cell(s) and neighboring cell(s). RAN1 also assumes the intention of the RAN4 conclusion is to protect the RRM measurement performance.

Previously RAN1 assumed that the dynamically scheduled UL transmission in serving cell can be prioritized over RRM measurements on SSBs in neighboring cell(s), such that network is allowed to schedule UL transmission even within SMTC window (as indicated in the previous LS in R1-1810008/R1-1814412).

From RAN1 perspective, the scheduling restriction introduced by the conclusion in RAN4 may have some negative impacts on latency-critical UL traffic, as they cannot be scheduled over symbols carrying consecutive SSBs (plus one symbol before/after) within the SMTC window.

With all above said, RAN1 respects RAN4 conclusions. RAN1 does not expect any changes to Rel-15 RAN1 specification due to RAN4 conclusions stated in the LS.

Actions to RAN4

RAN1 respectfully requests RAN4 to take the above into account.

Date of next TSG-RAN WG1 meetings:

TSG-RAN WG1 Meeting#97,			13th May to 17th May 2019, 				Reno, Nevada, USA.

TSG-RAN WG1 Meeting#98,			26th August to 30th August 2019,			Prague, Czech Republic.