3GPP TSG RAN WG1 #96bis		R1-1905862
Xi’an, China, 8th – 12th April, 2019

Agenda item:	7.2.13.3

Source:	Apple Inc.

Title:	Summary of offline discussions on remaining issues on potential enhancements to single Tx switched uplink for EN-DC

Document for:	Decision

# Introduction

In this document, we summarize the open issues and topics discussed in related contributions [1]-[9] to RAN1 #96bis under agenda item 7.2.13.3.

# Some previous agreements

In RAN #81 [14], it was further agreed to include the following as one of the objectives for the R16 work item on “MR-DC and CA enhancements”[15]:

- Study and, if found beneficial over the existing single Tx switched uplink solution, specify enhancements to single Tx switched uplink solution for EN-DC, such as allowing all DL and UL subframes for data transmission for both NR and LTE. [RAN1]. Note: UE in single Tx switched uplink mode is not expected to be scheduled simultaneous transmission on MCG and SCG

Later, in RAN1 Ad-Hoc #1901[16], the following agreement was made:

Agreements: - It is concluded that it is beneficial to specify enhancements to single Tx switched uplink solution for EN-DC over the existing single Tx switched uplink solution, e.g. via introducing a new reference HARQ configuration using NR/LTE Rel-15 or LTE eIMTA solution, etc.

In RAN1 #96, a similar “SUO case 1” with DL-reference HARQ timing as in the EN-DC with LTE FDD PCell was agreed for EN-DC with LTE TDD PCell [11]:

Agreements: For single tx switched UL in EN-DC with TDD PCell, the LTE PCell can be configured with DL-reference UL/DL config - For DL HARQ timing, the DL-reference UL/DL configuration is applied - Only LTE TDD Pattern 2, 4, 5 can be used as DL-reference - For UL HARQ timing is the same as without reference configuration for PCell’s UL/DL configuration other than TDD pattern 0/6, - FFS: whether/how to support TDD pattern 0/6 for LTE PCell - UE is not expected to transmit on the MCG and SCG simultaneously - For type 2 UE (i.e., UE without dynamic power sharing capability): - UE is allowed to transmit LTE PUSCH only in the UL subframes designated as UL in the DL-reference configuration - For type 1 UE (i.e., UE with dynamic power sharing capability): - The UE should not assume that LTE PUSCH is only scheduled in the UL subframes associated configured by the DL-reference configuration - The UE should not assume that NR PUSCH is only scheduled in the remaining UL subframes other than those configured by the DL-reference configuration - If there is a collision, - In the UL subframes designated as UL in the DL-reference configuration, UE is expected to drop NR PUSCH - In other UL subframes, UE behaviour to be expected to specified with details FFS - E.g., drop LTE PUSCH, drop NR PUSCH, etc. - FFS for the case of NR SRS & NR PRACH - Note: the impact of switching time (if non-zero) will be further studied. - Note: the above does not assume any restriction between gNB and eNB (e.g., tight coordination) - FFS: whether/how to support HARQ-offset (similar as in SUO case 1 in EN-DC with LTE FDD PCell)

From the above agreement, there are at least the following open issues to be further concluded for SUO case 1 in EN-DC with LTE TDD PCell:

- Supportable LTE TDD patterns

- How to handle the case of “collision” between UL transmissions over MCG and SCG

- Impact of switching (if non-zero)

- Support of HARQ-offset

# Clarification on DL HARQ timing for “SUO case1” in EN-DC with TDD PCell

The main open question is:

- some clarification needed for “For DL HARQ timing, the DL-reference UL/DL configuration is applied” in the agreement.

- Do we differentiate LTE PCell vs LTE FDD SCell vs LTE TDD SCell?

According to the latest agreement from RAN1 #96, the LTE DL HARQ timing for SUO case 1 in EN-DC with LTE TDD PCell cases use the HARQ timing corresponding to the DL-reference configuration. However, there still seems to be some ambiguity as to the exact HARQ timing (i.e. PDSCH to UL ACK mapping) to be used, as discussed in [7], out of two possible options:

- Option 1: use the same UL ACK timeline as for the SUO case 1 in EN-DC with LTE FDD PCell, as agreed in R15. Specifically, this corresponds to the DL-reference HARQ timing for FDD-TDD CA with TDD primary carrier, as specified in Table 10.1.3A-1 in 3GPP TS 36.213.

- Option 2: use the UL ACK timeline for the eIMTA, as specified in Table 10.1.3.1-1 in 3GPP 36.213.

For the LTE TDD PCell, both option 1 and option 2 technically work, as they both specify a “superset” of ACK timing for the possible DL subframes. The benefits of using option 1 is that it becomes a unified design as single Tx case 1 for the FDD PCell case, although it specifies ACK timing for all 10 DL subframes within a radio frame, which is more than what is needed for the TDD LTE PCell in the EN-DC scenario.

However, for the LTE FDD SCell, or LTE TDD SCell, option 2 may not always work, since the DL/UL subframe may not always align with the TDD PCell if they are in different band.

Below is summary of companies’ positions/proposals from contributions:

| Company | proposal |
| --- | --- |
| Apple | Proposal 1: For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell, RAN1 to further clarify/down-select among: - Option 1: use the PDSCH ACK timeline for LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213) Option 2: use the PDSCH ACK timeline for eIMTA (i.e., Table 10.1.3.1-1 in 36.213) |

Potential agreement: For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell, the following clarifications are agreed:

- For the LTE TDD PCell: use the PDSCH ACK timeline as in the case of eIMTA with the same DL-reference UL/DL configuration (i.e., Table 10.1.3.1-1 in 36.213).

- For LTE DL CA, the SCell uses the same DL-reference UL/DL configuration as the PCell

  - For the LTE FDD SCell: use the PDSCH ACK timeline defined for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

  - For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

  - For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213)

# PUCCH formats

Main open issue: should we limit the supportable LTE PUCCH formats on the LTE TDD PCell? Is fallback allowed?

Background: an issue related to single Tx operation with case 1 DL HARQ timing is the potential PUCCH resource collision between legacy UE and single Tx UE, if fallback operation is triggered, i.e., DAI=1. When fallback to PUCCH format 1a/1b, the resource is implicitly mapped from CCE. As mapping for the legacy UE and single Tx UE follow different TDD configurations (i.e. one corresponds to “subframeAssignment” in SIB1, and the other corresponds to “tdm-PatternConfig-r15”), two PDSCH from different subframes may possibly map to the same PUCCH resource.

To avoid potential PUCCH resource collision and simplify implementation, we may consider to allow only PUCCH format 3/4/5 without supporting PUCCH fallback for UE configured with SUO case 1.

Below is summary of companies’ positions/proposals from contributions:

| Company | proposal |
| --- | --- |
| ZTE | Proposal 4: For Rel-16 SUO for EN-DC with LTE TDD PCell, only PUCCH format 3/4/5 is supported |
| Samsung | Proposal 4: Only PUCCH format 3/4/5 is supported for EN-DC between LTE TDD PCell and NR SCell |
| Nokia | Proposal 3: If DL reference HARQ timing is configured for TDD PCell EN-DC, only PUCCH format 3/4/5 can be configured for HARQ-ACK feedback, the PUCCH fallback operation is not supported |
| Apple | Proposal 5: For single UL operation in EN-DC with LTE TDD PCell, where UE operates on only one carrier at a time, and UE is configured with DL-reference UL/DL configuration, only support PUCCH format 3/4/5 PUCCH on LTE PCell - fallback operation is not supported |

Potential agreement: For single UL operation in EN-DC with LTE TDD PCell, and UE is configured with DL-reference UL/DL configuration,

- Support PUCCH formats 3/4/5, PUCCH fallback operation based on implicit resource indication is not supported.

  - FFS PUCCH format and resource determination when the UE only receives DAI=1

# Supportable LTE TDD Patterns with “SUO case1” in EN-DC with TDD PCell

Main open issue: whether to support LTE TDD Pattern 0 or 6 on LTE PCell together with SUO case1 in EN-DC with LTE TDD PCell?

According to the latest agreement from RAN1 #96, the LTE PCell’s UL HARQ timing (i.e. gap between PUSCH to PUSCH retransmission) is the same as without the reference configuration.

However, for LTE TDD pattern 0 and 6, the PUSCH HARQ interval varies (e.g. 10, 11, 13, 14 ms) depending on the subframe number, as shown by Table 8-2 and Table 8.3-1 in TS 36.213. This makes it difficult to reuse the original PUSCH HARQ timing once a subset of UL subframes is configured using the DL-reference config. Meanwhile, there seems to be no existing commercial deployment using these two configurations.

Below is summary of companies’ positions/proposals:

| Company | Proposal |
| --- | --- |
| Apple | Proposal 2: When UE is configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC with LTE TDD PCell, the LTE PCell’s TDD pattern (as configured in SIB1) should not include TDD pattern 0 or 6 |
| Nokia | Proposal 1: TDD UL/DL configuration 0/6 as LTE Pcell in EN-DC is not supported |
| Intel | Proposal 1: To considering asynchronous HARQ operation for TDD configuration 0/6 if the latency concern is justified |
| Samsung | Proposal 1: For UL HARQ timing when PCell UL/DL configuration is #0 or #6, UL-reference UL/DL configuration for the LTE PCell can be considered |
| LGE | Proposal #1: Apply UL HARQ RTT of 10 msec for Pcell UL/DL configuration 0/6 and determine UL HARQ timing based on the UL/DL configuration (e.g. 1) having 10 msec HARQ RTT |
| ZTE | TDD pattern 0 or 6 is not supported in this case. |

Potential agreement: When UE is configured with DL-reference config on the LTE PCell to support single UL in EN-DC with LTE TDD PCell, the LTE PCell’s TDD pattern (as configured in SIB1) should not include TDD pattern 0 or 6 for R16.

Note: this does not preclude the consideration of TDD pattern 0 or 6 if commercial need arises in future.

# UE handling of “collision” between UL transmissions over MCG and SCG

Main open issue: for type 1 UE, if there is a collision in UL subframes other than those designated as UL in the DL-reference configuration, what is the UE behavior? Should it be specified or up to UE implementation?

Background:

- It was stated in WID [14] that “UE in single Tx switched uplink mode is not expected to be scheduled simultaneous transmission on MCG and SCG”

- It was agreed in RAN1 #96 that “for type 1 UE, If there is a collision in the UL subframes designated as UL in the DL-reference configuration, UE is expected to drop NR PUSCH”

Below is summary of companies’ positions/proposals:

| Company | Proposal |
| --- | --- |
| Huawei | Proposal 3: If there is a collision between LTE PUSCH and NR PUCCH/PRACH/PUSCH/SRS in UL subframes other than the subframes designated as UL by DL-reference TDD configuration, UE drops LTE PUSCH |
| ZTE | Observation 1: For SUO with EN-DC, always dropping NR PUSCH in case of collision will cause undesirable performance loss in NR side. Proposal 1: For SUO with EN-DC in case of collision, if the NR PUSCH grant arrives later than the LTE PUSCH grant, UE is expected to drop NR PUSCH; otherwise, UE is expected to drop LTE PUSCH |
| Intel | Proposal 2: In the UL subframes designated as UL in the DL-reference configuration, UE is expected to drop NR PUCCH if collision occurs Proposal 3: In the remaining UL subframes other than those configured by the DL-reference configuration, LTE PUSCH transmission is dropped if it collides with NR UL transmissions |
| Samsung | Proposal 2: Based on LTE/NR transmission powers and maximum EN-DC transmission power, NR PUSCH power can be reduced or NR PUSCH can be dropped if there is a collision between LTE PUSCH and NR PUSCH |
| LGE | Proposal #2: Consider how to handle PUSCH retransmission purely depending on PHICH without detecting UL grant PDCCH in LTE side for handling of PUSCH collision between LTE and NR |
| Apple | Proposal 3: When UE is configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC, but scheduled with overlapping transmissions on MCG and SCG, the UE behavior is not specified at least for type 2 UE - For type 1 UE: on UL/SF SF not indicated as UL in the DL-ref config, either leave up to UE implementation or UE can drop NR transmission |
| Nokia | Proposal 2: If there is collision in the subframe which is not designated as UL in the DL-reference configuration, UE is expected to drop NR UL transmission |
| Ericsson | Proposal 2: For Case 1 HARQ timing via UE-specific reference TDD configuration for TDD Pcell, for subframes not indicated as ‘UL’ by the reference TDD configuration, the following default UE behavior is supported - if there are colliding LTE and NR UL transmissions in the same subframe, the UE is expected to transmit LTE and drop NR |
| Qualcomm | Proposal 1: For single tx switched UL in EN-DC with TDD Pcell, the type 1 UE should consider LTE and NR uplink collisions either as an error case or should drop the NR transmission |

Potential agreement: for type 1 UE configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC, if there is overlapping transmission in the subframe which is not designated as UL in the DL-reference configuration, UE is expected to drop NR UL transmission.

# Impact from UL switching time

Main open issue: does the spec need to put any explicit constraint on eNB/gNB UL scheduling, so as to minimize the impact from UL switching time? Or purely leave it for network implementation when such optimization is desired?

Background: The switching time was discussed in RAN4, and final agreement was captured in [12]. For intra-band EN-DC with single PA architecture, the maximal switching time can be up to 120us, which is close to 2 OFDM symbols in LTE. For certain TDD patterns with limited UL opportunities, such switching time may lead to non-negligible overhead.

Figure 3-1 shows an example with the most popular deployment of TD-LTE using pattern 2, which includes 2 full UL subframes and 2 special subframes each containing an UL portion at the end of the subframe. In case UE needs to switch between different RATs among the UL portion of the special subframe and the immediately following UL subframe, the 120us switching time might take a couple symbols away from the UL subframe, and require NR transmissions use partial slot. On the other hand, the existing gap between two “UL clusters” (i.e. from end of symbol 2 to UL portion in symbol 6) is large enough to accommodate any switching gap necessary if there is indeed a need to change to a different RAT. Therefore, it is highly desirable that, from a single UE perspective, the UL portion in the special subframe uses the same RAT as the adjacent UL subframe that immediately follows it. This way, the impact of the switching gap, if needed, is minimal.

Figure 3-1 Example of LTE TDD pattern 2

Below is summary of companies’ positions/proposals from contributions:

| Company | proposal |
| --- | --- |
| Huawei | Proposal 2: For type 1 UE incapable of dual PA, LTE PUSCH is only allowed in the UL subframes designated as UL in the DL-reference configuration |
| Apple | Proposal 4: For a UE operating with single UL in EN-DC with LTE TDD PCell, the UE is not expected to transmit in different RAT between an UL portion of a special subframe and the immediately following UL subframe |

Potential agreement: further discuss whether any explicit constraint needed in spec to minimize impact from switching time for UE’s operating in single UL in EN-DC, such as:

- For type 1 UE incapable of dual PA, LTE PUSCH is only allowed in UL subframes designated as UL in the DL-ref config

- UE is not expected to transmit in different RAT between an UL portion of a special subframe and the immediately following UL subframe.

# Support of HARQ Offset

Main open issue: whether a similar “HARQ-offset” as introduced in SUO case 1 for EN-DC with LTE FDD PCell should also be introduced for SUO case 1 for EN-DC with LTE TDD PCell?

Background: the following agreement was made in RAN1 #90bis for EN-DC with LTE FDD PCell:

Agreements: - In Case 1, LTE TDD UL HARQ timing is supported and the UE is allowed to transmit only in the subframes designated as UL in the reference TDD configuration. Additionally, a UE-specific HARQ subframe offset can be configured. - The offset (HARQ_offset) is in the range [0…9] - The offset doesn’t change any subframe or slot number - If, before applying the offset, the HARQ feedback for DL SF number m was mapped to UL SF number n, then after the offset, the HARQ feedback for DL SF number (m + HARQ_offset) is mapped to UL SF number (n + HARQ_offset) - If, before applying the offset, the PUSCH in SF number m was scheduled (by UL grant or PHICH) in SF number n, then after the offset, the PUSCH in SF number (m + HARQ_offset) is scheduled (by UL grant or PHICH) in SF number (n + HARQ_offset) - If, before applying the offset, the HARQ-ACK (if carried on PHICH) corresponding to PUSCH in SF number m was transmitted in SF number n, then after the offset, the HARQ-ACK (if carried on PHICH) corresponding to PUSCH in SF number (m + HARQ_offset) is transmitted in SF number (n + HARQ_offset) - SRS and PRACH transmission may only occur in subframes indicated as UL in the reference TDD configuration but shifted by the configured non-zero HARQ_offset

Below is summary of companies’ positions/proposals from contributions:

| Company | Proposals |
| --- | --- |
| Huawei | Proposal 6: Do not support HARQ-offset for type 1 UE configured with DL-reference TDD configuration. Proposal 7: If HARQ-offset is supported by type 2 UE configured with DL-reference TDD configuration, specification should guarantee to avoid possible error cases regarding configuration of HARQ-offset |
| ZTE | Proposal 5: Do not support HARQ-offset for SUO for EN-DC with LTE TDD PCell in Rel-16 |
| Intel | Proposal 4: Support HARQ-offset for EN-DC with TDD PCell |
| Samsung | Proposal 3: HARQ-offset indices corresponding to LTE UL subframes designated as UL in the DL-reference UL-DL configuration can be considered |
| Nokia | Observation 1: HARQ-offset is only applicable to limited scenarios for TDD PCell EN-DC |
| Apple | Can be considered for the case of LTE TDD PCell as well. However, unlike LTE FDD PCell, the offset can only be applied to limited cases (e.g. pattern 1 and 2) and the offset values can only take limited values due to TDD UL/DL pattern contraints. |
| Qualcomm | Proposal 3: The HARQ offset should be configured for the users in case of EN-DC with TDD Pcell |

Potential agreement: support HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell

# Configuration of RACH resource

Main open issue: should network assign RACH resource differently for dual Tx UE and UE operating in SUO case 1? Should type 1 and type 2 UE be treated differently?

Background: it was agreed in RAN1 #90bis that PRACH resource configuration and transmission should satisfy the following:

Agreement: - For Case 1, FDD timing configuration (i.e., periodicity and offset) is applied to LTE PRACH and SRS on LTE UL carriers. - The UE is not required to support transmission of LTE PRACH or SRS transmission which does not coincide with the configured HARQ-ACK transmission occasions

Below is summary of companies’ positions/proposals from contributions:

| Company | proposal |
| --- | --- |
| Huawei | Proposal 5: For single UL operation in EN-DC with TDD PCell and the PCell is configured with DL-reference TDD configuration, UE is allowed to transmit LTE PRACH in all uplink subframes configured by cell-specific TDD configuration |
| Ericsson | Proposal 1: Following enhancement for PRACH should be supported - If a UE is configured with Case1 HARQ timing, UE’s PRACH transmissions are not limited to the UL subframes given by subframe-Assignment-r15 (i.e., for FDD, UE can transmit PRACH in any UL subframe). - If the UE’s LTE PRACH transmission collides with an NR UL transmission, PRACH transmission is prioritized |
| ZTE | Proposal 3: For Rel-16 SUO for EN-DC with LTE TDD PCell, LTE PRACH and SRS transmission are only allowed to transmit in the subframes designated as UL in DL-reference configuration |

Potential agreement: If a UE is configured with Case1 HARQ timing in EN-DC, UE’s PRACH resource configurations are not limited to the UL subframes given by the DL-reference config.

- For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s LTE PRACH transmission collides with an NR UL transmission, LTE PRACH transmission is prioritized

- For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of LTE PRACH transmission which does not coincide with the configured HARQ-ACK transmission occasions

# Configuration of SRS resource

Main open issue: should network config SRS resource differently for dual Tx UE and UE operating in SUO case 1? Should type 1 and type 2 UE be treated differently?

Background: it was agreed in RAN1 #90bis that SRS resource configuration and transmission should satisfy the following:

Agreement: - For Case 1, FDD timing configuration (i.e., periodicity and offset) is applied to LTE PRACH and SRS on LTE UL carriers. - The UE is not required to support transmission of LTE PRACH or SRS transmission which does not coincide with the configured HARQ-ACK transmission occasions

Below is summary of companies’ positions/proposals from contributions:

| Company | proposal |
| --- | --- |
| Huawei | Proposal 1: For single UL operation in EN-DC with TDD PCell and the PCell is configured with DL-reference TDD configuration, UE is allowed to transmit LTE SRS in subframes that designated as UL and special by DL-reference TDD configuration Proposal 4: For type 1 UE that is capable of dual PA, it can transmit LTE SRS in UL or special subframes that designated as DL by DL-reference TDD configuration, and it can transmit NR SRS in subframes that designated as UL or special Proposal 8: Allow EN-DC UE to transmit NR SRS overlapping in time with the last symbol of a LTE uplink subframe designated by subframeAssignment-r15 - The last LTE symbol thereof is reserved and not transmitted |
| ZTE | Proposal 3: For Rel-16 SUO for EN-DC with LTE TDD PCell, LTE PRACH and SRS transmission are only allowed to transmit in the subframes designated as UL in DL-reference configuration |

Potential agreement: If a UE is configured with Case1 HARQ timing in EN-DC, UE’s LTE SRS resource configurations are not limited to the UL or special subframes designated as UL by the DL-reference config.

- For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s LTE SRS transmission collides with an NR UL transmission, LTE SRS transmission is prioritized

- For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of LTE SRS transmission which are designated as DL by DL-reference configuration.

Potential agreement: If a UE is configured with Case1 HARQ timing in EN-DC, UE’s NR SRS resource configurations are not limited to the UL or special subframes designated as DL the by DL-reference config.

- For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s NR SRS transmission collides with an LTE UL transmission, LTE transmission is prioritized

- For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of NR SRS transmission which are designated as UL or special subframe by DL-reference configuration.

- UE can be configured to transmit NR SRS overlapping in time with the last symbol of UL subframe designated UL by the DL-reference config.

# Application of DL-reference config to dual Tx scenario for mitigating interference from harmonics

Main open issue: should network assign RACH resource differently for dual Tx UE and UE operating in SUO case 1? Should type 1 and type 2 UE be treated differently?

Background: as discussed in [10], interference due to harmonics will impact LTE DL, which is observed in EN-DC deployment with certain band combinations. A potential solution is to TDM LTE UL and NR DL, and configure a DL-ref HARQ timing to the LTE PCell so that HARQ feedback for all the LTE DL subframes can still be transmitted over the subset of UL subframes designated by the DL-ref config. The mechanism is similar to the SUO case 1 for EN-DC with LTE FDD PCell, but now applied to Dual Transmission scenario.

It turns out that there was actually an agreement from RAN1 #90bis to address this harmonic issue, referenced below:

Agreements: - DL/UL TDM (to avoid self-interference due to harmonics) is a network decision on a per-UE basis - No RRC configuration signalling is provided to the UE for this purpose (except FFS for the LTE RRC signalling to configure Case 1 HARQ timing) - UEs can be scheduled for NR or LTE uplink transmission in an arbitrary slot - UEs can be scheduled for NR or LTE downlink transmission in an arbitrary slot

Below is a summary of companies’ positions:

| Company | proposal |
| --- | --- |
| Vodafone | Proposal 1: For dual connectivity or CA scenarios involving LTE carriers (e.g. EN-DC, NE-DC, LTE-DC, LTE-CA) where the LTE carrier causes harmonic interference to the other carrier, regarding LTE HARQ timing, the existing LTE TDD-FDD CA reference configuration HARQ timing should be reused for the dual UL case. Proposal 2: Send a LS to RAN WG2 to inform them about the RAN 1 decision and ask them to introduce corresponding signalling |

Potential agreement: DL-reference config can be applied to dual Tx capable UE in EN-DC with LTE FDD PCell to avoid self-interference due to harmonics

- RRC configuration will be decided by RAN2 (send LS to RAN2)

- UEs can be scheduled for NR or LTE uplink transmission in an arbitrary slot

- UEs can be scheduled for NR or LTE downlink transmission in an arbitrary slot

- RAN1 to further study and conclude on remaining designs, including at least

  - LTE PUSCH HARQ timing

# Handling of LTE UL CA in the MCG

Main open issue: should R16 support an enhancement to allow for UL CA in MCG  for an U in EN-DC, when the single UL is applied? If so, what will be the available UL subframes for the LTE SCells?

Background: it was agreed that LTE UL CA would not be supported for SUO in R15, as shown in the following agreements in RAN1 #92bis.

Agreement: In Rel-15, Case 1 HARQ timing is supported for the LTE serving cells only for the following cases - A single FDD cell - DL CA with FDD-FDD CA and FDD-TDD CA, where the LTE PCell is FDD and there is only one UL carrier - The UE is not configured for cross-carrier scheduling Agreement: For a UE operating in EN-DC when configured with Case 1 HARQ timing on an FDD PCell, the following is supported for the LTE serving cells - For an FDD SCell, the same DL HARQ timing as the PCell is applied for the FDD SCell, - For a TDD SCell, the same DL HARQ timing as the PCell is applied restricted to the set of DL/special subframes on the TDD SCell - The UL HARQ timing for the FDD PCell is the same for the CA and non-CA cases

For the cases where LTE UL CA is feasible or can be supported, there would be the following two scenarios:

- The LTE SCell’s UL is in the same band as the LTE PCell (i.e. intra-band LTE UL CA). In this case, the LTE SCell’s UL is also part of the same “difficult band combination”, and therefore it is desirable that the UL subframes are aligned between the LTE PCell and LTE SCell, and they also follow the same UL HARQ timing.

- The LTE SCell’s UL is in a different band as the LTE PCell (i.e. inter-band LTE UL CA)

  - If the LTE SCell is not in a difficult band combination with NR, then obviously all UL subframes in the LTE SCell can be used

  - If the LTE SCell is also in a difficult band combination with NR, then the handling could be more complicated and potentially require different handling depending on whether LTE SCell and NR PSCell are in TDD or FDD band, respectively. Therefore, it is desirable to avoid such band combination in real deployment, unless there is clear market need, where the handling should be evaluated case by case.

Below is summary of companies’ positions/proposals from contributions:

| Company | proposal |
| --- | --- |
| Nokia | Observation 2: For the case that a EN-DC UE is configured with FDD PCell in single uplink operation mode, DL CA was already supported. Proposal 4: If Case 1 HARQ timing is configured, the UL subframe usage in a UL CA carrier is determined by whether its CA band is problematic band in EN-DC band combination. - For intra-band UL CA and inter-band CA with problematic CA band, the uplink subframes in Scell are aligned with the uplink subframe allocation in PCell For inter-band CA with non-problematic CA band, all UL subframe in the carrier of that band can be scheduled |
| Apple | Proposal 6: In case LTE UL CA is supportable by the UE operating with single Tx switched UL in EN-DC, the following can be supported: - LTE SCell’s UL is in the same band as the LTE PCell: the UL subframes are aligned between the LTE PCell and LTE SCell, and they follow the same UL HARQ timing. - LTE SCell’s UL is in a different band as the LTE PCell, but not in a difficult band combination with NR where single Tx is allowed: all UL subframes in the LTE SCell can be used |

Potential agreement:

# Appendix: Summary of proposals

[1] R1-1903994, “Enhancements for single UL operation for EN-DC”, Huawei, HiSilicon

Proposal 1: For single UL operation in EN-DC with TDD PCell and the PCell is configured with DL-reference TDD configuration, UE is allowed to transmit LTE SRS in subframes that designated as UL and special by DL-reference TDD configuration.

Proposal 2: For type 1 UE incapable of dual PA, LTE PUSCH is only allowed in the UL subframes designated as UL in the DL-reference configuration.

Proposal 3: If there is a collision between LTE PUSCH and NR PUCCH/PRACH/PUSCH/SRS in UL subframes other than the subframes designated as UL by DL-reference TDD configuration, UE drops LTE PUSCH.

Proposal 4: For type 1 UE that is capable of dual PA, it can transmit LTE SRS in UL or special subframes that designated as DL by DL-reference TDD configuration, and it can transmit NR SRS in subframes that designated as UL or special.

Proposal 5: For single UL operation in EN-DC with TDD PCell and the PCell is configured with DL-reference TDD configuration, UE is allowed to transmit LTE PRACH in all uplink subframes configured by cell-specific TDD configuration.

Proposal 6: Do not support HARQ-offset for type 1 UE configured with DL-reference TDD configuration.

Proposal 7: If HARQ-offset is supported by type 2 UE configured with DL-reference TDD configuration, specification should guarantee to avoid possible error cases regarding configuration of HARQ-offset.

Proposal 8: Allow EN-DC UE to transmit NR SRS overlapping in time with the last symbol of a LTE uplink subframe designated by subframeAssignment-r15

- The last LTE symbol thereof is reserved and not transmitted

[2] R1-1904155 “Discussion on single Tx switched uplink solution for EN-DC”, ZTE

Observation 1: For SUO with EN-DC, always dropping NR PUSCH in case of collision will cause undesirable performance loss in NR side.

Proposal 1: For SUO with EN-DC in case of collision, if the NR PUSCH grant arrives later than the LTE PUSCH grant, UE is expected to drop NR PUSCH; otherwise, UE is expected to drop LTE PUSCH.

Proposal 2: A UE is expected to drop LTE PUSCH if this LTE PUSCH collides with NR PUCCH resources configured by RRC.

Proposal 3: For Rel-16 SUO for EN-DC with LTE TDD PCell, LTE PRACH and SRS transmission are only allowed to transmit in the subframes designated as UL in DL-reference configuration.

Proposal 4: For Rel-16 SUO for EN-DC with LTE TDD PCell, only PUCCH format 3/4/5 is supported.

Proposal 5: Do not support HARQ-offset for SUO for EN-DC with LTE TDD PCell in Rel-16.

[3] R1-1904327, “Remaining issues for single uplink operation enhancement for EN-DC”, Intel

Proposal 1:

- To considering asynchronous HARQ operation for TDD configuration 0/6 if the latency concern is justified.

Proposal 2:

- In the UL subframes designated as UL in the DL-reference configuration, UE is expected to drop NR PUCCH if collision occurs.

Proposal 3:

- In the remaining UL subframes other than those configured by the DL-reference configuration, LTE PUSCH transmission is dropped if it collides with NR UL transmissions.

Proposal 4:

Support HARQ-offset for EN-DC with TDD PCell

[4] R1-1904400, “Enhancements to single Tx switched UL for EN-DC”, Samsung

Proposal 1: For UL HARQ timing when PCell UL/DL configuration is #0 or #6, UL-reference UL/DL configuration for the LTE PCell can be considered.

Proposal 2: Based on LTE/NR transmission powers and maximum EN-DC transmission power, NR PUSCH power can be reduced or NR PUSCH can be dropped if there is a collision between LTE PUSCH and NR PUSCH.

Proposal 3: HARQ-offset indices corresponding to LTE UL subframes designated as UL in the DL-reference UL-DL configuration can be considered.

Proposal 4: Only PUCCH format 3/4/5 is supported for EN-DC between LTE TDD PCell and NR SCell.

[5] R1-1904638, “HARQ related issues on single Tx-switched UL operation for EN-DC”, LG Electronics

Proposal #1: Apply UL HARQ RTT of 10 msec for Pcell UL/DL configuration 0/6 and determine UL HARQ timing based on the UL/DL configuration (e.g. 1) having 10 msec HARQ RTT.

Proposal #2: Consider how to handle PUSCH retransmission purely depending on PHICH without detecting UL grant PDCCH in LTE side for handling of PUSCH collision between LTE and NR.

Proposal #3: Discuss on 1) which PUCCH format is used for the case of HARQ-ACK fallback, 2) how to determine PUCCH resource in case without receiving ARI from DL grant PDCCH

[6] R1-1904721, “Discussion on single uplink operation enhancement for EN-DC”, Nokia, Nokia Shanghai Bell

Proposal 1: TDD UL/DL configuration 0/6 as LTE Pcell in EN-DC is not supported.

Observation 1: HARQ-offset is only applicable to limited scenarios for TDD PCell EN-DC.

Proposal 2: If there is collision in the subframe which is not designated as UL in the DL-reference configuration, UE is expected to drop NR UL transmission.

Proposal 3: If DL reference HARQ timing is configured for TDD PCell EN-DC, only PUCCH format 3/4/5 can be configured for HARQ-ACK feedback, the PUCCH fallback operation is not supported.

Observation 2: For the case that a EN-DC UE is configured with FDD PCell in single uplink operation mode, DL CA was already supported.

Proposal 4: If Case 1 HARQ timing is configured, the UL subframe usage in a UL CA carrier is determined by whether its CA band is problematic band in EN-DC band combination.

- For intra-band UL CA and inter-band CA with problematic CA band, the uplink subframes in Scell are aligned with the uplink subframe allocation in PCell

- For inter-band CA with non-problematic CA band, all UL subframe in the carrier of that band can be scheduled

[7] R1-1904987, “Remaining issues for single Tx switched uplink for EN-DC with TDD PCell”, Apple Inc

Proposal 1: For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell, RAN1 to further clarify/down-select among:

- Option 1: use the PDSCH ACK timeline for LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

- Option 2: use the PDSCH ACK timeline for eIMTA (i.e., Table 10.1.3.1-1 in 36.213)

Proposal 2: When UE is configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC with LTE TDD PCell, the LTE PCell’s TDD pattern (as configured in SIB1) should not include TDD pattern 0 or 6.

Proposal 3: When UE is configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC, but scheduled with overlapping transmissions on MCG and SCG, the UE behavior is not specified at least for type 2 UE.

- RAN1 to further decide whether LTE should always be prioritized for type 1 UE in this case.

Proposal 4: For a UE operating with single UL in EN-DC with LTE TDD PCell, the UE is not expected to transmit in different RAT between an UL portion of a special subframe and the immediately following UL subframe.

Proposal 5: For single UL operation in EN-DC with LTE TDD PCell, where UE operates on only one carrier at a time, and UE is configured with DL-reference UL/DL configuration, PUCCH fallback operation is not supported

Proposal 6: In case LTE UL CA is supportable by the UE operating with single Tx switched UL in EN-DC, the following can be supported:

- LTE SCell’s UL is in the same band as the LTE PCell: the UL subframes are aligned between the LTE PCell and LTE SCell, and they follow the same UL HARQ timing.

- LTE SCell’s UL is in a different band as the LTE PCell, but not in a difficult band combination with NR where single Tx is allowed: all UL subframes in the LTE SCell can be used.

[8] R1-1905143, “Enhancements to single-tx operation for EN-DC”, Ericsson

Proposal 1

- Following enhancement for PRACH should be supported

  - If a UE is configured with Case1 HARQ timing, UE’s PRACH transmissions are not limited to the UL subframes given by subframe-Assignment-r15 (i.e., for FDD, UE can transmit PRACH in any UL subframe).

    - If the UE’s LTE PRACH transmission collides with an NR UL transmission, PRACH transmission is prioritized.

Proposal 2

- For Case 1 HARQ timing via UE-specific reference TDD configuration for TDD Pcell, for subframes not indicated as ‘UL’ by the reference TDD configuration, the following default UE behavior is supported

  - if there are colliding LTE and NR UL transmissions in the same subframe, the UE is expected to transmit LTE and drop NR

[9] R1-1905280, “Enhancements for EN-DC Single-Tx TDM Operation”, Qualcomm

Proposal 1: For single tx switched UL in EN-DC with TDD Pcell, the type 1 UE should consider LTE and NR uplink collisions either as an error case or should drop the NR transmission.

Proposal 2: For single tx switched UL in EN-DC with FDD Pcell, the type 1 UE should not assume that LTE PUSCH is only scheduled in the UL subframes configured by the downlink reference configuration.

Proposal 3: The HARQ offset should be configured for the users in case of EN-DC with TDD Pcell.

Proposal 4: For a UE configured with EN-DC and more than one DL LTE serving cell:

- The downlink HARQ timing reference configuration of the FDD secondary cell is the RRC DL/UL configuration of the Pcell.

- The downlink HARQ timing reference configuration of the TDD secondary cell is given by the pair of its own SIB1 DL/UL configuration and the RRC DL/UL configuration of the Pcell similar to the LTE TDD-TDD CA with different DL/UL configurations

# References

- R1-1903994, “Enhancements for single UL operation for EN-DC”, Huawei, HiSilicon, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1904155, “Discussion on single Tx switched uplink solution for EN-DC”, ZTE, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1904327, “Remaining issues for single uplink operation enhancement for EN-DC”, Intel, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1904400, “Enhancements to single Tx switched UL for EN-DC”, Samsung, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1904638, “HARQ related issues on single Tx-switched UL operation for EN-DC”, LG Electronics, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1904721, “Discussion on single uplink operation enhancement for EN-DC”, Nokia, Nokia Shanghai Bell, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1904987, “Remaining issues for single Tx switched uplink for EN-DC with TDD PCell”, Apple Inc., 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1905143, “Enhancements to single-tx operation for EN-DC”, Ericsson, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1905280, “Enhancements for EN-DC Single-Tx TDM Operation”, Qualcomm, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1905677, “TDD Timing on FDD Bands for Dual Uplink”, Vodafone, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- “Chairman Note, RAN4#86bis”, 3GPP RAN4 #86bis, Melbourne, Australia, USA 16 - 20 April, 2018.

- “Chairman Notes, RAN1 #96”, 3GPP RAN1 Ad-Hoc #96, Athens, Greece, February 25th – March 1st, 2019.

- R4-1902496, “Draft CR for 38.101-3: Switching time for intra-band EN-DC upon dual PA UE capability”, Huawei, HiSilicon, 3GPP RAN4#90, Athens, Greece, February 25th – March 1st, 2019.

- RP-180680, “Solutions for single Tx TDM”, RAN #80, La Jolla, CA, U.S.A., June 11st-14th, 2018.

- “Chairman Note, RAN#81”, 3GPP RAN #81, Gold Coast, Australia, 10th – 13th September, 2018.

- PR-182076, “WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh-Core)”, RAN #81, Gold Coast, Australia, 10th – 13th September, 2018.

- “Chairman Notes, RAN1 Ad-Hoc #1901”, 3GPP RAN1 Ad-Hoc #1901, Taipei, Taiwan, 21st – 25th January 2019.