3GPP TSG-RAN WG1 Meeting #96bis	R1-1905903

Xi'an, China, 8th – 12th April 2019

Title:	[DRAFT] LS response on UL sharing applicability in different scenarios

Response to:	R1-1904182/R2-1902822 LS on UL sharing applicability in different scenarios

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Nokia [RAN1]

To:	To be RAN2

Cc:	To be RAN4

Contact Person:

#### Name:	Karri Ranta-aho

E-mail Address:	karri.ranta-aho@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 respectfully asked RAN1 the following 4 questions, and draft answers are provided for each question:

1.	Is the shared E-UTRA-NR FDD carrier supported in RAN1, i.e. the case where both FDD DL carrier and FDD UL carrier are shared from UE perspective by E-UTRA and NR?

[Draft answer] RAN1 supports spectrum sharing between E-UTRA and NR downlink carriers. According to the RAN decisions (see Annex), RAN1 was not tasked to consider DL sharing from the UE perspective, and UL sharing from the UE perspective was transparent to the RAN1 specifications. Thus RAN1 did not discuss the required UE capabilities for DL sharing from the UE perspective during the Rel-15 work, and during RAN1#96bis discussions there was no consensus on whether or not the DL sharing design defined in RAN1 is transparent to whether or not the sharing is from the UE perspective.

2.	Does the UL sharing from UE perspective (as indicated by the UL sharing capability) need to be supported together with any DL sharing capabilities that are required the shared E-UTRA-NR FDD carrier case?

[Draft answer] This would depend on the desired network deployment and whether or not the DL sharing from the UE perspective was supported by the 3GPP specifications and capabilities. The following example deployment scenarios support UL sharing from the UE perspective, without DL sharing from the UE perspective. From the RAN1 perspective, both these scenarios are supported.

Correspondingly, if DL sharing from the UE perspective was supported, the UL sharing from the UE perspective would be needed if the UE was configured to have a corresponding UL carrier on both LTE and NR. If the UE was not configured with the corresponding UL carrier on at least one of the two RATs (DL SCell without corresponding UL) or flexible duplex separation was used, no UL sharing from the UE perspective would be needed.

4.	Which of the rate matching mechanisms defined in RAN1 specifications were designed to be used to accomplish the DL sharing between E-UTRA FDD and NR FDD carriers?

[Draft answer] The CRS rate matching (rateMatchingLTE-CRS) was created solely for the purpose of E-UTRA – NR DL sharing. Bitmap based rate matching (rateMatchingResrcSetDynamic and rateMatchingResrcSetSemi-Static) were also motivated by E-UTRA – NR DL sharing, but is also used for forward compatibility as well as NR CORESET rate matching purposes. In addition an optional capability (additionalDMRS-DL-Alt) was introduced for DL sharing purposes, this capability introduces an alternate position for the additional DL DMRS symbol in order to avoid collision with LTE CRS.

2. Actions:

To RAN2 group.

ACTION: 	RAN1 respectfully asks RAN2 to take the provided information into account in their further work

3. Date of Next TSG-RAN WG1 Meetings:

3GPP RAN1#97		13 - 17 May 2019		Reno, USA

3GPP RAN1#98		26 - 30 August 2019		Prague, Czech Republic

4. Annex – Earlier RAN agreements related to the scope of the work in RAN WGs:

Agreement: (RP-172108)

NR-LTE Coexistence

  - Aiming December 2017 completion:

    - DL NR-LTE coexistence (only from network perspective) based on 15kHz numerology for control/data

    - UL NR-LTE coexistence, i.e., UL sharing only from network perspective

    - Single UL Tx according to RAN1 decisions

    - Solution to harmonic issue

    - Supplementary UL

  - Strive for specifying support for LTE/NR UL sharing from UE perspective in Rel-15 by June 2018

    - There is no NR L1/L2 impact after Dec 2017

    - Completion of standalone NR has priority until June 2018

No additional work specific to UL sharing from UE perspective in WGs until Dec 2017

LTE-NR DC

  - Aiming December 2017 completion:

    - Focus on LTE-NR DC for 2UL without NR-LTE coxistence and supplementary UL

      - LTE - NR (sub6) DC

        - Note that analysis for 2UL can cover that for single UL Tx for RF part (FFS: RRM and demodulation parts)

      - LTE - NR (above 24) DC

  - Strive for specifying support for LTE/NR UL sharing from UE perspective in Rel-15 by June 2018

    - There is no NR L1/L2 impact after Dec 2017

    - Completion of standalone NR has priority until June 2018

    - No additional work specific to UL sharing from UE perspective in WGs until Dec 2017

Agreement: (RP-172104)

- UL NR-LTE coexistence with UL sharing only from network perspective

  - Some UEs transmit NR UL on an NR UL carrier overlapping with the bandwidth of a carrier on which other UEs transmit LTE UL

  - A UE is not configured with a NR UL carrier overlapping with the bandwidth of a LTE component carrier configured for the same UE for LTE operation

- UL NR-LTE coexistence with UL sharing from UE perspective

  - A UE is configured with a NR UL carrier overlapping with the bandwidth of a LTE component carrier configured for the same UE for LTE operation

    - E.g. a UE supports transmitting NR UL and LTE UL within the bandwidth of an LTE component carrier configured for the UE

- LTE/NR UL subcarrier alignment is to be specified by Dec 2017

  - On-going work in RAN4

- Specify support for SUL with and without LTE/NR UL sharing only from network perspective in Rel-15 by Dec 2017

  - As in current RAN1/RAN4 prioritization list

- Strive for specifying support for LTE/NR UL sharing from UE perspective in Rel-15 by June 2018

  - There is no NR L1/L2 impact after Dec 2017

  - Completion of standalone NR has priority until June 2018

  - No additional work specific to UL sharing from UE perspective in WGs until Dec 2017

Agreement: (RP-180560)

- From RAN perspective agreed definition from RP- 172104:

UL NR-LTE coexistence with UL sharing from UE perspective

  - A UE is configured with a NR UL carrier overlapping with the bandwidth of a LTE component carrier configured for the same UE for LTE operation

    - E.g. a UE supports transmitting NR UL and LTE UL within the bandwidth of an LTE component carrier configured for the UE

- From RAN4 requirements perspective

  - ULSUP is applicable only to LTE/NR DC combinations including SUL bands, (as in able 8b in R4-1802135)

  - Intra-band LTE/NR DC without SUL band is not a case of ULSUP in current RAN4 work

- ULSUP with SUL band is in scope of Rel-15 NR WI targeting June 2018 completion

  - NR Standalone and leftover from NR December drop have higher priority than ULSUP with SUL band

- In Q2 2018, RAN4 will setup the agenda item for ULSUP with SUL band with following sub agenda items.

  - TDM based ULSUP with SUL bands

  - FDM based ULSUP with SUL bands

- ULSUP with SUL band relative contributions will be only discussed under these above agenda items